3GPP TSG RAN WG1 #122									R1-2505930

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:	10.1.1.2

Source:	NEC

Title: 	Discussion on other aspects of CSI compression

Document for:	Discussion and Decision

- Introduction

After Rel-18 and Rel-19 studies, the following objectives are defined for CSI compression use case in Rel-20 [1]. In this contribution, we provide our views on ‘other aspects’ of CSI compression in 5GA. More specifically, in Section 2, the discussion is on data collection for training, and in Section 3, the discussion is on model pairing procedure and Section 4 discusses performance monitoring.

CSI feedback enhancement, encompassing (two-sided model) [RAN1, RAN2]: - CSI spatial/frequency compression without temporal aspects (“Case 0”), - Specify necessary signalling/mechanism(s) as per the identified potential specification impacts in FS_NR_AIML_Air , including: - Model pairing procedure including ID and applicability reporting - Inference aspects including target CSI type, measurement and report configuration, CQI RI determination, payload determination, quantization configuration codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports - NW and UE side data collection for training, - Target CSI format - Note The framework defined in BM and CSI prediction use cases could be reused - Specify performance monitoring

- NW and UE side data collection for training

Rel-19 study has the following conclusion for NW and UE side data collection for training of CSI compression.

RAN1#121 NW side data collection for training is studied including data format, configuration of rank / layer, number of subbands and mechanisms for ground-truth reporting, but not all aspects are concluded. UE side data collection for training is studied including NW configuration or UE request, configuration for temporal aspects, but not all aspects are concluded. Aspects that were not concluded can be discussed in the normative phase.

For Rel-19 AI/ML based beam and CSI prediction, NW side data collection is based on MDT and UE data logging while the discussion is within RAN2 and RAN1’s task is to define what to be included for training. For UE side data collection, CSI-ReportConfig can be used for configuring the resources for data collection purpose without CSI report.

While for CSI compression, for data collection purpose, similar methods can be adopted, e.g., via MDT and UE data logging for NW side data collection and via CSI-ReportConfig for configuring the resources for data collection purpose. To further add flexibility to acquire field data, L1 report can also be supported. To our understanding, the target CSI can be legacy CSI codebooks (e.g., codebook-based Rel-16 eType2 or Rel-18 eType2 for PMI prediction) and the legacy CSI framework, even report quantity, can be reused for reporting the target CSI.

For data collection purpose, support CSI-ReportConfig for configuring both the resources and L1 reporting.

For target CSI format, only support legacy CSI codebooks.

- Model pairing procedure

During Rel-19 study, there are at least 3 model identification options available and they are all feasible to pair the encoder and decoder: MI-Option 1 (Model identification with data collection related configuration(s) and/or indication(s)), MI-Option 2 (Model identification with dataset transfer) and MI-Option 3 (Model identification in model transfer from NW to UE). And they are also highly dependent on the methods used to address the inter-vendor training collaboration, for example, model pairing procedure can be completed via the successful completion of model parameter transfer or dataset transfer.

According to the WID [1], model pairing procedure includes ID and applicability reporting. The specification work needs a clear definition of the “ID”. Based on different model identification methods, the ID may be a configuration ID, a dataset ID, or simply a model ID, or even “ID-X”, but it should be consistent across different procedures. Rel-19 RAN1 agreements also suggest that the same ID shall be used for data collection, applicability inquiry and reporting, inference configuration, and may be also for monitoring.

Model pairing procedure can be different based on the methods used for inter-vendor collaboration. But the ID used for model paring can be the same for different procedures.

Rel-19 LCM framework is defined for UE sided model, as in the figure below. While for CSI compression, it requires some improvements to accommodate the model pairing procedure. In particular, the applicability reporting procedure can be used to pair UE CSI generation part and NW reconstruction part. For example, NW provide NW reconstruction part information in step 3 RRC reconfiguration, and UE replies in Step 5 about applicability of the model pairing information (e.g., UE CSI generation part information) in RRC reconfiguration complete message.

Figure 1 LCM for UE sided model, according to a running TS 38.300

However, based on different inter-vendor collaboration method, it is not possible to deliver very large payload for model parameters or datasets via RRC reconfiguration and RRC reconfiguration complete message. If model parameter transfer or datasets transfer is needed for model pairing, additional steps may be introduced in the LCM (e.g., after step 5 in the figure above) and the applicability reporting is used as a triggering message for model parameter/dataset transfer. Only after successful transfer of parameter/dataset transfer, the inference configuration in step 3 can be considered as effective.

Applicability reporting can be used for model pairing, including applicability/inapplicability reporting of inference configuration with model pairing information (“ID”) and triggering parameter/dataset transfer for model pairing if needed.

- Performance monitoring

Rel-19 study has the following conclusion for performance monitoring of CSI compression.

RAN1#121 For NW-side performance monitoring, RAN1 concludes that target CSI reporting via legacy codebooks can be used. Target CSI reporting with CSI codebook enhancement via higher-resolution parameter combination may be beneficial for improving NW-side performance monitoring with additional cost of complexity and overhead at UE side. For UE-side side monitoring, RAN1 concludes that it is feasible, but some companies think that, at least in some UE-side monitoring options, NW-side monitoring with target CSI reporting is needed to check the reliability of UE-side monitoring reports.

According to the study and the conclusion, NW side performance monitoring and UE side performance monitoring are both feasible, but with some concerns. For NW side performance monitoring, the issue is mainly complexity and overhead. For UE side performance monitoring, the concern is on the reliability. A combination of NW and UE side monitoring is needed to achieve an efficient and reliable mechanism.

Both NW and UE side performance monitoring are feasible and a combination of NW and UE side monitoring can be supported.

To avoid the overhead issue of NW-side monitoring, UE-side monitoring can be the baseline solution, and the performance metric can be calculated at UE side by using a proxy NW side decoder. The performance metric can be SGCS based on target CSI and reconstructed target CSI at UE. SGCS quantization can be the one defined for CSI prediction in Rel-19, i.e., 4 bits. The overall reporting overhead of UE side performance monitoring is acceptable especially considering only wideband, per layer SGCS.

Moreover, to further reduce the reporting overhead, event triggered performance monitoring report can be supported. The event can be defined based on the condition of SGCS being lower than a threshold value for several times or for a time duration.

For UE-side monitoring, support SGCS as performance metric, SGCS is calculated based on target CSI and reconstructed CSI.

For UE-side monitoring, support event triggered performance monitoring report. The event is defined based on the SGCS lower than a threshold value for a time duration.

To provide further reliability, NW-side monitoring can be used in addition to UE reported SGCS. For this monitoring type, UE needs to provide target CSI to NW via a CSI report. To our understanding, legacy codebooks are sufficient. Specifically, UE can report the compressed CSI and corresponding target CSI in the same report, or two separate reports with linkage. To avoid the possible ambiguity of the mapping relationship between compressed CSI and target CSI, a single report is preferred.

Furthermore, according to the conditions for performance monitoring for beam prediction and CSI prediction in Rel-19, the monitoring report and inference report must be sufficiently close in the time domain; otherwise, the comparison results would be meaningless. Correspondingly, in CSI compression, for NW-side monitoring, reporting compressed CSI and reporting target CSI also need to be sufficiently close in the time domain. This may raise an issue: if the two are confirmed to be sufficiently close, whether it is still necessary to report other information besides PMI (e.g., RI, CQI) for the latter report in the time domain needs further discussion.

For NW-side monitoring, support target CSI reporting via legacy codebooks, and support paired target CSI and compressed CSI in a single report.

For NW-side monitoring, for reporting compressed CSI and reporting (corresponding) target CSI, discuss whether it is still necessary to report other information besides PMI (e.g., RI, CQI) for the latter report in the time domain.

Unlike single-sided models, performance degradation in two-sided models can stem from various causes, such as poor encoder or decoder quality, issues with the proxy decoder at the UE side or proxy encoder at the NW side, or data drift in the input. Two-sided models introduce a collaborative structure between the UE and NW, which increases flexibility but also introduces more points of failure. Each of these causes can affect the model differently—for example, encoder issues may distort latent representations, while proxy decoder mismatches can lead to reconstruction errors. Since different causes may trigger different LCM actions and impact protocol design, it is essential to study how to accurately detect these root causes.

Study the mechanism and specification impact of identifying root causes for performance degradation, enabling accurate diagnosis across different root causes with corresponding impacts on LCM actions.

- Conclusion

In this contribution, we provided our views on‘other aspects’ of CSI compression in 5GA including data collection for training, model pairing procedure performance monitoring, and we have the following observations and proposals:

Observation 1:	Model pairing procedure can be different based on the methods used for inter-vendor collaboration. But the ID used for model paring can be the same for different procedures.

Observation 2:	Both NW and UE side performance monitoring are feasible and a combination of NW and UE side monitoring can be supported.

Proposal 1:	For data collection purpose, support CSI-ReportConfig for configuring both the resources and L1 reporting.

Proposal 2:	For target CSI format, only support legacy CSI codebooks.

Proposal 3:	Applicability reporting can be used for model pairing, including applicability/inapplicability reporting of inference configuration with model pairing information (“ID”) and triggering parameter/dataset transfer for model pairing if needed.

Proposal 4:	For UE-side monitoring, support SGCS as performance metric, SGCS is calculated based on target CSI and reconstructed CSI.

Proposal 5:	For UE-side monitoring, support event triggered performance monitoring report. The event is defined based on the SGCS lower than a threshold value for a time duration.

Proposal 6:	For NW-side monitoring, support target CSI reporting via legacy codebooks, and support paired target CSI and compressed CSI in a single report.

Proposal 7:	For NW-side monitoring, for reporting compressed CSI and reporting (corresponding) target CSI, discuss whether it is still necessary to report other information besides PMI (e.g., RI, CQI) for the latter report in the time domain.

Proposal 8:	Study the mechanism and specification impact of identifying root causes for performance degradation, enabling accurate diagnosis across different root causes with corresponding impacts on LCM actions.

- References

- RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025