3GPP TSG RAN WG1 #122	R1-2506022

Bengaluru, India, Aug 25th – 29th, 2025

Source:	MediaTek Inc.

Title: Channel coding for 6GR interface

Agenda item:		11.4.1

Document for:		Discussion and Decision

# Introduction

This contribution provides a review of 5G channel coding (LDPC and Polar) and an overview on the aspects to be improved and studied for 6G channel coding together with evaluation methodology.

# General view on 6G channel coding

To facilitate the specification for 6G, several new use cases and capabilities have been discussed and specified in IMT-2030 to reflect the vision of 6G communication. In addition to the newly introduced capability, enhanced capabilities from 5G are also captured as part of evolution to future communication system, e.g., peak data rate, latency, etc. Channel coding, as a crucial block in PHY layer, should be carefully designed to address the stringent challenges from new 6G requirements and improved capabilities from 5G. On the other hand, 5G specification deployment has been massively invested and commercialized by operators. Therefore, leveraging 5G infrastructure to maximize the reutilization of existing implementation can greatly accelerate the migration to 6G.

Meanwhile, study scope of 6G channel coding is captured in [1] where 5G channel coding schemes, LDPC and Polar codes, are considered as baseline with acceptable trade-off to satisfy 6G requirements. To proceed the study under the guideline of channel coding study scope, the investigation should revisit 5G channel coding designs and reexamining with 6G requirements. It is observed that 5G channel coding design provides pragmatic features for the use scenarios, e.g., rate-matching and HARQ for data channels, and it has been optimized in term of error correcting performance. On the other hand, 6G requirements covers various aspects and it might not be feasible for 6G channel coding design to accommodate all of them while achieving the same optimized error correcting capability of 5G design. Therefore, the design principle of 6G channel coding schemes should leverage the 5G design as much as possible while considering necessary modification and performance tradeoff to address 6G requirements. To facilitate the study, and methodology to reflect the 6G requirement from channel coding perspective should be discussed and prioritized in the very beginning stage. In addition to meeting 6G requirements, issues or limitation identified in 5G channel coding design should also be addressed in 6G design. For example, DCI payload size and the insufficient early termination rate in Polar code design.

Another aspect to investigate in 6G channel coding is the joint design across different PHY blocks. Channel coding technology has been quite mature and 5G LDPC and Polar codes error correction capability has been pushed to limit with the consideration of practical features, e.g., HARQ, rate-matching, etc. Further dB level coding gain from 5G design is extremely challenging without excessive implementation overhead. To achieve further performance enhancement and better trade-off with complexity, it is feasible to jointly design channel coding block with other PHY blocks, for example, modulation. It has been shown in several literatures the joint design of channel coding and modulation can grasp additional shaping gain toward the ultimate 1.53dB shaping gain [4].

In conclusion, 6G LDPC and Polar study should 1. investigate candidates which provide justified gain over 5G design in terms of 6G requirements with the consideration of necessary performance trade-off 2. validate issues identified in 5G design and provide solutions3. Investigate the feasibility of joint design with other PHY blocks with consideration of performance and implementation complexity trade-off.

# 6G LDPC

## 5G LDPC overview

LDPC codes are adopted as the primary channel coding scheme for the 5G physical layer data channels, as specified in [3]. In particular, QC-LDPC codes are selected for their excellent error correction capability and suitability for high-throughput, low-latency hardware implementations. These properties are essential for supporting the diverse requirements of 5G services, including enhanced Mobile Broadband (eMBB), Ultra-Reliable Low-Latency Communications (URLLC), and massive Machine-Type Communications (mMTC).

The QC-LDPC code structure in 5G NR is based on two base graphs, referred to as Base Graph 1 (BG1) and Base Graph 2 (BG2). BG1 is optimized for large code block sizes and higher code rates to enable large data payloads and high spectral efficiency. The full size of BG1 is 46×68 where 22 columns are associated with information bits. BG2, on the other hand, is designed for smaller code block sizes and lower code rates, addressing the scenarios where reliability and low latency are prioritized. The full size of BG2 is 42×52 where 10 columns are associated with information bits. The selection of the base graph is determined by the TB size and the target code rate, as defined in [3] This flexible approach allows 5G NR to efficiently support a wide range of data rates and service requirements.

The QC-LDPC parity-check matrix is constructed by expanding the selected base graph using a lifting size Z, which is chosen from a predefined set of values as listed in [3]. The quasi-cyclic (QC) structure of the LDPC matrix enables efficient encoding and decoding, as each non-zero entry in the base graph is mapped to a cyclically shifted identity matrix of size Z. This structure is highly advantageous for hardware implementations, as it allows for parallel processing and reduces the complexity of both encoding and decoding operations. For large transport block sizes, the input data is segmented into multiple code blocks, each encoded separately using LDPC codes.

The encoding process in 5G QC-LDPC leverages the QC structure for hardware efficiency. The parity-check matrix is divided into a systematic part, which is associated with the information bits, and a parity part, which is associated with the parity bits. The parity part features a dual diagonal structure, which facilitates low-latency encoding by enabling the use of simple shift registers and accumulators. For scenarios requiring very low code rates, a Raptor-like extension is applied to the base graph, further enhancing error correction performance and robustness against channel impairments. Decoding is typically performed using layered belief propagation or min-sum algorithms, which exploit the QC structure to enable parallel message updates and reduce decoding latency. These algorithms iteratively update the messages passed between variable nodes and check nodes in the Tanner graph representation of the LDPC code, converging on the most likely codeword.

## 6G LDPC enhancement

### Motivation

Peak data rate is one of the KPIs to enable 6G applications, e.g., AR/XR, immersive communication. In IMT-2030 and company contributions from RAN WG1#122, peak data rate in the range of 30-200 Gbps are being actively discussed. To achieve high peak data rate within the range, LDPC codes remain a strong candidate due to the efficient parallel decoder structure and error correction capability close to Shannon limit. In particular, 5G data channel employs a QC-LDPC structure with a sufficiently large lifting size (Z) and optimized base graphs (BGs) to deliver robust error correction and support peak data rates for eMBB use cases.

However, the design goal of 5G QC-LDPC design is approximately ~20Gbps, which implies at least >1.5X enhancement is expected in 6G. One straightforward approach is to scale the existing 5G LDPC design by deploying additional parallel decoders. Despite the achievability, such approach might not be the most efficient way in terms of area efficiency. Furthermore, implementing extra decoders solely to meet peak data rate requirements introduces inefficiencies, as these resources remain underutilized during lower-throughput operation.

Observation 1: 1.5x-10x peak data rate of the 5G QC-LDPC design is under discussion for 6G throughput requirements.

To address the 6G throughput challenge more efficiently, it is useful to analyze the key factors influencing QC-LDPC decoder throughput. The Tput of QC-LDPC decoder can be approximated as

Tput≈NI∙(Kb×Z)∙fnitr×Citr

,

where NI is the number of decoders, Kb is the number of information column in BG. It can be observed that there are several design factors of QC-LDPC which contribute to Tput, i.e., Kb, Z, nitr, Citr. At the first glance, larger Kb and/or Z can benefit the Tput outcome. However, larger Kb generally increases Citr accordingly and the associated Tput improvement might not be as expected. Another direction to improve Tput is to reduce nitr and/or Citr. The factor nitr is a random variable which depends on various factors such as channel realization, code rate, CB size, etc. Due to the randomness and the dependency across several factors, contribution of nitr to Tput might not be feasible to quantify and the impact is less predictable.

Based on the above analysis, increasing Z and/or reducing Citr from current 5G design can be promising Tput enhancement directions and further study on these two aspects for 6G QC-LDPC design should be pursued. Increasing Z, the lifting size, provides the feasibility of processing information exchange between more variable and check nodes simultaneously, which has direct impact on the peak data rate, i.e., double the max lifting size Zmax from 5G design can directly double the peak data rate, regardless of BG design in QC-LDPC structure. The side-effect of large Z is the implementation cost and how to achieve the trade-off of Tput enhancements from larger lifting size and implementation cost should be further studied. The factor Citr provides another dimension of Tput enhancement and it solely depends on BG design. Unlike the implementation cost from the Tput enhancement of large lifting size, reducing Citr generally provides implementation benefits given a certain type of decoder implementation. In addition to the peak Tput enhancements, reducing Citr also provides benefit to daily use scenario in terms of decoding latency.

Observation 2: Increasing maximum lifting size Z and/or reducing decoding cycles per decoding iteration Citr  from current 5G design provides potential Tput enhancements.

Observation 3: Reducing decoding cycles per decoding iteration Citr from current 5G design provides potential decoding latency benefit.

As for the encoding latency, 5G design already addresses this aspect by incorporating efficient structure, such as an accumulator (dual diagonal structure) in the high code part of parity check matrix and a Raptor code structure in the low code rate extension part of parity check matrix. Such encoding latency beneficial structure should be preserved in 6G design.

Observation 4: Dual diagonal structure and Raptor code structures introduced in 5G QC-LDPC code can be leveraged in 6G QC-LDPC code design to achieve encoding latency reduction.

### Evaluation methodology

To evaluate Tput, the approximation introduced in section 3.2.1 can serve as a starting point. In particular, the peak Tput of a single LDPC decoder given the maximum operating clock cycle rate fmax can be approximated as (Kb×Z)∙fmaxnitr,MCS=27×Citr,MCS=27. Note that such approximation is the maximum peak data rate one single LDPC decoder can achieve and how much the max peak data rate can be translated to actual peak data rate depending on the operating bandwidth BW. In particular, for a given bandwidth, the peak data rate is estimated as CBsize∙NCB,BW,MCS=27Ts, where CBsize is the CB size, NCB,BW,MCS=27 is the number of CB within a bandwidth BW and time duration Ts. A more comprehensive formula for Tput considering carrier aggregation is captured in TS 38.306. In another word, the actual peak data rate CBsize∙NCB,BW,MCS=27Ts  can be achieved by operating at the bandwidth BW provided that the LDPC decoder can sustain the required Tput, i.e., (Kb×Z)∙fmaxnitr,MCS=27×Citr,MCS=27≥CBsize∙NCB,BW,MCS=27Ts. However, to operate on such bandwidth, the same inequality should be satisfied for all MCSs. That is, (Kb×Z)∙fmaxnitr,MCS=27×Citr,MCS=27≥CBsize∙NCB,BW,MCS=27Ts  for all MCS, or 
fmax≥nitr,MCS∙Citr,MCS∙NCB,BW,MCSTs for all MCS.

From the BG1 design, it can be observed that the critical MCS to satisfy the inequality is MCS=20, where Citr,MCS∙NCB,MCS is the largest across all MCSs (assuming nitr,MCS is fixed), i.e., MCS=20 requires the most decoding cycles to finish decoding of all CBs at a given bandwidth compared with other MCSs. For example, for 100MHz, 4×4 MIMO, the Citr,MCS∙NCB,BW=100MHz,MCS information is provided in Table 1, where NCB,BW=100MHz,MCS is acquired based on current 5G LDPC CB segmentation mechanism considering REs within a subframe and Citr,MCS is estimated by the number of edges in the associated submatrix of BG1 at given MCS.

Table 1 BG1 decoding cycles within 100MHz bandwidth across MCSs

Observation 5: For BG1, MCS20 requires the most decoding cycles to decode all CBs within a given bandwidth

Based on previous example and the observation, it can be concluded that the maximum bandwidth an LDPC decoder can operate is determined by the MCS having the largest Citr,MCS∙NCB,MCS, the largest total decoding cycle per iteration within a bandwidth, which is MCS20 in BG1 design. In particular, the maximum bandwidth roughly can be proportional to the inverse of the decoding cycles required for the most demanding MCS, i.e., fmaxnitr,MCS=20∙Citr,MCS=20∙NCB,BW=100MHz,MCS=20 ×100MHz. In general, since fmax is fixed and considering the worst case for decoding iterations (the channel realization which requires the maximum decoding iteration), the maximum bandwidth is roughly proportional to 1Citr,MCS∙NCB,BW,MCS and the peak data rate can be approximated to minMCSαCitr,MCS∙NCB,BW,MCS given the linear relation stated in TS 38.306 between Tput and bandwidth.

To facilitate the Tput enhancement discussion based on the metric minMCS1Citr,MCS∙NCB,BW,MCS , assumption of each of the terms given a BG design is elaborated as follows. For Citr,MCS, it can be estimated by the number of edges in the associated submatrix of BG design at given MCS for simplicity or by the number of edges divided by the parallelism factor associated with the submatrix of BG design which has parallelism structure. NCB,BW,MCS can be obtained from 5G LDPC CB segmentation mechanism given the maximum TB size within the bandwidth. Due to the scalability of NCB,BW,MCS to bandwidth, we can consider following setup for calculating NCB,BW,MCS without loss of generality.

| BW | 100MHz |
| --- | --- |
| RB number | 273 |
| RE number per PRB within subframe | 156 |
| Subcarrier spacing | 30kHz |

Proposal: Consider the following metric to facilitate peak data rate evaluation

- minMCS1Citr,MCS×NCB,BW=100MHz,MCS

  - Citr,MCS: decoding cycle per iteration at given MCS

    - Citr,MCS can be approximated by e/M, where e is the number of edge and M is the parallelism factor exiting in the BG design, if any

  - NCB,BW=100MHz,MCS: number of CB at given bandwidth (100MHz) at given MCS with following setup

| BW | 100MHz |
| --- | --- |
| RB number | 273 |
| RE number per PRB within subframe | 156 |
| Subcarrier spacing | 30kHz |

### Enhancement directions

To further enhance Tput of 6G QC-LDPC, it is essential to investigate approaches to reduce Citr,MCS and NCB,BW,MCS compared to 5G QC-LDPC design, as highlighted by the proposed metric in the previous section.

Recall that Citr,MCS, the decoding cycle per LDPC decoding iteration, can be estimated by e/M, where e is the number of edges in the associated submatrix of BG at given MCS and M is the parallelism factor exiting in the BG design. Therefore, to reduce Citr,MCS, a new BG to address 6G Tput enhancement should aim to introduce less edges than 5G design. The feature of less edges within a BG not only provides the benefit of throughput, but the associated implementation complexity can also be reduced, which will be discussed with more details later in this section. The impact of reducing edges in a BG is the potential performance loss. Therefore, the trade-off between Tput, implementation advantage, and performance should be further studied.

Another direction to improve Citr,MCS can be introducing parallelism structure inside the BG such that the decoder can inherit the parallelism and boost the Tput. One possible parallelism structure to be considered can be improving the orthogonality between rows of a BG. One example is shown in Figure 1, where the 2i-th row and 2i+1-th row are orthogonal, i.e., no common “1”s between the two rows in any column. Such orthogonality provides the feasibility of enabling processing two CN nodes concurrently and accordingly the Citr,MCS is reduced by half (due to the parallelism factor M=2) comparing with legacy layer decoder processing CN nodes one by one sequentially. In addition to the decoding latency and Tput benefits, the encoder also can enjoy the orthogonality structure to reduce encoding latency by allowing parallel computation of parity check bits, further reducing encoding latency. The 5G BG1 design already includes the pair-wised orthogonality in the extension part of BG, i.e., the additional rows within the submatrix associated with lower code rate. However, the submatrix of BG1 associated with high code rate remain very dense and extending the pair-wised orthogonality to high code rate part in 6G QC-LDPC BG design should greatly improve the Tput and latency.

Figure  1 Example of pair-wise orthogonal structure inside a BG

To reduce NCB,BW,MCS, one straight forward way is to increase maximum CB size limit from 5G, which is 384×22=8448 information bits in 5G BG1. Since the maximum CB size can be represented by Kb×Zmax, increasing Kb or Zmax can achieve the similar impact on improving maximum CB size. However, increasing Kb typically introduces more edges into the BG, which can negatively impact the Citr,MCS, and potentially reduce the anticipated Tput gain. On the other hand, enlarging Zmax improves the decoder implementation parallelism and Tput accordingly. The trade-off from larger lifting size is the increased implementation complexity, which is discussed in the following using simplified metrics.

For a commonly used block-parallel min-sum decoder for QC-LDPC, two types of implementation complexities are considered, memory area and logic area. The memory area can be roughly categorized into 4 blocks: LLR memory, R memory, Q memory, and sign memory. The LLR memory is used to store the input LLRs and the intermediate LLRs during the decoding and it can be estimated by c×(8+8)×Zmax, where c is the number of columns (VNs) in the BG and 8 bits are assumed to represent each of LLRs. R memory is used to store the minimum and second minimum of the absolute value of input LLR of a check node and the corresponding indices of minimal LLRs. The estimated R memory is r ×(8+8+ceil(log2(c))) ×Zmax, where r is the number of rows in the BG. The Q memory is used to store the extrinsic LLR from a VN to a CN and it can be estimated by w×(8)×Zmax, where w is the maximum row weight in the BG. The sign memory is used to store the sign of extrinsic LLR of each edge from a CN to a VN and it can be estimated by e×(1)×Zmax, where e is the number of edges(1s) in the BG. For logic area, the estimation is more complicated, but it can be generally assumed to be proportional to Zmax. From the above area estimation, it can be concluded that although increasing maximum lifting size Zmax directly improves the Tput, both memory area and logic area of a decoder also increase proportionally. Therefore, careful study on the trade-off between Tput and memory area cost is necessary while considering increasing maximum lifting size.

Proposal: Study following design aspects of QC-LDPC from the perspectives of Tput enhancement, error correction performance, and implementation complexity

- Reduced BG edges

- Parallelism structure

- Maximum lifting size>384

# 6G Polar

## 5G polar overview

Polar codes are adopted in 5G NR as the channel coding scheme for physical layer control channels, including Downlink Control Information (DCI), Uplink Control Information (UCI), and the Physical Broadcast Channel (PBCH. Their selection is based on their capacity-achieving properties, efficient implementation for short block lengths, and suitability for the stringent reliability and latency requirements of control signaling.

The construction of Polar codes in 5G NR is based on the principle of channel polarization. The code length N, ranging from 32 to 1024, is selected based on payload size and code rate, as defined in [3]. For a given payload size, the number of information bits K and the number of CRC bits are determined according to the control channel type and the required error correction capability. The most reliable bit positions, identified through reliability sequences standardized in [3], are assigned to information and CRC bits, while the remaining positions are filled with frozen bits with zero values.

To further enhance the error correction performance, 5G NR employs CRC-aided Polar coding. For DCI, a distributed CRC design is used, where the CRC bits are not simply appended at the end of the information bits but are interleaved or distributed among the information bits before encoding. This approach improves the effectiveness of CRC-aided successive cancellation list (SCL) decoding by increasing the likelihood that erroneous paths are detected and discarded early in the decoding process. The distributed CRC design is specified in [3,Section 5.3.3.1], and the CRC polynomial and length are chosen according to the control channel type and payload size.

For DCI transmission, an additional layer of security and user-specific identification is provided by RNTI (Radio Network Temporary Identifier) scrambling. After CRC generation, 16 RNTI bits are scrambled with the last 16 CRC bits for DCI encoding followed by polar encoding. Another scrambling process is described in [3,Section 7.3.2], and it is applied to the encoded output prior to rate matching and modulation. After Polar encoding and before modulation, the encoded bits are scrambled using a sequence derived from cell IDs and RNTI value assigned to the user for USS and from cell IDs for CSS. The scrambling ensures that only the intended receiver, configured with the correct RNTI, can successfully decode the control information.

Rate matching is a critical step in adapting the encoded Polar codeword to the required transmission length, which may differ from the code length N. 5G NR supports three rate matching techniques: puncturing, shortening, and repetition.

Puncturing: Selected bits from the encoded output are omitted, reducing the transmitted codeword length.

Shortening: Certain input bits are set to zero and their corresponding output bits are not transmitted, effectively reducing the code rate.

Repetition: When the required transmission length exceeds the code length, some bits are repeated to fill the available resources.

The choice of rate matching technique depends on the control channel type, payload size, and available resources.

## 6G Polar enhancement

### Motivation

While Polar codes have been adopted in 5G NR for Downlink Control Information (DCI) transmission due to their strong error correction performance at short block lengths, several limitations and issues have been identified in practical deployments, both from the base station and UE perspectives.

From the base station perspective, the 5G Polar code design for DCI exhibits significant limitations in terms of resource efficiency. A primary drawback is the restricted payload size, as the code construction supports only a narrow range of payload lengths up to 140 bits. This constraint results in suboptimal utilization of control channel resources and limits scheduling flexibility, making it challenging to efficiently accommodate diverse DCI payload requirements in potential 6G application, e.g., multi-carrier scheduling for larger bandwidth. Furthermore, due to the false alarm requirements, each DCI is appended with 24 bits CRC regardless of the payload size, which introduce another layer of resource inefficiency when considering implementing multi-carrier scheduling through several DCIs. In addition, the current 5G Polar code implementation for DCI is confined to QPSK modulation. This limitation further hinders resource efficiency, especially in high-SINR scenarios where higher-order modulation could significantly enhance control channel spectral utilization. Such drawbacks in the 5G Polar code design are fundamentally misaligned with the anticipated demands of 6G applications, which will require greater flexibility, scalability, and efficiency to support a broader range of use cases. Addressing these issues is therefore essential for the evolution of Polar code design in 6G.

Observation 6: Due to the limitation of maximum 140bit payload sizes and the support of only QPSK QAM, the 5G Polar code design for DCI is limited on scheduling flexibility and resource efficiency to support 6G applications.

From the UE perspective, the 5G Polar code for DCI is not optimized with respect to decoder complexity and early termination. The decoding process, especially for blind decoding of DCI, can be computationally intensive, leading to increased power consumption and latency at the UE. To address the blind decoding complexity issue, the 5G polar code introduces distributed CRC bits to enable early termination functionality when all the candidates in the decoding list failed to comply with the distributed CRC bits. Due to the encoding complexity concern, a single interleaver is employed to distribute CRC bits across payload regardless of the payload size. This design choice often results in an insufficient number of distributed CRC bits, which significantly diminishes the effectiveness of early termination. In particular, as shown in the interleaving table [3], only the first 3 CRC bits (the 141st, 142nd, and 143rd bits from the input of interleaver) are distributed to achieve limited amount of early termination when payload size is large. For a typical SCL Polar decoder with list size of 8, the probability that at least one of the eight candidates passes the check for these three CRC bits is relatively high. As a result, UEs frequently process the entire codeword, even when a decoding failure is inevitable, which fail the intended benefits of early termination and further impact UE decoding efficiency. Moreover, the distributed CRC interleaver is subject to maximum payload size of 140bits, which further limit the applicability in 6G application. Addressing the blind decoding effectiveness will be essential for 6G, where UEs are expected to support even more demanding use cases with tighter power and latency constraints.

Observation 7: The 5G distributed CRC design for DCI delivers limited early termination benefit at the UE, resulting in increased power consumption and latency.

Additionally, the 5G Polar code design for DCI is susceptible to RNTI (Radio Network Temporary Identifier) false alarm issues. During blind decoding, the UE must examine multiple candidate codewords, each subjected to RNTI descrambling. In the current 5G design, RNTI bits are directly scrambled with the CRC bits at the end of the codeword, which increases the likelihood of false alarms—where a UE erroneously detects a DCI intended for another UE. In a typical SCL decoder, a list of candidates is checked against CRC bits after RNTI descrambling. Under high SNR conditions, the candidate codewords in the list often differ by only a few bits, with errors typically accumulate at the end of the payload. As a result, when neighboring UEs have similar RNTIs, differing by only a few bits, there is a significant probability that a neighboring UE may incorrectly detect and act upon an unintended DCI after RNTI descrambling. Such false alarms can lead to unnecessary processing and increased power consumption at the UE, and also raise concerns regarding the reliability of control signaling. For example, neighbor UE might mistakenly decode an uplink grant intended for the target UE and cause resource collisions on the uplink, which degrade system performance and potential service disruptions. As 6G networks are expected to support even denser deployments and more stringent reliability requirements, addressing RNTI false alarm issues will be essential to ensure robust and efficient control channel operation.

Observation 8: The 5G RNTI scrambling mechanism for DCI is not optimized for RNTI-based false alarm rate among UEs with different RNTIs and endanger DCI reliability especially in denser and more demanding 6G scenarios.

In summary, while 5G Polar codes provide robust error correction for DCI, their current design presents notable challenges in terms of resource efficiency, decoder complexity, and reliability. Addressing these issues is essential for the evolution of Polar code design in 6G, where greater flexibility, efficiency, and robustness will be required to meet more demanding control channel requirements.

### Outer code design enhancements

#### Scalable data integrity check design with early termination feature

In 5G polar code design for DCI, the outer code consists of 24 bits CRC encoding followed by a fixed size interleaver to realize distributed CRC feature for a maximum payload size of 140bits. Such payload size restriction from the distributed CRC design limits the flexibility of multi-carrier scheduling where potential more carriers are considered in 6G. Moreover, each DCI is attached with 24 bits CRC bits and segmentation of payloads for multi-carrier scheduling introduce additional CRC overhead. Therefore, a revisit on the DCI payload size limit should be further discussed in 6G.

Proposal: Study the feasibility of increasing DCI payload size limit to >140bits

To accommodate a potentially larger payload size in 6G, current 5G distributed CRC should be redesigned since the current design has no scalability to larger payload size. The intention of the 5G distributed CRC design is to provide the early termination feature in polar successive cancellation list decoding, where payload bits are decoded in a sequential manner. Particularly, the distributed CRC bits can be used to perform early check on the list of decoding candidates and execute the decoding termination when all the candidates fail to comply with the CRC checks. Such feature is crucial when considering the computationally intensive DCI blind detection procedure. However, the distributed CRC design in 5G only distributed limited CRC bits and the distributed locations are not at the beginning part of payload. Consequently, the early termination benefit is very limited especially when the list size is large. In particular, the probability that a random candidate can pass an m CRC bits check is around 2-m and the probability that at least one of L random candidates pass an m CRC bits check is around L2-m. Therefore, there is a very low possibility of early termination when considering typical polar decoder of list size 8 under current distributed CRC design where less than 4 CRC bits are distributed in most of cases. In Figure 2, we provide the evaluation result of early termination rate for current 5G design. The early termination rate is defined as 
RET=Ekterminatek+m

where k is the number of total information bits excluding CRC bits, m is the number of total CRC bits, and kterminate the smallest number of decoded information bits (including CRC bits) when none of the candidates in the list can pass the current CRC check after decoding kterminate informations bits. As shown in Figure 2, >50% information bits are processed before decoding termination, which should be further enhanced in 6G.

Figure 2 RET  performance of 5G DCI

In addition to the inferior early termination rate, the implementation of distributed CRC check at decoder requires extra efforts in addition to legacy CRC check due to the irregular distributed CRC bit locations according to different payload size and ALs.

To address the early termination rate and scalability in 6G design, a new data integrity check mechanism which accommodates larger payload size and allows the data integrity check to be performed at earlier decoding stage to provide reasonable early termination benefit should be further studied.

Proposal: Study a new data integrity check mechanism with the aim to improve payload size scalability and early termination rate over 5G distributed CRC design

Proposal: Consider following metric and methodology to facilitate early termination rate evaluation

- Early termination rate: RET=Ekterminatek+m

  - k : payload size

  - m : number of total data integrity check bits

  - kterminate: the smallest number of decoded information bits (including data intefrity check bits) when none of the candidates in the list can pass the current CRC check(s) after decoding kterminate informations bits

  - E[kterminate] is estimated under SCL with the assumption of list size=8 and pure AWGN noise is transmitted.

#### RNTI-FA reduction design

In current 5G outer code encoding for DCI, 24 CRC bits are first generated followed by scrambling the last 16CRC bits with 16 RNTI bits. During typical polar list decoder, a list of candidate codewords is generated and each candidate is examined by descrambling the last 16 CRC bits using the RNTI. The candidate with the highest likelihood that passes the CRC check after RNTI descrambling is selected as the decoder output. The primary objective of RNTI scrambling is to ensure that only the intended UE, possessing the correct RNTI, can successfully detect and decode the transmitted DCI.

However, due to the nature of list decoding, both the target UE and neighboring UEs process the same multiple candidates in the list, including the transmitted DCI and other incorrect DCIs. While neighboring UEs typically fail the CRC check for the transmitted DCI due to incorrect RNTI descrambling, there remains a non-negligible probability that a neighbor UE may accidentally pass the CRC check for an incorrect DCI after applying its own RNTI, resulting in an RNTI false alarm. This issue is particularly observed during blind decoding, where the UE must evaluate multiple candidate codewords without prior knowledge whether there is an intended DCI or not. For example, consider two UEs, UE-A and UE-B, assigned RNTIs of 0000000000000000 and 0000000000000001, respectively, which differ by only a single bit. In high SNR conditions, the candidates produced by polar list decoding may differ by only a few bits, and the similarity in RNTI patterns increases the likelihood that UE-B may incorrectly pass the CRC check for a DCI intended for UE-A, leading to an RNTI false alarm. In this example, if any of the incorrect candidates in the list and the intended DCI is differed by the last bit, then UE-B can “correctly” decode the incorrect candidate after incorrect RNTI descrambling. Unlike other types of false alarms, the RNTI false alarm rate tends to worsen at high SNR, as the “noise” is introduced by the difference among list of candidates rather than by channel. Figure 3 shows the RNTI FA rate of the worst RNTI patterns across several payload sizes for AL=1. The worst RNTI pattern FA is defined as follows.

PRNTIWorst=maxRNTI=0216-1P∃u∈L s.t  ϕu,RNTI passes CRC checks,

where ϕu,RNTI is the function that recovers the original unscrambled information sequence from a candidate u in the list and RNTI, L is the list of candidates from SCL decoding. It can be observed that 5G DCI design encounter >10-2 FA rate for the payload size 40-60 bits, which significantly impacts the system reliability for typical DCI configuration. This phenomenon has also been observed in practical studies, such as [5], where the RNTI false alarm rate of the current 5G RNTI scrambling design can remain significantly high, e.g., 10-1 at high SNR region.

The implications of RNTI false alarms are substantial, as they can lead to unintended resource allocation, increased power consumption, and potential uplink collisions, thereby impacting overall system reliability and efficiency. As 6G networks are expected to support even higher densities and more stringent reliability requirements, it is crucial to address the reliability issue of the current RNTI scrambling mechanism and explore enhanced designs to mitigate false alarm risks in future systems.

Figure 3 Worst case RNTI FAR from 5G DCI Polar design

Proposal: Study new mechanism of RNTI scrambling to reduce RNTI-FA rate

Proposal: Consider following methodology to facilitate DCI RNTI false alarm evaluation.

- Given a target UE RNTItarget, random payload with size K, aggregation level AL, and AWGN channel

- Evaluate PRNTIWorst=maxRNTI not equal RNTItarget Pr∃u∈L s.t  ϕu,RNTI passes CRC checks

  - L is the list of 8 candidates from SCL decoding

  - ϕu,RNTI is the function that recovers the original unscrambled information sequence from a candidate u in the list and RNTI

  - Opration @SNR with BLER=1%

### Polar+high order QAM for DL

It is well established that constellation shaping can increase the achievable capacity in AWGN channels, particularly when the modulation order exceeds two. To realize the shaping gain, probabilistic constellation shaping schemes, such as Constant Composition Distribution Matching (CCDM), trellis-coded shaping, and shell mapping, have been investigated in literatures. However, these schemes require extra building blocks or hardware to facilitate the shaping functionality. Alternatively, it has been shown in [4] that polar code can be naturally integrated with shaping through the “decode-then-encode” procedure without extra building blocks. The overall procedures of the polar-coded modulation with shaping are shown in Figure 4, where Bj is the shaped polar codeword of the jth layer.

Figure 4 Multi-level coding polar code structure with shaping [4]

The main concept of doing shaping with polar code is based on the idea of reversed source polarization. In the problem of source polarization, n binary i.i.d random variables Xi∼PX ∀i∈1;n are polarized into variables with entropy being either 0 or 1, where the fraction of polarized variables with 1 entropy is HPX. Specifically, let G denote the generator matrix of a length n polar code and U1n=X1nG be the polarized vector, then for any δ>0,

limn→∞{j:1-HUj≤δ}n=HPX.

Since G-1=G, the result from polar source polarization implies that it is possible to put information bits U1k (with entropy 1) on the bit locations I={j∈1;n:1-HUj≤δ} and shaping bits S1n-k on locations J=1;n∖I forming U1n such that X1n=U1nG are almost i.i.d distributed with probability PX, where PX is the Bernoulli distributed with entropy related to the number of shaping bits, i.e., HPX≈1-Jn. Based on such observation, [4] combines shaping and polar coding into a single structure by partitioning the polar bit indices at the input of polar encoder into the frozen set F, the information set I, and the shaping set J. With the additional ns=Rs×n shaping bits, where Rs=1-HPX, the resulting polar codeword is shown to achieve a desired empirical distribution PX with HPX≈1-Jn. It is shown in [4] that such joint polar coding and shaping scheme can be decoded reliably with the code rate R=In matching the underlying capacity IPX;WY|X.

A key advantage of this approach is that constellation shaping is seamlessly integrated into the existing polar code structure, with shaping performed at the transmitter side by executing a Successive Cancellation List (SCL) or Successive Cancellation (SC) decoder. At receiver side, conventional SCL decoder is applied to decode both information and shaping bits, which provides relatively low additional complexity at the receiver compared to other probabilistic shaping schemes, such as CCDM, which requires arithmetic decoding at the receiver side. In Figure 5, scheme proposed in [4] is compared with 5G polar code+ BICM interleaving under 16QAM. It can be observed that a 0.5dB gain from shaping and multi-level coding can be achieved by the structure proposed by [4] at a cost of extra decoding step at transmitter side. Another benefit of considering the multi-level coding structure is the feasibility of maintaining compact size of mother polar code. For instance, generating more than 2,000 coded bits for 500 16QAM symbols with BICM would require a mother polar code of length greater than 2,048. In contrast, multi-level coding allows the same coded bit length to be achieved by encoding each layer with a smaller mother polar code, such as a length-512 polar code. A brief comparison of 5G BICM and [4] are summarized as follows.

|  | Pros | Cons |
| --- | --- | --- |
| 5G-BICM | Low complexity at the transmitter side | BLER performance; Need larger polar sequence for larger payload size |
| [4] | Better Performance; Can Accommodate more payload with current polar sequence | Decoding latency |

Figure 5 Error correction performance comparison between 5G BICM and multi-level coding+shaping[4]

Proposal: Study high order QAM polar code for DCI spectral efficiency enhancement

# Conclusion

In summary, followings are the observations and proposals.

Observation 1: 1.5x-10x peak data rate of the 5G QC-LDPC design is under discussion for 6G Tput requirements

Observation 2: Increasing maximum lifting size Z and/or reducing decoding cycles per decoding iteration Citr  from current 5G design provides potential Tput enhancements

Observation 3: Reducing decoding cycles per decoding iteration Citr from current 5G design provides potential decoding latency benefit

Observation 4: Dual diagonal structure and Raptor code structures introduced in 5G QC-LDPC code can be leveraged in 6G QC-LDPC code design to achieve encoding latency reduction

Observation 5: For BG1, MCS20 requires the most decoding cycles to decode all CBs within a given bandwidth

Observation 6: Due to the limitation of maximum 140bit payload sizes and the support of only QPSK QAM, the 5G Polar code design for DCI is limited on scheduling flexibility and resource efficiency to support 6G applications.

Observation 7: The 5G distributed CRC design for DCI delivers limited early termination benefit at the UE, resulting in increased power consumption and latency

Observation 8: The 5G RNTI scrambling mechanism for DCI is not optimized for RNTI-based false alarm rate among UEs with different RNTIs and endanger DCI reliability especially in denser and more demanding 6G scenarios.

Proposal: Consider the following metric to facilitate peak data rate evaluation

- minMCS1Citr,MCS×NCB,BW=100MHz,MCS

  - Citr,MCS: decoding cycle per iteration at given MCS

    - Citr,MCS can be approximated by e/M, where e is the number of edge and M is the parallelism factor exiting in the BG design, if any

  - NCB,BW=100MHz,MCS: number of CB at given bandwidth (100MHz) at given MCS with following setup

| BW | 100MHz |
| --- | --- |
| RB number | 273 |
| RE number per PRB within subframe | 156 |
| Subcarrier spacing | 30kHz |

Proposal: Study following design aspects of QC-LDPC from the perspectives of Tput enhancement, error correction performance, and implementation complexity

- Reduced BG edges

- Parallelism structure

- Maximum lifting size>384

Proposal: Study the feasibility of increasing DCI payload size limit to >140bits

Proposal: Study a new data integrity check mechanism with the aim to improve payload size scalability and early termination rate over 5G distributed CRC design

Proposal: Consider following metric and methodology to facilitate early termination rate evaluation

- Early termination rate: RET=Ekterminatek+m

  - k : payload size

  - m : number of total data integrity check bits

  - kterminate: the smallest number of decoded information bits (including data intefrity check bits) when none of the candidates in the list can pass the current CRC check(s) after decoding kterminate informations bits

  - E[kterminate] is estimated under SCL with the assumption of list size=8 and pure AWGN noise is transmitted.

Proposal: Study new mechanism of RNTI scrambling to reduce RNTI-FA rate

Proposal: Consider following methodology to facilitate DCI RNTI false alarm evaluation.

- Given a target UE RNTItarget, random payload with size K, aggregation level AL, and AWGN channel

- Evaluate PRNTIWorst=maxRNTI not equal RNTItarget Pr∃u∈L s.t  ϕu,RNTI passes CRC checks

  - L is the list of 8 candidates from SCL decoding

  - ϕu,RNTI is the function that recovers the original unscrambled information sequence from a candidate u in the list and RNTI

  - Opration @SNR with BLER=1%

Proposal: Study high order QAM polar code for DCI spectral efficiency enhancement

# Reference

- RP-251881, New SID: Study on 6G Radio

- 3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities"

- 3GPP TS 38.212: "NR; Multiplexing and channel coding"

- J. Honda and H. Yamamoto, "Polar Coding Without Alphabet Extension for Asymmetric Models," in IEEE Transactions on Information Theory, vol. 59, no. 12, pp. 7829-7838, Dec. 2013

- K. Tian, H. Wang, M. Zhang and X. Yang, "A Novel Scheme to Mitigate the RNTI-FA in Blind Detection of 5G Polar Codes," 2022 IEEE 95th Vehicular Technology Conference