3GPP TSG RAN WG1 #122									R1-2506113

Bengaluru, India, 25 August – 29 August 2025

Agenda Item 	:	10.3.2

Source 	:	Sony

Title 	:	Air interface for device C and 2b

Document for 	:	Discussion and decision

# Introduction

In RANP#108, a new study item on enhancement of solutions for Ambient IoT in NR outdoor for active devices was approved [1]. The objective of the study is as the following:

- Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]: - Deployment scenario 4 with topology 1 with no external carrier wave. - Traffic types DO-DTT, DT, DO-A. - Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command). - Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. - Assumption of the same air interface for Device 2b and Device C. - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769

On the RAN1 level, this objective includes:

- Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios - Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C. - While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming - Maximum distance between Reader and Device of 50-500 m - Maximum transmit power between -3 dBm and +5 dBm for device C - RAN1 to recommend feasible values within this range - Maximum transmit power of -20 dBm and -10 dBm for device 2b Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources. - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations. - Define link budget calculation for coverage.

This document considers the necessary and feasible changes to the Rel-19 air interface to support Device 2b/C.

# Characteristics of device 2b and C

The most significant differences of device 2b and C compared to device type 1 that was considered in Rel-19 are:

Table 1 - Characteristics of device type C and 2b

| Characteristic | Device 1 | Device 2b / C |
| --- | --- | --- |
| Receiver | RF-ED | IF-ED based receiver or ZIF receiver Device C could implement the ZIF receiver without envelope detection. The receiver could incorporate an LNA |
| Transmitter | Backscattering transmitter | Active signal generation based on LO |
| Energy storage | Minimal | Greater energy storage than device type 1 Assume energy storage for dev C is greater than for dev 2b |
| Peak power consumption | ~1 µW | 2b: ≤ a few hundred µW C: ≤ 1 mW to ≤ 10 mW |
| SFO for initial sampling | ~ 105 ppm | 2b: ~ 103 - 104 ppm C: we assume that more accurate sampling can be applied based on higher peak power consumption |
| CFO | N/A | 2b: ~ 103 - 104 ppm C: We assume that CFO in the region of 0.1ppm, based on [TC]XO |

# Necessary changes to air interface to support device type C and DO-A traffic

Active waveform transmission. The waveform for device type C can be transmitted actively. The waveform is generated based on a local oscillator locked to (and frequency shifted from) a DL reference signal, such as a synchronisation signal. Transmitting at an UL frequency that uses a DL synchronisation signal as a reference allows the type C device to transmit at a known UL frequency accurately. This will allow:

- UL transmission is within the bandwidth of the operator

- Frequency division multiplexing of the UL signal

- Minimisation of interference with NR signals in the same UL carrier, assuming a transmit accuracy of 0.1ppm

In order to support active waveform transmission, the A-IoT air interface needs to support a DL synchronisation signal.

Proposal 1: The air interface for device type C and 2b supports a DL synchronisation signal.

UL waveform. The UL transmission needs to be power efficient. A low PAPR waveform aids power efficient transmission. It is hence proposed that a single-tone waveform is supported for UL transmission. RAN1 should study the subcarrier bandwidth for single-tone transmission. A baseline of 3.75kHz can be considered.

Proposal 2: The air interface for device type C and 2b UL single-tone transmission waveform. RAN1 considers a baseline single-tone bandwidth of 3.75kHz.

UL modulation. The UL modulation for device type 1 is either BPSK or OOK. These modulations are compatible with a backscattering device. For an active waveform transmission, we propose that RAN1 considers the MSK and CPFSK proposed for device type 2b in Rel-19 and additionally BPSK and QPSK waveforms that can be transmitted accurately by a type C device.

Proposal 3: RAN1 considers MSK, CPFSK, BPSK and QPSK modulations for UL modulation.

Multiple access for DO-A. It is expected active device, e.g., device type 2b and C support DO-A. For those low complexity devices, grant-free channel access is a good mechanism to reduce the overhead and latency for devices to enter the channel.

Proposal 4: RAN1 considers supporting the grant-free channel access for DO-A traffic for active devices in Rel-20.

However, the D2R data transmission in DO-DTT and DT rely on resources indicated by the R2D triggering message, and there may be many devices trying to access the medium at the same time in the DO-A traffic case in a grant free occasion. It would hence be beneficial if colliding transmissions could be separated. A candidate for such a grant-free D2R, is using non-orthogonal code-based messages allowing for multiple devices transmitting messages at same resources. This technique has recently been a topic of interest in academia, showing good link level and spectral efficiency performance [3][4].

Proposal 5: RAN1 studies the use of grant-free non-orthogonal multiple access for DO-A traffic.

DL multiple access. A ZIF-based receiver allows the device to separate DL transmissions in the frequency domain, allowing for frequency division multiplexing in the DL. This will increase system capacity in the DL.

Proposal 6: RAN1 support DL frequency division multiplexing.

System information. For DO-DTT traffic, any system information type information can be transmitted in the R2D signal that allocates the DO-DTT traffic. For DO-A traffic, there is no R2D signal allocating the DO-A message. Hence, some form of system information is required for the support of DO-A traffic.

Proposal 7: RAN1 provides physical channels that enable the transmission of system information to devices for the support of DO-A traffic.

Energy harvesting / intermittent transmissions. While device type 1 has small power consumption and small energy storage, device type C devices will have larger power consumption and larger energy storage. To avoid the UE having to support large energy storage devices, it should be possible to interrupt transmissions from the UE to allow the UE to replenish its energy storage or to allow the energy store to recover (for example in the case where the device needs to harvest the energy to charge its storage device or where the energy storage device is a battery). Discontinuous monitoring (e.g. associated with DL synchronisation signal timing) should also be supported, such that the device does not need to drain its energy store continuously to monitor for DL transmissions.

Proposal 8: RAN1 support intermittent UL transmission and DL discontinuous monitoring to allow for device operating on limited energy stores.

# Conclusion

This document has considered the necessary changes required to support device type C and 2b for Ambient-IoT. The following proposals are made:

Proposal 1: The air interface for device type C and 2b supports a DL synchronisation signal.

Proposal 2: The air interface for device type C and 2b UL single-tone transmission waveform. RAN1 considers a baseline single-tone bandwidth of 3.75kHz.

Proposal 3: RAN1 considers MSK, CPFSK, BPSK and QPSK modulations for UL modulation.

Proposal 4: RAN1 considers supporting the grant-free channel access for DO-A traffic for active devices in Rel-20.

Proposal 5: RAN1 studies the use of grant-free non-orthogonal multiple access for DO-A traffic.

Proposal 6: RAN1 support DL frequency division multiplexing.

Proposal 7: RAN1 provides physical channels that enable the transmission of system information to devices for the support of DO-A traffic.

Proposal 8: RAN1 support intermittent UL transmission and DL discontinuous monitoring to allow for device operating on limited energy stores.

References

[1]	RP-251884. “New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices”

[2]	TR 38.769 V19.0.0. “Study on solutions for Ambient IoT (Internet of Things) in NR”.

[3]	Z. Gao, L. Dai, S. Han, C. -L. I, Z. Wang and L. Hanzo, "Compressive Sensing Techniques for Next-Generation Wireless Communications," in IEEE Wireless Communications, vol. 25, no. 3, pp. 144-153, JUNE 2018

[4]	“Massive IoT Access for 6G”. Zhen Gao, Malong Ke, Li Qiao and Yikun Mei. Springer. 2022.