3GPP TSG RAN WG1 Meeting #122				  	     R1-2506154

Bangalore, India, August 25 – 29, 2025

Title:	Considerations for GNSS resilient NR-NTN operation

Source: 	ST Engineering iDirect

Type:	discussion

Agenda Item:	10.6.1 NR-NTN GNSS resilience

Abstract

This tdoc reviews GNSS dependence in 5G NTN and presents solutions to address GNSS resilience in NR-NTN.

# Introduction

In the current 3GPP specifications, it is mandated that a UE supporting NTN is GNSS-capable (TS 38.300 § 16.14.1), and that the UE shall only transmit when it has a valid GNSS position (TS 36.300 § 23.21.2.2).

In the RAN#108 plenary meeting in Prague (June 2025), a Rel-20 5G-Advanced study item is approved to study GNSS resilient NR-NTN operation [1]. The study will focus on a scenario where the GNSS information is temporarily unavailable. The impact on initial access and connected mode procedures for NR-NTN will be assessed.

In this contribution, we discuss current issues with the GNSS dependence, and present general considerations that should be addressed in the study.

# GNSS dependency considerations

Compared to terrestrial links, a satellite link suffers from a long round-trip time delay (due to the link distance) and large Doppler frequency offset (due to the fast satellite movement, mainly for non-geosynchronous orbits). There is also a frequency offset caused by the drift of the crystal oscillators at the satellite and UE.

Based on its GNSS position and satellite ephemeris data (that is received in SIB19), the UE can pre-compensate time and frequency offsets caused by the satellite movement (TS 38.300 § 16.14.2.2). If the UE does not have a valid GNSS position, it shall transition to the idle state (TS 36.300 § 23.21.2.2).

In current 5G NTN deployments, GNSS information is required during

- Initial access (i.e., logon procedure)

- Tracking of time and frequency offsets

The UE and satellite position can also be used to derive pointing information, i.e., define the beam steering coefficients for a UE’s electronically steerable antenna.

The dependence on GNSS has several drawbacks, e.g., a GNSS can be easily spoofed or jammed and impacts the reliability of the NTN system. It should also be emphasized that the current GNSS-based solution has considerable challenges, e.g., to deal with the frequency offsets and timing drifts (even for GEO satellites) [2].

Observation 1: In current 5G-NTN systems, GNSS information is used for pre-compensation of time and frequency offsets in initial access and connected mode.

Observation 2: In practical deployments, the current GNSS-based system has challenges for dealing with time and frequency offsets.

# Requirements

The objective of the study is to assess the impact on initial access and connected mode procedures for NR-NTN, including potential implications if any on RAN4 RRM specifications [1], considering:

- Aim to minimize physical layer procedures impact

- Avoid physical layer channel/signal changes

- Backward compatible issues with legacy NTN capable UEs will be assessed and should be prevented

- Applicable to NGSO and GSO constellations, NTN operating in below and in above 10 GHz frequency bands, transparent and regenerative satellites

More specifically, we propose that a solution to enable GNSS-resilient NR-NTN operation should meet the following requirements

- It should enhance tracking of time and frequency offsets in connected mode

- It should enable GNSS-resilient operation of different satellite deployments, including

  - A deployment with only a single geostationary satellite

  - A deployment of a full non-geosynchronous orbit (NGSO) constellation

  - A small NGSO deployment where only a single satellite is in view

  - An NGSO constellation with discontinuous coverage

  - Deployments with multiple NGSO/GEO satellites in view

It should be noted that IRIS2 will be a multi-orbit constellation consisting of 264 LEO satellites at 1200 km altitude, 10 LEO satellites below 750 km, and 18 MEO satellites at 8000 km altitude [4]. At many locations, there won’t be multiple satellites in view. Furthermore, in typical deployments, the UE will only have a single beam, limiting the visibility to a single satellite.

Proposal 1: Consider different satellite deployment scenarios, including a scenario where only a single (GEO) satellite is visible to the UE.

Furthermore, it is expected that the outcome of the study can be reused for the 6G Radio, cf. the Rel-20 study on 6G Radio [3], and that it will be verified whether a potential solution can be applied to IoT-NTN.

Proposal 2: Consider study outcomes for 6G Radio.

# Possible solutions

Two possible solutions that could enable GNSS-resilient operation are the following.

- Enhance 5G NR-NTN positioning reference signals (PRS) so that a UE can determine its approximate position based on 5G NR control signaling, rather than using a GNSS.

- Enhance closed loops for control of frequency and timing offsets.

The first solution can be considered a first step towards a positioning, navigation, and timing (PNT) system, which can also be relevant for NTN-based location verification, and which solves the GNSS-dependence for initial access (i.e., no PRACH changes are required). On the other hand, it increases the complexity at the satellite, as the satellites need to be accurately synchronized and the signaling load increases. This solution might not work to connect to a single GEO satellite.

The second solution can be seen as an enhancement of the current timing advance mechanism, i.e., by adding signaling for frequency offset compensation. This solution does not introduce positioning complexity in physical-layer synchronization, e.g., to cope with error propagation, and only requires the introduction of a new MAC CE.

Both solutions could complement each other, e.g., using PRS info during initial access to get a rough estimate of a UE’s location, and for network-verified UE location purposes, and enhancing time and frequency correction loops for time and frequency tracking in connected mode.

Use case: distance accuracy requirement

For numerology 4, with a subcarrier spacing of 240 kHz, the length of the cyclic prefix is 0.293 µs. Therefore, the distance error in connected mode should be less than 88 m. Considering a 50% margin, the distance accuracy should be ±22 m. The current timing advance mechanism is a first-order loop and does not allow correcting timing offsets in high-speed scenarios such as NTN. Considering that the speed of a LEO satellite at an altitude of 400 km is 8000 m/s, it is challenging to have such a tight requirement. For initial access, PRACH formats exist with a longer cyclic prefix and guard time, to enable relaxed accuracy.

Observation 3: The required position accuracy to maintain an NTN link is challenging to obtain with a PNT-based system.

Proposal 3: Investigate two possible solutions (enhancement of positioning reference signaling and enabling closed loops for time and frequency tracking), and how they could complement each other.

# Conclusion

In this contribution, requirements for GNSS-resilient operation are discussed, and an overview of two possible solutions is provided.

Observation 1: In current 5G-NTN systems, GNSS information is used for pre-compensation of time and frequency offsets in initial access and connected mode.

Observation 2: In practical deployments, the current GNSS-based system has challenges for dealing with time and frequency offsets.

Observation 3: The required position accuracy to maintain an NTN link is challenging to obtain with a PNT-based system.

Proposal 1: Consider different satellite deployment scenarios, including a scenario where only a single (GEO) satellite is visible to the UE.

Proposal 2: Consider study outcomes for 6G Radio.

Proposal 3: Investigate two possible solutions (enhancement of positioning reference signaling and enabling closed loops for time and frequency tracking), and how they could complement each other.

# References

[1] RP-251863, “New SID: Study on GNSS resilient NR-NTN operation”, Prague, Czech Republic, June 9 – 13, 2025

[2] S. Kumar et al., "5G-NR Over Transparent Payload GEO Satellites: A Hands-On Approach Using OpenAirInterface5G-NTN," in IEEE Open Journal of the Communications Society, doi: 10.1109/ OJCOMS.2025.3586871

[3] RP-251881, “New SID: Study on 6G Radio”, Prague, Czech Republic, June 9 – 13, 2025

[4] RP-250765, “IRIS²: the first fully-fledged 5G NR NTN multi-orbit satellite communications system”, Incheon, Korea, 12 – 14 March, 2025

END