3GPP TSG RAN WG1 #122	R1-2506251

Bengaluru, India, Aug 25th – 29th, 2025

Source:	Sharp

Title:	Maintenance on adaptation of common signal/channel transmissions

Agenda Item:	8.5.3

Document for:	Discussion and Decision

Introduction

In this paper, we share the view on adaptation of common signal/channel transmissions for network energy saving.

Discussions

  - Indication for availability of additional PRACH resource in DCI format 1_0 with P-RNTI

Agreement (RAN1#120bis) For DCI-based adaptation for additional PRACH resources, the following 1-bit field is used for adaptation indication in DCI format 1_0 with P-RNTI - Use one bit from the Bits 5-8 within the Short Message field (from upper layers) - Send LS to RAN2 to confirm the use of this bit. Above applies for cell that transmits the DCI for connected UEs and IDLE/INACTIVE mode UEs Agreement (RAN1#121) Additional PRACH availability indication can be carried by a DCI 1_0 with P-RNTI with Short Messages Indicator set to 00, 01,10,11. - Note: Above is already reflected in the endorsed editor CR 38.212 Running CR v1 on TS38.331 for RAN2 post 130 [NES] Table 6.5-1: Short Messages Bit; Short Message. 1; systemInfoModification If set to 1: indication of a BCCH modification other than SIB6, SIB7, SIB8 and posSIBs.. 2; etwsAndCmasIndication If set to 1: indication of an ETWS primary notification and/or an ETWS secondary notification and/or a CMAS notification.. 3; stopPagingMonitoring This bit can be used for only operation with shared spectrum channel access and if nrofPDCCH-MonitoringOccasionPerSSB-InPO is present. If set to 1: indication that the UE may stop monitoring PDCCH occasion(s) for paging in this Paging Occasion as specified in TS 38.304 [20], clause 7.1.. 4; systemInfoModification-eDRX If set to 1: indication of a BCCH modification other than SIB6, SIB7, SIB8 and posSIBs. This indication applies only to UEs using IDLE eDRX cycle longer than the BCCH modification period.. 5; adaptiveRAResources If set to 1: Indication of activation for adaptive random access resources.. 6 – 8; Not used in this release of the specification, and shall be ignored by UE if received..

After RAN1#121 meeting, CR for TS38.212 was created by the editor based on previous agreements related to Rel-19 NES [1]. In email discussions related to the CR, the following issues were raised concerning the indication for availability of additional PRACH resource in DCI format 1_0 with P-RNTI. However, these points remain unresolved:

- Description of the Short Messages Indicator field (including Table 7.3.1.2.1-1)

- Description of the Short Messages field

Before considering the detailed descriptions, it should be clarified whether there are any discrepancies between the RAN1 understanding and the description of running CR on TS38.331. RAN1 agreed that the 1-bit indication for availability of additional PRACH resource exists regardless of whether the short messages indicator is set to 00, 01, 10, or 11 [2]. It has also been agreed that this 1 bit is included within the short messages field [3]. The intention behind the two agreements suggests that the 1-bit indication exists within the Short Messages field but is not one content of the short messages themselves. However, according to the description of the short messages in the running CR for TS38.331, the 1-bit indication seems to be considered one content of the short messages, as the 1 bit is described in the same manner as the other contents. Therefore, it should first be clarified which of the following options should be applied:

Option 1: TS38.212 describes the 1-bit indication as one of short messages

Option 2: TS38.212 describes the 1-bit indication as separate from the short messages, which may require modifying the description on TS38.331 accordingly

Option 3: The 1-bit indication is supported as both of one of short messages and separate from the short messages. TS38.212 describes as separate from the short messages, and TS38.331 describes as one of short messages

- Proposal 1: RAN1 should clarify whether the 1-bit indication for availability of additional PRACH resource is one content of the short messages or not

Regarding the motivation for treating the indication separately from the short messages, there is the intention to process the indication at the physical layer whereas short messages is handled at the higher layer. Therefore, it is advisable to reflect this intention in the description in TS38.212, handling the indication separately from the short messages.

- Proposal 2: In TS38.212, the description of the 1-bit indication for the availability of additional PRACH adaptation should ensure that the corresponding operations can be completed at the physical layer

- Regarding the description of the Short Messages Indicator field, our understanding is that the agreement—the 1-bit indication for availability of additional PRACH resource exists regardless of whether the short messages indicator is set to 00, 01, 10, or 11— reflects the intention that the 1 bit is independent from other information in the paging DCI. Specifically, UEs can read the 1-bit indication without the referring to the Short Message Indicator field. Based on this interpretation, the Short Messages Indicator field (including Table 7.3.1.2.1-1) does not require additional description, as it is not relevant to the 1-bit indication.

- Regarding the description of the Short Messages field, in order to reflect the intention that the 1-bit indication is processed at the physical layer, a description as this 1 bit is determined by referring to TS38.212 is necessary. Regarding this, the intention can be clarified by adding the description that the 1 bit used for the additional PRACH availability indication (referred to as the 5th bit in the following TP) is present irrespective of the value of the Short Messages Indicator and according to TS38.212 (i.e., Table 7.3.1.2.1-5). For Table 7.3.1.2.1-5 in the corresponding TP, although it is currently defined for PDCCH order, it can be shared with Paging DCI by removing the part of the description that is specific to PDCCH order.

Based on the above, the following TP should be considered

- 7.3.1.2.1 Format 1_0 *** Unchanged parts are omitted *** The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by P-RNTI: - Short Messages Indicator - 2 bits according to Table 7.3.1.2.1-1. - Short Messages - 8 bits, according to Clause 6.5 of [9, TS38.331]. If addl-RACH-Config-Adaptation is configured, 5th bit in this bit field is present for the PRACH availability indication according to Table 7.3.1.2.1-5, irrespective of the value of Short Messages Indicator, otherwise, reserved. If only the scheduling information for Paging, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, all the bits in this bit field are reserved, except the 5th bit indicating the availability of the PRACH resource configured by addl-RACH-Config-Adaptation according to Clause 6.5 of [9, TS38.331],. *** Unchanged parts are omitted *** Table 7.3.1.2.1-5: PRACH resource indicator Bit field; PRACH resource indicator. 0; The PRACH resource configured by addl-RACH-Config-Adaptation is not available for the PRACH transmission initiated by the PDCCH order. 1; The PRACH resource configured by addl-RACH-Config-Adaptation is available for the PRACH transmission initiated by the PDCCH order. *** Unchanged parts are omitted ***

Proposal 3: Adopt the following TP in clause 7.3.1.2.1 of TS38.212

- 7.3.1.2.1 Format 1_0 *** Unchanged parts are omitted *** The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by P-RNTI: - Short Messages Indicator - 2 bits according to Table 7.3.1.2.1-1. - Short Messages - 8 bits, according to Clause 6.5 of [9, TS38.331]. If addl-RACH-Config-Adaptation is configured, 5th bit in this bit field is present for the PRACH availability indication according to Table 7.3.1.2.1-5, irrespective of the value of Short Messages Indicator, otherwise, reserved. If only the scheduling information for Paging, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, all the bits in this bit field are reserved, except the 5th bit indicating the availability of the PRACH resource configured by addl-RACH-Config-Adaptation according to Clause 6.5 of [9, TS38.331],. *** Unchanged parts are omitted *** Table 7.3.1.2.1-5: PRACH resource indicator Bit field; PRACH resource indicator. 0; The PRACH resource configured by addl-RACH-Config-Adaptation is not available for the PRACH transmission initiated by the PDCCH order. 1; The PRACH resource configured by addl-RACH-Config-Adaptation is available for the PRACH transmission initiated by the PDCCH order. *** Unchanged parts are omitted ***

Conclusion

In this contribution, we have the following proposal:

- Proposal 1: RAN1 should clarify whether the 1-bit indication for availability of additional PRACH resource is one content of the short messages or not

Proposal 2: In TS38.212, the description of the 1-bit indication for the availability of additional PRACH adaptation should ensure that the corresponding operations can be completed at the physical layer

Proposal 3: Adopt the following TP in clause 7.3.1.2.1 of TS38.212

- 7.3.1.2.1 Format 1_0 *** Unchanged parts are omitted *** The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by P-RNTI: - Short Messages Indicator - 2 bits according to Table 7.3.1.2.1-1. - Short Messages - 8 bits, according to Clause 6.5 of [9, TS38.331]. If addl-RACH-Config-Adaptation is configured, 5th bit in this bit field is present for the PRACH availability indication according to Table 7.3.1.2.1-5, irrespective of the value of Short Messages Indicator, otherwise, reserved. If only the scheduling information for Paging, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, all the bits in this bit field are reserved, except the 5th bit indicating the availability of the PRACH resource configured by addl-RACH-Config-Adaptation according to Clause 6.5 of [9, TS38.331],. *** Unchanged parts are omitted *** Table 7.3.1.2.1-5: PRACH resource indicator Bit field; PRACH resource indicator. 0; The PRACH resource configured by addl-RACH-Config-Adaptation is not available for the PRACH transmission initiated by the PDCCH order. 1; The PRACH resource configured by addl-RACH-Config-Adaptation is available for the PRACH transmission initiated by the PDCCH order. *** Unchanged parts are omitted ***

References

R1-2504984, “Introduction of Rel-19 enhancements of network energy savings for NR,” Huawei, RAN1#121, May 2025.

“RAN1 Chairman’s Notes,” RAN1 #121, May 2025

“RAN1 Chairman’s Notes,” RAN1 #120bis, April 2025