3GPP TSG RAN WG1 #122	R1-2506307

Bengaluru, India, Aug 25th – 29th, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on Frame structure for 6GR

Agenda Item:	11.3.2

Document for: 	Discussion and Decision

- Introduction

At the RAN1#121 meeting, it was concluded that R19 NR-NTN WI was completed. In this contribution, maintenance of DL coverage enhancement will be discussed, and several proposals will be submitted.

- Discussion

As most people may consider, frame structure defined in 5G NR is well-scalable/flexible design and therefore, the same structure could be reused for 6GR. Some aspects may be different between 5G NR and 6GR; RAN1 should focus on such essential modifications only. Impractical change is not preferable based on lessons learned from 5G discussion and commercial services.

Proposal 1:

- Reuse numerology and frame structure in 5G NR as baseline, and focus on essential modifications only

One important point for 6GR is FR3 support. For 6GR, FR3 will be new frequency spectrum to boost mobile communication performance though availability could be different dependent on countries/regions. Here, it is noted that 4.4 to 4.8 GHz, 7.125 to 8.4 GHz, 14.8 to 15.35 GHz are the candidate frequencies to be discussed in WRC-27. For FR3, 3GPP should consider both 7 to 8 GHz band and 15 GHz band.

Observation 1:

- 4.4 to 4.8 GHz, 7.125 to 8.4 GHz, 14.8 to 15.35 GHz are the candidate frequencies to be discussed in WRC-27.

  - FR3 discussion in 3GPP will cover both 7 to 8 GHz band and 15 GHz band.

  - Sub-carrier spacing

    - # of SCSs per band

Although baseline for SCS should be the definition in 5G NR as abovementioned, we believe that # of SCSs per band is an important topic, which could be different between 5G NR and 6GR. In 5G NR, 15/30/60 kHz SCSs are available for FR1 bands and 60/120 kHz SCSs are available for FR2 bands. For example, it was assumed in 5G NR that multiple SCSs could be beneficial to support various service types such as eMBB and URLLC. However, as a result, a single/common SCS has been widely used for each band in practical. At least in our understanding, 15 kHz SCS is used for FR1 FDD bands such as 800 MHz/2GHz, 30 kHz SCS is used for FR1 TDD bands such as 3.5 GHz/4.5 GHz, and 120 kHz SCS is used for FR2 bands such as 28 GHz (except for SSB with 240 kHz SCS for large # of beams); there could be no other case.

Given the real situation, it is better to define only a single SCS per band or per sub-FR or per carrier type. This limitation does not have any disadvantages in practical NW and brings both easier NW/UE implementation and simpler 3GPP specifications/discussion. The following will be the further discussion points:

- Regarding which SCS is selected for each band/sub-FR/carrier type, the above combinations should be considered/agreed, unless some NW operator has strong concern on them. For SCS for SSB at FR2, although support of 240 kHz SCS for SSB is beneficial to increase # of SSBs for beam sweeping, 120 kHz SCS could be OK with mechanisms to avoid reductions of # of SSBs.

- For FR3 bands, it may be better to use different SCS between lower FR3 (e.g., 7GHz) and higher FR3 (e.g., 15 GHz). Especially, inter-FR CA with FR3 bands will not be rare cases; from this perspective, common SCS among CCs is beneficial as # of SCSs for CA could be an important perspective. For example, 30 kHz SCS is applied for lower FR3 so that FR1-FR3 (e.g., 7GHz) CA is performed with less than 3 different SCSs; 120 kHz SCS is applied for higher FR3 so that FR2-FR3 (e.g., 15 GHz) CA is performed with only a single SCS; etc.

- Another point could be NTN bands. For FR1-NTN in NR, only 15 kHz SCS has been studied as only FDD bands are defined while all 15/30/60 kHz are available in 38.101-5. For FR2-NTN in NR (UL around 30 GHz, DL around 20 GHz), there has not been a lot of studies and thereby which SCS is more reasonable is unclear; at least 60/120 kHz are available in 38.101-5. It is necessary to have discussion on whether the same limitation between TN and NTN is applicable or not.

Observation 2:

- In 5G NR, although multiple SCSs are available for each band, it seems that this flexibility has not been used effectively in real network.

Proposal 2:

- Down-select a single SCS per band or sub-FR or carrier type

  - E.g., 15 kHz SCS for FR1 FDD, 30 kHz SCS for FR1 TDD, 120 kHz SCS for FR2

  - FFS: SCS for FR3. E.g., 30 kHz SCS for lower FR3 and 120 kHz SCS for higher FR3

  - FFS: whether the definition is common or separate between TN and NTN

    - SCS for FR3

RAN1 needs to discuss which SCS is better for FR3 bands. For the discussion, the following two perspectives will be considered/analyzed:

- BLER performance evaluation: LLS can be performed taking into consideration phase noise as done in R14 for 5G NR. If there is a large difference, SCS with better performance could be selected; otherwise, SCS should be determined based on other perspectives as below.

- # of different SCSs in case of inter-FR CA: When CA is performed, less # of numerologies among carriers is preferable for easier NW/UE implementation and simpler 3GPP specifications/discussion. FR1-FR3 CA, FR2-FR3 CA, or even FR1-FR2-FR3 CA will be required in 6G NW; in this case, SCS alignment between the existing bands and new FR3 bands is beneficial for the reduction.

Proposal 3:

- Study which SCS is applied to FR3 from the following perspectives

  - BLER performance taking into consideration phase noise

  - # of different SCSs in case of inter-FR CA

Then, the remaining in this section shows BLER performance taking into consideration phase noise. SCSs are 30/60/120 kHz SCS, carrier frequencies are 7/15 GHz, bandwidths are 100/200 MHz, and MCSs are 64QAM with coding rate R = 666/1024 and 256QAM with R = 948/1024. The other simulation assumptions can be found in Table A-1 of appendix.

64QAM with R = 666/1024

(a) 7 GHz / 100 MHz	(b) 15 GHz / 100 MHz

(c) 7 GHz / 200 MHz	(d) 15 GHz / 200 MHz

Figure 1: BLER performance with 64QAM and R = 666/1024.

Figure 1 shows the BLER performance with 64QAM and R = 666/1024. In Figure 1, similar performance are observed among different SCSs configurations in all parameter sets. It seems that there is no/little impact of phase noise in 64QAM cases.

Observation 3:

- For 64QAM with R = 666/1024, taking into consideration phase noise,

  - No/little difference of BLER performance is observed among 30/60/120 kHz SCSs with 100 or 200 MHz bandwidth

256QAM with R = 948/1024

(a) 7 GHz / 100 MHz	(b) 15 GHz / 100 MHz

Figure 2: BLER performance with 256QAM and R = 948/1024.

Figure 2 shows the BLER performance with 256QAM and R = 948/1024. In both cases of Figure 2, performance degradation is observed at 30 kHz SCS in comparison to 60/120 kHz SCSs. The degradation could be due to phase noise.

Observation 4:

- For 256QAM with R = 948/1024, taking into consideration phase noise,

  - Degradation of BLER performance is observed at 30 kHz SCS in comparison to 60/120 kHz SCSs, with 100 MHz bandwidth.

  - No significant degradation of BLER performance is observed at 60 kHz and 120 kHz, with 100 MHz bandwidth.

Based on the above evaluation results, it can be said that smaller SCS may not be a good choice if using higher modulation order such as 256 QAM or even 1024 QAM is a typical situation. Although a simple solution is just using higher SCS, another potential solution to apply smaller SCS is to introduce PTRS as in FR2.

Proposal 4:

- Study which SCS is used for FR3, including the following alternatives:

  - Alt 1: 30 kHz SCS with PTRS

  - Alt 2: 60 kHz SCS or 120 kHz SCS

  - Channel bandwidth

This section is the same as what we describe in 2.1.2 of [1] for agenda 11.1. There is a high relationship with this agenda item as well, thus we include this in both contributions.

    - Max BW

As stated in the SID, 6GR is assumed to support wider channel bandwidth (at least 200MHz) for at least above 2 GHz, around 7 GHz. In this regard, we propose to support at least 200MHz CBW for FR1 and FR3 (the range between FR1 and FR2-1 (including around ~7GHz)). For FR1, this should be supported especially in 6.425 to 7.125 GHz. Regarding how to support 200MHz CBW for FR1 and FR3, there are following two alternatives:

- Alt 1: 8k FFT / cons – implementation efforts

- Alt 2: 60 kHz SCS / cons – larger # of SCSs in the case of inter-band/FR CA

The pros/cons for each alternative need further study. Regarding FR2-1, as in NR, 400 MHz CBW should be supported.

Proposal 5:

- For the maximum CBW,

  - FR1 especially in 6.425 to 7.125 GHz: Support at least 200MHz CBW

  - FR3 (the range between FR1 and FR2-1): Support at least 200MHz CBW

  - FR2-1: Support 400 MHz CBW

- Study how to support 200MHz CBW for FR1 and FR3

  - Alt 1: 8k FFT

  - Alt 2: 60 kHz SCS

    - Min BW

As discussed in Section 2.7.2 of [1] for agenda 11.1, we target to support 6G IoT (low-tier device) which can be used as the migration from 4G LTE Cat.M, and 3 – 5 MHz CBW is the candidate considering the common 6G Radio design with MBB at least for low FR1 bands with 15kHz SCS. RAN1 needs to consider the trade-off between complexity reduction gain for BW reduction and the impact on the 6GR initial access design. We have observed that there are two camps as follows:

- A: Cost is the same b/w UE with 20 MHz BW and UE with narrower BW, which is motivated by band support with 3 to 5 MHz BW

- B: Cost can be reduced when bandwidth is reduced from 20 MHz to narrower BW

Meanwhile, cost reduction (max approx. 15%) has been studied/agreed in Rel-18 eRedCap SI (TR 38.865).

Proposal 6:

- For the minimum CBW,

  - Study 3 – 5 MHz CBW at least for low FR1 bands with 15kHz SCS, considering

    - 6G IoT (low-tier device) support, including complexity reduction gain

    - Impact on the 6GR initial access design

  - Bandwidth part

This section is also the same as what we describe in 2.6 of [1] for agenda 11.1. For the same reason as mentioned in 2.2 of this contribution, both contributions include the same contents.

For BWP, our view for 6GR is that simplified/convenient BWP mechanism should be designed/specified only with essential factors. Although BWP concept was discussed from the beginning of 5G NR and introduced to achieve various service types/UE power saving/etc., our feeling in this stage is that unfortunately the BWP mechanism is too flexible and complicated from both NW/UE implementation perspective and 3GPP specifications/discussion perspective. As a result, it seems that BWP has been used in real NW less effectively than what 3GPP had expected. For example, the following can be pointed out as inconvenient perspectives, which are lessons learned from 5G discussion and commercial services.

- SCS switching: Complicated, but less motivated. As discussed in the previous section, a single SCS is sufficient for each band/sub-FR/carrier type.

- Association with CORESET/Search space (SS): Once CORESET/SS is configured by RRC parameters associated with BWP, basically they are applied until RRC reconfigurations or BWP switching. If NW side would like to change CORESET based on e.g., traffic variation, either of them must be performed, which is inefficient.

- BWP switching delay: When BWP switching is indicated dynamically, the switching is completed after UE processing time. The UE processing time defined in 5G NR is too large, depending on UE capability, e.g., 1 or 3 slots for 15 kHz SCS.

- RRC configuration overhead: A lot of RRC parameters are under BWP configuration. This results in unnecessarily large overhead as common parameters among multiple BWPs must be duplicated.

- BWP types: A lot of types, but excessive. For example, it is questionable whether there is any NW operating with default BWP or dormant BWP.

- Frequency location: Although BWP in 5G NR is a kind of too flexible feature as abovementioned, a less flexible part is that center frequency must be common between DL BWP and UL BWP.

From our perspective, motivation to define BWP in 6G is to support multiple UE types, e.g., smartphone and IoT, in the same band. Some kind of BWP feature is necessary also in 6G as bandwidth available in a specific UE type could be narrower than a carrier bandwidth. For example, NW could configure a carrier with 100 MHz bandwidth, but IoT UE may support only 5 or 20 MHz bandwidth. In this case, some kind of configuration to allow the IoT UE to access the carrier with a part of the 100 MHz bandwidth. BWP concept can be used for this purpose, and 6G BWP can focus on elements relevant to this purpose. The proposal below is submitted based on this direction.

One controversial aspect could be that whether 6G BWP should be defined for power saving as well as support of the various UE types in the same band. Our current understanding is that PDCCH monitoring is a dominant factor in UE power consumption; with this assumption, 6G BWP definition can be separated from UE power saving discussion as long as some kind of dynamic CORESET/SS switching features is available. However, UE/chip vendors may have different opinions; if 6G BWP is designed also for power saving purpose and the corresponding elements are practically/effectively used by NW, that direction is also fine for us. The necessity of BWP for power saving purpose could be discussed in agenda 11.5 (EE).

Proposal 7:

- In 6GR, design BWP to support various UE types in the same band.

Proposal 8:

- Define BWP with the following details/restrictions.

  - Single SCS per BWP

  - More than one CORESET/Search space configurations with dynamic switching feature in a single BWP

  - Configuration per UE or per UE type

  - Only essential/relevant configurations under BWP configurations

  - No dynamic BWP switching, as long as dynamic CORESET/SS switching is available

  - Only initial BWP and a dedicated BWP for each UE or each UE type

    - No other BWP types such as default BWP and dormant BWP

  - Duplex

This section is also the same as what we describe in 2.10 of [1] for agenda 11.1. For the same reason as mentioned in 2.2 of this contribution, both contributions include the same contents.

In 5G NR, FDD/semi-static TDD/dynamic TDD has been specified at R15 and SBFD has been introduced at R19. However, at least we would point out that it is difficult to say 5G dynamic TDD has been successful. As known well probably, it could be typical that NW operators with adjacent TDD carriers use a completely aligned/fixed TDD pattern to avoid inter-operator interference, and changing the pattern is significantly challenging. We believe that this situation will not be different in 6G era, therefore, our view is that dynamic TDD can be deprioritized. For semi-static TDD, D/U/F will be configured in RRC layer, but F will not be used for any TX/RX, which means “F” is just a kind of DL/UL gap. Given that dynamic TDD is not so attractive for 6GR considering the real situation, at least we can skip discussion on complex features for dynamic TDD such as SFI and slot formats.

For FDD and semi-static TDD, basically we believe that the same definition as in NR can be reused for 6GR. Periodicity including one or two patterns, configuration format, FDD as a special case of semi-static TDD, etc. One question could be the necessity of UE-dedicated semi-static TDD configuration. Cell common semi-static TDD configuration may be sufficient.

Regarding SBFD, it is better to consider whether/how to support SBFD with its feasibility and realistic performance in practical deployments. For example, SBFD is beneficial to increase UL resources without causing severe CLI to adjacent carriers; thus, applying SBFD to the existing TDD bands could be possible, which is different from dynamic TDD. On the other hand, for the existing TDD bands, given current aligned/fixed TDD pattern among adjacent carriers it would only be possible to introduce SBFD symbols in current DL slot(s) without changing TDD pattern in the bands. Due to the limited SBFD symbols as well as performance degradation caused by self-CLI, the practical gain may be limited.

When the practical gain of SBFD is well justified for the existing TDD bands and/or 6G new bands, then another discussion could be whether SBFD is the same as in NR or different, e.g., dynamic SBFD, SBFD at UE side, etc. In our view, SBFD at BS side is enough at least in 6G Day 1. Regarding dynamic SBFD, we do not see any strong/realistic motivation to change SBFD pattern dynamically. Alternatively, a simple switching between SBFD and TDD (non-SBFD) without RRC reconfiguration is a possible direction to change the ratio of DL/UL resources according to actual DL/UL traffic, without excessive/complicated mechanisms. For example, it can be considered that a single SBFD pattern and a single TDD pattern are configured in RRC layer, and 1 bit switching indication is provided via lower layer signaling.

Proposal 9:

- Support at least FDD and semi-static TDD.

- Deprioritize dynamic TDD.

- Discuss whether/how to support SBFD with considering its feasibility and realistic performance in practical deployments.

  - If supported, study the following two alternatives:

    - Alt 1: support semi-static SBFD only and not support dynamic SBFD

    - Alt 2: support semi-static SBFD and a simple SBFD/TDD switch w/o RRC reconfiguration.

  - Resource grid

Resource grid, i.e., definitions of time-domain resource and frequency-domain resource, will be a discussion point in this agenda item. Basically, our feeling is that resource grid defined for 5G NR is well-designed/scalable without excessive aspects. 6GR can reuse the definition as baseline.

For time-domain, we believe that any change is unnecessary. 10 ms radio frame / 1 ms subframe / 2μ slots per subframe / 14 symbols per slot, with subframe-level boundary alignment should be reused as they are. This scalable frame structure can cover FR3 and sensing cases, which are the key differences from between 5G NR and 6GR that could have impacts on resource grid.

For frequency-domain, the same PRB definition, i.e., 12 subcarriers per PRB, is the priority perhaps. On the other hand, scheduling based on 1 PRB with 12 SCs may not be the best for coverage limit, especially for NTN scenario. 1 PRB with other than 12 SCs can be another option with the disadvantage of a lot of spec impacts. Further alternative is keeping the resource grid with sub-PRB-level allocations, which has been raised previously for NR-NTN at RAN plenary level. One note is that waveform discussion in agenda 11.3.1 could have some kind of impact on frequency-domain resource grid. If this is the case, revisit in this agenda is required.

Resource grid for MRSS support will be discussed in the next section.

Proposal 10:

- For time-domain resource grid, reuse the scalable frame structure with subframe-level boundary alignment as in NR.

  - 10 ms radio frame, 1 ms subframe, 2μ slots per subframe, 14 symbols per slot.

Proposal 11:

- For frequency-domain resource grid, study the following for UL coverage especially in NTN scenario:

  - Alt 1: 1 PRB with 12 SCs as in NR.

    - Alt 1-1: Allow sub-PRB-level resource allocation

  - Alt 2: 1 PRB with other than 12 SCs, e.g., 4 or 6 SCs

  - MRSS

This section is also the same as what we describe in 2.4 of [1] for agenda 11.1. For the same reason as mentioned in 2.2 of this contribution, both contributions include the same contents.

As explicitly described in the 6G SID, MRSS is one of the important topics for 6GR. Regardless of migration options such as RAN/CN aggregation and dual registration, during the migration phase, both 5G BS and 6G BS will be necessarily operated e.g., for coverage band such as 800 MHz / 2 GHz; otherwise, connectivity of either 5G or 6G could be degraded. MRSS will be better to use the limited bandwidth efficiently, compared to e.g., fixed FDM between 5G and 6G.

Meanwhile, we know well that 5G NR specification has been designed with super-flexibility. In other words, it would be true that not so many aspects of 6GR should carefully be specified to support MRSS between 5G NR and 6G. This point is a clear difference from DSS between 4G LTE and 5G NR, where it seems that achievable performance is not so high unfortunately.

Proposal 12:

- For MRSS support, RAN1 to introduce only essential features in consideration of high NR flexibility.

Then, for frame structure perspectives taking into consideration MRSS, we do not see any necessity to redesign resource grid. As proposed in the previous section, scalable/aligned frame structure with OFDM-based waveform defined in 5G NR should be reused for 6GR, and the structure is applicable to 6GR as it is.

Proposal 13:

- For MRSS support, no special handling of frame structure is expected.

  - As long as scalable/aligned frame structure with OFDM-based waveform is applied as in NR.

Another factor of MRSS is, though this aspect may be handled by other agenda item, handling of unremovable signals in 5G NR, e.g., SSB/TRS/SIBs/PRACH/etc. In our view, there are two possibilities in high-level:

- Alt 1: Signal sharing. For example, 6G UEs monitor 5G signals and uses them for 6G communication such as sync, RA, RRM, BM. This alternative is beneficial to achieve better spectrum efficiency, but whether this alternative is a reasonable choice or not needs further discussion in several aspects.

  - When the same signal design is undesirable/impossible, it may be difficult/complicated for 6G UEs to support both 5G signals-based access and 6G signals-based access. For example, it could be the case that PBCH contents are different between 5G NR and 6GR, and as a result, structure/resource sharing of SSB is not applicable in this case.

  - For signals where the same design is possible, this alternative will be one of the feasible candidates. For example, it may be considered that periodic CSI-RS structure/resource can be shared between 5G NR and 6GR. Having said that, this leads to problems, e.g., how to handle cell ID-based parameter for sequence generation, how to ensure CSI-RS-based sensing without any structure enhancement, etc.

- Alt 2: Rate-matching (similar to 4G/5G DSS). For example, the locations of 5G signals are indicated to 6G UEs, and 6G signals are mapped while avoiding overlap with the indicated REs. Although throughput performance is not the best, this alternative could make sense for any signals.

Observation 5:

- For MRSS support,

  - Whether signal structure/resource sharing between 5G NR and 6GR is applicable or not could be different among signal types.

  - Even for signals that are applicable without any change, it could not be desirable to preclude any other design, e.g., CSI-RS design for both communication and sensing.

Based on this analysis, we submit the following proposal.

Proposal 14:

- For MRSS support, study the following alternatives for handling of unremovable signals in 5G NR.

  - Alt 1: Signal sharing

  - Alt 2: Rate-matching (similar to 4G/5G DSS)

Proposal 15:

- For handling of unremovable signals in 5G NR in MRSS case, rate-matching is supported at least for some signal types.

  - FFS: whether signal sharing is feasible for each signal type, in consideration of various usages, how to reuse, etc.

  - Other features that could be kept as they are in NR

CP

We do not see any motivation to introduce new definitions of CP. Rather, necessity of ECP is questionable for us. ECP has not widely been used in actual 5G NW in our understanding, and furthermore, one possibility of 6GR is no support of 60 kHz SCS. Our suggestion is to focus on NCP only and to deprioritize ECP, unless any strong opinion of the practical necessity is found.

Proposal 16:

- Reuse at least NCP in 5G NR.

- Discuss practical necessities of ECP. Deprioritize ECP as long as the practical necessities are unclear.

DC subcarrier handling

DC subcarrier handling is probably a discussion topic in this agenda item, and just reuse of the 5G NR definition may be fine. It is noted that this is not our strong preference; if some kind of change is requested by BS/UE/chip vendors, we are open to discuss it.

Proposal 17:

- Reuse the same DC subcarrier handling in NR, if no reason to change the rule is found, i.e.,

  - For DL: TX DC subcarrier is indicated to UE; no RX-side behavior is defined

  - For UL: TX DC subcarrier (w/ or w/o 7.5 kHz offset) is reported to NW in RRC reconfiguration complete message if reporting is requested, and no TX-side behavior is defined

- Conclusion

In this contribution, we discussed frame structure for 6GR. Observations/Proposals are summarized as follows:

Proposal 1:

- Reuse numerology and frame structure in 5G NR as baseline, and focus on essential modifications only

Observation 1:

- 4.4 to 4.8 GHz, 7.125 to 8.4 GHz, 14.8 to 15.35 GHz are the candidate frequencies to be discussed in WRC-27.

  - FR3 discussion in 3GPP will cover both 7 to 8 GHz band and 15 GHz band.

Observation 2:

- In 5G NR, although multiple SCSs are available for each band, it seems that this flexibility has not been used effectively in real network.

Proposal 2:

- Down-select a single SCS per band or sub-FR or carrier type

  - E.g., 15 kHz SCS for FR1 FDD, 30 kHz SCS for FR1 TDD, 120 kHz SCS for FR2

  - FFS: SCS for FR3. E.g., 30 kHz SCS for lower FR3 and 120 kHz SCS for higher FR3

  - FFS: whether the definition is common or separate between TN and NTN

Proposal 3:

- Study which SCS is applied to FR3 from the following perspectives

  - BLER performance taking into consideration phase noise

  - # of different SCSs in case of inter-FR CA

Observation 3:

- For 64QAM with R = 666/1024, taking into consideration phase noise,

  - No/little difference of BLER performance is observed among 30/60/120 kHz SCSs with 100 or 200 MHz bandwidth

Observation 4:

- For 256QAM with R = 948/1024, taking into consideration phase noise,

  - Degradation of BLER performance is observed at 30 kHz SCS in comparison to 60/120 kHz SCSs, with 100 MHz bandwidth.

  - No significant degradation of BLER performance is observed at 60 kHz and 120 kHz, with 100 MHz bandwidth.

Proposal 4:

- Study which SCS is used for FR3, including the following alternatives:

  - Alt 1: 30 kHz SCS with PTRS

  - Alt 2: 60 kHz SCS or 120 kHz SCS

Proposal 5:

- For the maximum CBW,

  - FR1 especially in 6.425 to 7.125 GHz: Support at least 200MHz CBW

  - FR3 (the range between FR1 and FR2-1): Support at least 200MHz CBW

  - FR2-1: Support 400 MHz CBW

- Study how to support 200MHz CBW for FR1 and FR3

  - Alt 1: 8k FFT

  - Alt 2: 60 kHz SCS

Proposal 6:

- For the minimum CBW,

  - Study 3 – 5 MHz CBW at least for low FR1 bands with 15kHz SCS, considering

    - 6G IoT (low-tier device) support, including complexity reduction gain

    - Impact on the 6GR initial access design

Proposal 7:

- In 6GR, design BWP to support various UE types in the same band.

Proposal 8:

- Define BWP with the following details/restrictions.

  - Single SCS per BWP

  - More than one CORESET/Search space configurations with dynamic switching feature in a single BWP

  - Configuration per UE or per UE type

  - Only essential/relevant configurations under BWP configurations

  - No dynamic BWP switching, as long as dynamic CORESET/SS switching is available

  - Only initial BWP and a dedicated BWP for each UE or each UE type

    - No other BWP types such as default BWP and dormant BWP

Proposal 9:

- Support at least FDD and semi-static TDD.

- Deprioritize dynamic TDD.

- Discuss whether/how to support SBFD with considering its feasibility and realistic performance in practical deployments.

  - If supported, study the following two alternatives:

    - Alt 1: support semi-static SBFD only and not support dynamic SBFD

    - Alt 2: support semi-static SBFD and a simple SBFD/TDD switch w/o RRC reconfiguration.

Proposal 10:

- For time-domain resource grid, reuse the scalable frame structure with subframe-level boundary alignment as in NR.

  - 10 ms radio frame, 1 ms subframe, 2μ slots per subframe, 14 symbols per slot.

Proposal 11:

- For frequency-domain resource grid, study the following for UL coverage especially in NTN scenario:

  - Alt 1: 1 PRB with 12 SCs as in NR.

    - Alt 1-1: Allow sub-PRB-level resource allocation

  - Alt 2: 1 PRB with other than 12 SCs, e.g., 4 or 6 SCs

Proposal 12:

- For MRSS support, RAN1 to introduce only essential features in consideration of high NR flexibility.

Proposal 13:

- For MRSS support, no special handling of frame structure is expected.

  - As long as scalable/aligned frame structure with OFDM-based waveform is applied as in NR.

Observation 5:

- For MRSS support,

  - Whether signal structure/resource sharing between 5G NR and 6GR is applicable or not could be different among signal types.

  - Even for signals that are applicable without any change, it could not be desirable to preclude any other design, e.g., CSI-RS design for both communication and sensing.

Proposal 14:

- For MRSS support, study the following alternatives for handling of unremovable signals in 5G NR.

  - Alt 1: Signal sharing

  - Alt 2: Rate-matching (similar to 4G/5G DSS)

Proposal 15:

- For handling of unremovable signals in 5G NR in MRSS case, rate-matching is supported at least for some signal types.

  - FFS: whether signal sharing is feasible for each signal type, in consideration of various usages, how to reuse, etc.

Proposal 16:

- Reuse at least NCP in 5G NR.

- Discuss practical necessities of ECP. Deprioritize ECP as long as the practical necessities are unclear.

Proposal 17:

- Reuse the same DC subcarrier handling in NR, if no reason to change the rule is found, i.e.,

  - For DL: TX DC subcarrier is indicated to UE; no RX-side behavior is defined

  - For UL: TX DC subcarrier (w/ or w/o 7.5 kHz offset) is reported to NW in RRC reconfiguration complete message if reporting is requested, and no TX-side behavior is defined

References

- R1-2506304	Discussion on overview of 6GR air interface	NTT DOCOMO, INC.

Appendix

Table A-1 below shows the evaluation assumptions used in 2.1.2. SCS for FR3 link-level evaluation.

Table A-1: Simulation assumption for link-level evaluation

| Parameter | Value |
| --- | --- |
| Carrier frequency | 7 GHz/15 GHz (FR3) |
| Carrier bandwidth | 100/200 MHz |
| Subcarrier spacing (SCS) | 30/60/120 kHz |
| Number of RB (for 120/60/30 kHz SCS) | 66/132/273 (BW = 100 MHz) 132/264/540 (BW = 200 MHz) |
| Waveform | CP-OFDM |
| CP | NCP defined in 5G NR for all SCSs |
| Channel model | TDL-B (DS = 100 ns) |
| Antenna configuration | 4x4 |
| UE speed | 3 km/h |
| PN model | Example 2 in TR38.803 |
| DMRS configuration | 2 symbols (index 2, 11) |
| PTRS configuration | No |
| Channel code | LDPC |
| Channel estimation | MMSE |
| MCS | 64QAM (code rate = 666/1024) 256QAM (code rate = 948/1024) |