3GPP TSG RAN WG1 #122                                                                  R1-2506331

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:		8.3.1

Source: 		Google

Title:			Discussion on SBFD TX/RX/measurement procedures

Document for: 	Discussion and Decision

# Introduction

In this document, we provide our views on remaining issues of SBFD TX/RX/measurement procedures.

# Discussion

In clause 11.1 in TS 38.213, a general description for available symbol type (SBFD or non-SBFD) of transmission is introduced. However, we found that the content is duplicated with descriptions in clauses 5.1.2.1a and 6.1.2.1a in TS 38.214. Furthermore, there are missing parts in clause 11.1 in TS 38.213. For example, in the description when a UE is not provided with sbfd-Configuration2-Transmission, the first bullet includes two cases, first one is the PUCCH or PUSCH transmission with repetitions, and the second one is PDSCH reception with repetitions. But the multiple PUSCH and PDSCH transmissions/receptions scheduled by single DCI is missing. In this regard, we propose the following text proposal to further simplify the description in TS 38.213, and leave the detailed descriptions in TS 38.214.

Text proposal 1:

38.213 clause 11.1 /* Unrelated parts are omitted */ When the UE is provided sbfd-Configuration2-Transmission, the UE can - transmit a first PUCCH or PUSCH or a first repetition of a PUCCH or PUSCH, or receive a first PDSCH or a first repetition of a PDSCH, in non-SBFD symbols, and - transmit a second PUCCH or PUSCH or a second repetition of the PUCCH or PUSCH, or receive a second PDSCH or a second repetition of the PDSCH, in SBFD symbols When a UE is not provided sbfd-Configuration2-Transmission - available symbol types is determined according to clauses 5.1.2.1a and 6.1.2.1a of [6, TS 38.214] - /* Unrelated parts are omitted */

In clause 6.1.2.1a in TS 38.214, available symbol types for PUSCH transmission is addressed. However, the multiple PUSCH transmissions scheduled by single DCI is missing. Thus, we propose to have following text proposal for the missing part.

Text proposal 2:

/* Unrelated part are omitted */ #### 6.1.2.1a Resource allocation in time domain for SBFD For a UE scheduled with PUSCH transmission occasions across SBFD symbols and non-SBFD symbols in different slots, - if the UE is not configured with [sbfd-Configuration2-Transmission], - the UE transmits only the PUSCH in a valid symbol type; - For Type 1 PUSCH transmissions with a configured grant, the valid symbol type is provided by symbolType in rrc-ConfiguredUplinkGrant in ConfiguredGrantConfig. - For Type 2 PUSCH transmissions with a configured grant or PUSCH transmissions scheduled by DCI scrambled with SP-CSI-RNTI, the valid symbol type is the symbol type of the first PUSCH transmission occasion associated with activation DCI. For Type 2 PUSCH transmissions with a configured grant of PUSCH repetition type B, the valid symbol type is the symbol type of the first actual repetition associated with activation DCI. - For PUSCH transmissions scheduled by DCI format 0_1, 0_2, 0_3, 0_0 with CRC scrambled by TC-RNTI, RAR UL grant or fallbackRAR UL grant, the valid symbol type is the symbol type of the first PUSCH transmission occasion indicated by the scheduling DCI, the RAR UL grant or the fallbackRAR UL grant. For PUSCH repetition type B scheduled by DCI format 0_1 or 0_2, the valid symbol type is the symbol type of the first actual repetition occasion indicated by scheduling DCI. For PUSCH transmissions across different slots scheduled by a DCI using pusch-TimeDomainAllocationListForMultiPUSCH in which one or more rows contain multiple SLIVs, the valid symbol type is the symbol type of the first PUSCH transmission occasion indicated by the scheduling DCI. The UE does not expect that the first PUSCH transmission occasion indicated by scheduling DCI, the RAR UL grant or the fallbackRAR UL grant is mapped to both SBFD symbols and non-SBFD symbols, except for PUSCH repetition type B. /* Unrelated part are omitted */

# Conclusion

Text proposal 1:

38.213 clause 11.1 /* Unrelated parts are omitted */ When the UE is provided sbfd-Configuration2-Transmission, the UE can - transmit a first PUCCH or PUSCH or a first repetition of a PUCCH or PUSCH, or receive a first PDSCH or a first repetition of a PDSCH, in non-SBFD symbols, and - transmit a second PUCCH or PUSCH or a second repetition of the PUCCH or PUSCH, or receive a second PDSCH or a second repetition of the PDSCH, in SBFD symbols When a UE is not provided sbfd-Configuration2-Transmission - available symbol types is determined according to clauses 5.1.2.1a and 6.1.2.1a of [6, TS 38.214] - /* Unrelated parts are omitted */

Text proposal 2:

/* Unrelated part are omitted */ #### 6.1.2.1a Resource allocation in time domain for SBFD For a UE scheduled with PUSCH transmission occasions across SBFD symbols and non-SBFD symbols in different slots, - if the UE is not configured with [sbfd-Configuration2-Transmission], - the UE transmits only the PUSCH in a valid symbol type; - For Type 1 PUSCH transmissions with a configured grant, the valid symbol type is provided by symbolType in rrc-ConfiguredUplinkGrant in ConfiguredGrantConfig. - For Type 2 PUSCH transmissions with a configured grant or PUSCH transmissions scheduled by DCI scrambled with SP-CSI-RNTI, the valid symbol type is the symbol type of the first PUSCH transmission occasion associated with activation DCI. For Type 2 PUSCH transmissions with a configured grant of PUSCH repetition type B, the valid symbol type is the symbol type of the first actual repetition associated with activation DCI. - For PUSCH transmissions scheduled by DCI format 0_1, 0_2, 0_3, 0_0 with CRC scrambled by TC-RNTI, RAR UL grant or fallbackRAR UL grant, the valid symbol type is the symbol type of the first PUSCH transmission occasion indicated by the scheduling DCI, the RAR UL grant or the fallbackRAR UL grant. For PUSCH repetition type B scheduled by DCI format 0_1 or 0_2, the valid symbol type is the symbol type of the first actual repetition occasion indicated by scheduling DCI. For PUSCH transmissions across different slots scheduled by a DCI using pusch-TimeDomainAllocationListForMultiPUSCH in which one or more rows contain multiple SLIVs, the valid symbol type is the symbol type of the first PUSCH transmission occasion indicated by the scheduling DCI. The UE does not expect that the first PUSCH transmission occasion indicated by scheduling DCI, the RAR UL grant or the fallbackRAR UL grant is mapped to both SBFD symbols and non-SBFD symbols, except for PUSCH repetition type B. /* Unrelated part are omitted */

# References

- 3GPP TS 38.214 V19.0.0, June, 2025.

- 3GPP TS 38.213 V19.0.0, June, 2025.