3GPP TSG-RAN WG1 Meeting #122	R1-2506602

Bengaluru, India, August 25th – 29th 2025

Agenda Item:	11.5

Source:	Moderators (Ericsson, MediaTek)

Title:	Final Summary of 6GR Energy Efficiency Study

Document for:	Discussion, Decision

Introduction

This summary synthesizes observations and proposals from contributions submitted to 3GPP TSG RAN WG1 #122, focusing on energy efficiency for 6G Radio (6GR) in AI 11.5 and as described in the SID. The contributions address network energy savings (NES), user equipment power saving (UEPS), and joint NW-UE energy efficiency, emphasizing a holistic, day-one integration to overcome 5G’s limitations, such as backward compatibility constraints and fragmented feature deployment, etc.

Work plan

The energy efficiency work in the 6G Radio SI has been scheduled for three meetings and the objective is to come up with recommendations to consider for further studies in different agenda items. That means that after the present meeting, only two meetings remain. Rapporteur’s work plan for AI 11.5 Energy Efficiency is as follows [2]:

RAN1#122 (8 TU) - Energy efficiency - Identify candidate technologies for NW power saving, UE power saving, and joint mechanisms taking both NW and UE into account for power saving. RAN1#122bis (10 TU) - Energy efficiency - Continue identifying candidate technologies for NW power saving, UE power saving, and joint mechanisms taking both NW and UE into account for power saving. RAN1#123 (10 TU) - Energy efficiency - Complete identifying candidate technologies for NW power saving, UE power saving, and joint mechanisms taking both NW and UE into account for power saving, to be distributed to respective related agenda.

In FL’s understanding, the term candidate technology represents a deliverable for the initial energy efficiency phase of the 6G radio SI.

The FL’s detailed deconstruction of the work plan is to divide the work into the following parts:

RAN1 #122: Agree on topics for discussion (during 3 meeting EE phase) as candidate technologies for NW/UE/joint power savings (i.e., in other AIs). In parallel, start discussion on energy efficiency models, metrics and scenarios for future use.

RAN1 #122bis: Agree on candidate technologies for NW/UE/joint power savings, evolve/refine agreed topics for discussion, if needed, and agree on further topics for discussion. Evolve discussions on models, metrics and scenarios. Discuss and capture, if available, observations on issues and evaluation results.

RAN1 #123: Additional agreements on candidate technologies for NW/UE/joint power savings. Finalize models metrics and scenarios for evaluation of energy efficiency. Discuss and capture, if available, observations on issues and evaluation results to assist further studies in specific agenda items.

The following are example categories and features where companies can submit contributions related to 6G EE study. In case of any proposal not categorized below, please contact moderators for assistance.

| Example categories | Example features |
| --- | --- |
| SSB, SIB1 | Periodicity adaptation, on-demand operation including UL-WUS trigger, low-power transmission |
| RO/RACH and PO/Paging | RO and PO adaptation, preamble with large pool size, paging with fast UE identification |
| TRP on/off | Dynamic adaptation/On-off, |
| Multi-carrier/CA | SCell dormancy, fast SCell (de)activation, SSB-less SCell, UL-only/DL-only cell activation, overall procedure of multi-carrier operation |
| Cell DTX/DRX | On/off, adaptation, Joint op w/ UE cDRX |
| BS Power models & evaluation metric | QoS |
| Example categories | Example features |
| PDCCH monitoring and adaptation | cDRX, SSSG switching, PDCCH skipping, decoupling RF/BB PDSCH/PDCCH power states, PDCCH monitoring for multi-CC, restricting maximum coreset size, restricting minimum SS period |
| Wake-up mechanism | DCP, PEI, LP-WUS |
| Scheduling and data transmission | Cross-slot, UE UL DTX, UL skipping, sub-CC configuration, aggregated/efficient data transmission, wideband transmission with low PAPR, decoupling RF/BB power states, no late changes to UL, Autonomous PUSCH transmission, cDRX |
| RRM/RLM/BFD | RRM Relaxation, RLM/BFD Relaxation |
| UAI | DRX setting, MIMO layer #, #CC, Overheat |
| Idle/inactive optimization | idle TRS, inactive SDT |
| BWP, and other BW/rank adaptation | BW adaptation, MIMO layer adaptation, coarse supported BWP granularity |
| MIMO | Spatial domain/antenna adaptation in BS, NES CSI calculation in UE, |
| UE Power models & evaluation methodologies & evaluation metric | Methodology includes scenarios, baselines, etc. |

For the upcoming two RAN1 meetings, the moderators would like to suggest the following structure of contributions, which can help companies’ understanding on the identified issues and solutions related to 6G energy efficiency:

- BS and UE power consumption models (including reference configurations), energy efficiency metric(s) and baseline settings, and evaluation assumption updates, if any.

- Evaluation of baseline settings and show power consumption issues.

- Introduce enhancement techniques/solutions and provide the corresponding evaluation results on both BS and UE energy efficiency metrics to justify BS/UE/joint energy efficiency improvement.

- Discussion on other aspects including performance and complexity characteristics/trade-offs.

Considering there are only two meetings left, for the sake of efficiency, it is the moderators’ intention to proceed with the power model updates and enhancement techniques in parallel. Ideally, evaluations would be made with the updated BS and UE power consumption models in point 1 above. However, with only two meetings left, that may prove challenging. Moreover, it is the moderators’ view that enhancement techniques should show gains with existing power models. If that is not the case and before updated models have been agreed, it is up to companies to use their own models.

Discussion (Part 1)

Day 1 functionality

Companies’ views

Below is a composition of proposals relating to Day 1 requirements:

Nokia - R1-2505131 - Proposal 2: 6GR to consider NES-native design from the first release, with mandatory UE support of the corresponding energy-saving design and features. - Proposal 2: 6G should support cell DTX/DRX type of operation from day one to allow for sufficient BS sleep opportunities and achieve meaningful NES. - Proposal 3: The first 6G release should support a leaner carrier and signaling reduction (of SS/PBCH synchronization signals, system information, PRACH and paging occasions) including on-demand provisioning mechanisms for NES operations of cells operating under low/no load. - Proposal 4: The first 6GR release should support spatial adaptation (of Tx antenna ports/chains) and power adaptation, with enhanced CSI / SRS frameworks for NES operations. - Proposal 5: The first 6G release should support Cell DTX/DRX for all RRC states, including enhancements during Cell DTX/DRX inactive time and interactions with other features (such as spatial/power-domain adaptation). - Proposal 13: 3GPP specifies mandatory support for UE energy saving features from the first release of 6G. FUTUREWEI - R1-2505145 - Proposal 3: Adopt from day one 5G UE power saving techniques as baseline mechanisms in 6G, such as: - Duty-cycled based operations (iDRX, eDRX, cDRX), - LP-WUS with at least PEI and DCP functionality replacement, - Relaxed RRM measurements of neighboring cells, and - Relaxed/Offloading to LP-WUR of serving cell measurements. - Proposal 4: Adopt and further enhance from day one the gNB power saving solutions developed in 5G (e.g., 5G NES features SCell with on-demand SSB or no SSB, on-demand SIB1, Cell DTX/DRX, etc.). Huawei, HiSilicon - R1-2505187 - Proposal 1: 6GR ES design should take the following guidance principles: - Key ES techniques for network and UE should be considered in all the aspects of the system design for 6GR Day1 as mandatory requirement - including signal/waveform generation, initial access procedure, reference signal measurement/report, UL/DL control/data communication procedure, and UE state design etc. - Proposal 2: NES designs for single carrier deployment should be first studied in Day 1, in order to achieve comprehensive energy savings and provide future proof design for initial access, and NES for multi-carrier deployment can be studied later than single carrier since it will not impact initial access design. Ofinno - R1-2505677 - Proposal 4: 6GR should support cell DTX/DRX for PCell and SCell from day-1. Oppo - R1-2505761 - Proposal 1: 6GR energy efficiency design should consider the balance of network energy efficiency and UE efficiency. The design towards optimizing network energy efficiency should not significantly comprise UE energy efficiency and vice versa. - Proposal 2: A methodology for energy efficiency evaluation should be discussed and concluded in the earlier stage of 6GR study. - Proposal 3: For network energy efficiency, the 6GR should study the NES solutions that are applicable for coverage layer cells and idle UEs with high priority to ensure a good forward compatibility in future. - Proposal 4: For UE energy efficiency, the 6GR should consider energy efficient design for DL control channel and related procedure. UE energy efficiency should avoid duplication of design similar function, i.e., strive for a single power saving signal. TCL - R1-2505698 - Proposal 4: Discussion on how to include the impact of physical air interface design (e.g., waveform, coding, frame structure, sequence design, etc.) in 6G energy efficiency from day 1. - Proposal 7: Discuss potential power domain transmission technologies to improve sustainability in 6G day 1, at least including lower PAPR considerations, power control considerations, AI-assisted control and expanded deep sleep for both BS and UE PAs, adaptive emission masks and context-based EVM in RAN 1. - Proposal 8: Discuss whether/how to consider low-power signal design in 6G day 1 for 6G energy savings. Apple - R1-2505917 - Proposal 4: Reduction/adaptation of common signals/channels should be considered in 6G day-1 to avoid backward compatibility issue and fully achieve the NES benefit. - For OD-SIB1, how much additional NES gain can be obtained through SIB1 reduction in single cell case, under the assumption of increased SSB periodicity. - Proposal 5: Cell DTX/DRX should be considered in 6G day-1 to avoid backward compatibility issue and fully achieve the NES benefit. Lenovo - R1-2505995 - Proposal 1: 6GR should aim to reduce TCO by utilizing the non-backward compatible opportunity provided by once in a decade new generation refresh by designing native energy efficient solution using various techniques such as time, frequency, spatial, power domain. - Proposal 17: Study a unified device power saving mechanism using following techniques to support diverse device types from day-1 - Time domain technique - Frequency domain technique - Spatial domain technique - Measurement relaxations - Processing domain technique CMCC - R1-2506101 - Proposal 4: Support Cell DTX/DRX applies for both IDLE/CONNECTED mode, and more channels/signals from Day-1 in 6G. Vodafone, Bouygues Telecom, Deutsche Telekom - R1-2506134 - Proposal 3: Study introduction of LP-WUS/WUR for all device types in 6GR air interface as a day-1 considering potential impacts and benefits. InterDigital - R1-2506146 - Proposal 1: Support energy saving schemes for 6GR with following consideration: - Support of power saving features from 6G Day-1 - Always on signal with longer periodicity - Joint NW and UE energy saving - Study all energy saving domains - Flexible bandwidth adaptation - Proposal 10: Support LP-WUS targeting low power receiver capability from 6G Day-1. SK Telecom - R1-2506152 - Proposal 1: For 6G energy efficiency, at least the following aspects should be studied: - SSB/SIB1 transmission (longer periodicity, on-demand) - Enhanced BWP mechanism - Time-domain enhancement (UE-basis C-DRX vs. cell-basis DRX/DTX, LP-WUS/WUR) - Reduced RRM measurement - PEI AT&T - R1-2506237 - Proposal 1: Energy efficiency is leveraged across the 6G RAN design and supported as a Day 1 mandatory feature of 6GR. - Proposal 11: Energy Efficiency metric(s) are included as 6GR key performance metrics from day 1. IIT Kanpur - R1-2506392 - Proposal 2: 6GR Day 1 design, shall apply time/frequency adaptations of SS/PBCH transmission with more on-demand signal transmission and less always-on signals. Also on-demand SIB1 operation shall be supported from the Day 1 in 6G. - Proposal 4: 6GR Day 1 design shall enhance the low-power wake-up signal/receiver (LP-WUS/WUR) designs for improving UE power efficiency.

Summary

The Day 1 functionality proposals collectively address the critical need to integrate energy efficiency as a foundational principle in the initial 6G Radio (6GR) specification, overcoming the limitations of 5G NR where energy-saving features were introduced incrementally in later releases (e.g., Rel-16 to Rel-19), leading to fragmented adoption, optional implementation, and suboptimal effectiveness due to backward compatibility constraints. These proposals emphasize embedding energy-efficient mechanisms from Day 1, including:

- Network Energy Savings (NES): Proposals advocate for leaner carrier designs, reduced signaling (e.g., on-demand SS/PBCH, SIB1, PRACH, paging), and Cell DTX/DRX to maximize base station (gNB) sleep opportunities, addressing 5G’s inefficiencies where always-on signals (e.g., SS/PBCH every 20ms or SIB1 every 160ms) limited deep sleep modes, achieving 83.7% NES for Cat-1 and 52.5% for Cat-2 under low load (TR 38.864) (Nokia Prop. 3, FUTUREWEI Prop. 4, Apple Prop. 4, IIT Kanpur Prop. 2).

- UE Power Saving (UEPS): Mandatory UE energy-saving features, such as low-power wake-up signals/receivers (LP-WUS/WUR), duty-cycled operations (iDRX, eDRX, cDRX), relaxed RRM measurements, and unified power-saving mechanisms across time, frequency, spatial, and processing domains, are proposed to support diverse device types (e.g., IoT, XR/AR) from Day 1 (Nokia Prop. 13, FUTUREWEI Prop. 3, Lenovo Prop. 17, Vodafone Prop. 3, InterDigital Prop. 10).

- System Design: Proposals emphasize integrating energy efficiency into all aspects of 6GR design, including waveform, coding, frame structure, initial access, and power domain techniques (e.g., low-PAPR, adaptive emission masks, AI-assisted control), to ensure a sustainable air interface that balances performance, latency, and coverage (Huawei Prop. 1, TCL Prop. 4, Prop. 7, Prop. 8).

- Performance Metrics: Energy efficiency is proposed as a mandatory key performance indicator (KPI) from Day 1 to ensure evaluations reflect total system power consumption, avoiding 5G’s fragmented approach (AT&T Prop. 11).

- Avoiding Backward Compatibility Issues: By prioritizing single-carrier deployments and non-backward compatible designs, the proposals aim to embed energy-saving features natively, avoiding 5G’s challenges where features like PDCCH skipping and on-demand signaling were not universally adopted due to varying UE capabilities and complex network configurations (Huawei Prop. 2, Apple Prop. 4, Prop. 5).

These proposals collectively aim to create a lean, sustainable 6GR design that maximizes energy savings for both network and UE, reduces total cost of ownership (TCO), supports diverse use cases (e.g., IoT, NTN) in new spectrum (~7 GHz), and ensures energy efficiency is a core metric from the outset, enabling a more effective and unified approach compared to 5G’s incremental and constrained deployments.

1st round of FL comments and proposals

Many companies discuss the importance for Day 1 functionality for energy savings. In FL’s view, the whole purpose with the EE phase of the 6G Radio SI is to identify what such Day 1 functionality should be. Hence, there is little need to explicitly discuss this area further. Nevertheless, FL has one proposal related to Day 1 functionality, to get companies aligned regarding the ambitions with the EE phase.

FL Proposal 2.1-1:

RAN1 to strive for energy efficiency features that are mandatory from Day 1 to maximize energy gains.

| Company | View |
| --- | --- |
| Google | Yes, as companies views arranged by FL, we should avoid backward compatibility issues, which has been observed in 5G deployment. In such way, 6GR can benefit more from EE features. |
| InterDigital | Support |
| TCL | We supports this proposal, as making key energy-saving features mandatory from 6G Day-1 ensures uniform adoption across operators and devices for maximum network-wide gains. This addresses a major 5G gap (many energy efficiency features were optional, limiting impact). A Day-1 mandate would encourage early integration and optimization, benefiting both network energy costs and UE battery life. However, we stress that any mandatory features must be carefully vetted for feasibility and minimal added complexity, so as not to unduly burden device design or deployment. |
| Spreadtrum | We think the wording of “mandatory” is too strong. We would like to change the proposal as following: FL Proposal 2.1-2: RAN1 to strive for energy efficiency features that are critical for mandatory from Day 1 to maximize energy gains of UE and Network. |
| Panasonic | We support to have some mandatory EE features from Day 1. Just to clarify that, the intention is to discuss proposals/directions first and then to select some of the features and only to support them as mandatory. Or is the intention to assume from the beginning that only mandatory features would be discussed? |
| Qualcomm | In general, we don’t think the proposal is necessary. It is premature to discuss whether features are mandatory or not. At this state, we should follow Rapporteur’s work plan for AI 11.5 Energy Efficiency. If majority would like to make some conclusion, we suggest the following updated proposal: FL Proposal 2.1-3 (updated): RAN1 to strive for energy efficiency features that are mandatory from Day 1 to maximize energy gains. |
| Fujitsu | Agree in principle. Based on the online discussion, we believe there is a common understanding that maximizing energy saving gains should not be done recklessly, the trade-off between energy saving gains and negative impact on the NW and UE sides needs to be considered. Therefore, we would like to propose the following updates: RAN1 to strive for energy efficiency features that are mandatory from Day 1 to maximize energy gains without significantly negative impact on both NW and UE sides. |
| Fainity | Clarify the proposals are applied for all device types? Especially all RRC IDLE UE can enable the energy efficiency features without UE capability negotiation signaling? |
| Ofinno | Support |
| CEWiT | Support, some mandatory enhancements without backward compatibility issues, which has been a restriction in 5G NES enhancements. However, there should be scope for other enhancement which are critical but not mandatory for DAY 1. |
| Nokia | Support. Important to ensure broad support to energy efficiency features in 6G from the first release. |
| LG Electronics | Support |
| Sharp | Support |
| Huawei, HiSilicon | Support |
| DCM | Support. |
| CMCC | Support |
| CATT | We know the intention of this proposal, but we think this proposal may not be needed. |
| ETRI | Support. We prefer Qualcomm’s version. |
| NEC | Support |
| Xiaomi | We support to have some mandatory EE feautres from 6G Day1 in order to avoid non-compability issue and make EE techniques come to reality as soon as possible. However, as mentioned by lots of companies, balance betwteen performance and EE is very important. Keep this in mind, ‘to maximize energy gains’ is too strong and partial. The key is to achieve a common understanding that EE should happen from Day1. Hence, we propose the following modification of the proposal: FL Proposal 2.1-4: RAN1 to strive for energy efficiency features that are mandatory from Day 1 to maximize energy gains. |
| Ericsson | Support. It is important that the features we standardize gets implemented. |
| vivo | We support the direction generally. However, we need to select the energy efficiency feature set carefully consider the individual energy saving gain for each feature and the accumulative gain when multiple features are combined . |
| ZTE, Sanechips | Support in principle but need to clarify the maximum energy gain is targeting UE side or NW side or both sides. For example, for OOK based WUS, it has largest UE PS gain, does it mean this is our target? So, suggest to update it as follows RAN1 to strive for energy efficiency features that are mandatory from Day 1 to maximize energy gains. |
| Samsung | As a usual practice, we understand ‘mandatory’ is from UE perspective. However, what important is to make energy efficiency features both for UE and NW from 6G day 1 and to enable them as basic features as much as possible. |
| IIT Kanpur | We support the proposal. We also prefer to study other NES features that are not part of Day 1 6GR. |
| Apple | Understand the intention but we also do not see the need of the proposal as CATT. We do not think maximizing energy gains is the only goal we should strive for. Energy saving gains should be jointly considered with complexity, overhead, latency at both BS and UE. If a proposal is indeed needed, we propose to have a more general guideline as follows: FL Proposal 2.1-5(updated): RAN1 to strive for energy efficientcy design features that are mandatory from Day 1 to maximize energy gains. |
| Lenovo | Support , since it is a conclusion, addition from Fujitsu looks fine too. |
| Fraunhofer | We support the proposal. Some of the edits suggested above can be incorporated, e.g. the suggestion from Fujitsu to add ‘without significant negative impact on both NW and UE sides.’ |
| Tejas | Ok with the updated proposal from fujitsu. |
| OPPO | First, our proposals 1-4 on the general design for 6G in contribution R1-2505761 were not captured in FL summary. We appreciate if feature lead could add our views also in the summary. We agree with QC’s comment. At this stage is to good to go with the proposal suggested by QC. We don’t need to decide the features are mandatory at the moment. |
| Futurewei | OK in principle. The proposal in this formulation has little value. It would be a more valuable proposal if the word “strive” is replaced by “should define”. Suggested change: RAN1 to strive for should define energy efficiency features that are mandatory from Day 1 to maximize energy saving gains. |

2nd round of FL comments and proposals

FL can agree that this proposal may not strictly be needed but may be good that companies agree on the general direction of the energy efficiency work.

FL Proposal 2.1-6:

RAN1 to define energy efficiency features from Day 1 to maximize energy savings.

SSB requirements

Companies’ views

Below is a composition of proposals relating to SSB requirements:

Nokia - R1-2505131 - Proposal 3: The first 6G release should support a leaner carrier and signaling reduction (of SS/PBCH synchronization signals, system information, PRACH and paging occasions) including on-demand provisioning mechanisms for NES operations of cells operating under low/no load. - Proposal 6: 6G studies to consider trade-off between network energy saving and UE complexity for initial access, including relaxing the default SS/PBCH periodicity. - Proposal 7: On-demand reference signal, e.g. SS/PBCH, operation shall be studied in 6G. - Proposal 8: For 6G design with SS/PBCH-less SCell operation, it is proposed to consider more flexible and scalable solutions that can fit in with different deployment scenarios. FUTUREWEI - R1-2505145 - Proposal 4: Adopt and further enhance from day one the gNB power saving solutions developed in 5G (e.g., 5G NES features SCell with on-demand SSB or no SSB, on-demand SIB1, Cell DTX/DRX, etc.). CATT - R1-2505297 - Proposal 3: In order to reduce the initial access delay due to sparse SSB, the solution of increasing the detection probability of SSB in one period of SSB burst set for initial cell selection should be studied, i.e., the UE can detect the PSS/SSS and decode the PBCH successfully within only one period of SSB burst set. - Proposal 4: To increase the detection probability of SSB in one period of SSB burst set for initial cell selection, SSB repetition with one SSB period, PBCH repetition within one SSB or PBCH with a lower coding rate should be studied. - Proposal 5: In 6GR, both network-triggered on-demand SSB and UE-triggered on-demand SSB for UEs in idle mode and connected mode should be introduced. Spreadtrum (UNISOC) - R1-2505176 - Proposal 5: The following technologies can be studied for 6GR joint NW&UE power saving: - Reduce always-on signal and improve one-shot detecting performance of common signal/channel. Xiaomi - R1-2505467 - Proposal 7: Default SSB periodicity extension and/or sparse synchronization raster should be considered for 6GR with taking UE requirement into account. - Proposal 8: Enhancements on RO adaptation and joint adaptation between RO and SSB can be considered in 6G to further reduce energy consumption. - Proposal 9: Joint condensed common signal/channel design and L1-based paging adaptation can be considered for 6GR NES. Samsung - R1-2505589 - Proposal 1: Extend ‘Default’ SSB transmission periodicity longer than that of NR where a UE assumes the default value for initial cell selection. - Proposal 2: Set Day 1 SSB design principle accommodating coverage, diverse devise type, and energy efficiency. ZTE - R1-2505607 - Proposal 23: Two stage SSB can be considered for UE and NW energy saving. Ericsson - R1-2505625 - Proposal 3: 6GR should support default SSB periodicity of 160 ms to enable network deep sleep states while maintaining acceptable UE Idle mode performance. - Proposal 5: Study periodic bursts of SSB and SIB1 repetitions, where in each period multiple SSB and SIB1 repetitions are provided during a short time interval followed by a longer inactive interval. - Proposal 6: 6GR should support coordinating system information broadcast with other common signals/channels in order not to interrupt sleeping opportunities. - Proposal 7: Study a design with two GSCN raster subsets: a primary raster where UE assumes extended SSB periodicity and a secondary raster where UE assumes 20 ms SSB periodicity. - Proposal 8: Study enhancements of on-demand SSB to extend its applicability. - Proposal 10: SSB-less SCells operation should be included as baseline functionality in 6GR. Tejas Networks Ltd. - R1-2505631 - Proposal 19: Enhancements to the SSB RO mapping and RAR can be studied to reduce the overhead of indicating SSBs to the base station and reduce the RAR overhead associated with each RO. Ofinno - R1-2505677 - Proposal 6: 6GR should support OD-SSB and RAN1 to study cases where OD-SSB can be supported (e.g., PCell, SCell, on/off synch raster). Oppo - R1-2505761 - Proposal 10: The following studies can be considered to reduce the UE power consumption towards 6GR cell search - to design a smaller SSB bandwidth, resulting in wider step between sync raster points, - to study a two-step cell search procedure: UE first searches high priority sync raster points and gets prior information for a next step cell search for initial access. - Proposal 12: 6GR can consider the following candidate schemes - introduce large default SS periodicity for coverage layer cells  the large default SS periodicity can be considered to be limited to a part of the sync raster points - explore more advanced and flexible SS periodicity adaptation strategies - expand the applicability of on-demand SS Quectel - R1-2505769 - Proposal 1: The OD-SSB/OD-SIB1 structure simplifying SSB/SIB1 needs discussion in 6G. Panasonic - R1-2505789 - Proposal 1: To study synchronization signal design with options to be on-demand and adaptive in proper use cases. Fraunhofer IIS, Fraunhofer HHI - R1-2505834 - Proposal 5: The 6GR study should investigate the possibility of increasing common signal periodicity and enable on demand common signals. - Proposal 8: The 6GR study should investigate the possibility of limiting the search area for synchronization signals so the UE does not have to search for the entire frequency grid. LG Electronics - R1-2505858 - Proposal 1: Study the default periodicity of synchronization signal (SS) larger than 20 msec. - Proposal 2: Study on-demand SS and/or PBCH procedure and how to utilize SS/PBCH with adaptation for measurement. Apple - R1-2505917 - Proposal 4: Reduction/adaptation of common signals/channels should be considered in 6G day-1 to avoid backward compatibility issue and fully achieve the NES benefit. - For SSB, study to introduce two types of SSBs, one is an always-on signal and designed for RRC_IDLE UEs and the other is on-demand SSB for RRC_CONNECTED UEs. Fujitsu - R1-2505972 - Proposal 3: Study the methods to enable on-demand transmission of cell common signals, such as SS, PBCH and SIB1, without limitations on applicable scenarios. - The above aspects can be included in the initial access related discussions. CAICT - R1-2506005 - Proposal 1: the coverage and the capacity carrier can be defined to enable dynamic on/off carrier, the SSB signal with long periodicity and on demand SSB can be transmitted in the capacity carrier ,and the always on RS can be transmitted in the coverage carrier. Sharp - R1-2506014 - Proposal 2: To reduce unnecessary power consumption and signalling overhead, on demand reference signal in Pcell should be supported. CMCC - R1-2506101 - Proposal 2: RAN1 to further consider and study the following case for single carrier scenario in 6GR: - Case 1: no “6G SSB” transmitted on a carrier by default, and UE triggers “6G SSB” transmission (and other common channels/signals if needed) by demand: - UE can pre-receives or pre-configures the uplink wake-up signal (UL-WUS) configuration, and transmits UL-WUS on the carrier with the assist of pre-stored information or other information (e.g. GNSS). - 6G BS can turn off TX part while enabling RX part for UL-WUS reception (e.g. sliding window detection). - Case 2: always-on “6G SSB” transmitted on a carrier with large periodicity by default (e.g. 160ms), and UE on-demand triggers short period SS transmission (and other common channels/signals if needed): - The always-on “6G SSB” is used for cell detection, basic sync and provide necessary info (e.g. UL-WUS configuration), while the on-demand short period SS can be used for finer sync or fast measurement. - Proposal 3: RAN1 to further consider and study the following case for multi-carrier scenario in 6GR: - The common signal transmission/reception procedure (e.g. SIB1 transmission, RACH reception) for multiple carriers can converge to one anchor carrier. Therefore, other carrier (i.e. NES carrier) can turning off or only transmit long period SS by default, so as to obtain more NES gain. - NES carrier can be activated per NW guidance or UE demand and UE can initiate access on NES carrier, so as to achieve better UE experience or load balancing for network. ETRI - R1-2506069 - Proposal 9: Study the definition of a default SSB periodicity substantially longer than 20 ms (e.g., 160 ms). - Proposal 10: Study increasing the SSB resource size to enable one-shot SSB detection. - Proposal 12: Study the use of on-demand SSB for several use cases. InterDigital - R1-2506146 - Proposal 5: Minimize the need to configure always-on signals (e.g., only present with long periodicities of at least 160 ms). Support configurations that do not require provisioning of always-on signals on all carriers. - Proposal 6: Support on-demand signals/channels (e.g., configurable, dynamic (de)activation, and/or UE-requested) where it is applicable. SK Telecom - R1-2506152 - Proposal 1: For 6G energy efficiency, at least the following aspects should be studied: - SSB/SIB1 transmission (longer periodicity, on-demand) Qualcomm - R1-2506222 - Proposal 16: Sync signal design should consider both NW energy saving, UE complexity and user experience. AT&T - R1-2506237 - Proposal 16: Derive an updated 6GR initial access procedure with SSB periodicity extended beyond 20ms, followed by evaluation of the performance and the underlying energy savings. - Proposal 17: Study the pros and cons of the UE monitoring relatively fewer frequency raster points during the prospective 6GR initial access procedure. NTT DOCOMO - R1-2506310 - Proposal 4: Study the following directions to achieve better energy efficiency than NR for initial access procedure: - Study placing sync-raster on specific band - Study longer periodicity from NR such as 40 ms, 80 ms - To achieve the above, study reducing the number of defined sync raster, thereby mitigating delays and UE burden through initial cell search. - Coarser sync raster locations - Limiting bands with sync raster by the specification - Proposal 5: Study a mechanism to provide OD-RS transmission dynamically with less signalling overhead on PCell. - Consider at least NW triggering mechanism, and UE triggering including the necessity and its criteria. - Study dense RS transmission within one periodicity and use of each RS as a measurement sample for RRM - Assumption: Static AO-SSB is transmitted with a long periodicity (e.g., ~160ms). WILUS Inc. - R1-2506324 - Proposal 2: Study On-Demand SSB/SSB1 for 6GR - Enable on-demand SSB/SIB1 transmission for UEs in Idle, Inactive, or RRC_Connected modes to maximize energy savings and deep-sleep opportunities for gNBs. - Proposal 3: Study Extended SSB Periodicity for 6GR - Introduce longer SSB periodicities to enable deeper gNB sleep modes, leading to substantial energy savings. Rakuten Mobile, Inc. - R1-2506346 - Proposal 1.1: RAN1 to define a study topic under the 6GR SID to evaluate the Beacon/Anchor/Data carrier architecture, including much higher sync signal periodicity, carrier roles, and SSB/SIB transmission strategies. - Proposal 1.2: RAN1 to study implications of, SIB1 and paging transmitted by Anchor Carriers on demand or at ultra-low periodicity and Data Carriers remain dormant until scheduled user activity is detected. CEWiT - R1-2506363 - Proposal 1: 6G should support energy efficiency enhancements for common signals including - On-Demand Signals for initial access including OD-SSB & OD-SIB1 a. Simplified SSB - SSB periodicity extension beyond 20ms. IIT Kanpur - R1-2506392 - Proposal 2: 6GR Day 1 design, shall apply time/frequency adaptations of SS/PBCH transmission with more on-demand signal transmission and less always-on signals. Also on-demand SIB1 operation shall be supported from the Day 1 in 6G.

Summary

The above proposals collectively address the challenge of reducing energy consumption in 6G Radio (6GR) networks by minimizing the overhead of always-on Synchronization Signal/Physical Broadcast Channel (SS/PBCH, or SSB) transmissions, which contribute to network energy inefficiency in 5G NR, particularly under low or no traffic conditions. In 5G, the default 20 ms SSB periodicity prevents base stations (gNBs) from entering deep sleep modes, leading to substantial energy usage (e.g., limiting energy savings to less than 84.8% as per TR 38.864) (Ericsson Obs. 1, Samsung Obs. 1, MediaTek Obs. 1). The proposals advocate for solutions such as extending SSB periodicity (e.g., to 40 ms, 80 ms, or 160 ms), cf. Figure 1. When needed, on-demand SSB (OD-SSB) may be provided, triggered by either the network or a UE. Additionally, SSB-less SCell operations may be used in certain deployments, allowing deeper network sleep states and reduce UE complexity.

Figure 1: Zero-traffic energy consumption with increased SSB periodicity [13].

Additional enhancements include sparser sync rasters, SSB repetition, two-stage SSB designs, and simplified SSB structures to reduce UE search latency and complexity while maintaining coverage, particularly for new spectrum (~7 GHz) and diverse device types (Spreadtrum Prop. 5, NTT DOCOMO Prop. 4, Samsung Prop. 2). These proposals aim to overcome 5G’s backward compatibility constraints, which restricted on-demand SSB to SCells or non-standalone scenarios, ensuring a leaner, energy-efficient 6GR design from Day 1 that balances network energy savings (NES) with UE accessibility and user experience (Ofinno Prop. 6, Sharp Prop. 2, Qualcomm Prop. 16).

1st round FL comments and proposals

It is the FL’s understanding that companies have very different views on how an increased SSB periodicity would work. No company propose to only increase SSB to 160 ms and do nothing else to compensate the resulting decreased UE performance. For that reason, FL proposes that RAN1 focuses on what mitigation techniques would be useful to achieve 160 ms SSB periodicity with minimal impact on the UE performance.

Some companies object to the long SSB periodicity and instead propose a LP radio. In FL’s view, there is not necessarily any contradiction between LP radio and 160 ms (AO-)SSBs if these are combined with 20 ms OD-SSBs. In that case, the LP Radio would still have the same power performance as 20 ms (AO-)SSBs.

FL Proposal 2.2-7:

Study NW energy saving from increasing the default periodicity of cell-defining SSB on synchronization raster. Additionally, study UE performance impact and mechanisms to mitigate UE performance degradations in important use-cases, considering:

- SBB types (always-on SSB, on-demand SSB),

- SSB periodicity(ies),

- Synchronization raster granularity, incl. prioritized raster points,

- SSB detection performance,

- SCell operation,

- Etc.

Companies are welcome to share their views on the above FL proposal.

| Company | View |
| --- | --- |
| Google | On UE performance impact, we understand it at least includes latency of accessing a cell. In addition to UE performance impact, the NES on CD-SSB in 6GR should also consider UE complexity, e.g., detection number increasing with the larger default SSB period. |
| InterDigital | Generally fine with the intention of the proposal. But, we have some comments on the original proposal. 1. Increasing the default periodicity is not the only issue which needs to be considered. In addition, we don’t need to restrict the consideration on cell defining SSB or SSBs on sync raster. 2. For sync raster granularity, we do not want to highlight one specific solution (i.e., the prioritized raster points). We believe that it would be better to have a generalized study area on sync raster granularity/location rather than pointing out one specific solution. 3. In our view, sync signals, MIB and PBCH may have different performance requirements. Having said that, in the current stage, we believe that it would be better to generalize the discussion. 4. As this discussion is triggered for IDLE modes, we prefer to focus on PCell operation in this discussion. FL Proposal 2.2-8: Study NW energy saving aspects considering from increasing the default periodicity of cell-defining SSB on synchronization raster. Additionally, study UE performance impact and mechanisms to mitigate UE performance degradations in important use-cases, considering: - SBB types (e.g. always-on SSB, on-demand SSB), - SSB periodicity(ies), - Synchronization raster granularity/location, incl. prioritized raster points, - SSB dDetection performance of sync signals, MIB and PBCH, - SCell operation, - Etc. |
| TCL | Three comments, thanks - We agree that studying a longer default SSB periodicity (beyond the current ~20 ms, e.g. up to 160 ms) could unlock significant base station sleep periods, boosting network energy savings. However, this must be balanced against UE impacts–infrequent SSBs may slow initial cell detection and neighbor measurements or require UEs to wait longer, affecting latency and coverage. Any periodicity extension should be tuned so idle UEs can still efficiently find cells without excessive scanning energy or degraded user experience. - New SSB structure for 6G would be considered for power saving and low complexity, which would be more simplified and more flexible than 5G SSB. Thus, we suggest this issue should consider the impact of 6G SSB structure. - Beam sweeping is an important functionality of SSB. 6G may support narrower beams and a greater number of beams. When we study SSB, high-efficiency beam detection/sweeping could be considered. |
| Spreadtrum | In our view, SSB transmission adaptation and SSB structure/pattern also should considered. In addition, Scell operation only exists in RRC connected CA scenario. We prefer change “Scell operation” to “Multi-carriers operation” We prefer to modify this proposal into the following version. FL Proposal 2.2-9: Study NW energy saving from increasing the default periodicity of cell-defining SSB on synchronization raster. Additionally, study UE performance impact and mechanisms to mitigate UE performance degradations in important use-cases, considering: - SBB types (e.g., always-on SSB, on-demand SSB), - SSB transmission adaptation - SSB structure/pattern - SSB periodicity(ies), - Synchronization raster granularity, incl. prioritized raster points, - SSB detection performance, - Multi-carriers operation, SCell operation, - Etc. |
| Panasonic | We are supportive in general. First bullet has a typo of “SBB”. However, as the detailed design of SS, PBCH and so on has not started yet for 6GR, the usage of the term SSB is not so clear. We propose adding one bullet to clarify that: - Detailed design of the synchronization signal and PBCH is FFS. Also, as this is IDLE mode session, the bullet of SCell operation should be modified to: SCell operation multi-carrier operation. |
| Qualcomm | It is unclear whether cell-defining or non-cell-defining SSB needs to be defined for 6G yet. Per our understanding, this proposal concerns the SSB periodicity assumed for initial cell selection. This SSB should be always-on SSB. In addition to UE performance, it is critical to understand the impact of new default periodicity of SSB to cell search latency, UE’s complexity and power consumption. Furthermore, the sub-bullets are not necessary. FL Proposal 2.2-10 (updated): Study NW energy saving from increasing the default for different periodicity values of cell-defining SSB on synchronization raster SSB for initial cell selection. Additionally, study impacts on cell search latency/performance and UE’s complexity/power consumption/ performance impact and mechanisms to mitigate the impacts UE performance degradations in important use-cases, considering: - SBB types (always-on SSB, on-demand SSB), - SSB periodicity(ies), - Synchronization raster granularity, incl. prioritized raster points, - SSB detection performance, - SCell operation, Etc. |
| Fujitsu | In our understanding, SCell operation should be categorized to the connected mode. Additionally, the SSB transmitted on the SCell is not limited to sync rater. Thus, we suggest to remove SCell operation: Study NW energy saving from increasing the default periodicity of cell-defining SSB on synchronization raster. Additionally, study UE performance impact and mechanisms to mitigate UE performance degradations in important use-cases, considering: - SBB types (always-on SSB, on-demand SSB), - SSB periodicity(ies), - Synchronization raster granularity, incl. prioritized raster points, - SSB detection performance, - SCell operation, Etc. |
| Fainity | The bandwidth of SSB is suggested to take into account as well. In addition, Scell operation should be removed since the scope here is for Idle mode. |
| Ofinno | Support in general. Pefer to keep high level and remove “incl. priorizted rater points” or call whole bullet “sync raster prioritization. Two comments: - On SCell operation we are okay to include but for clarification are we studying SCell for Idle mode? - We think that low power SS and/or 2 stage SSB could also be included in the list of techniques to consider on the UE impact. |
| CEWiT | SSB structure and transmissions pattern need to be redefined for 6G. For e.g., simplified SSB or enhancements in transmissions patterns along with SSB periodicity extension can be studied which may improve SSB detection performance at the UE. Further, the Scell operation is intended for connected mode UEs, hence the enhancements should be both for single cell and multi cell operations especially for PCell. Hence we suggest following update in the proposal. FL Proposal 2.2-11: Study NW energy saving from increasing the default periodicity of cell-defining SSB on synchronization raster. Additionally, study UE performance impact and mechanisms to mitigate UE performance degradations in important use-cases, considering: - SBB types (e.g., always-on SSB, on-demand SSB), - SSB structure/pattern - SSB periodicity(ies), - enhancements to synchronization incl. raster granularity, incl. prioritized raster points, etc - SSB detection performance, - Single/Multi-carriers operation, SCell operation, Etc. |
| Nokia | Support in principle. The (intention) under 11.1 is to study the aspect related to synch signal structure, thus we could generalize to study the types of synchronization signals needed (always-on, on-demand etc.), periodicity etc. Also whether to consider the synchronization detection performance in this AI, 11.1 or 11.7 could be considered. |
| LG Electronics | The main bullet of the proposal looks good to us. However, we have several comments for several sub-bullets. - “SCell operation” seems to be contradictory to the main bullet where SSB is cell-defining and located on sync raster. Unless clarified, we prefer removing “SCell operation” - “SSB periodicity(ies)” can be modified to “Periodicity(ies) of SS and/or PBCH” considering the possibility of different periodicities of SS and PBCH. - Typo in the first sub-bullet (i.e., SBB SSB) With that, our suggested modification is as follows. Study NW energy saving from increasing the default periodicity of cell-defining SSB on synchronization raster. Additionally, study UE performance impact and mechanisms to mitigate UE performance degradations in important use-cases, considering: - SSBB types (always-on SSB, on-demand SSB), - SSB pPeriodicity(ies) of SS and/or PBCH, - Synchronization raster granularity, incl. prioritized raster points, - SSB detection performance, - SCell operation, - Etc. |
| Sharp | Support in general. For the purpose of NW energy saving, how to minimize the number of SSBs (periodicity/On-demand/etc.) in the time domain while still satisfying UE performance requirements should be studied. From this perspective, the comparison between SSB detection performance and the defined requirements should serve as the key metric for evaluating any proposed approach. Furthermore, as 6GR initial access procedure is not yet determined, whether ‘SSB’ will still be used in 6GR is not clear. Thus, we suggest change ‘SSB’ to ‘SS/PBCH synchronization signals’, which is in Nokia’s proposal. |
| Huawei, HiSilicon | The starting point of the SSB discussion should be to identify what aspects could be enhanced to achieve all potential of NW energy savings and study impact of the corresponding techniques. For that reason, we believe it should be FL Proposal 2.2-12-Huawei update: Study NW energy saving for SSB transmission from various domains for different procedures/functions, and UE performance impact, including at least - Time domain: increasing the default periodicity of SSB beyond 20ms - Spatial domain: reducing the TRxUs - Power domain: lower PAPR signal generation - Frequency domain: reduced transmission BW - Necessary signaling provision for e.g. on-demand SSB - At least initial access (including e.g. synch. Raster design), RRM measurement procedure, LP-WUS procedure for IDLE UEs should be considered - UE performance impact includes at least access latency, coverage, sync. accuracy, UE power consumption increasing the default periodicity of cell-defining SSB on synchronization raster. Additionally, study UE performance impact and mechanisms to mitigate UE performance degradations in important use-cases, considering: - SBB types (always-on SSB, on-demand SSB), - SSB periodicity(ies), - Synchronization raster granularity, incl. prioritized raster points, - SSB detection performance, - SCell operation, - Etc. |
| DCM | Generally ok. Meanwhile, it isn’t clear what needs to be captured in the list of bullets. We first saw it as aspects for related features, while after seeing Google comments, seems like it is for capturing KPI to assess e.g., cons of any change for SSB. |
| CMCC | We generally fine with this proposal but since this agenda is for idle mode, it is more appropriate to replace SCell operations to multi-carrier/multi-TRP operation. Therefore, we suggest the following revisions: FL Proposal 2.2-13-CMCC rev1: Study NW energy saving from increasing the default periodicity of cell-defining SSB on synchronization raster. Additionally, study UE performance impact and mechanisms to mitigate UE performance degradations in important use-cases, considering: - SSBB types (always-on SSB, on-demand SSB), - SSB periodicity(ies), - Synchronization raster granularity, incl. prioritized raster points, - SSB detection performance, - Multi-carrier/multi-TRP SCell operation, Etc. |
| CATT | Support in principle. First, in the main bullet, whether cell-defining or non-cell-defining SSB should not be restricted. Seonce in the sync raster sub-bullet, the ‘incl. prioritized raster points ’ should be delated to make it more general. |
| ETRI | Support the proposal in general. SCell operation can be discussed separately from the idle mode/initial access procedure. |
| NEC | We support this proposal. There is a broad consensus that extending the default SSB periodicity beyond 20 ms (e.g., to 160 ms) is a critical enabler for network deep sleep and offers significant NES gains. However, it is equally important to study the potential negative impacts on UE performance, such as increased access latency and complexity. Therefore, we agree that this study must be coupled with an investigation of mitigation techniques, such as one-shot SSB detection, sparser synchronization rasters, and on-demand SSB, to ensure a balanced solution that benefits both the network and the UE. |
| Xiaomi | We are generally fine with the proposal. It is a good starting point for discussion but include too many details. From our understanding, the first step should try to agree on SSB periodicity extension with taking UE performance into account. For the detailed SSB design, it should be handled under SSB agenda. Regarding to Scell operation, it is a bit confusing and can be removed considering it is already covered by ‚etc‘. FL Proposal 2.2-14: Study NW energy saving from increasing the default periodicity of cell-defining SSB on synchronization raster. Additionally, study UE performance impact and mechanisms to mitigate UE performance degradations in important use-cases, considering the following aspects can be starting point for SSB design: - SBB types (always-on SSB, on-demand SSB), - SSB periodicity(ies), - SSB adaptation - Synchronization raster granularity, incl. prioritized raster points, - SSB detection performance, - SCell operation, - Etc. Other mechanisms are not precluded depending on SSB discussion |
| Ericsson | Support. SCell operation may fit better in the CONNECTED mode discussions. |
| vivo | Before study of the mechanism for increasing SSB period, we need to determine which Cat of NW power model is the baseline for 6G study since it will have large impact on the NES gain of increasing SSB periods. Based on a single baseline Cat, NES gain of increasing SSB period should be evaluated to see whether it is deserved for further study. So we suggest the updated proposal: Study and evaluate NW energy saving from increasing the default periodicity of cell-defining SSB on synchronization raster with respect to NW energy saving gain and UE impact (e.g., frequency scan and cell search latency, access latency, processing complexity, power consumption and etc.). Additionally, study UE performance impact and mechanisms to mitigate UE performance degradations in important use-cases, considering: - SBB types (always-on SSB, on-demand SSB), - SSB periodicity(ies), - Synchronization raster granularity, incl. prioritized raster points, - SSB detection performance, - SCell operation, Etc. |
| ZTE, Sanechips | We support the intention of the proposal and we agree to introduce the larger default SSB periodicity (e.g., 160ms). While, some updates with red are suggested with following reasons: - Important use cases are not clear - SSB type definition currently is not clear, we can set always-on SSB, on-demand SSB as examples for information. - Regarding SCell operation, CMCC’s update is more clear to us. - SSB structure, e.g., PSS, SSS, and PBCH decoupling, new SSB structure with better performance, also could be used to mitigate the impacts on UE side. - SSB adaptation also could be considered to mitigate the UE impacts as needed. FL Proposal 2.2-15: Study NW energy saving from increasing the default periodicity of cell-defining SSB on synchronization raster. Additionally, study UE performance impact and mechanisms to mitigate UE performance degradations in important use-cases, considering: - SBB types (e.g., always-on SSB, on-demand SSB), - SSB periodicity(ies), - Synchronization raster granularity, incl. prioritized raster points, - SSB detection performance, - Multi-carrier/multi-TRP SCell operation, - SSB structure, - SSB adaptation, - Etc. |
| Samsung | We suggest the following changes to the proposal for clarity: Study NW energy saving from increasing the default periodicity of cell-defining SSB on synchronization raster for initial cell search. Additionally, study UE performance impact and potential mechanisms to mitigate UE performance degradations in important use-cases, considering: - SBSB types (e.g., always-on SSB, on-demand SSB), - SSB periodicity(ies), - Synchronization raster interval granularity, incl. prioritized raster points, - SSB detection performance, - SCell operation, - SSB structure(s), - SSB pattern, - Etc. |
| IIT Kanpur | Support the proposal. There is a broad consensus to support increased periodicity SSB for significant NES gains. We are propose to include the design of light SSB in this proposal. In addition the NES techniques should be jointly studied with UE energy saving techniques such as improved SSB detection, sparser/prioritized synch raster points. |
| Apple | We believe that any study evaluating network energy savings from increased SSB periodicity should holistically address the associated negative impact at UE—such as increased UE initial cell search complexity, larger memory demands, power consumption, and cell access latency—rather than examining these factors in isolation or focusing solely on UE performance impact. Moreover, any proposal to adopt higher SSB periodicity values should be accompanied by mitigation strategies to manage the resulting increase in UE complexity. More importantly, beyond its impact on initial access operations, increased SSB periodicity directly affects other functional blocks in RRC_IDLE—such as SIB monitoring and paging reception. A higher SSB periodicity often necessitates more frequent SSB monitoring to compensate for TO/FO drift. These procedures, performed periodically for RRC_IDLE UEs, are just as critical as the initial cell search process. We therefore suggest the following modification: Study the NW energy saving and the associated impact on UE including increased cell search complexity, higher memory requirement, larger power consumption for SIB/Paging reception and potential performance degradation (e.g., cell search latency) from increasing the default periodicity of cell-defining SSB on synchronization raster. Additionally, study UE performance impact and mechanisms to mitigate these impacts at UE side. UE performance degradations in important use-cases, considering: - SBB types (always-on SSB, on-demand SSB), - SSB periodicity(ies), - Synchronization raster granularity, incl. prioritized raster points, - SSB detection performance, - SCell operation, - Etc. |
| Lenovo | Our preference is to keep the bullet at high level without going into details of the schemes which can be discussed later in the respective initial access agenda item Study NW energy savings from increasing the default periodicity for the initial cell search on the synchronization raster and study UE performance impact and mechanisms to mitigate UE performance degradations. |
| Fraunhofer | We support this proposal. Mechanisms to mitigate the impact of NW energy saving (NES) techniques on UE and network performance are key to enhance the scope of maximizing NES gains. In this case, specifically, sparse SS/PBCH transmission for NES (achieved via increased periodicity and/or on-demand transmission) should ensure faster synchronization and cell search mechanisms in the design for given sparseness compared to 5G. We agree with the modifications suggested by LGE and Samsung. |
| Tejas | Our preference is to study different deployment scenarios particularly non standalone case deployment and we prefer to study simplified SSB, such as SSB with partial PBCH and partial MIB in non-standalone. Hence, we suggest the following modification. Study NW energy saving by adapting the SSB on or off the synchronisation. from increasing the default periodicity of cell-defining SSB on synchronization raster. Additionally, study UE performance impact and mechanisms to mitigate UE performance degradations in important use-cases, considering one or more of the following: - SBB types (always-on SSB, on-demand SSB, CD-SSB, NCD-SSB), - SSB periodicity(ies), - NEW SSB structure/pattern. - Synchronization raster granularity, incl. prioritized raster points, - SSB detection performance, - Deployment scenarios (Standalone deployment, non-standalone deployment, single carrier, multicarrier). - SSB with partial PBCH and/or partial MIB. - SCell operation, Etc. |
| OPPO | Our proposal 10 and proposal 12 in R1-2505761 provided our views 6GR design for SSB. But these were not captured in FL summary. We appreciate if feature lead could add our proposal to the summary. Regarding the proposal, we have the following comments: - we think listing the bullets is good for the study guideline. But we think that different aspects should be put at the same level for the study, - it is not clear for the term SCell operation or multi-carrier operation. To our understanding, if this implies to, e.g., study whether OD-SSB can be considered for SCell or PCell. Maybe we should make this clear instead of saying simply SCell operation, so we agree with Fujitsu’s comment. But if it reflects whether the OD-SSB should rely on another cell, e.g., Cell-A in 5G R19, then we would suggest to use single-carrier or multi-carrier instead of SCell operation. - SSB structure should be added in the bullet as it can directly impact the UE detection complexity. Suggested updated proposal: Study NW energy saving aspects on SSB from increasing the default periodicity of cell-defining SSB on synchronization raster. Additionally, study UE performance impact and mechanisms to mitigate UE performance degradations in important use-cases, considering: - SBB types (always-on SSB, on-demand SSB), - SSB structure, - SSB default periodicity and SSB periodicity(ies), - Synchronization raster granularity, incl. prioritized raster points, - SSB detection performance (including UE complexity and latency), - SCell operation,multi-carrier and single-carrier scenarios, - Etc. |
| Futurewei | This is a good starting point. We think that there are other possible design decisions in 6GR to factor in such as C-DTX/DRX mode extension to SSB transmission, and SSB periodicity adaptation (time/spatial). |

2nd round of FL comments and proposals

In general there is a strong support for studying changes to SSB periodicity and it is acknowledged to be a key aspect for NES. FL proposal is updated to accommodate companies’ views.

FL Proposal 2.2-16:

Study and evaluate NW energy saving from increasing the default periodicity of cell-defining SS/PBCH on synchronization raster. Additionally, study and evaluate UE performance impact and mechanisms to mitigate UE performance degradations in important use-cases, considering:

- SS/PBCH types (e.g., always-on, on-demand),

- SS/PBCH transmission adaptation

- SS/PBCH periodicity(ies),

- Synch raster search complexity,

- SS/PBCH detection performance (incl. #RBs, #symbols, beamforming),

- Other mechanisms are not precluded,

for at least least initial access, RRM measurement procedure.

SIB-1 availability

Companies’ views

Nokia - R1-2505131 - Proposal 3: The first 6G release should support a leaner carrier and signaling reduction (of SS/PBCH synchronization signals, system information, PRACH and paging occasions) including on-demand provisioning mechanisms for NES operations of cells operating under low/no load. - Proposal 9: Consider extending the Rel-19 OD-SIB1 for different deployment scenarios, including the single cell scenario. - Proposal 12: On-demand SIB1 operation shall be studied in 6G, including support for legacy operation in PCell and other applicable scenarios. FUTUREWEI - R1-2505145 - Proposal 4: Adopt and further enhance from day one the gNB power saving solutions developed in 5G (e.g., 5G NES features SCell with on-demand SSB or no SSB, on-demand SIB1, Cell DTX/DRX, etc.). - Proposal 5: Develop consistent energy efficiency solutions among all UE states. For instance, consistent on-demand control SIB1 signaling for all IDLE, INACTIVE and ACTIVE UEs. CATT - R1-2505297 - Proposal 6: In 6GR, on-demand SIB1 should be supported for both homogeneous network and heterogeneous network. - Proposal 10: To simplify the on-demand mechanism of multiple common signals, a unified common signal request mechanism can be considered. Spreadtrum (UNISOC) - R1-2505176 - No SIB1 or OD-SIB1-related proposals. Xiaomi - R1-2505467 - Proposal 9: Joint condensed common signal/channel design and L1-based paging adaptation can be considered for 6GR NES. Samsung - R1-2505589 - No SIB1 or OD-SIB1-related proposals. ZTE - R1-2505607 - No SIB1 or OD-SIB1-related proposals. Ericsson - R1-2505625 - Proposal 5: Study periodic bursts of SSB and SIB1 repetitions, where in each period multiple SSB and SIB1 repetitions are provided during a short time interval followed by a longer inactive interval. - Proposal 6: 6GR should support coordinating system information broadcast with other common signals/channels in order not to interrupt sleeping opportunities. Tejas Networks Ltd. - R1-2505631 - Proposal 1: Transmission of SIB1/SI corresponding only to one SSB beam will improve NES and is targeted for the low load scenario, and transmission of SI corresponding to all the active SSB beams will reduce the number of requests from different UEs and is used during the high load scenario. - Proposal 22: Standalone OD-SIB1 needs to be studied in 6G to determine potential benefits as compared to the non-standalone case defined in 5G NR. Ofinno - R1-2505677 - Proposal 5: 6GR should support OD-SIB1 and RAN1 to study supporting OD-SIB1 for a standalone cell. Oppo - R1-2505761 - Proposal 13: For the 6GR, OD-SIB1 without relying on cell A can be studied with the following considerations: - further simplify the OD-SIB1 procedure compared to the 5G OD-SIB1 counterpart, - consider adopting default configurations, allowing the necessary OD-SIB1 parameters to be inferred with minimal signaling, or consider introducing an extended MIB to provide necessary configuration Quectel - R1-2505769 - Proposal 1: The OD-SSB/OD-SIB1 structure simplifying SSB/SIB1 needs discussion in 6G. Panasonic - R1-2505789 - Proposal 3: To study system information design and framework facilitating common channel/signal ON/OFF and adaptation. Fraunhofer IIS, Fraunhofer HHI - R1-2505834 - Proposal 5: The 6GR study should investigate the possibility of increasing common signal periodicity and enable on demand common signals. LG Electronics - R1-2505858 - Proposal 3: Study on-demand SIB1 procedure for single-cell scenario (where a cell provides UL WUS configuration for its own on-demand SIB1) as well as for multi-cell scenario (as introduced in Rel-19 NES). - Proposal 6: Study a unified/integrated on-demand procedure for multiple common signals/channels. Apple - R1-2505917 - Proposal 4: Reduction/adaptation of common signals/channels should be considered in 6G day-1 to avoid backward compatibility issue and fully achieve the NES benefit. - For OD-SIB1, how much additional NES gain can be obtained through SIB1 reduction in single cell case, under the assumption of increased SSB periodicity. Fujitsu - R1-2505972 - Proposal 3: Study the methods to enable on-demand transmission of cell common signals, such as SS, PBCH and SIB1, without limitations on applicable scenarios. - The above aspects can be included in the initial access related discussions. Lenovo - R1-2505995 - Proposal 2: 6GR should enhance common signals/channels transmission and periodicity for network energy saving under various cell load. HONOR - R1-2506003 - No SIB1 or OD-SIB1-related proposals. CAICT - R1-2506005 - Proposal 1: the coverage and the capacity carrier can be defined to enable dynamic on/off carrier, the SSB signal with long periodicity and on demand SSB can be transmitted in the capacity carrier ,and the always on RS can be transmitted in the coverage carrier. Sharp - R1-2506014 - Proposal 2: To reduce unnecessary power consumption and signalling overhead, on demand reference signal in Pcell should be supported. - Proposal 7: On demand signalling for dual-side joint mechanisms for network and UE should be supported in energy efficiency. ETRI - R1-2506069 - Proposal 13: Study on-demand system information transmission as a complementary mechanism to periodic transmission. Vodafone, Bouygues Telecom, Deutsche Telekom - R1-2506134 - No SIB1 or OD-SIB1-related proposals. InterDigital - R1-2506146 - Proposal 6: Support on-demand signals/channels (e.g., configurable, dynamic (de)activation, and/or UE-requested) where it is applicable. SK Telecom - R1-2506152 - Proposal 1: For 6G energy efficiency, at least the following aspects should be studied: - SSB/SIB1 transmission (longer periodicity, on-demand) Qualcomm - R1-2506222 - No SIB1 or OD-SIB1-related proposals. AT&T - R1-2506237 - No SIB1 or OD-SIB1-related proposals. NTT DOCOMO - R1-2506310 - Proposal 6: Study both of the following directions for OD-SIB1 operation - Direction1: Standalone OD-SIB1 operation with introducing pre-defined/pre-configured UL WUS. - Direction2: Representative cell/carrier to deliver NES cells’ system information. WILUS Inc. - R1-2506324 - Proposal 2: Study On-Demand SSB/SSB1 for 6GR - Enable on-demand SSB/SIB1 transmission for UEs in Idle, Inactive, or RRC_Connected modes to maximize energy savings and deep-sleep opportunities for gNBs. Rakuten Mobile, Inc. - R1-2506346 - Proposal 1.2: RAN1 to study implications of, SIB1 and paging transmitted by Anchor Carriers on demand or at ultra-low periodicity and Data Carriers remain dormant until scheduled user activity is detected. CEWiT - R1-2506363 - Proposal 1: 6G should support energy efficiency enhancements for common signals including - On-Demand Signals for initial access including OD-SSB & OD-SIB1 a. Simplified SSB IIT Kanpur - R1-2506392 - Proposal 2: 6GR Day 1 design, shall apply time/frequency adaptations of SS/PBCH transmission with more on-demand signal transmission and less always-on signals. Also on-demand SIB1 operation shall be supported from the Day 1 in 6G.

Summary

The above proposals address the energy inefficiency caused by the frequent periodic transmission of SIB1 in 5G NR, typically every 20 ms or up to 160 ms, which consumes network resources and prevents gNBs from entering deep sleep modes, particularly in low or no-traffic (Sharp, Nokia Obs. 1, Ericsson Obs. 1, MediaTek Obs. 1). This always-on broadcasting limits network energy savings (NES), which could be reduced by up to 83.7% for Cat-1 and 52.5% for Cat-2 base stations under low load, as per TR 38.864. The proposals aim to mitigate this by enabling OD-SIB1, possibly triggered by UE requests via uplink wake-up signals (UL-WUS) or network conditions, allowing gNBs to remain in low-power states during idle periods (Sharp Prop. 7, NTT DOCOMO Prop. 6, FUTUREWEI Prop. 5). They address 5G’s backward compatibility constraints, which restricted OD-SIB1 to capacity cells in non-standalone scenarios, by advocating for support in standalone operation of on-demand SIB1, and both homogeneous and heterogeneous networks (Ofinno Prop. 5, Tejas Prop. 22, CATT Prop. 6). Additional enhancements include simplifying OD-SIB1 procedures, using default configurations or extended Master Information Blocks (MIB) to reduce signaling overhead, coordinating SIB1 with other common signals to avoid interrupting sleep opportunities, optimizing SIB1 transmission to specific SSB beams based on traffic load, and integrating with unified common signal request mechanisms to streamline operations (Ericsson Prop. 6, OPPO Prop. 13, CATT Prop. 10, Tejas Prop. 1). These changes ensure minimal impact on UE performance, such as access latency, while supporting diverse device types and new spectrum (~7 GHz) for a leaner, energy-efficient 6GR design from Day 1 (IIT Kanpur Prop. 2, Lenovo Prop. 2).

1st round FL comments and proposals

Companies agree that SIB1, as the only periodically transmitted SIB, is an obstacle to NW energy efficiency. The views on what to do about it differ, though. A UE-centric approach is a UE-initiated OD-SIB1 (and OD-SSB), as presented in Figure 2, whereas a NW-centric approach combines a more efficient SIB1 transmission with NW-initiated OD-SIB1 transmissions.

Figure 2: Illustration of UE-centric OD-SIB functionality [9].

In FL’s understanding, OD-SIB provisioning is not necessarily limited to SIB1 for which reason the FL proposes to generalize the discussion to overall system information.

FL proposes that companies study more detailed alternatives for OD-SIB and their respective potential gains.

FL Proposal 2.3-17:

Study and evaluate on-demand system information operation with respect to, e.g.,

- NW and UE energy savings potential,

- Acquisition delay

- Applicable deployment scenarios

- NW and UE complexity

Companies are welcome to share their views on the above FL proposal.

| Company | View |
| --- | --- |
| Google | On the FL Proposal 2.3-1, we would like to have a revision as below, to emphasize the concept in Figure 2 above, which is really helpful on reducing the cell accessing latency. In addition, we can also consider integration of UL-WUS with MSG1 in the subsequent RA procedure, since the UL-WUS can be PRACH as well in 6GR. FL Proposal 2.3-18: Study and evaluate on-demand system information operation with respect to, e.g., - NW and UE energy savings potential, - Acquisition delay, e.g., integration of one request for different OD common signal. - Applicable deployment scenarios - NW and UE complexity |
| InterDigital | Support |
| TCL | We see clear benefit in on-demand system information to eliminate constant SIB1 broadcasts and reduce idle energy waste. Triggering SIB only when needed can significantly save network energy (5G evaluations showed skipping SIB1 yields substantial savings in low-load cells). It could also let UEs skip unnecessary system-info decoding. Nevertheless, we still concern about added access latency and signaling complexity–UEs requesting SI might face delays or contention. Thus, like proposal mentioned, acquisition delay needs carefully considered. |
| Spreadtrum | We think “request signaling” should be added. FL Proposal 2.3-19: Study and evaluate on-demand system information operation with respect to, e.g., - NW and UE energy savings potential, - Request signaling, - Acquisition delay - Applicable deployment scenarios - NW and UE complexity |
| Panasonic | Support |
| Qualcomm | We are ok with this proposal |
| Fujitsu | We are fine with the proposal. |
| Fainity | OK |
| Ofinno | Generally supportive. As many of the proposals from companies focus on SIB1 specifically we wonder if it would be helpful to have a note which says that the study includes SIB1 and OSI |
| CEWiT | The on demand SI operation should be studied including request signalling details between UE and Gnb. It can also be considered in conjuction with other operations especially common signals. We support the proposal with following modification FL Proposal 2.3-20: Study and evaluate on-demand system information operation with respect to, e.g., - NW and UE energy savings potential, - Request signaling, - Acquisition delay - Applicable deployment scenarios - Interaction with other common signal/channel operations - NW and UE complexity |
| Nokia | While on-demand system information in general is relevant for NES in 6GR, we need to be careful with the potential overlap between scope of the work in RAN1 and RAN2. For that reason, we propose to focus the RAN1 studies on SIB1, as its content has direct impact on physical layer procedures. In addition, we would like to clarify if “applicable deployment scenarios” includes other cases such as SIB1 offloading to an anchor cell, coexistence between OD-SIB1 and regular (but infrequent) SIB1. |
| LG Electronics | In general, the proposal is okay. Evaluation methodology that was defined in Rel-19 on-demand SIB1 can be considered as starting point. |
| Sharp | We are fine with the proposal. |
| Huawei, HiSilicon | The proposal should, probably with higher priority, look into single carrier scenario since it relates to the SSB discussion and initial access procedure, although the proposal does not limit to multi-carrier scenario. For that reason, we do not prefer to start with on-demand SIB; furthermore, NR has already established good design for on-demand SIB1, we would like to see all potential for SIB1 delivery for 6GR from Day1. NW complexity for SIB1/SIBx transmission does not seem to be concerned. We provide the following modifications FL Proposal 2.3-21-Huawei update: Study and evaluate on-demand system information delivery operation with respect to, e.g., - NW and UE energy savings potential, - Acquisition delay, common PDCCH/SIB coverage, UE complexity - Applicable deployment scenarios - For multi-carrier scenario, On-demand SIB1 framework can be starting point - NW and UE complexity |
| DCM | Generally fine. In Rel-19, some of the scenarios (e.g., standalone OD-SIB1 operation) were not supported due to backward compatibility and lack of time for the study. So, in 6G, we would like to focus on the scenario which is not supported in NR. Study and evaluate on-demand system information operation with respect to, e.g., - NW and UE energy savings potential, - Acquisition delay - Applicable deployment scenarios, e.g., standalone OD-SIB1 cell - NW and UE complexity |
| CMCC | We are fine with the proposal. |
| CATT | OK with the proposal. |
| ETRI | Support |
| NEC | We support studying on-demand system information. A key limitation of the on-demand SIB1 feature in 5G NR was its reliance on an assisting anchor cell, which restricted its applicability. We agree with the general consensus that 6GR should study extending on-demand SIB1 to standalone/single-cell scenarios. Evaluating the potential energy savings, acquisition delay, and complexity, as proposed by the FL, is the correct approach to developing a more universally applicable and effective mechanism for 6GR. |
| Xiaomi | We support the intention of support OD-SIB1 to save energy. However, we feel a little bit lost on the proposal. Actually OD-SIB1 was comprehensively discussed during Rel-19, we should not repeat previous discussion as much as possible. For us, the main bullet with some modification is sufficient: FL Proposal 2.3-22: Study and evaluate on-demand system information operation with respect to, e.g., and evaluate if necessary. - NW and UE energy savings potential, - Acquisition delay - Applicable deployment scenarios - NW and UE complexity |
| Ericsson | Support From network EE perspective, on-demand system information is only meaningful if SSB periodicity is large. Regarding acquisition delay, on-demand operation can in some cases reduce the acquisition delay compared to periodic transmission with long periodicity. Regarding deployment scenarios – standalone operation (i.e. not relying on assisting cell) will benefit a wider range of practical network deployments. For SIB1 however, the question is how to provide the configuration. |
| Vivo | In our view, there are two different directions based on Rel-19 OD-SIB1 procedure: one is to extend OD-SIB1 to all cell types by providing WUS configuration within or together with NES cell’s SSB; the other is to reduce SSB transmission in capacity cells in addition to the SIB1 to achieve larger NES gain. Besides, on-demand system information is based on UL WUS. So we suggest the following updates Study and evaluate on-demand system information (potentially together with SSB) operation with respect to, e.g., - NW and UE energy savings potential, - Acquisition delay - Applicable deployment scenarios - NW and UE complexity |
| ZTE, Sanechips | We think not only OD-SIB1, also OD-SSB should be studied considering some aspects, e.g., NES gain or acquisition delay. Therefore, this proposal should be more general, as following FL Proposal 2.3-1: Study and evaluate on-demand signal/channel system information operation with respect to, e.g., - NW and UE energy savings potential, - Acquisition delay - Applicable deployment scenarios - NW and UE complexity |
| Samsung | We want to clarify that the proposal is for OD-SIB1 only or generally for all SIB (e.g., SIB1 and SIBx, x>1). If it is only for OD-SIB1, the main bullet may need to clarify this point. |
| IIT Kanpur | We support the proposal. |
| Apple | In general, we would try to avoid leaving an impression in the summary part that the benefits/gains of the proposals have been commonly recognized. We propose to capture our proposal to further justify the necessity of OD-SIB1 by evaluating how much additional NES gain can be obtained through SIB1 reduction, under the assumption of increased SSB periodicity.(Prop.4 in R1-2505917). Proposed update to the Summary part: The above proposals address the energy inefficiency caused by the frequent periodic transmission of SIB1 in 5G NR, typically every 20 ms or up to 160 ms, which consumes network resources and prevents gNBs from entering deep sleep modes, particularly in low or no-traffic (Sharp, Nokia Obs. 1, Ericsson Obs. 1, MediaTek Obs. 1). This always-on broadcasting together with SSB transmission limits network energy savings (NES), which could be reduced by up to 83.7% for Cat-1 and 52.5% for Cat-2 base stations under low load, as per TR 38.864. The proposals aim to mitigate this by enabling OD-SIB1, possibly triggered by UE requests via uplink wake-up signals (UL-WUS) or network conditions, allowing gNBs to remain in low-power states during idle periods (Sharp Prop. 7, NTT DOCOMO Prop. 6, FUTUREWEI Prop. 5). They intend to address 5G’s backward compatibility constraints, which restricted OD-SIB1 to capacity cells in non-standalone scenarios, by advocating for support in standalone operation of on-demand SIB1, and both homogeneous and heterogeneous networks (Ofinno Prop. 5, Tejas Prop. 22, CATT Prop. 6). However, there were also proposals to further justify the need to reduce SIB1 broadcast, considering if the SSB periodicity is increased and SIB1 is broadcast with the same periodicity as SSB, additional NES gain for OD-SIB1 could be marginal (Apple Figure 4, Prop.4). Additional enhancements include simplifying OD-SIB1 procedures, using default configurations or extended Master Information Blocks (MIB) to reduce signaling overhead, coordinating SIB1 with other common signals to avoid interrupting sleep opportunities, optimizing SIB1 transmission to specific SSB beams based on traffic load, and integrating with unified common signal request mechanisms to streamline operations (Ericsson Prop. 6, OPPO Prop. 13, CATT Prop. 10, Tejas Prop. 1). These changes ensure minimal impact on UE performance, such as access latency, while supporting diverse device types and new spectrum (~7 GHz) for a leaner, energy-efficient 6GR design from Day 1 (Apple Prop. 4, IIT Kanpur Prop. 2, Lenovo Prop. 2). For the proposal, Huawei’s version provides a more general starting point FL Proposal 2.3-23(Updated): Study and evaluate on-demand system information delivery, including broadcast and on-demand provisioning operation with respect to, e.g., - NW and UE energy savings potential, - SIB Acquisition delay, and complexity - Applicable deployment scenarios |
| Lenovo | We are fine with the proposal I think the on-demand system information operation should be changed to on-demand minimum system information |
| Fraunhofer | We support this direction and agree with DCM, NEC and Ericsson to specifically address the standalone scenario without relying assistance from anchor cell has wider scope and greater potential for NES. |
| Tejas | Our preference is to include joint request of SI and other common signal such as SSB. Study and evaluate the on-demand system information (OD-SI) or joint on demand system information and other common signal/channel operation with respect to, e.g., - NW and UE energy savings potential, - Acquisition delay - Applicable deployment scenarios - NW and UE complexity |
| OPPO | Our proposal 13 in R1-2505761 and the corresponding text provide some of our views on 6G SIB transmission. This is inline with the discussion item in FL summary section 2.3. Unfortunately, our views were not captured in FL summary. We appreciate if feature lead could add our proposal to the summary. Regarding FL proposal, we are fine. |
| Futurewei | OK in principle. The FL refers to Fig 1, should it be Fig 2? Without additional specifics the formulation is quite like 5G NR study. For instance, we could differentiate with the following change: - Applicable deployment scenarios in addition to those supported in 5G NR. |

2nd round FL comments and proposals

There is strong support for studying on-demand SI. Per Nokia’s comment, to not interfere with RAN2 responsibilities, FL propose to limit studies to SIB1.

FL Proposal 2.3-24:

Study and evaluate on-demand SIB1 delivery with respect to, e.g.,

- NW and UE energy savings potential,

- Acquisition delay

- SIB coverage

- NW and UE complexity

- UE requested vs NW provided

- Applicable deployment scenarios

  - OD-SIB1 for standalone and/or Scells

DL WUS/WUR requirements

Companies’ views

FUTUREWEI - R1-2505145 - Proposal 3: Adopt from day one 5G UE power saving techniques as baseline mechanisms in 6G, such as: - Duty-cycled based operations (iDRX, eDRX, cDRX), - LP-WUS with at least PEI and DCP functionality replacement, - Relaxed RRM measurements of neighboring cells, and - Relaxed/Offloading to LP-WUR of serving cell measurements. Spreadtrum (UNISOC) - R1-2505176 - Proposal 3: The following technologies can be studied for 6GR UE power saving: - Bandwidth adaptation, SCell dormancy, TX/RX antenna adaptation, paging enhancement, WUS/WUR, etc. vivo - R1-2505420 - Proposal 3: Study the introduction of wake-up signal and wake-up receiver for paging, PDCCH monitoring, and serving cell measurement for 6GR. ZTE - R1-2505607 - Proposal 25: Low power WUS should be supported in 6GR. Ericsson - R1-2505625 - Proposal 17: 6GR should adopt a simplified OFDM-based LP-WUS/WUR design for idle/connected mode UE power saving, with improved coverage and spectral efficiency compared to NR. NEC - R1-2505641 - Proposal 13: Study the enhancement of low-power wake-up signal/receiver (LP-WUS/WUR) for paging and PDCCH monitoring for 6GR. - Proposal 14: Study the design of low-power synchronization signal (LP-SS) for serving cell measurement with low-power wake-up receiver for 6GR. - Proposal 19: Study the enhancements of PDCCH skipping and SSSG switching with wake-up signal for 6GR. Ofinno - R1-2505677 - Proposal 8: 6GR should support a framework for LP-WUS as a baseline for power saving for RRC_IDLE/INACTIVE UEs. - Proposal 9: RAN1 to study improving coverage for LP-WUS, e.g., by introducing repetition and/or low modulation order and coding rate, without increasing UE complexity. OPPO - R1-2505761 - Proposal 4: For UE energy efficiency, the 6GR should consider energy efficient design for DL control channel and related procedure. UE energy efficiency should avoid duplication of design similar function, i.e., strive for a single power saving signal. Panasonic - R1-2505789 - Proposal 8: To study possible LP-WUS/LP-SS integration with 6GR and the support by MRSS. LG Electronics - R1-2505858 - Proposal 13: Study DL WUS to control UE wake-up during active time, e.g., PDCCH monitoring, as well as wake-up for paging/measurement for idle/inactive mode. Apple - R1-2505917 - Proposal 1: Time domain adaptation techniques like C-DRX, PDCCH skipping, SSSG switching, and LP-WUS should be considered for a simplified/harmonized design to reduce UE power consumption for PDCCH monitoring. - Proposal 2: LP-WUS/WUR design targeting all device types should be based on OFDM waveform to achieve better coverage and spectral efficiency, and should be applicable to both idle/inactive and connected mode UEs. Fujitsu - R1-2505972 - Proposal 6: For UE power saving, the following should be considered: C-DRX, PDCCH monitoring reduction, WUS for idle mode, etc. KT Corp. - R1-2505991 - Proposal 3: A low-power receiver is mandatory for both 6GR BS and UE. HONOR - R1-2506003 - Proposal 6: The design of low-power WUS should be considered on both the UE side and the network side to save energy. Sharp - R1-2506014 - Proposal 6: To complement the paging mechanism for UE energy efficiency, low-power WUS should be introduced in the 6GR design. MediaTek - R1-2506024 - Proposal 5: Study WUR for UE synchronization and measurements in addition to wake-up functionality. - Proposal 19: For 6G, WUS/WUR should consider OFDM-based design to achieve better coverage and spectral efficiency. CMCC - R1-2506101 - Proposal 5: Support the following UE power saving techniques to be further adopted in 6GR: - Time domain: - PDCCH Skipping/SSSG switching/cross-slot scheduling - I-DRX, Extended-DRX including PTW - RRM/RLM/BFD relaxation - LP-WUS/WUR for paging, PDCCH monitoring and serving cell measurement - Frequency domain: SCell activation/deactivation/dormancy for CA scenario, BWP switching - Proposal 6: RAN1 to further consider and study the enhancement of LP-WUS/WUR in 6GR: - Signal design aspect, aim for better performance on coverage/robustness/efficiency: - Better receiver detection method for 6G LP-WUR more than envelope detection, can be considered. - The receiver accuracy, e.g., option 3 or 4 in 3GPP TR 38.845 (~0.1-1mW) can be considered as start point with justified power saving gain. - Overlaid-wise signals are not necessary to avoid unnecessary design trade-offs. - Both RRC IDLE / CONNECTED mode are supported. - Procedure design aspect, aim for extend the usage: - For 6G LP-WUS, - It can be considered to carry small payload size data or extra indications, which can further reduce the turning-on time of MR and the complexity of PDCCH monitoring. - In addition, it can be considered together with the usage of PDCCH skipping to control PDCCH monitoring in a more power efficient way. - For 6G LP-SS, - It can be considered for neighbor cell/TRP measurement at least for cell selection/reselection procedure in addition to serving cell measurement. - It should consider a harmonized design of UE measurement can be targeted between 6G LP-SS for LP-WUR and “6G SSB” for Main Radio, in order to avoid too much always-on signals being transmitted by BS. Vodafone, Bouygues Telecom, Deutsche Telekom - R1-2506134 - Proposal 3: Study introduction of LP-WUS/WUR for all device types in 6GR air interface as a day-1 considering potential impacts and benefits. InterDigital - R1-2506146 - Proposal 10: Support LP-WUS targeting low power receiver capability from 6G Day-1. - Proposal 11: Support multiple types of modulations with different receiver architectures considering different power consumption levels and achievable coverages. - Proposal 12: Support low power receiver operations for mobility and cell (re)selection. SK Telecom - R1-2506152 - Proposal 1: For 6G energy efficiency, at least the following aspects should be studied: - SSB/SIB1 transmission (longer periodicity, on-demand) - Enhanced BWP mechanism - Time-domain enhancement (UE-basis C-DRX vs. cell-basis DRX/DTX, LP-WUS/WUR) - Reduced RRM measurement - PEI NTT DOCOMO - R1-2506310 - Proposal 13: Study LP-WUS/WUR to control UE wake-up for PDCCH monitoring. - Proposal 14: Study LP-WUS/WUR for RRM measurement. - Proposal 15: Study extension of LP-WUS/WUR for neighbor cell measurement in idle/inactive mode, as well as serving cell measurement. IIT Kanpur - R1-2506392 - Proposal 4: 6GR Day 1 design shall enhance the low-power wake-up signal/receiver (LP-WUS/WUR) designs for improving UE power efficiency.

Summary

The DL WUS and WUR proposals address the significant UE energy consumption in 5G NR, particularly during PDCCH monitoring, paging, and RRM measurements, which account for approximately 30% of UE power usage due to frequent wake-ups that prevent the main radio (MR) from entering deep sleep states (Apple Obs. 1, MediaTek Obs. 2). These proposals aim to enhance UE power saving by introducing low-power wake-up signal (LP-WUS) and wake-up receiver (WUR) mechanisms as mandatory Day 1 features in 6GR, using OFDM-based designs or advanced detection methods (e.g., beyond envelope detection) to improve coverage, robustness, spectral efficiency, and reduced overhead/NW energy consumption compared to 5G NR (Ericsson Prop. 17, MediaTek Prop. 19, CMCC Prop. 6). They target minimizing unnecessary UE wake-ups for PDCCH monitoring, paging, and serving/neighbor cell measurements in both RRC_CONNECTED and RRC_IDLE/INACTIVE modes, enabling UEs to remain in low-power states longer (LG Prop. 13, Ofinno Prop. 8, NTT DOCOMO Prop. 15). Proposals advocate harmonizing LP-WUS/WUR with other power-saving techniques like C-DRX, PDCCH skipping, and SSSG switching, unifying designs with Paging Early Indication (PEI) and DCI-based Power Saving (DCP) to reduce complexity (Apple Prop. 2, OPPO Prop. 4, NEC Prop. 14). They address 5G’s limitations, such as underutilized WUS due to late standardization and multiple waveform variants (e.g., OOK vs. OFDM), by proposing simplified, robust designs with techniques like repetition or low modulation order, supporting diverse device types (e.g., IoT, XR/AR) in new spectrum (~7 GHz), and enabling mobility and cell (re)selection, achieving significant power savings (e.g., 10–20 times lower than MR per 3GPP Rel-18 studies, up to 80–90% compared to 5G paging) while maintaining low latency and compatibility with diverse 6GR use cases (Ofinno Prop. 9, InterDigital Prop. 10, vivo Prop. 3).

1st round FL comments and proposals

Most companies support an OFDM-based WUS/WUR due to its superior coverage, reduced overhead, and greater additional energy efficiency potential from, e.g., synchronization and RRM. For that reason, agreeing to an OFDM-based WUS is the first firm agreement put forward.

A natural consequence of such an agreement is for RAN1 to further study additional use for an OFDM-based WUS. This is presented in a follow-up proposal in FL Proposal 2.4-28.

- FL Proposal 2.4-25:

Propose OFDM-based DL WUS as a candidate technology for further studies in the 6G Radio SI.

Companies are welcome to share their views on the above FL proposal.

| Company | View |
| --- | --- |
| Google | We are fine with studying OFDM-based DL WUS for enhancing WUS performance, especially for coverage consideration. But meanwhile, we should also consider whether OFDM-based DL WUS is applicable for diverse device types anticipated in 6GR. |
| TCL | A unified OFDM WUS can replace multiple legacy indicators (PEI, DCP, etc.), simplifying protocols and benefiting both network and device vendors. It leverages standard OFDM receivers, avoiding the need for a separate waveform. However, some concerns are concluded as below - If any enhanced WUR should be considered in 6G day 1 or following LP-WUR discussed in Rel-18 SI (like OFDM-based overlaid sequence detection or OFDM-based sequence detection) - What CP-OFDM or DFT-s-OFDM or both is used to generate OFDM-based DL WUS needs to be discussed if considering enhanced WUR. |
| Spreadtrum | We are fine with the proposal. |
| Panasonic | We are open to study the potential of OFDM-based DL WUS in the IDLE mode. However, the use/scenario and function should be looked into before such a conclusive proposal. For example, what is the target function, what is the UE procedure of receiving and use it and what is the relation with SS are all not discussed yet. But the wording of the proposal reads like it is already concluded. Thus, we propose to only discuss the next proposal. |
| Qualcomm | We are ok with the proposal, but would like to avoid the “/WUR” in the discussion. It is up to the UE how to implement the receiver for the OFDM-based DL WUS. |
| Fujitsu | We are fine with the proposal. |
| Fainity | OK with the proposal |
| Ofinno | Support |
| Nokia | The proposal should be reformulated in terms of study on DL-WUS. In the current format it is suggesting we first agree on DL WUS support, regardless of evaluations. Revised proposal: “Consider DL WUS in the studies performed in this AI.“ |
| LG Electronics | We support a principle pursuing a unified solution for PEI, DCP, or PDCCH monitoring adaptation. However, it would be premature to take OFDM-based DL WUS as a single candidate solution. Before narrowing down to a solution, we would suggest to study and compare pros and cons for several candidate schemes, i.e., DCI-based approach, OFDM-based DL WUS, and OOK-based DL WUS. |
| Sharp | Support |
| Huawei, HiSilicon | We agree enhancements on NR LP-WUS is beneficial. We suggest to modify the proposal as below, - FL Proposal 2.4-26-Huawei updated: Study Propose OFDM/DFT-based DL WUS as a candidate technology for further studies in the 6G Radio SI, aspects to be considered include at least - Coverage, synchronization accuracy, capability - Small control and/or data information - Associated functions, e.g. serving cell/neighboring cell measurement. |
| DCM | We are supportive with studying OFDM-based DL WUS While the OOK-based LP-WUS signal in release 19 had the advantage of being usable with low performance OOK-based receivers, it caused large overhead regardless of the receiver type. Furthermore, low synchronization accuracy and BLER target limited coverage. This is a significant issue regarding commercial deployment. For this reason, in release 20, we need to study OFDM-based LP-WUS signals to achieve minimized NW resource overhead and maximized use cases with good UE PS gain. |
| CMCC | Support |
| ETRI | To avoid multiple waveform variants and devices, we prefer to focus the study on OFDM-based DL WUS. |
| NEC | We are generally fine with the proposal, OFDM-based WUS is a good solution to extend the coverage of LP signal. However, we think OOK-based WUS can also be studied for the scenario that extremely low power consumption is demanded, if such a scenario is identified. |
| Ericsson | Support |
| vivo | We are generally ok to this proposal and suggest slight change to the wording. In addition, it is important to target unified design of DL WUS across IDLE and CONNETED mode - FL Proposal 2.4-27: Propose Study OFDM-based DL WUS as a candidate technology for further studies in the 6G Radio SI. Target unified design of DL WUS across IDLE and CONNECTED mode usage. |
| ZTE, Sanechips | Support |
| Samsung | Suggest to reword: Propose Study OFDM-based DL WUS as a candidate technology for further studies in the 6G Radio SI. |
| IIT Kanpur | We support the proposal for downlink WUS design in 6GR. |
| Apple | We are fine with the proposal, but we would like to leave it open to still consider other DL WUS waveform(s) at this early stage. |
| Lenovo | support |
| Fraunhofer | support |
| Tejas | Support |
| OPPO | First, the proposal 4 under the company name OPPO captured in FL summary is not exactly our text. We have” Proposal 4: For UE energy efficiency, the 6GR should consider energy efficient design for DL control channel and related procedure. UE energy efficiency should avoid duplication of design similar function, i.e., strive for a single power saving signal. ” Regarding OFDM-based DL WUS, we are open to study. That would reduce the duplicated functionality like DCP based PEI and LP-WUS based DCP. And the coverage would be much better for OFDM-based. |
| Futurewei | OK to study. We do not see any reason to restrict the study at this stage. We would like the following reformulation: Propose OFDM-based DL LP-WUS as a candidate technology for further studies in the 6G Radio SI. |

- FL Proposal 2.4-28:

Study further use cases and potential energy efficiency gains for an OFDM-based DL WUS/WUR, apart from wake-up indication, e.g.,

- Synchronization,

- RRM measurements (e.g., neighbor cells),

- Small control information and/or data,

- Etc.

Companies are welcome to share their views on the above FL proposal.

| Company | View |
| --- | --- |
| Google | We are supportive of use case of synchronization. Given that likely increased SSB periodicity, an additional signal in between two SSBs would be helpful for UE’s sync performance. |
| TCL | We partially agree with this proposal. In our understanding, expanding WUS roles may introduce complexity and new failure modes. The DL WUS design must remain lightweight–any extra payload or usage should bring clear net energy gains and minimal added UE processing burden. We thus supports study but urges cautious evaluation of the cost-benefit trade-offs. |
| Spreadtrum | We are fine with the proposal. A small modification is as follows - FL Proposal 2.4-29: Study further use cases and potential energy efficiency gains for an OFDM-based DL WUS/WUR, apart from wake-up indication, e.g., - Synchronization, - RRM measurements (e.g., serving cell and/or neighbor cells), - Small control information and/or data, - Etc. |
| Panasonic | We believe this is regarding basic IDLE mode UE procedures and operations. So cell identification should also be added. Also, the meaning of the wake-up indication should also be open at this moment. Thus, to put wake-up indication to the bullet is suggested: - Wake-up indication and function |
| Qualcomm | Ok with the proposal with the same note on removing “/WUR” |
| Fujitsu | We are fine with the proposal. |
| Fainity | We suggest the redirect behavior upon cell selection and on-demand SSB behavior should be included in this study. |
| Ofinno | Support the main bullet. Could we clarify what we mean by small control information and/or data? If we understand right it may be simpler to say “enhance information LP-WUS can indicate/carry” |
| CEWiT | We are open to discuss the enhancements needed for the proposal |
| Nokia | Inline what was commented above, the intention seems to be to study the EE benefits (of extending) the WUR capabilities for further power saving. In addition, one needs to consider, e.g. energy penalty at network side from DL WUS transmission, potential spectral efficiency loss (due to higher system overhead), potential coverage difference between DL WUS and other DL signals/channels, transmitter impairments (especially if there is power difference between other channels and DL-WUS). |
| LG Electronics | As we stated in Proposal 2.4-1, we would like to study/compare candidate techniques on the table. In that sense, our proposal can be as follow, Study further use cases and potential energy efficiency gains for an OFDM-based DL WUS/WUR candidate DL WUS/WUR techniques (i.e., DCI-based approach, OFDM-based DL WUS, and OOK-based DL WUS), apart from wake-up indication, e.g., - Synchronization, - RRM measurements (e.g., neighbor cells), - Small control information and/or data, - Etc. |
| Sharp | We are fine with the proposal. |
| CMCC | Support |
| ETRI | We support studying synchronization and RRM measurements including neighbor cells. Regarding the third one, we think it is not necessary to include the study on small control information and data as it will complicate LP-WUS design and will not align with 6GR design principles. |
| NEC | We support this proposed study. Utilizing a more capable OFDM-based WUR for functions beyond simple wake-up indication is a promising direction for maximizing UE power saving. There is wide support for investigating the use of the WUR for offloading main radio operations, such as performing RRM measurements on serving and neighbor cells, maintaining synchronization, and even receiving small data packets. This would allow the UE's main receiver to remain in a deep sleep state for longer periods, leading to significant energy savings. |
| Xiaomi | We are generally fine with the direction. DL WUS is confused, no sure the extent, e.g., LP-WUS, DCP, PEI. If the intention is to cover all kinds of WUS, it’s better to clarify as we don’t have a terminology of DL WUS before. |
| Ericsson | Support. RRM measurements would be beneficial. |
| vivo | We agree to study further use cases besides wake-up indication and corresponding energy efficiency gains for DL WUS/WUR. |
| ZTE, Sanechips | Support |
| Samsung | There should be a preliminary proposal on whether to study DL WUR, and then discuss the details about further procedures. |
| IIT Kanpur | We are open to discuss these aspects. |
| Apple | We support the proposal |
| Lenovo | We can keep it at high level by removing the sub-bullets. Add impact to the NW energy savings, if any. Study further use cases and potential energy efficiency gains for an OFDM-based DL WUS/WUR, apart from wake-up indication, and impact to the NW energy savings |
| Tejas | We support |
| OPPO | Suggest not to have a standalone proposal 2.4-1. Please see our comment above. |
| Futurewei | OK to study, however the specific scenarios for the usage must be defined first |

2nd round of FL comments and proposals

It seems like companies are not prepared to agree to WUS as a candidate technology, why the first FL proposal is scrapped. Instead, FL propose to study OFDM WUS in more detail.

FL Proposal 2.4-30:

Study further use cases and potential energy efficiency gains for an OFDM-based DL WUS, apart from wake-up indication, e.g.,

- UE energy savings potential,

- gNB energy costs,

- Synchronization,

- Coverage,

- RRM measurements (e.g., serving cell and/or neighbor cells),

- Small control information and/or data,

- [Cell identification]

- Etc.

Cell DTX/DRX and sleep mechanisms

Companies’ views

Nokia - R1-2505131 - Proposal 2: 6G should support cell DTX/DRX type of operation from day one to allow for sufficient BS sleep opportunities and achieve meaningful NES. - Proposal 3: The first 6G release should support a leaner carrier and signaling reduction (of SS/PBCH synchronization signals, system information, PRACH and paging occasions) including on-demand provisioning mechanisms for NES operations of cells operating under low/no load. - Proposal 4: 6G should support lean carrier operation in capacity cells where always-on signals can be turned off in the absence of traffic. - Proposal 9: Consider extending the Rel-19 OD-SIB1 for different deployment scenarios, including the single cell scenario. FUTUREWEI - R1-2505145 - Proposal 4: Adopt and further enhance from day one the gNB power saving solutions developed in 5G (e.g., 5G NES features SCell with on-demand SSB or no SSB, on-demand SIB1, Cell DTX/DRX, etc.). - Proposal 5: Develop consistent energy efficiency solutions among all UE states. For instance, consistent on-demand control SIB1 signaling for all IDLE, INACTIVE and ACTIVE UEs. CATT - R1-2505297 - Proposal 2: In 6GR, cell DTX/DRX should be supported for both homogeneous network and heterogeneous network. - Proposal 6: In 6GR, on-demand SIB1 should be supported for both homogeneous network and heterogeneous network. - Proposal 10: To simplify the on-demand mechanism of multiple common signals, a unified common signal request mechanism can be considered. Xiaomi - R1-2505467 - Proposal 9: Joint condensed common signal/channel design and L1-based paging adaptation can be considered for 6GR NES. ZTE - R1-2505607 - Proposal 4: Cell DTX/DRX should be supported in 6GR to allow sufficient BS sleep opportunities. Ericsson - R1-2505625 - Proposal 4: Study cell DTX/DRX to allow sufficient BS sleep opportunities, including support for legacy operation. Tejas Networks Ltd. - R1-2505631 - Proposal 6: Cell DTX/DRX should be supported in 6GR to enhance energy efficiency for BS under various traffic load. - Proposal 22: Standalone OD-SIB1 needs to be studied in 6G to determine potential benefits as compared to the non-standalone case defined in 5G NR. Ofinno - R1-2505677 - Proposal 4: 6GR should support cell DTX/DRX for PCell and SCell from day-1. OPPO - R1-2505761 - Proposal 2: Cell DTX/DRX can be studied for 6GR to allow sufficient BS sleep opportunities and achieve meaningful BS energy saving. - Proposal 13: For the 6GR, OD-SIB1 without relying on cell A can be studied with the following considerations: - further simplify the OD-SIB1 procedure compared to the 5G OD-SIB1 counterpart, - consider adopting default configurations, allowing the necessary OD-SIB1 parameters to be inferred with minimal signaling, or consider introducing an extended MIB to provide necessary configuration. Quectel - R1-2505769 - Proposal 1: The OD-SSB/OD-SIB1 structure simplifying SSB/SIB1 needs discussion in 6G. Panasonic - R1-2505789 - Proposal 3: To study system information design and framework facilitating common channel/signal ON/OFF and adaptation. Fraunhofer IIS, Fraunhofer HHI - R1-2505834 - Proposal 3: The 6GR study should investigate the possibility to deactivate the capacity cell in case of no or low traffic load. LG Electronics - R1-2505858 - Proposal 3: Study on-demand SIB1 procedure for single-cell scenario (where a cell provides UL WUS configuration for its own on-demand SIB1) as well as for multi-cell scenario (as introduced in Rel-19 NES). - Proposal 6: Study a unified/integrated on-demand procedure for multiple common signals/channels. Apple - R1-2505917 - Proposal 5: Cell DTX/DRX should be considered in 6G day-1 to avoid backward compatibility issue and fully achieve the NES benefit. Fujitsu - R1-2505972 - Proposal 2: Study the methods to turn off always-on signals in capacity cells without traffic for energy saving, and enable fast activation of the cell when traffic arrives. - Proposal 3: Study the methods to enable on-demand transmission of cell common signals, such as SS, PBCH and SIB1, without limitations on applicable scenarios. - The above aspects can be included in the initial access related discussions. Lenovo - R1-2505995 - Proposal 2: 6GR should enhance common signals/channels transmission and periodicity for network energy saving under various cell load. CAICT - R1-2506005 - Proposal 1: the coverage and the capacity carrier can be defined to enable dynamic on/off carrier, the SSB signal with long periodicity and on demand SSB can be transmitted in the capacity carrier ,and the always on RS can be transmitted in the coverage carrier. Sharp - R1-2506014 - Proposal 5: Cell DTX/DRX should be supported in 6GR to allow sufficient BS sleep opportunities and achieve meaningful BS energy saving. CMCC - R1-2506101 - Proposal 3: RAN1 to further consider and study the following case for multi-carrier scenario in 6GR: - The common signal transmission/reception procedure (e.g. SIB1 transmission, RACH reception) for multiple carriers can converge to one anchor carrier. Therefore, other carrier (i.e. NES carrier) can turning off or only transmit long period SS by default, so as to obtain more NES gain. - NES carrier can be activated per NW guidance or UE demand and UE can initiate access on NES carrier, so as to achieve better UE experience or load balancing for network. InterDigital - R1-2506146 - Proposal 3: Support cell DTX/DRX to allow sufficient BS sleep opportunities where it is applicable. - Proposal 6: Support on-demand signals/channels (e.g., configurable, dynamic (de)activation, and/or UE-requested) where it is applicable. SK Telecom - R1-2506152 - Proposal 1: For 6G energy efficiency, at least the following aspects should be studied: - SSB/SIB1 transmission (longer periodicity, on-demand) - Enhanced BWP mechanism - Time-domain enhancement (UE-basis C-DRX vs. cell-basis DRX/DTX, LP-WUS/WUR) - Reduced RRM measurement - PEI NTT DOCOMO - R1-2506310 - Proposal 2: Study cell DTX/DRX operation for 6GR, including support for legacy operation in PCell. WILUS Inc. - R1-2506324 - Proposal 2: Study On-Demand SSB/SSB1 for 6GR - Enable on-demand SSB/SIB1 transmission for UEs in Idle, Inactive, or RRC_Connected modes to maximize energy savings and deep-sleep opportunities for gNBs. Rakuten Mobile, Inc. - R1-2506346 - Proposal 1.2: RAN1 to study implications of, SIB1 and paging transmitted by Anchor Carriers on demand or at ultra-low periodicity and Data Carriers remain dormant until scheduled user activity is detected. CEWiT - R1-2506363 - Proposal 1: 6G should support energy efficiency enhancements for common signals including - On-Demand Signals for initial access including OD-SSB & OD-SIB1 a. Simplified SSB - SSB periodicity extension beyond 20ms. ETRI – R1-2506069 - Proposal 1: For DTX/DRX in 6GR, aim to design a single framework, or a minimal set of frameworks, that can address diverse scenarios. - Proposal 2: Address both TN and NTN scenarios for DTX/DRX in 6GR, taking into account the following key operational differences: - TN scenario: Certain essential transmissions may be allowed outside active duration. - NTN scenario: In beam-hopping use cases, transmissions are inherently not possible outside active duration. - Proposal 3: Allow configurability in 6G DTX/DRX so that parameters can be flexibly adjusted for different use cases, traffic characteristics, and performance–energy saving priorities. - Proposal 4: For the purpose of pursuing a single, unified DTX/DRX framework, study the following objectives: - Classify signals and channels into multiple categories - For example, (1) not impacted by DTX/DRX operation, (2) impacted by DTX/DRX operation, and (3) configurable (connected mode only) - Configurable timer operation (e.g., extension of active duration based on inactivity timer) - For connected mode, UE-specific configuration of DTX/DRX parameters - Proposal 5: Study beam-level DTX/DRX operation and analyze its necessity for specific use cases (e.g., NTN, mTRP in TN). - Proposal 6: Study use cases and analyze applicability of multiple DTX/DRX patterns for one carrier. - Proposal 7: Study both idle mode and connected mode operations for DTX/DRX in 6GR. - Proposal 8: Study potential enhancements to the random access procedure and paging operation for idle mode DTX/DRX operation.

Summary

The proposals for cell DTX/DRX, cell activation/inactivation, and System Information (SI) signaling for capacity cells address the energy inefficiency in gNB, driven by the continuous transmission of always-on signals like Synchronization Signal Blocks (SSBs) and System Information (SI, e.g., SIB1), which prevent gNBs from entering deep sleep modes, particularly in low or no-traffic scenarios, prohibiting network energy savings (NES) of 83.7% for Cat-1 and 52.5% for Cat-2 base stations under low load, as per TR 38.864 (Nokia Obs. 1, Ericsson Obs. 1). These proposals aim to enhance 6GR energy efficiency by implementing cell DTX/DRX to enable gNB sleep states during idle periods, supporting both primary (PCells) and secondary cells (SCells) in homogeneous and heterogeneous networks (Nokia Prop. 2, CATT Prop. 2, Ofinno Prop. 4). They advocate dynamic activation/inactivation of capacity cells, turning off always-on signals when traffic is absent and enabling rapid reactivation to maintain responsiveness (Fujitsu Prop. 2, Fraunhofer Prop. 3). Additionally, they propose converging SI signaling to anchor or coverage carriers in multi-carrier scenarios, allowing capacity (NES) carriers to deactivate or use minimal, long-periodicity signals, enhancing NES and load balancing (CAICT Prop. 1, CMCC Prop. 3, Rakuten Prop. 1.2). Further enhancements include on-demand SIB1 transmission across all UE states (IDLE, INACTIVE, CONNECTED), simplified procedures with default configurations or extended Master Information Blocks (MIB), and unified common signal request mechanisms to reduce signaling overhead (FUTUREWEI Prop. 5, OPPO Prop. 13, CATT Prop. 10). These changes address 5G’s backward compatibility constraints, which restricted such mechanisms to specific scenarios like non-standalone deployments and promote Day 1 integration in 6GR to support diverse deployments and new spectrum (~7 GHz) while ensuring UE accessibility and system performance (Apple Prop. 5, NTT DOCOMO Prop. 2).

1st round FL comments and proposal

There is large support for cell DTX/DRX among companies although the understandings of what cell DTX/DRX implies and includes differ as well as what gains may result from it. It is also pointed out by companies that a joint framework for both UE and gNB DTX/DRX should be included in this study. For that reason, FL proposes to further study such a joint framework.

- FL Proposal 2.5-31:

Study joint Cell DTX/DRX and UE DTX/DRX regarding,

- Common (idle mode) signal adaptation and clustering,

- UE effects (latency and synchronization),

- Etc.

Companies are welcome to share their views on the above FL proposal.

| Company | View |
| --- | --- |
| Google | We support this proposal, which is a good starting point of joint NW/UE DTX/DRX. |
| InterDigital | Support |
| TCL | We support a unified approach to idle-mode DTX/DRX that jointly optimizes base station and UE sleep cycles. |
| Spreadtrum | We are fine to study Cell DTX/DRX in RRC idle/inactive mode. In RRC idle/inactive mode, the UE receives the paging message based on the paging cycle. At present, there is no UE DTX/DRX mechanism in RRC idle/inactive mode. In addition, common (idle mode) signal adaptation and clustering is not clear to us. We prefer to modify this proposal into the following version. - FL Proposal 2.5-32: Study joint Cell DTX/DRX and UE DTX/DRX regardingin idle mode, at least considering: - Impacts on common (idle mode) signals/channels adaptation and clustering transmission(e.g., SSB, SIB1, Paging, etc.), - Impacts on UE effects (access latency and synchronization), - Etc. |
| Panasonic | We support the main proposal but think the bullets are a bit challenging to agree at this moment. In addition, we propose to strive for such joint Cell/UE DTX/DRX design framework applicable for both IDLE and CONNECTED mode. |
| Qualcomm | We suggest the following update. - We suggest to UE DTX to be aligned with legacy term “UE DRX”. - The 1st bullet can be generalized to “Mechanisms to achieve joint Cell DTX/DRX and UE DRX” - FL Proposal 2.5-1 (updated): Study joint Cell DTX/DRX and UE DTX/DRX at least including regarding, - Common (idle mode) signal adaptation and clustering, - UE impact effects (latency and synchronization), - Energy efficiency analysis - Applicable UE RRC states - Mechanisms to achieve joint Cell DTX/DRX and UE DRX Etc. |
| Fujitsu | We have one question for clarification. In our view, joint cell DTX/DRX and UE DTX/DRX is also a promising energy saving solution for connected mode. Does this discussion solely focus on idle mode? |
| Fainity | OK with the proposal |
| Ofinno | We are supportive in general. Agree with Qualcomm’s comment on removing UE DTX. While we agree that it is worthwhile to study joint techniques we wonder if we should also include study of cell DTX/DRX and UE DRX individually as well at this early stage. Suggest this update: Study Cell DTX/DRX and UE DRX including, - Joint cell DTX/DRX and UE DRX - Common (idle mode) signal adaptation and clustering, - UE effects (latency and synchronization), - Etc. |
| CEWiT | We are fine with the proposal. |
| Nokia | Support, joint operation of Cell DTX/DRX and UE DTX/DRX is important aspect to be addressed for 6G. Besides, during the non-active period of cell DTX/DRX, further “relaxation” of transmission/reception (e.g. longer periodicity of NW transmission of signal/channel and UE monitoring/measuring) could be specifically considered for 6G compared with active period of cell DTX/DRX. Moreover, the interaction of feature designs, i.e. between cell DTX/DRX and on-demand SSB or on-demand SIB1, could also be jointly investigated. |
| LG Electronics | Agree with FL that cell DTX/DRX operation needs to be considered with UE DTX/DRX operation from the beginning of 6GR. In addition, alignment of cell DTX/DRX active time and UE DTX/DRX active time can be considered as well. “Clustering” in the first sub-bullet is quite hard to understand. With this regard, our suggestion is as follows. Study joint Cell DTX/DRX and UE DTX/DRX regarding, - Common (idle mode) signal adaptation and clustering, - UE effects (latency and synchronization), - Alignment between cell DTX/DRX active time and UE DTX/DRX active time - Etc. |
| Sharp | Supoort the proposal. The cordination would contributes improvemen both UE power saving and NW energy saving. |
| Huawei, HiSilicon | To be discussed later. Cell DTRX is just a container that include behavior of cell or UEs. Critical things at the moment is to identify what signal/channels and what functions/procedures are to be enhanced. With the above, we see the need of another proposal: - FL Proposal 2.5-33- Huawei update: Study necessary enhancements to signals for at least IDLE UEs in addition to SSB/SIBx, including - Preamble - RAR - PUSCH - Paging - Common PDCCH - New signal/channels, e.g. for LP WUS - configuration provision methods (e.g. for UE identification), signaling (e.g. for adaptation) - UE impact on e.g. detection complexity, sync. accuracy - etc. joint Cell DTX/DRX and UE DTX/DRX regarding, - Common (idle mode) signal adaptation and clustering, - UE effects (latency and synchronization), - Etc. |
| DCM | We think we should study first what kind of signals are targeted for Cell DTX/DRX and UE DTX/DRX. Also, we would like to study Cell DTX/DRX and UE DTX/DRX operation depending on the deployment scenario (for coverage cell or capacity cell) because turning off SSB within non-active period on capacity cell would be feasible, while coverage cell which offers initial access (initial attach) cannot turn off SSB within the period. |
| CMCC | We generally fine with the bullets listed below, but whether such effect should be bundled with joint Cell DTX/DRX and UE DTX/DRX may need further discuss. There can be multiple ways to achieve such dense pattern. |
| CATT | The Cell DTX/DRX should be discussed for IDLE mode, then whether/how to joint Cell DTX/DRX and UE DRX can be further studied. |
| ETRI | We support the proposal in general, and prefer Spreadtrum’s version. We want to add one more aspect from TN and NTN harmonization perspective as follows: - FL Proposal 2.5-34: Study joint Cell DTX/DRX and UE DTX/DRX regardingin idle mode, at least considering: - Impacts on common (idle mode) signals/channels adaptation and clustering transmission(e.g., SSB, SIB1, Paging, etc.), - Impacts on UE effects (access latency and synchronization), - Applicability for TN and NTN Etc. |
| NEC | We support this proposal. Our view is that a holistic and joint optimization of network energy saving and UE power saving is a critical principle for 6G. A proper evaluation methodology must be able to assess the direct impact of network-side procedures on UE power consumption. This includes quantifying the interplay between enhanced Cell DTX/DRX on the network side and the UE's own DRX cycles. Therefore, studying a joint framework as proposed is essential. |
| Ericsson | We think that also scheduling of system information (both SIB1 and OSI) should be covered here, irrespective of capacity or coverage cell scenario. In NR, the scheduling of OSI in sequential, non-overlapping SI windows leads to spread transmissions. SIB1 repetitions are also spread in time in NR. For 6GR, we should study improvements in this area. |
| vivo | As far as we understand, cell DTX/DRX and UE DRX are connected mode procedure in 5G NR. The need of extension to idle mode should be first studied. So we suggest the following updates: Study and evaluate Cell DTX/DRX and UE DRX in IDLE mode operation, regarding, - Common (idle mode) signal adaptation and clustering, - UE effects (latency and synchronization), - Etc. Further, the concept of UE DTX operation in IDLE mode is not clear to us. |
| ZTE, Sanechips | - Besides SSB such signal can be adapted, the PRACH channel also could be adapted. - Besides latency and sync issue, UE effect also includes the impacts on measurement. - Therefore, following updates is suggested - FL Proposal 2.5-35: Study joint Cell DTX/DRX and UE DTX/DRX regarding, - Common (idle mode) signal/channel adaptation and clustering, - UE effects (e.g., measurement, latency and synchronization), - Etc. |
| Samsung | This pertains to UE idle and inactive modes, UE DTX/DRX refers to when UE receives SI and paging message and can transmit RACH. We don’t have cell DTX/DRX in UE idle for NR, so the first step can be to study cell DTX/DRX for UEs in idle and inactive modes. We suggest the following wording: Study joint Cell DTX/DRX and UE DTX/DRX in idle mode regarding, - Common (idle mode) signal adaptation and clustering (e.g., SSB, SIB, paging, RACH) transmission/reception, - UE effects (latency and synchronization), - Joint operation with UE DRX - Etc. |
| IIT Kanpur | We are open to discuss these aspects. |
| Apple | We think cell DTX/DRX should be first discussed in CONNECTED mode. Whether/how cell DTX/DRX has to be supported for IDLE/INACTIVE mode needs further investigation. One possible way is that there is no cell DTX/DRX configuration in IDLE mode, but the common signal/channels, e.g. SSB, SIB1, PRACH and Paging are designed naturally to form a clustered pattern so that energy efficiency can be achieved. We propose to update the proposal: - FL Proposal 2.5-xx: Study joint Cell schemes to achieve NW side DTX/DRX and UE DTX/DRX regarding for common signals/channels used for IDLE UEs, including, - Common (idle mode) signal/channel adaptation and/or clustering, etc - Impacts on UE effects (access latency and synchronization), should be considered - Etc. |
| Lenovo | We can keep the main bullet at high level, sub-bullets can be discussed separately as they can be configured separately too. We are fine Qualcomm wording. |
| Fraunhofer | Support. We think mechanisms to achieve joint Cell DTX/DRX and UE DRX is important in idle mode (concerning common signals/channels) and in connected mode. Both needs to be studied. |
| Tejas | Same view as Huawei. |
| OPPO | First, the proposal 2 under the company name OPPO captured in FL summary section 2.5.1 is not from our contribution. So this proposal may need to be re-allocated to a right place. Second, our proposal 13 captured in FL summary section 2.5.1 is related to SIB1 design thus should be moved to FL summary in section 2.3.1. Moreover, our proposal 17 in R1-2505761 is about our views on 6GR Cell DTX/DRX and is missing from the FL summary. We would appreciate if this proposal can be captured to reflect our views. Regarding the proposal, we think that ‘UE DTX/DRX’ and ‘joint’ are bit confusing. We don’t understand what it means. If the joint means joint energy saving, we should just say it. Study joint Cell DTX/DRX for joint network and UE energy saving and UE DTX/DRX regarding, - Common (idle mode) signal adaptation and clustering, - UE effects (latency and synchronization), - Etc. |
| Futurewei | OK in principle, not clear what does mean “latency” for the idle mode. Is it the initial access latency? If so, it should be clarified in the proposal. Suggested Change: Study joint Cell DTX/DRX and UE DTX/DRX regarding, - Common (idle mode) signal adaptation and clustering, - UE effects (paging/initial access latency and synchronization), - Etc. |

Moreover, an LP radio has been proposed by companies, e.g., for transmitting SSB or SIB1 or receiving UL WUS or requests for OD-SSB or OD-SIB1. It is the FL’s view that the RAN1 spec does not concern implementation matters like particular radio implementations. For that reason, FL proposes as a first step to study the spec impact, if any, with such a radio.

- FL Proposal 2.5-36:

Study the spec impact, if any, of a gNB implementation with an LP stage for idle mode signal support.

Companies are welcome to share their views on the above FL proposal.

| Company | View |
| --- | --- |
| InterDigital | In our view, we prefer to have the proposal in the same level with proposal 2.5-1. Given the situation, we suggest the following proposal. Study utilization of low power transmitter/receiver at gNB in IDLE mode. |
| TCL | While implementing a low-power radio stage for idle-mode signals is a promising hardware approach, we agree with the FL that RAN1 should remain agnostic to specific gNB implementations. We are cautious about dedicating standardization effort here – ideally, if a separate “LP” receiver/transmitter chain is used for SSB, SIB1 or WUS, it should function under existing spec assumptions. We support clarifying whether any spec changes are needed to enable such designs. If so, they must be minimal and justified, since introducing hardware-specific hooks could increase complexity without clear broad benefit. |
| Panasonic | We are open to discuss. |
| Qualcomm | As we discuss for the UE side, we’d rather avoid implying any implementation architecture choice here. We also do not think that the focus should be on spec impact yet, but the utility of a proposal. - FL Proposal 2.5-37: Study the spec impact, if any, of a gNB implementation with an LP stage for idle mode signal support. Study a base station low-power state for idle mode signal support |
| Fujitsu | If LP radio requires special design such as the waveform and target performance, then there will be spec impact. Otherwise, we think it can be left to gNB implementation. |
| Ofinno | Okay to study. |
| CEWiT | We are open to discuss. |
| Nokia | Do not support. This proposal is unclear, we should not be discussing gNB implementation aspects, but potential spec impacts to support particular features. Moreover, the definition of “LP stage” is unclear. |
| CMCC | Can be further discussed. |
| CATT | Agree with Qualcomm’s proposal. The low-power state for base station can be first studied before go to the spec impac. |
| NEC | Support |
| Ericsson | Not support. The concept is too vague. There is no common understanding of the function of this LP entity. Besides, we see several potential issues, like increased latency when it has to wake-up the MR (especially if it is a CAT2 BS). And the market-penetration of this type of radio will be small, if any. It will rather drive investment costs for operators investments, while the objective with network EE is to reduce costs. We should prioritize the models for widely deployed radios. |
| vivo | We are open to study this. |
| ZTE, Sanechips | We are jumping to the spec discussion which is confusing to us. At the SI beginning, we firstly need to consider whether it is feasible and necessary. For example, whether the gNB could have a low power mode but also operate with monitoring PRACH with same sensitivity. |
| IIT Kanpur | We are open to discuss these aspects. |
| Apple | We prefer to study what signals/channels may need to be transmitted/received with a LP mode first to understand how this mode works and then study whether there is a spec impact. - FL Proposal 2.5-18(update): Study the feasibility spec impact, if any, of using a LP-Tx/RX for BS gNB implementation with an LP stage mode transmission/reception for idle mode signal/channel support. |
| Lenovo | We are fine with the Qualcomm wordings |
| Tejas | Ok to study |
| OPPO | Ok for the update |
| Futurewei | The proposal is confusing, are we going to study the spec impact or the performance impact, or both? Our understanding is that if the design of SSB, PDCCH/PDSCH indicating/carrying SIB1, or UL WUS is not impacted by such a radio then it won’t lead to a spec impact, so maybe the proposal can be formulated to study a new design for common signals (e.g., SSB, UL WUS) compatible with a low power radio. |

- FL Proposal 2.5-38:

Study and evaluate anchor cell SI signaling for capacity cells.

Companies are welcome to share their views on the above FL proposal.

| Company | View |
| --- | --- |
| Google | Although understanding the intention, perhaps we should have definition of anchor cell and capacity cell first, to avoid possible different understanding across companies. |
| InterDigital | Fine |
| TCL | Cross-cell SI coordination may burden networks and confuse UE implementations (e.g. requiring UEs to monitor a different cell for essential info). We support studying feasibility and performance – any anchor-based approach must demonstrably simplify overall signaling and avoid introducing undue overhead or coverage gaps. |
| Panasonic | Okay. |
| Qualcomm | We believe this proposal is already included in Proposal 2.3-1 |
| Fujitsu | We are open to studying offloading SIs of capacity cell(s) to an anchor cell. |
| Ofinno | Is the intention to say the anchor cell SI includes all the SI of the capacity cells or more generally anchor cell SI includes information on the capacity cells? We support the latter at this stage. |
| CEWiT | We are Okay |
| Nokia | Support |
| Sharp | We are open to discuss. |
| CMCC | Generally support, and from our understanding such mechanism may but only restrict to the concept of cell, but also can be further studied from carrier/TRP-level aspect. Therefore, we suggest the following revisions: - FL Proposal 2.5-39-CMCC rev1: Study and evaluate anchor cell/carrier/TRP SI signaling for capacity cells/carriers/TRPs. |
| ETRI | Support in general. This issue seems more relevant to system information delivery in section 2.3. |
| NEC | Support |
| Xiaomi | We support the direction of jointly considering cell-DTX/DRXa and C-DRX. It is a systematic design and will impact very aspects on communication. Hence, we think the main bullet itself is sufficient at this stage. |
| Ericsson | We agree to study transmission of SI signaling from coverage/anchor cells on behalf of capacity cells. Furthermore, we think that evaluations should be done for PAGING and PRACH where for example the coverage/anchor cell pages the UE and directed to respond in either anchor or capacity cell. |
| Vivo | We need to clarify the concept of anchor cell SI signaling before agreeing this proposal. |
| ZTE, Sanechips | Open to consider but anchor cell is not clear to us. |
| Samsung | Anchor cell signaling can include signaling other than SI for cell. For example, anchor cell signaling include activation/deactivation of capacity cells Suggest the following wording: Study and evaluate power savings benefit of anchor cell SI signaling for communication on capacity cells. |
| IIT Kanpur | We are open to discuss. |
| Apple | We think this proposal can be discussed together with Proposal 2.3-1. |
| Lenovo | Are we suggesting to study usage of anchor cell in the idle mode ? Study and evaluate anchor cell SI signaling in the idle mode |
| Tejas | Can be discussed in proposal 2.3-9. |
| OPPO | We think 6G Day-1 should focus on coverage cell energy saving. Because the coverage layer is always on and cannot be switched off. While operators have already many solutions by implementation to save power for capacity cell. So we suggest that the capacity cell energy saving should be put lower priority. |
| Futurewei | It is not clear the context of the proposal. Is the proposal referring to SI signaling in SCells? What are the scenarios that this proposal considers? The FL text above refers “to anchor or coverage carriers in multi-carrier scenarios”, is it the same as the anchor cell? Should we also address whether the anchor cell and the capacity cells can be the same cell to simplify the system configuration? |

2nd round of FL comments and proposals

- The intention here is to also (at least initially) conder connected mode. For that reason, UE DTX is maintained in the proposal. For purely idle mode discussions, that would not be necessary.

- FL Proposal 2.5-40:

Study joint Cell DTX/DRX and UE DTX/DRX at least including,

- Enhancements of common signals and channels transmissions, e.g., PRACH, RAR, PUSCH, CPDCCH, WUS, …

- Impact on UE, e.g., latency and synchronization,

- Applicable RRC states

- There is a strong support for assessment of anchor cells, some changes are provided in the update to accommodate companies’ views.

- FL Proposal 2.5-41:

Study and evaluate power savings benefit of anchor cells/carriers signaling, e.g., SI, RA and paging, for capacity cells/carriers.

Models, metrics and baseline scheme(s)

Companies’ views

Nokia - R1-2505131 - Proposal 1: 6G should target meaningful energy efficiency improvements over Rel-18 for all load conditions and consider the following for NES evaluation: - Energy consumption for BS and UE. - Both data transmission/reception and other operations (e.g., monitoring, measurements, and signaling). - A metric combining performance and energy efficiency, e.g., capacity or throughput per energy unit, for different load conditions and deployment scenarios. - Proposal 17: 6G SI to use the 5G BS power model Cat.2 in TR 38.864 as a starting point for network energy saving evaluations. - Proposal 18: 6G SI to use the 5G UE power consumption model (e.g. in TR 38.840 and in TS38.869) as a starting point for UE energy saving evaluations. FUTUREWEI - R1-2505145 - Proposal 1: Adopt a comprehensive energy efficiency evaluation methodology for 6GR, which considers: - Energy consumption for all operations (e.g., data transmission/reception, monitoring, measurements, and signaling) for both BS and UE. - Energy efficiency metric combining spectral efficiency and energy consumption, e.g., bits/Joule, for different load conditions and deployment scenarios. - Proposal 2: Develop new energy efficiency metrics for 6GR, which consider new use cases, new frequency bands, and deployment scenarios, including: - Energy efficiency for sporadic traffic. - Energy efficiency for multi-band operation. - Energy efficiency for diverse device types and capabilities. - Proposal 8: Consider studying energy consumption evaluation methodologies for AI/ML-based approaches in 6G. - Proposal 9: Include energy efficiency as part of the evaluation KPIs for all 6G AI/ML-based use cases based on studied and agreed methodologies. Huawei, HiSilicon - R1-2505187 - Proposal 4: Study proper KPI for Energy-efficiency (EE) oriented 6GR system design which allows network and UE can have more opportunities for energy saving - Study energy saving evaluation based on user experience, compared with UPT. Xiaomi - R1-2505467 - Proposal 11: Discuss metrics on joint energy saving between network and UE. Ericsson - R1-2505625 - Proposal 1: 6GR should target meaningful EE improvements for all load conditions, considering the following aspects for evaluation: - Energy consumption for BS and UE, covering data transmission/reception and other operations (e.g., monitoring, measurements, and signaling). - A metric combining performance and EE, e.g., capacity or throughput per energy unit, for different load conditions and deployment scenarios. - Proposal 2: Study EE evaluation methodology for 6GR considering new use cases, new spectrum, and diverse device types: - EE for sporadic/low load traffic. - EE for multi-band operation. - EE for diverse device types (e.g., XR/AR, IoT). Tejas Networks Ltd. - R1-2505631 - Proposal 2: In 6GR, EE evaluation methodology should consider new use cases, new spectrum and diverse device types for BS and UE energy efficiency improvements. - Proposal 3: In 6GR, EE metric should combine spectral efficiency and energy consumption (e.g., bit/Joule) for different load conditions and deployment scenarios. NEC - R1-2505641 - Proposal 11: The evaluation methodology for 6G must account for the net energy impact, considering the power consumption of real-time model inference and monitoring operations within the RAN. TCL - R1-2505698 - Proposal 3: Consider whether/how to define energy efficiency parameters in 6G systems, including UE side energy efficiency or NW side energy efficiency or end-to-end (including NW+UE) energy efficiency. OPPO - R1-2505761 - Proposal 1: 6GR should adopt a comprehensive EE evaluation methodology, considering: - Energy consumption for BS and UE for all operations (e.g., data transmission/reception, monitoring, measurements, signaling, etc.). - EE metric combining spectral efficiency and energy consumption (e.g., bit/Joule) for different load conditions and deployment scenarios. Fraunhofer IIS, Fraunhofer HHI - R1-2505834 - Proposal 1: Energy saving gains at network and UE sides must be considered critical metrics, alongside traditional performance indicators such as data rate and latency. These metrics should be explicitly included in performance evaluations across several key agenda items in the 6G study. - Proposal 3: RAN1 should focus on revising the evaluation methodology for 6GR energy savings to accommodate relevant scenarios, novel technologies like AI and ISAC, and the increased flexibility provided by the foundational design aspects like initial access. - Proposal 4: Once a revised version of the 6GR power saving evaluation methodology, including the power consumption models and KPIs are finalized, all relevant agenda items should employ them for evaluation. Apple - R1-2505917 - Proposal 3: 6G should target meaningful EE improvements over Rel-18, considering: - Energy consumption for BS and UE for all operations. - EE metric combining spectral efficiency and energy consumption for various load conditions and deployment scenarios. KT Corp. - R1-2505991 - Proposal 1: Procedures for managing the energy-saving balance between NW and UE are adopted in 6GR. CAICT - R1-2506005 - Proposal 2: EE evaluation methodology for 6GR should consider energy consumption of BS and UE, and EE metric should combine spectral efficiency and energy consumption for various load conditions. ETRI - R1-2506069 - Proposal 1: For 6GR, a comprehensive EE evaluation methodology should be developed considering: - Energy consumption for BS and UE for data transmission/reception and other operations (e.g., monitoring, measurements, signaling). - EE metric combining performance and energy consumption (e.g., bit/Joule) for various load conditions and deployment scenarios. CMCC - R1-2506101 - Proposal 1: 6GR should target meaningful EE improvement, considering: - Energy consumption for BS and UE for all operations, including data transmission/reception, monitoring, measurements, and signaling. - EE metric combining spectral efficiency and energy consumption (e.g., bit/Joule) for various load conditions and deployment scenarios. InterDigital - R1-2506146 - Proposal 1: Adopt comprehensive EE evaluation methodology for 6GR, considering energy consumption for BS and UE, and EE metrics combining spectral efficiency and energy consumption for various load conditions. - Proposal 2: Study EE evaluation for new 6GR use cases, including sporadic traffic, multi-band operation, and diverse device types. AT&T - R1-2506237 - Proposal 11: Energy Efficiency metric(s) are included as 6GR key performance metrics from day 1. - Proposal 12: An energy efficiency metric based on aggregate throughput normalized by the total system power at the transmitter and/or receiver side, including power needed to operate different transmitter/receiver modules, is considered as a starting point for evaluation of energy efficiency in 6GR air interface.

Summary

The proposals for energy efficiency (EE) modeling and metrics address the limitations of 5G NR energy efficiency evaluation frameworks, which were introduced late (e.g., Rel-16 and Rel-18) and primarily focused on data transmission/reception, often neglecting energy consumption from other operations like monitoring, measurements, and signaling, particularly under diverse load conditions and emerging use cases such as AI/ML, Integrated Sensing and Communication (ISAC), and XR/AR (Nokia Obs. 1, Ericsson Obs. 1, Fraunhofer Prop. 3).

Proposals aim to establish EE as a key performance indicator (KPI) from Day 1 in 6GR, leveraging 5G models like the gNB power models Cat.1 or  Cat.2 (TR 38.864) and UE power model (TR 38.840, TS 38.869) as baselines, while introducing new metrics (e.g., aggregate throughput normalized by total system power) and methodologies that account for all gNB and UE operations, including real-time AI/ML inference, and support new spectrum (~7 GHz) and diverse deployment scenarios (Nokia Prop. 17, AT&T Prop. 12, FUTUREWEI Prop. 8). They emphasize joint NW-UE energy saving metrics, user experience-based evaluations, and end-to-end EE parameters to ensure a holistic, standardized approach, overcoming 5G’s fragmented implementation and enabling balanced trade-offs between performance and energy consumption across all 6GR use cases (Huawei Prop. 4, KT Corp. Prop. 1, TCL Prop. 3).

1st round FL comments and proposals

In order to diligently assess energy efficiency techniques, models, metrics and scenarios are needed. Regarding the models, it is FL’s view, based on companies’ inputs, that RAN1 needs to discuss whether the existing models in TR 38.840 (5G NR), TR 38.875 (RedCap), TR 38.869 (UE WUR) and TR 38.864 (gNB) need to be updated, and if so, how.

Evaluation assumptions for 6G Radio are discussed in AI 11.2. The 11.2 FL has come with the following suggestion for the continued discussion:

Observations and suggestions from moderator [of AI 11.2]: - The traffic model could be discussed as a common assumption in section Error! Reference source not found.. - The power consumption or energy efficiency for AI related use cases will be discussed in the agenda of AI/ML study. - Other assumptions, e.g., power model, metrics for energy efficiency evaluation, and baseline schemes could be discussed in the agenda of energy efficiency study.

From the above it is clear that the focus of AI 11.5 should focus on power model, evaluation metrics and baseline schemes for said evaluation metrics. Furthermore, EE FLs have agreed that the majority of power models will be discussed in the connected mode discussions. Based on this, and companies’ input, the following is proposed regarding power models:

FL Proposal 2.6-42:

Study if and how the existing UE and network power consumption models need to be enhanced for (UE) idle mode 6G radio.

Companies are welcome to share their views on the above FL proposal.

| Company | View |
| --- | --- |
| InterDigital | Fine |
| TCL | Support. Many new idle-mode features (e.g. low-power wake-up receivers, cell DTX, multi-band sleep strategies) won’t be captured by legacy 5G models. By enhancing these models, we ensure proposed techniques are assessed with realistic assumptions about idle power draw and sleep/wake overhead. Also, we think that incorporating new components (like ultra-low-power radio stages and higher-frequency bands) into the models as needed. |
| Spreadtrum | In our view, “the existing UE and network power consumption models” is not clear in the proposal. We prefer to modify this proposal into the following version. FL Proposal 2.6-43: Study if and how the existing UE and network power consumption models in TR 38.840, TR 38.864 and TR38.869 need to be enhanced for (UE) idle mode 6G radio. |
| Panasonic | Agree |
| Qualcomm | We are ok with the proposal |
| Fujitsu | We are fine with the proposal |
| Ofinno | Support |
| CEWiT | Support |
| Nokia | Support if proposal is modified to remove brackets around “UE”. The existing models can be used for UEs in idle mode, but there is no need to define a “network idle mode radio”. |
| LG Electronics | In our view, it would be better to strive for a common power consumption model for idle and connected mode, and to discuss evaluation assumptions altogether in one FL summary. |
| Sharp | Support |
| Huawei, HiSilicon | Considering new spectrum, BS/UE hardware evolution and some joint consideration of BS/UE EE, it is evident that the current modeling of both sides need update, even for IDLE mode UEs. On the other hand, it is a bit unclear how to evaluate gNB power savings with only IDLE UEs assumed. Is it to assume empty load/common signal only? For the moment, we provide a general update applicable to all scenarios. FL Proposal 2.6-44-Huawei update: Study if and how the existing UE and network power consumption models need to be enhanced for (UE) idle mode 6G radio. |
| CMCC | Support the proposal. Moreover, from our point of view, the following aspects can be further considered and studied: - Specific values of relative power, transition time and additional transition energy should be revisited considering the evolution of product at the year of 2030 (i.e. the year that 6GR is potentially ready for commercial use). - More accurate scaling method for UE power model to jointly consider RU in frequency/spatial/power domain, and further consider aspects like PDCCH blind decoding. - Consider both Tx and Rx at the same time for evaluation. - More accurate model for PA efficiency. |
| CATT | OK with the proposal. |
| ETRI | Support |
| Xiaomi | OK |
| Ericsson | Support |
| vivo | We think there is no dedicated power model for IDLE mode operation, thus suggest the following revision. Study if and how the existing UE and network power consumption models that are applicable to IDLE mode operations need to be enhanced for (UE) idle mode 6G radio. |
| ZTE, Sanechips | The power model in 38.840 and 38.864 are applied for both idle/inactive mode and connected mode. We do not think we need to restrict it to idle mode. FL Proposal 2.6-45: Study if and how the existing UE and network power consumption models need to be enhanced for (UE) idle mode 6G radio. |
| Samsung | OK |
| IIT Kanpur | We support the proposal. |
| Apple | We agree with the principle of this proposal. However, we think we need to consider in general what needs to be enhanced for UE and NW power models, not only limited to IDLE mode. Huawei’s updated version is preferred. |
| Lenovo | ok |
| Fraunhofer | Support |
| Tejas | Same view as Huawei |
| OPPO | We are fine with the proposal |
| Futurewei | Not sure how to understand the proposal. Will there be another similar study for the connected mode? How will they be combined given that a cell will support UEs in both idle and connected modes |

Regarding the evaluation metrics, and taking the discussion in AI 11.2 into consideration, it is FL’s understanding from 11.2 that the intention is to evaluate relative metrics compared to a baseline scheme. Additionally, in idle mode, it is not really relevant to discuss detailed transmission-centric metrics like bits/joule but more fundamental properties like total power consumption, duty cycle, deep sleep duration, UE access latency (in relation to energy consumption) etc. For that reason, the following is proposed for evaluation metrics:

FL Proposal 2.6-46:

Study idle mode energy efficiency metrics for UE EE, network EE, and joint UE and NW EE.

Companies are welcome to share their views on the above FL proposal.

| Company | View |
| --- | --- |
| InterDigital | In our reading, energy efficiency related metrics are proposed to reflect both system performance (e.g., throughput) and energy consumption/saving. However, as this discussion is for IDLE mode, we are not sure that we can consider system performance in this discussion. It may be better to discuss in CONNECTED mode discussion. |
| TCL | We support developing metrics for UE-side, network-side, and joint UE–NW energy efficiency. These could include average idle power consumption, duty cycle (sleep ratio), wake-up latency penalties, or an end-to-end energy score. |
| Spreadtrum | We are fine with the proposal. |
| Panasonic | Agree. |
| Qualcomm | We support the direction, but we would like to have more discussion on what a metric for joint energy looks like before agreeing to the proposal. |
| Fujitsu | We are fine with the proposal |
| Ofinno | Support |
| CEWiT | We are fine to study defining metrics for UE EE, network EE, and joint UE and NW EE. For e.g., relative network energy saving gain w.r.t. baseling can be a metric for network EE. |
| Nokia | Support, it is important to have clear evaluation metrics so that energy efficiency impact of different proposals for 6GR can be evaluated later on. |
| LG Electronics | As commented in Proposal 2.6-1, it would be better to strive for a common power consumption model for idle and connected mode, and to discuss evaluation assumptions altogether in one FL summary. |
| Sharp | Support |
| Huawei, HiSilicon | We are generally not in favor of the proposal since idle mode for UE does not necessarily mean e.g. empty load of BS. Perhaps a general proposal is sufficient and more proper. For more details, the metrics for both energy and performance should be studied. For energy evaluation, energy consumption/energy efficiency can be considered. For performance, instead of using latency/UPT, we propose a new one, i.e., the QoS based metric. As explained in our contribution, the user QoS satisfaction is a common and generic performance metric to justify how the system works for user experience. FL Proposal 2.6-47 – Huawei update: Study idle mode energy efficiency metrics for UE EE, network EE, and joint UE and NW EE, including energy consumption/energy efficiency, QoS based metric, and etc. |
| DCM | support |
| CMCC | Support. For joint NW and UE energy saving, we suggest to further study the framework for supporting features that are bilateral beneficial for both network and UE. For example, some features can be supported as a package and enabled together, e.g. UE wake up BS signal and some UE power saving features. |
| ETRI | Support |
| Ericsson | Support |
| Xiaomi | Fine For Idle mode, as there is no UE data transmission, power consumption and power saving gain could be the candidate metrics. |
| vivo | We agree to study energy efficiency metrics for UE and network. However, for current wording on joint UE and NW EE, it sounds like there will be dedicated metrics to joint UE and NW EE. Before study, we are not sure such joint metrics are realistic. UE EE metrics and network EE metrics can also work together to reflect the joint UE and NW EE, and thus, we suggest a more general way as below: FL Proposal 2.6-48: Study idle mode energy efficiency metrics for UE EE, and network EE., and joint UE and NW EE. |
| ZTE, Sanechips | Energy efficiency is not clear to us, if we do not have the definition, how we could study? |
| Samsung | OK |
| IIT Kanpur | Support |
| Apple | We think both IDLE and CONNECTED mode needs to be considered. Currently, we do not see the need to differentiate EE metrics for different UE modes. |
| Lenovo | ok |
| Fraunhofer | We are open to discuss. It is important, however, to clarify the scope and usefulness of the joint UE and NW metric. In addition to EE, we should consider energy consumption, energy saving gains and QoS-based metric as suggested by Huawei. |
| Tejas | Support |
| OPPO | We are fine with the proposal |
| Futurewei | Open to discuss. |

Finally, RAN1 needs to agree on baseline scheme(s) for the evaluation. In FL’s understanding, this may include default NW and UE configurations, network load, deployment type, frequency ranges etc.

FL Proposal 2.6-49:

Study relevant baseline schemes for network and UE energy efficiency assessment, including

- Network and UE configurations,

- UE traffic types,

- Network load (in the range from empty to high),

- Network deployment, e.g. single carrier, multi-carrier

- Frequency ranges FR1, FR2, FR3

- etc.

Companies are welcome to share their views on the above FL proposal.

| Company | View |
| --- | --- |
| Google | We think UE types should also be considered, given that diverse UE types would be introduced in 6GR Day-1. In addition, we can also consider UE status. For example, if UE is under overheating situation, the power usage status would be impacted. |
| InterDigital | Fine |
| TCL | Actually, EE features should not undermine meeting QoS targets, security, or other key performance objectives. We are confusing what is UE traffic types, is there any relationship with time delay or data rate or reliability? |
| Spreadtrum | We are fine with the proposal. |
| Panasonic | Is this for both RRC modes or only for IDLE mode? UE traffic types only apply to connected mode. |
| Qualcomm | We propose to also capture total energy over a day to better reflect the total gains from energy savings designs. Study relevant baseline schemes for network and UE energy efficiency assessment, including - Network and UE configurations, - UE traffic types, - Network load (in the range from empty to high), - A combination of network load values. - Network deployment, e.g. single carrier, multi-carrier - Frequency ranges FR1, FR2, FR3 |
| Fujitsu | We are fine with the proposal |
| CEWiT | We are fine with proposal in general. However, we are not clear whether the UE traffic type is meant for inactive UEs. |
| Nokia | Support, though one should not refer to FR3 here as there is no such FR defined in 3GPP. |
| LG Electronics | OK with the proposal |
| Sharp | We are fine with the proposal. |
| Huawei, HiSilicon | This proposal in general does not fit IDLE UEs or empty load gNB. And we do not see how the sub-bullets are relevant to “schemes” – is the intention to discussion baseline evaluation assumptions? Nevertheless, several other comments: - For IDLE UEs, instead of UE configurations, the basic UE capabilities/UE types are more relevant. - FR3 is not yet defined. FL Proposal 2.6-50 – Huawei update: Study relevant assumptions schemes for network and UE energy efficiency assessment, including - Network and UE configurations/capabilities, - UE traffic types, - Network load (in the range from empty to high), - Network deployment, e.g. single carrier, multi-carrier - Frequency ranges FR1, FR2, and other spetrumFR3 - etc. |
| CMCC | Support, and we think multi-TRP can also be included in Network deployment. FL Proposal 2.6-51-CMCC rev1: Study relevant baseline schemes for network and UE energy efficiency assessment, including - Network and UE configurations, - UE traffic types, - Network load (in the range from empty to high), - Network deployment, e.g. single carrier, multi-carrier, multi-TRP - Frequency ranges FR1, FR2, FR3 - etc. |
| ETRI | Fine with the proposal. |
| Ericsson | We can also consider diverse device types. |
| Xiaomi | Similar question as Panasonic. |
| vivo | Since there will be different baseline schemes for network and UE involve different aspects to be studied, we suggest separate this proposal for UE and gNB respectively. In addition, it should be clarified if the study of this proposal includes IDLE mode operation only or both IDLE and CONNECTED? |
| ZTE, Sanechips | Baseline assumption or scheme is confusing. For example, if both single carrier and multi-carrier deployment are considered, which one is the baseline? So, if the intention of the proposal is to discuss the baseline assumption, then we can just choose one assumption as baseline. If the intention of the proposal is to capture the evaluation assumptions, then we can list all the relevant items. Based on the later way, we have the following updates FL Proposal 2.6-3: Study relevant baseline assumptionsschemes for network and UE energy efficiency assessment, including - Network and UE configurations, - UE traffic types, - Network load (in the range from empty to high), - Network deployment, e.g. single carrier, multi-carrier, multi-TRP, RIS - Frequency ranges FR1, FR2, FR3 - etc. |
| Samsung | 1. Suggest to replace “schemes” by “scenarios” 2. To be generic, just having ‘traffic types’ should be enough. ------------- Study relevant baseline schemes scenarios for network and UE energy efficiency assessment, including - Network and UE configurations, - UE traffic types, - Network load (in the range from empty to high), - Network deployment, e.g. single carrier, multi-carrier - Frequency ranges FR1, FR2, FR3 - etc. |
| IIT Kanpur | We support the proposal in general. However, in IDLE/Inactive mode, it is not clear what UE traffic type mean. |
| Apple | Generally fine with the proposal. The last bullet ‘’ Frequency ranges FR1, FR2, FR3’’ is already included in the first bullet. |
| Lenovo | Generally fine with the proposal, mention ‘combination of UE traffic types’ |
| Fraunhofer | Support and agree with Ericsson to add diverse device types. |
| Tejas | Same view as Panasonic |
| OPPO | We are fine with the proposal |
| Futurewei | Open to discuss. In our opinion, for the assessment/evaluation this group should coordinate (be consistent) with evaluation methodology AI assumptions. |

2nd round of FL comments and proposals

Companies are generally in agreement with the first round of FL proposal. Some minor changes are included in the update.

FL Proposal 2.6-52:

Study if and how the existing UE and network power consumption models in TR 38.840, TR 38.864 and TR38.869 need to be enhanced for (UE) idle mode 6G radio.

Similarly, minor updates for the below proposal.

FL Proposal 2.6-53:

Study idle mode energy efficiency metrics for UE EE, network EE, and joint UE and NW EE.

As pointed out in the first comment, UE traffic patterns will be discussed in AI 11.2. For that reason, it is overstricken here (i.e., to be deleted) but companies’ comments are appreciated.

FL Proposal 2.6-54:

Study relevant baseline schemes for network and UE energy efficiency assessment, including

- Network and UE configurations and capabilities,

- UE traffic types,

- Network load (in the range from empty to high),

- A combination of network load values.

- Network deployment, e.g. single carrier, multi-carrier, multi-TRP, etc.

- Frequency ranges FR1, FR2, cmW

Omitted topics

It is the FL’s understanding that the following topics that have been discussed among contributions will be discussed elsewhere and for that reason will not be discussed in AI 11.5:

- Waveforms, that will be specifically discussed in AI 11.3.1, and

- AI/ML, that will be discussed in AI 11.6.

Discussions (Part 2)

Energy Efficiency Related Metrics

Energy Efficiency

Multiple companies support defining energy efficiency metrics based on data rate and power consumption. [MediaTek], [Ericsson], [AT&T], [InterDigital], [Fraunhofer IIS/HHI], [MediaTek] , et al., emphasize the importance of energy efficiency metrics as critical KPIs alongside traditional performance indicators. [TCL] and [FUTUREWEI] reference the IMT-2020 definition of bits per Joule. [Xiaomi], [OPPO], and [vivo] stress the need for joint consideration of network and UE energy efficiency. Based on companies’ inputs, it is suggested to define a unified EE metric that can be utilized for BS or UE. To consider reuse/extend 3GPP power consumption model, “3GPP unit” instead of “Walt/Joule” is utilized. Also to include consideration of latency requirement when counting the delivered information amount, as highlighted in [Huawei], the following proposal is suggested:

Proposal 3.1.1-1:  Define the energy efficiency (EE) metric as total valid bits subject to traffic latency requirement over energy consumption, evaluated over a common duration T.

- EE ≜ Average data rate (Mbps) / Average power consumption (3GPP unit)

- FFS: T definition (simulation time vs. traffic-latency-dependent window)

| Company | Support? (Y/N) | Comments/Revisions |
| --- | --- | --- |
| LG Electronics |  | Our preference is to define average power consumption as metric, as we did in NR. Companies can report throughput and latency in addition. |
| CMCC |  | We can understand the intention for FL and companies to have such EE definition, but it seems 1% average data rate here equals to 1% power consumption, is it feasible to do the direct calculation like this? Also note that currently both RAN 6G SI and TPR for IMT-2030 are discussing EE definition. We prefer to have a unify design between RAN1 and RAN (and ITU-R, if possible:)), that is: - The relative target of certain metric (e.g. threshold value for the metric, or certain performance loss compared with the benchmark) can be set. Constant like certain percent (e.g. 90%) of the UE fulfils the target should be achieved while evaluating the relative power saving for each load case. In addition, since different service may sensitive to different type of metric, it is recommended that the certain metric can be selected according to the service type, for instance, latency for Vo6G (i.e. VoNR-like) service, latency or data rate for XR service, QoS or UPT for burst-buffer service. |
| Fujitsu | Y |  |
| Spreadtrum |  | It is unclear what the “total valid bits subject to traffic latency requirement over energy consumption” means. For definition of T, we need further discussion and “(simulation time vs. traffic-latency-dependent window)” should be removed. |
| Panasonic |  | To clarify our understanding, the reading of the physical meaning is like bits/per energy unit and then divided by average packet latency? |
| Xiaomi |  | We are fine to define the energy efficiency (EE) metric for connected mode evaluation, but for this proposal, we have the following questions: 1. Does the valid bits mean the bits successfully decoded? A clear definition may be needed. 2. The definition of “traffic-latency-dependent window” is not clear, we prefer to remove the whole bracket of the FFS part for now. |
| vivo |  | Clarification is needed for the following - How to understand the total valid bits subject to traffic latency requirement? - What is the traffic-latency-dependent window? |
| Nokia |  | We think that the metrics used in past studies should be used at least initially and then developed if needed for certain cases. |
| Ericsson |  | UPT shall not be the only measure. It depends on what type of traffic it is, and therefore the proper KPI shall be considered per service type. We need more discussion before agreement. |
| InterDigital |  | We are fine with defining energy efficiency metric, but would like to be careful for the decision. As this is the first meeting, it is too early to finalize the definition. |
| Apple |  | We wonder which way is better: EE as defined here, or energy per bit (Eb=Average power consumption (3GPP unit)/Average data rate (Mbps)). If we use energy per bit, the relatively energy saving gain would be defined as (1 - Eb_with_scheme / Eb_baseline), which is similar to the power saving gain that has been used so far in NR UEPS and NES evaluations. We also wonder if comparing the energy consumption directly for different schemes under the same load would be sufficient already. We are open to hear views from companies. “Subject to traffic latency requirement” may not be necessary here. We will need to discuss the exact performance requirements, which may be different for different service types. |
| Qualcomm | N | It is not clear why we need to include data rate here. It also is not clear that the equation corresponds to the proposal. The proposal states that the valid bits are subject to latency requirement, which would be a binary pass/fail. The equation scales with the actual latency. Finally, it is not clear how this would apply to cases where PDCCH is monitored without data transmission. |
| Huawei, HiSilicon | Clarification needed | For fair comparison of different solutions, the total amount of data (to be transmitted in the simulation duration time T) should be fixed and the duration T should also be common, which means the average data rate is fixed. Then the only variable would be the average power consumption. Direct use of average power consumption seems natural. In addition, we recommend to compare EE or average power consumption for different energy saving techniques, with a given user’s QoS satisfaction rate. |
| Lenovo | N | Metric from 5G NR TR38.864 is taken as baseline to evaluate BS power consumption and we can discuss what is the cons and further improvement needs. |
| ZTE, Sanechips |  | Firstly, we need to consider whether/how to use the 5G metrics, e.g., power saving gain, latency, UPT or some other metrics. Secondly, we think for some cases, the EE metric can not reflect whether some schemes are beneficial or not. For example, for a delay tolerant UE, e.g., tolerant up to 80ms, with LP-WUS scheme, the latency may be increased from 1ms to 40ms due to wake-up delay. In this case, the latency is increased, and the average data rate is decreased, and then EE may be decreased. However, the UE can tolerant this and no much impact. Therefore, we think we should take this into account. |
| CEWiT |  | We share views with LG. Our preference also is to define average energy consumption as metric, as was done in NR. Further throughput and latency can be reported in addition. |
| Samsung |  | Need to be consistent with on-going RAN discussion for TPR |
| Fraunhofer | N | We share concerns raised by Qualcomm. Especially when there is no data to transmit with only the common signals and the sleep states in between, how would this metric represent energy consumption? As Lenovo and others pointed out, the metric from TR38.864 should be the basis. |
| TCL |  | It seems EE definition in bullet 1 could be baseline to further study. In our understanding, some significant metrics could be combined with EE to further define, e.g., energy density for whole system during different time periodicity, and latency for URLLC traffic, UPT, sleep mode power consumption with monitoring function, etc. |
| OPPO |  | We need to clarify if the EE value would be simulated by system level simulation or not. The data rat is highly related to the geometry for both UE side and Network side. The detail methodology would behind the metrics should also be determined. |
| Vodafone | Clarification needed | Agree to specify a energy efficiency metric. Is there a common understanding that this metric is to be used across all different areas of study in 6GR and not only limited to energy saving techniques? |
| Futurewei |  | To clarify, are the bits both control and data bits? Will each company make some assumptions on the control size necessary to transport some data bits and then report the results? |
| DCM |  | We are OK with the current formula as a starting point. At the same time, we are wondering if the average TP is always the best KPI to express EE because, as long as service requirements are met, not so high TP is necessary for e.g., non-real time and light weight service. |
| Tejas |  | Definition of valid bits needs to be clarified. |
| AT&T |  | EE metric may differ based on the aspect discussed. We are open in general to the definition of EE metric(s) that can be used for multiple scenarios/ feature evaluation. Also whether the metric is agreed in 11.2 agenda or in EE agenda needs further guidance from the chair |

Moderator thanks companies’ valuable feedback. On this topic, the following are identified issues and suggested proposal revision for further consideration (next meeting):

Issues Identified:

- Metric Definition Concerns: Multiple companies ([LG Electronics], [CMCC], [Qualcomm], [Huawei, HiSilicon], [Lenovo], [Fraunhofer], [CEWiT]) expressed reservations about the proposed EE metric, with key concerns including:

  - Preference for average power consumption as the primary metric, similar to NR, with throughput and latency reported separately [LG Electronics, CEWiT]

  - Feasibility of direct calculation where 1% data rate equals 1% power consumption [CMCC]

  - Need for alignment with ongoing discussions in RAN and ITU-R [CMCC, Samsung]

  - Ambiguity of "total valid bits subject to traffic latency requirement" and "traffic-latency-dependent window" [Spreadtrum, Xiaomi, vivo]

  - Applicability in scenarios with no data transmission [Qualcomm, Fraunhofer]

  - Fixed average data rate in simulations making average power consumption the key variable [Huawei, HiSilicon]

- Service-Specific Requirements: [CMCC], [Ericsson], and [Apple] emphasized that metrics should be service-dependent, avoiding single-metric decisions across diverse services

- Alternative Metric Proposals: [Apple] suggested considering energy per bit (Eb) for better comparison with existing power saving gain metrics, while [Lenovo] and [Fraunhofer] supported alignment with existing NR metrics (e.g., TR 38.864)

- Control-Only Scenarios: [Qualcomm] highlighted the need to handle cases with control-only monitoring (no data transmission)

Revised Proposal:

Study to define complementary energy efficiency metrics for 6G, considering the following aspects:

- Primary metrics: Energy per bit (Eb) = Average power consumption (3GPP unit) / Average data rate (Mbps), and Energy efficiency (EE) = Average data rate (Mbps) / Average power consumption (3GPP unit)

  - Alignment with metrics used in past 3GPP studies (e.g., average power consumption in TR 38.864)

  - Consistency with EE definitions in other relevant bodies (e.g., ITU-R)

  - Service KPIs (e.g., latency, UPT) reported alongside metrics

  - Valid bits defined as payload bits successfully delivered under applicable service QoS constraints

  - Common averaging duration T without coupling to latency windows

- FFS: control-only monitoring cases

[AT&T], [Fraunhofer IIS/HHI], et al., support defining comparative energy-efficiency gain (EEG) metrics across baselines to enable scheme comparisons. Since the “3GPP” units for BS and UE are actually different, defining relative EEG also helpful for checking the degree of EE improvement on BS and UE.

Proposal 3.1.1-2: Define energy efficiency gain (EEG) for BS and UE relative to a baseline setting:

- EEG_X = (EE_with_scheme_X / EE_baseline_X − 1) × 100, where X ∈ {BS, UE}

- FFS: Baseline configurations for BS/UE.

| Company | Support? (Y/N) | Comments/Revisions |
| --- | --- | --- |
| LG Electronics | Y |  |
| CMCC | Y | Generally fine |
| Fujitsu | Y |  |
| Panasonic | Y |  |
| Xiaomi |  | Whether the EEG is the only metric for the scheme comparison? If so, SLS is the only evaluation method, other methodology, e.g., numerical analysis used in 5G phase is precluded in some sense. For baseline setting, we wondering how to consider the existing 5G power saving schemes, should these schemes be a part of the comparison baseline? |
| vivo |  | We think this proposal is straightforward, but may not be needed as EEG definition is clear. The more important discussion is the FFS Baseline configurations for BS/UE. We suggest directly discussing the Baseline configurations |
| Nokia | Y |  |
| Ericsson |  | In general we think that a relative metric like this is good, but this proposal depends on 3.1.1-1, and we want to discuss that more (see comment to 3.1.1-1). |
| InterDigital |  | The definition itself may be straightforward, but no need to hurry. The definition can be agreed with the EE definition when the definition is ready to be approved. |
| Apple |  | Please see comment for P2.1-1. |
| Qualcomm | Y |  |
| Huawei, HiSilicon |  | With our understanding of Proposal 3.1.1-1, it is equivalent to the power saving gain. |
| Lenovo | Y |  |
| ZTE, Sanechips |  | Based on energy efficiency definition discussion. We do not think EE should be the only metric. |
| CEWiT | Y | Fine |
| Samsung |  | First, need to stabilize above Proposal 3.1.1-1 |
| TCL | Y |  |
| OPPO | Y | We can consider the NR as baseline, could be FFS for exact release. |
| Vodafone | Y | Although it depends on Proposal 3.1.1-1 |
| Futurewei |  | Let’s agree on the metric first. Open to discuss. |
| DCM | Y | Support |
| Tejas | N | The definition of EE_baseline_X need clarification. |
| AT&T |  | Baseline EE needs further clarification |

Moderator thanks companies’ valuable feedback. On this topic, the following are identified issues and suggested proposal revision for further consideration (next meeting):

Issues Identified:

- Dependency on EE Metric: [Ericsson], [Samsung], and [Futurewei] noted that this proposal depends on the definition of the EE metric in Proposal 2.1-1, which is not yet agreed upon

- Baseline Definition Requirements: [Xiaomi], [vivo], [Tejas], and [Ericsson] emphasized that baseline configurations for BS/UE need to be discussed and defined, including relevant NR power-saving features with release alignment as FFS

- Evaluation Methodology Scope: [Xiaomi] and [ZTE, Sanechips] questioned whether EEG would be the only metric, suggesting it should not preclude other evaluation methodologies like numerical analysis

- NR Baseline Alignment: [OPPO] and [CMCC] stressed the importance of aligning with NR baseline including existing power-saving features

Revised Proposal:

With EE metric(s) defined, the energy efficiency gain (EEG) for BS or UE is defined relative to agreed baseline settings:

- EEG_X = (EE_with_scheme_X / EE_baseline_X − 1) × 100, where X ∈ {BS, UE}

- EEG is used together with Eb/EE and service KPIs, as accompany metric

Note the baseline is agreed to be further studied as per the agreement below:

Agreement

- Study baseline BS setting(s) for evaluating 6G BS EE improvement/impact, considering NR features and 6G BS reference configuration(s)

- Study baseline UE setting(s) for evaluating 6G UE EE improvement/impact, considering NR features and 6G UE reference configuration(s)

[AT&T], [Ericsson], [Xiaomi], [OPPO], [CMCC], [vivo], [FUTUREWEI], [Nokia], [Huawei], [CATT], [Samsung], [Ericsson], [NTT DOCOMO], [Apple], [Fujitsu], [ZTE], [InterDigital] strongly support joint consideration of network and UE energy efficiency. There are some proposals trying to define a single joint EE metric for both BS and UE, but moderator suggests to report EEGs for both BS and UE as starting point:

Proposal 3.1.1-3:  To study joint EE improvement for given EE optimizing scheme(s), both EEGBS and EEGUE are considered.

- FFS: Whether/how to define a joint EE metric for both BS and UE

| Company | Support? (Y/N) | Comments/Revisions |
| --- | --- | --- |
| LG Electronics | Y |  |
| CMCC | Y | Generally fine |
| Fujitsu | Y |  |
| Panasonic | Y |  |
| Xiaomi | Y |  |
| vivo |  | First of all, we do not think define the joint EE is realistic, considering following: - The power unit difference between BS and UE, they cannot be aligned - Different number of UEs with different service requirements served by one BS. Assuming there is no joint EE definition, when use separate EEGBS and EEGUE, how to understand the joint EE improvement and what is the criterion to determine there is improvement if the EE is separate between BS and UE.for example, if the solution provide 20% gain for UE EE and 10% loss for BS, do we consider this is joint EE improvement or loss? |
| Nokia |  | At this stage we do not see the need for a joint metric. The issue with any joint metric, is the relative weighting of BS and UE EE metrics is arbitrary and therefore not helpful. Therefore, it is better to consider separate metrics for BS and UE. |
| Ericsson |  | See comment on 2.1-1 |
| Google | Y |  |
| InterDigital |  | Fine to consider the joint metrics, but it can be agreed with the EE definition when the definition is ready to be approved. |
| Apple |  | Generally OK with the direction. |
| Qualcomm | Y |  |
| Huawei, HiSilicon |  | When stating “both EEGbs and EEGue are considered”, we interpret that both EEG should be used for a same evaluation. However, we agree that solutions beneficial to both sides are aimed, where the benefits on one side may not be EE for some cases, for example, it might be complexity reduction, coverage etc. Thus mandating the use of both EEGs may not be optimal. For the joint EE metric, discussion could be diverse on how to define the weight between BS EE and UE EE for different use cases, for which we tend to avoid. |
| Lenovo |  | Relative power values forBS and UE are quite different and hence a joint metric is difficult to obtain. Discuss whether possible to obtain joint metric should be the proposal, since it not clear how to obtain it. |
| ZTE, Sanechips |  | We also don’t think joint EE metric is realistic. The weight for UE side or NW side can not converge. |
| CEWiT | Y | We comply with moderator suggestion to report EEGs for both BS and UE independently |
| Samsung |  | OK to consider both network and UE. However, as any UE power savings scheme is up to the NW to deploy and a NW will not do so if NES takes a “significant” hit, it will be good to also discuss prioritizations in case scheme 1 has larger EEGBS and smaller EEGUE and scheme 2 is the reverse. |
| TCL | Y |  |
| OPPO | Y | Would be better to have to explore the energy consumption model with the Joint energy state of UE and BS. |
| Vodafone |  | It is more transparent for the evaluation and for the comparison of different techniques to have separate metrics for UE and gNB. |
| Futurewei |  | We want to make sure that the same the metrics are captured for UEs in both RRC Idle and Connected modes. |
| DCM | Y | Support |
| Tejas | Y | Support |
| NEC |  | We also agree with some other comments that we do not see the need for this joint metric. We should focus on utilizing separate EE metrics for UEs and the network, rather than defining a single joint EE metric. |

Moderator thanks companies’ valuable feedback. On this topic, the following are identified issues and suggested proposal revision for further consideration (next meeting):

Issues Identified:

- Feasibility of Joint Metric: Multiple companies ([vivo], [Nokia], [Huawei, HiSilicon], [Lenovo], [ZTE, Sanechips], [NEC]) expressed doubts about the feasibility and necessity of a joint EE metric for BS and UE, citing:

  - Different power units making direct combination challenging

  - Arbitrary weighting between BS and UE components

  - Difficulty in creating meaningful single joint metrics due to units and weighting issues

- Improvement Criteria: [vivo] raised the important question of how to determine joint improvement when one entity's EE improves while the other's degrades

- Prioritization and Trade-offs: [Samsung] suggested that discussions on prioritization are needed when different schemes have conflicting EEG for BS and UE

- Reporting Preference: [CEWiT], [Samsung], and [Vodafone] preferred reporting separate BS/UE benefits and targeting Pareto improvements rather than forced joint metrics

Revised Proposal:

For joint EE improvement studies, at least report both EEG_BS and EEG_UE for the same evaluation

6G Power Consumption Models

6G BS and UE power consumption models

[Nokia], [Samsung], [Ericsson], [FUTUREWEI], [Fraunhofer IIS/HHI], [MediaTek], et al. support reusing and updating BS power models from TR 38.864. [Samsung] and [MediaTek] specifically propose adding Set 4 FR3 configuration. [Lenovo] suggests considering wider bandwidth (200MHz) and increased Tx & Rx RUs (256) for XL-MIMO operations. With the above inputs, moderator suggests the following proposal to proceed:

Proposal 3.2.1-1: Study how to reuse and update reference configurations in TR 38.864 for 6G BS, including adding new Set 4 FR3 configuration.

- Discuss and decide the yellow marked values in the below table in RAN1#122-bis meeting

|  | Set 1 FR1 | Set 2 FR1 | Set 3 FR2 | Set 4 FR3 |
| --- | --- | --- | --- | --- |
| Duplex | TDD | FDD | TDD | TDD |
| System BW | 100 MHz | 20 MHz | 100 MHz | 200 MHz |
| SCS | 30 kHz | 15 kHz | 120 kHz | 30 kHz |
| Number of TRP | 1 | 1 | 1 | 1 |
| Total number of DL TX RUs | 64 | 32 | 2 | 256 |
| Total DL power level | 55 dBm | 49 dBm | 33 dBm | TBD |
| Total number of UL Rx RUs | 64 | 32 | 2 | 256 |

| Company | Support? (Y/N) | Comments/Revisions |
| --- | --- | --- |
| Ofinno |  | Given the discussion that happened during the frame structure AI it may be better to postpone the FR/set discussion |
| LG Electronics |  | OK in general. However, some values (e.g., SCS, # TXRUs) can be decided after RAN1#122-bis since they are related with other agenda items’ decision. |
| CMCC |  | Generally fine with the structure, but a small issue here: whether U6G (i.e. 6.425GHz-7.125GHz) is for FR1 or FR3? From our knowledge it belongs to FR1 according to TS 38.104 |
| Fujitsu | Y | Although we understand that alignment across different agenda items is required, the discussion can be conducted in parallel, considering that this agenda item has only been scheduled for three meetings. |
| Spreadtrum | Y |  |
| Panasonic | Y | We propose to prioritize just one set for each of FR1/2/3 is helpful for further investigation. |
| CATT | Y |  |
| Xiaomi | Y with comments | The table in the proposal should be the starting point. Rows and columns can be added, if any. |
| vivo |  | What is FR1 range needs to be clarified, e.g. whether 6425-7125MHz is included in FR1 or not |
| Nokia | Y | OK to reusing Set 1-3 from TR38.864 and to add Set 4. The values can be discussed in next meeting e.g. if there is need to update set 1-3 and what values to use for set 4 (omitted for time being). |
| Ericsson | Support with modification | Support with following modifications: - Set 4 FR3, Total number of DL TX RUs should be “TBD” instead of 256. - Set 4 FR3, Total number of UL RX RUs should be “TBD” instead of 256. |
| InterDigital | Y |  |
| Apple | Y |  |
| Qualcomm |  | For Set 4, we’d like to consider 400MHz instead of 200MHz bandwidth, and 60kHz SCS instead of 30. On the number of antennas and DL power, our understanding is that those will be based on the discussions in Agenda Item 11.2. We submitted our energy evaluation assumptions under the evaluation assumption agenda (11.2) in R1-2506217. There we provided input on UE and BS power models |
| Huawei, HiSilicon | Minor | On “system BW” row, FR3 is not officially defined yet. Besides, suggest [100MHz, 200MHz] for further discussion/down-selection. Reason: the max BW for FR2 is 400M while the system BW in the reference configuration for FR2 was defined as100MHz in Rel-18 NES, due to the reason that scaling can be used. Thus, system BW is not necessarily same as the max BW in a FR. |
| Lenovo | Y |  |
| ZTE, Sanechips | Y |  |
| CEWiT | Y | We are ok to discuss the proposal starting from RAN1#122 |
| Samsung | Y |  |
| Fraunhofer | Y |  |
| TCL |  | Some parameters needs to be carefully checked especially for FR 3 setting. |
| OPPO | Y |  |
| Vodafone |  | We propose to also have a new set for the evaluation of SBFD base station and ISAC base station, as the extra hardware components required to perform both techniques may generate extra energy consumption |
| DCM | Y | Some of the parameters, such as numerologies, system BW should be discussed in different agenda, so these parameters should be referred. |
| Tejas | Y | We support |
| NEC | Y | We support having new additions for the FR3 evaluation, however some of the aspects may be covered by 11.2 (e.g. DL TX RUs), so it may be more pragmatic to wait for progress of 11.2 before deciding on such parameters. |
| AT&T | Y | Support, agree with Ericsson’s modifications to set #ports for FR3 to FFS |

Moderator thanks companies’ valuable feedback. On this topic, the following are identified issues for further consideration (next meeting):

Issues Identified:

- FR3 Definition and Frequency Range: [CMCC] and [vivo] questioned the definition of FR3 and whether the 6.425-7.125GHz band belongs to FR1 or FR3, requiring alignment with related agenda items

- Parameter Values for Set 4 FR3: Multiple companies identified that several parameters need to be TBD or aligned with other agenda items:

  - [Ericsson] and [AT&T] suggested that the number of TX/RX RUs should be TBD

  - [Qualcomm] proposed different values for bandwidth (400MHz) and SCS (60kHz)

  - [Huawei, HiSilicon] suggested a range for system BW ([100MHz, 200MHz])

  - [LG Electronics], [DCM], and [NEC] noted that parameters like SCS and number of TXRUs should be decided after progress in other agenda items

- Interdependency with Other Agenda Items: Strong consensus that parameters interdependent with other AIs should be marked TBD and aligned in AI 11.2 before finalization

- Additional Configuration Sets: [Vodafone] proposed adding new sets for SBFD and ISAC base stations to reflect additional hardware energy requirements

After further online discussion, companies’ think to agree to consider TR38.864 as starting point for further characterizing 6G BS reference configurations:

Agreement

Study how to reuse and update reference configurations in TR 38.864 for 6G BS.

[Nokia], [FUTUREWEI], [Ericsson] support reusing the BS power consumption formula from TR 38.864. [Nokia] specifically recommends using Cat.2 model as starting point, and [Vodafone], [Lenovo], [Rakuten], [NEC] suggests modeling of PA/PAPR effects. With the above considerations, moderator would like to suggest the following proposal:

Proposal 3.2.1-2: Study how to reuse and update the formula of BS power consumption in TR 38.864 for 6G BS, including aspects BS Cat.1/Cat.2 definition or selection/prioritization, PA/PAPR effects, etc.

| Company | Support? (Y/N) | Comments/Revisions |
| --- | --- | --- |
| LG Electronics | Y |  |
| CMCC | Y | We support to reuse the general structure of BS power consumption model while: - Revisit specific values of relative power, transition time and additional transition energy considering the evolution of product at the year of 2030 (i.e. the year that 6GR is potentially ready for commercial use). - Consider more accurate model for PA efficiency, if any. |
| Fujitsu | Y |  |
| Spreadtrum | Y |  |
| Panasonic | Y | We support to prioritize one BS category or merging both categories to make the picture clearer. |
| CATT | Y |  |
| Xiaomi | Y |  |
| vivo | Y | We think this study should be prioritized within the 3 meetings before initial access study starts. |
| Nokia |  | Suggested update: Study how to reuse and update the model of formula of BS power consumption in TR 38.864 for 6G BS, including aspects BS Cat.1/Cat.2 definition or selection/prioritization, PA/PAPR effects, etc. To simplify the evaluation effort, we could attempt to conclude a single BS category |
| Ericsson | Y | For BS modelling, Category 1 in TR 38.864 should be the baseline for 6GR evaluations. Category 1 reflects future-oriented BSs with faster and deeper transitions into energy-saving states, aligned with advanced and flexible 6G deployments (which is expected to be after 2030). By contrast, Category 2 assumes slower and less efficient transitions, which are not representative of anticipated 6G HW capabilities. |
| InterDigital | Y |  |
| Apple |  | We think it is important that we converge to a single power model for BS, and suggest the following change: Proposal 3.2.1-2: Study how to reuse and update the formula of BS power consumption in TR 38.864 for 6G BS, including aspects BS Cat.1/Cat.2 definition or selection/prioritization, PA/PAPR effects, etc, targeting for a single BS category. |
| Qualcomm | Y |  |
| Huawei, HiSilicon | Partially | With development evolution, we believe update to the BS power model of TR 38.864 is necessary to achieve an up-to-date BS model. We don’t prefer keep both Cat1 and Cat2, but understand that selection is part of the discussion. Relative power values as well as transition time/power states can also be revisited. Additionally, scaling is also one aspect to be discussed. As explained in our contribution, with large number of antenna equipped at BS and advanced implementation, switch-off of antennas/TRxU can also reduce the static power part of the total power, different from that in NR. We see this may be related to proposal 3.2.2-1, however, to separately define a power model specific to a mode, an implementation or configuration may not be desirable. Given the above, the below is sufficient Study necessary update to the power model, formula in TR 38.864 for evaluating BS power consumption for 6G BS |
| Lenovo |  | A single BS category should be used for evaluation to minimize effort. |
| ZTE, Sanechips | Y |  |
| CEWiT | Y |  |
| Samsung | Y |  |
| TCL | Y | Same views like CMCC. |
| OPPO | Y |  |
| Vodafone |  | There should be at least an update on the PA efficiency model for a more accurate evaluation of NES techniques on the power domain |
| Futurewei |  | OK in principle. |
| DCM | Y | We think that the BS cat model may need to be considered based on a realistic value. Meanwhile, the value should be decided considering potential future HW improvement since the NES model and features in 6G day1 will be used and deployed in real field in the next generation, between 2030 and 2040. |
| Tejas | Y | Support |
| NEC | Y |  |
| AT&T |  | Agree with using a single model for evaluation |

Moderator thanks companies’ valuable feedback. On this topic, the following are identified issues for further consideration (next meeting):

Issues Identified:

- Single BS Category Convergence: Strong consensus from [Nokia], [Apple], [Lenovo], [AT&T], and [Ericsson] to target a single BS category for 6G to simplify evaluations, with [Ericsson] specifically advocating for Category 1 as the baseline

- Model Updates for 2030 Technology: [CMCC], [Huawei, HiSilicon], and [Vodafone] emphasized that the power model needs updating to reflect technology evolution by 2030, including:

  - Revisiting relative power values and transition times/energies

  - Updating PA efficiency models

  - Considering scaling behaviors with large antenna counts and partial TRxU shutdown

- Scaling Considerations: [Huawei, HiSilicon] highlighted that scaling of the power model, especially the static power part, needs reconsideration for large antenna systems

After further online discussion, companies agreed to start with reusing/updating the power model in TR38.864 for 6G BS:

Agreement

Study how/whether to reuse or update the power model in TR 38.864 for evaluating BS power consumption for 6G BS.

[Nokia], [FUTUREWEI] support updating UE reference configurations and power consumption model for 6G. [TCL] and [MediaTek] suggest configurations of different device types. The following proposal is therefore suggested:

Proposal 3.2.1-3: Study how to reuse and update reference configurations and power consumption model in TR 38.840 for 6G UE, including 6G FR3 UE and 6G C-IOT UE

- Discuss and decide device types for 6G UE and corresponding reference configurations in RAN1#122-bis

| New 6G Device types | 6G Cellular IOT | 6G Mobile computing | 6G CPE/FWA |
| --- | --- | --- | --- |
| Frequency band | FR1 | FR1/FR3/FR2 | FR1/FR3/FR2 |
| Subcarrier space | 15 kHz | 30 kHz/30 kHz/120 kHz | 30 kHz/30 kHz/120 kHz |
| Maximum bandwidth | 20 MHz | 100MHz/200 MHz/100MHz | 100MHz/200 MHz/100MHz |
| Maximum Modulation order | 64 QAM | 256 QAM | 1024 QAM |
| MIMO configuration | 1R × 1-layer | 4R × 4-layer | 8R × 8-layer |
| Tx antenna configuration | 1TX | 2TX | 4TX |
| Tx power level | TBD | 26 dBm | 29 dBm |

| Company | Support? (Y/N) | Comments/Revisions |
| --- | --- | --- |
| LG Electronics |  | We are fine with considering TR 38.840 as the baseline for UE power modelling. However, what kind of NEW device types needs to be evaluated under this agenda item requires further discussion. |
| CMCC | Y | Okay for further study. Furthermore, we think that the 6G UE power model shall: - Consider more accurate scaling method to jointly evaluate RU in frequency/spatial/power domain, and further consider aspects like PDCCH blind decoding. - Consider both Tx and Rx at the same time for evaluation. - Consider More accurate model for PA efficiency, if any. Also, the issue on frequency should be clarified, which is same as Proposal 3.2.1-1. |
| Fujitsu | Y |  |
| Spreadtrum |  | A little revision. Discuss and decide device types for 6G UE and corresponding reference configurations the yellow marked values in the below table in RAN1#122-bis |
| Panasonic |  | We support to reuse and update reference configurations, but a bit concerned about too many variations of UE types, which would possibly lead to not so clear results, especially when checking together with EEG_BS. Thus we propose to prioritize a most typical type of device and can include other types as optional. |
| CATT | Y |  |
| Xiaomi |  | The configurations for PDCCH, PDSCH, SSB etc., are important for the determination of the relative power for the corresponding state, but the PDCCH, PDSCH, SSB design are unclear for now. How to consider this part? |
| vivo |  | It’s important to study reference configurations and power consumption model for 6G UE. And regarding the device types, we suggest to study reference configurations and power consumption model for eMBB UE first, and we can further study on other device types depending on progress in other agenda for device type discussion. |
| Nokia |  | Whether to study/identify necessary updates to the UE power consumption model seems reasonable, but what is not fully clear, are the 6G UE descriptions. Is the intention to discuss valid values for device types (including possible device types/categories)? If that is the intention, should we, or another AI, step back and identify first the device types and then agree related parameter values? |
| Ericsson |  | The table indicates that the maximum bandwidth for “6G Cellular IoT” is assumed to be 20 MHz, but this has not been decided yet. In agenda item 11.1, study of 3, 5, 10, and 20 MHz is proposed. |
| Google |  | We have concern on maximum bandwidth of Cellular IoT. 20 MHz would be too large. We would prefer to consider 10 MHz, 5 MHz or even 3 MHz. Or is it FL’s intention to mean that among those 6G IoT devices, the maximum bandwidth of some IoT devices is 20 MHz, but there could be other 6G IoT devices with maximum bandwidth as 5 or 3 MHz? |
| Apple |  | This is related to the discussion on diverse device types. Ideally, we should follow the outcome of that discussion in terms of definition of different device types. We are not sure if it is necessary to consider CPE/FWA devices in energy efficiency discussion, because we do not think they are the main focus for energy saving features. The definition of mobile computing devices and the necessity of including it in energy efficiency discussion is also not clear to us. This needs to be clarified. We are open to discuss IOT devices. |
| Qualcomm | N | It is not clear to us that there is a need new to define additional device types in this study and propose to not do that. |
| Huawei, HiSilicon | Almost | Our understanding is that device types are to be decided in e.g. initial access agenda. We can refer to that once progress of that is made, while for the moment we consider the names could be moderate, e.g. lower tier, medium tier and high end device. Alternatively, if the intention is just for EE evaluation purpose, we could be fine to consider 3 types – the 6GR device types are to be decided separately. Note FR3 is undefined and to be updated in the proposal. Proposal 3.2.1-3- Huawei: Study necessary update to reference configurations and power consumption model in TR 38.840 for 6G UE, including 6G FR3 UE and 6G C-IOT UE considering new spectrum and different UE types - For EE evaluation purpose, three types of device are considered, noting that the 6GR device types are to be separately decided in other agenda - Discuss and decide device types for 6G UE and corresponding reference configurations in RAN1#122-bis |
| Lenovo |  | As Ericsson suggested, IoT bandwidth can start from 3, …, 20 MHz as it depends on the MNO carrier bandwidth., suggest to add coverage for the IoT too, as NES impact on supporting a larger coverage should be studied. |
| ZTE, Sanechips | Y |  |
| CEWiT | Y | We support the proposal in general, however we suggest prioritizing the most typical UE type as mandatory for evaluations and making others optional. |
| Samsung | Y |  |
| TCL |  | Basically support this proposal. Two suggestions, -- We are not sure if modulation order could be 1024QAM for FR 3. -- Consider minimum BW limitation especially for 6G cellular IoT device. |
| OPPO |  | We’d better to only focus fewer type of devices. 1 device like Immersive Communication(mobile computing) would be desirable. |
| Futurewei |  | Not sure that defining the UE types are part of this agenda. This can wait until they are defined. |
| DCM | Y | Discussing device types is fine, but numerologies should be referred from other agenda’s output once it is decided. Note that we should rely on realistic values. |
| Tejas | Y | Support |
| NEC | Y (in principle) | In principle we agree to define different assumptions for different device types but we should keep the parameters (like antenna configuration) aligned between this and 11.2 |
| AT&T |  | Fine with the categorization but not necessarily the values, e.g., why SCS of 15 kHz is not considered in FR1, given that most 6G study docs for frame structure are proposing a simgle numerology per band. |

Moderator thanks companies’ valuable feedback. On this topic, the following are identified issues for further consideration (next meeting):

Issues Identified (Merged):

- Device Type Definition Concerns: Significant debate about the definition and necessity of new device types for 6G:

  - [Nokia], [Futurewei], and [Huawei, HiSilicon] questioned whether this agenda item is appropriate for defining new device types

  - [Apple] expressed doubts about focusing on CPE/FWA devices for energy efficiency

  - [Qualcomm] suggested not defining additional device types in this study

  - [vivo], [CEWiT], [Panasonic], and [OPPO] proposed focusing first on a typical eMBB UE type

- Parameter Value Concerns: Several companies raised issues with specific parameter values:

  - [Ericsson] and [Google] questioned the 20 MHz bandwidth for Cellular IoT, suggesting 3/5/10/20 MHz options

  - [TCL] was unsure about 1024QAM for FR3

  - [AT&T] questioned the absence of 15 kHz SCS in FR1

- Power Model Accuracy: [CMCC] emphasized the need for a more accurate scaling method for the UE power model

- Alignment Requirements: [Lenovo] stressed the need to align device type definitions with other AIs

After further online discussion, companies think to start with the following generic proposal for 6G UE power consumption model:

Agreement

Study reference configurations and power consumption model for 6G UE, considering but not restricted to the following:

- TR 38.840 (UEPS), TR38.875 (RedCap), TR38.865 (eRedCap), and TR38.869 (LP-WUS/WUR) for reference configurations

- TR 38.840 (UEPS), TR38.875 (RedCap), and TR38.869 (LP-WUS/WUR) for power consumption models

Power consumption model for low-power transmitter and receiver (LP-TX/RX)

[CATT], [Huawei], [MediaTek] propose studying power consumption models for BS LP-TX/RX with functionality restricted to simple signals. Since LP-TX/RX look important BS power saving techniques, particularly for Cat.2 BS, moderator suggests the following proposal for further study:

Proposal 3.2.2-1: Study power consumption model for 6G BS LP-TX/RX utilized for only transmitting/receiving specific signals (e.g., SSB, paging/WUS, PRACH triggers).

- FFS: Specific signals for BS TX and RX

| Company | Support? (Y/N) | Comments/Revisions |
| --- | --- | --- |
| Ofinno |  | We are open to discuss but not sure if there is a common understanding what BS LP-TX/RX means |
| LG Electronics | Y |  |
| CMCC | Y, but | We are fine with 6G BS LP-RX study. Regarding 6G BS LP-TX, since 6GR is potential deployed on a higher frequency (e.g. U6G), the coverage issue should also be carefully considered. |
| Fujitsu |  | We wonder what the difference is between this proposal and Proposal 3.2.1-2 (how to reuse and update the formula of BS power consumption in TR 38.864). |
| Spreadtrum |  | Firstly, LP-TX/RX in UE side should also be considered. Secondly, we think “(e.g., SSB, paging/WUS, PRACH triggers).” should be removed. We suggest to revise the proposal as follow Proposal 3.2.2-1: Study power consumption model for 6G BS and UE LP-TX/RX utilized for only transmitting/receiving specific signals (e.g., SSB, paging/WUS, PRACH triggers). - FFS: Specific signals for BS TX and RX |
| Panasonic | Y | We support to study this, but is a bit concerned on how to properly assume the signal/channel structure, as SS, PBCH, WUS, PRACH and so on are not discussed yet. Or do we assume the NR signal/channel design and then adapt to potential new design in later phase? |
| CATT | Y | The LP-Tx/Rx state of network and UE can be implemented by using low-power consumption of components or configurations. For the low-power consumption of components, similar to Rel-19 LP-WUR in 5G, low power consumption of transmitter/receiver could be considered, e.g., low complexity baseband components, low complexity signal detection, less antenna configuration, etc. For low power consumption of configuration, narrower bandwidth, low frequency, and low power consumption of spatial domain configuration could be considered for network and UE. On the other hand, low power consumption of algorithm and signal design suitable to low power consumption of components or configurations should also be studied. The corresponding low power consumption of procedure should also be studied for both network and UE to match LP-Tx/Rx state well. |
| vivo |  | Suggest following change since BS LP-TX/RX may be misunderstood as a separate radio. Proposal 3.2.2-1: Study power consumption model for 6G BS LP-TX/RX utilized for only transmitting/receiving specific signals (e.g., SSB, paging/WUS, PRACH triggers). - FFS: Specific signals for BS TX and RX |
| Nokia |  | Before we study the model, it should be first clarified what a BS LP-TX/RX entails (what to model) and what possible enhancements it offers over other methods discussed e.g. later, it is noted that the current BS power model already covers SSB, paging/WUS, PRACH, etc. |
| Ericsson | N | It is too early to study a power consumption model because the concept is too vague at this point. There is no common understanding of the function of this 6G BS LP-TX/RX entity. Is it a separate radio or a mode of operation? How does it interact with the main-radio? What are the requirements regarding link-performance? Moreover, the proposal suggests that CAT2 BS model should be used for MR but we disagree, CAT2 BS model is not representative for 6G. BS LP-TX/RX that wakes up a main-radio can lead to long latency. If Cat.2. BS is assumed for MR (as suggested), then the delays will be ~ 10s. The market-penetration of this type of radio will be small, if any. We should prioritize the models for widely deployed radios. |
| InterDigital |  | We support the study. |
| Apple |  | We are open to study. Suggested changes: Proposal 3.2.2-1: Study the feasibility of using a LP-TX/RX for BS to transmit/receive some specific signals (e.g., SSB, paging/WUS, PRACH triggers) and the corresponding power consumption model. for 6G BS LP-TX/RX utilized for only transmitting/receiving specific signals (e.g., SSB, paging/WUS, PRACH triggers). - FFS: Specific signals for BS TX and RX |
| Qualcomm |  | We generally support the proposal but would like to remove the LP-Tx/LP-Rx terminology. We can follow what Huawei used in the evaluation contribution and use “low-power configuration” or what we used in our contribution and use “energy-efficient mode”. We discuss this in our evaluation assumption contribution (R1-2506217) |
| Huawei, HiSilicon | Almost | Proposal 3.2.2-1-Huawei: Study and aim for a unified power consumption model for 6G BS LP-TX/RX utilized for only transmitting/receiving specific signals (e.g., SSB, paging/WUS, PRACH triggers). - FFS: Specific signals for BS TX and RX - FFS: applicable functions/procedures/scenarios |
| Lenovo |  | It is not clear how LP signal structure with OFDM WUR Tx/Rx looks like, and first, we can discuss how much of the signals from the main radio can be reused and later discuss this proposal. |
| ZTE, Sanechips |  | We should firstly study the feasibility and necessity. In our understanding, it would be quite challenging for gNB to keep the same PRACH receiving performance but have the less power consumption. |
| Samsung |  | At first, need to clarify 6G BS LP-TX/RX. Suggest rewording: Proposal 3.2.2-1: Study power consumption model for 6G BS LP-TX/RX utilized for only transmitting/receiving only specific signals (e.g., SSB, paging/WUS, PRACH triggers) to/from UEs in CONNECTED mode. - FFS: which Specific signals for BS TX and RX |
| Fraunhofer |  | We support studying this. |
| TCL | Y |  |
| OPPO | Y |  |
| Futurewei |  | Not sure how this works. We prefer that these signals and behavior to be defined first, i.e., which signals will exactly be considered for the LP-TX/RX? Are we going to consider the impact on the UE in the IDLE state too? |
| DCM |  | We do not see the necessity of introducing LP-TX/RX for BS side since this leads to providing different types of common signals, which is not a unified solution. |
| Tejas | Y | If there is impact on the coverage, it should be reported during the evaluation. |
| NEC | Y |  |

Moderator thanks companies’ valuable feedback. On this topic, the following are identified issues and suggested proposal revision for further consideration (next meeting):

Issues Identified:

- Concept Clarity: Major confusion raised by [Ofinno], [Nokia], [Ericsson], [Samsung], [Qualcomm], [Apple], and [vivo] regarding:

  - Lack of common understanding of what "BS LP-TX/RX" entails

  - Whether it represents a separate radio or a low-power configuration/mode

  - Need to avoid premature architecture choices

- Feasibility and Market Relevance: [Ericsson] and [ZTE, Sanechips] questioned the feasibility and market penetration, especially considering potential long latencies, while [DCM] did not see the necessity

- Scope Expansion: [Spreadtrum] and [Panasonic] suggested that UE side should also be considered, with [Nokia] emphasizing CONNECTED scope alignment

- Relationship to Existing Models: [Fujitsu] wondered about the difference between this proposal and the general BS power model in Proposal 3.1-2

Revised Proposal:

Study feasibility and power consumption model of low-power TX/RX configuration for BS utilized for specific signals (e.g., SSB, paging/WUS, PRACH triggers), considering:

- Low-power configuration/mode approach, not mandating separate radio

- Potential use cases and power consumption implications

- Specific signals and performance targets are FFS and should be aligned with related Agenda Items

[CMCC], [vivo], [NTT DOCOMO], [Ericsson], [Samsung], [FUTUREWEI], [InterDigital], [Vodafone], [Apple], [Qualcomm], [MediaTek], et al. support OFDM-based WUR design. Since the definition/characteristics of OFDM-based WUR for 6G needs to be clarified, the following proposal is therefore suggested by moderator:

Proposal 3.2.2-2: Study the definition and power consumption model for 6G OFDM-based WUR, considering at least the following aspects: RX antenna setting, CFO characteristics, restrictions in processing capability (e.g., 3/5MHz, sequence matching only), etc.

| Company | Support? (Y/N) | Comments/Revisions |
| --- | --- | --- |
| Ofinno |  | Is this only for UE side? |
| LG Electronics | Y |  |
| CMCC | Y |  |
| Spreadtrum |  | We think this proposal is for UE side, and it is better to clarify “Study the definition and power consumption model for 6G OFDM-based WUR for UE” |
| Panasonic | Y | Similar comments for above proposal. |
| vivo |  | This is from UE side, so main bullet should add at UE side after OFDM-based WUR. In addition, the definition for OFDM-based WUR should be clear based on NR Rel-19 OFDM-based WUR TR 38.869 and NR Rel-19 OFDM-based WURshould be the starting point for 6G OFDM-based WUR. Suggest following Proposal 3.2.2-2: Study the definition and power consumption model for 6G OFDM-based WUR at UE side using NR Rel-19 OFDM-based WUR in TR 38.869 as starting point, considering at least the following aspects: RX antenna setting, CFO characteristics, restrictions in processing capability (e.g., 3/5MHz, sequence matching only), etc. |
| Nokia |  | Is the intention to identify the afore aspects for OFDM-based WUR (as considered in TS38.869) with the purpose to be able to establish/identify the benefit and applicability of different use cases in scope of the CONNECTED mode? |
| Ericsson |  | Why the “definition” is needed? In Rel-19, there are power consumption values for OFDM-WUR which can be reused. Also, suggest to remove “(e.g., 3/5MHz, sequence matching only)” |
| Google | Y |  |
| InterDigital |  | Fine with the proposal with the clarification that this proposal applies to UE. |
| Apple |  | We support using OFDM-based LP-WUS design in NR as the starting point, but do not want to limit to it at this stage. Proposal 3.2.2-2: Study the potential functions/capabilities of low-power receiver at the UE and the corresponding power consumption model for 6G OFDM-based WUR - considering at least the following aspects: RX antenna setting, CFO characteristics, restrictions in processing capability (e.g., 3/5MHz, sequence matching only), etc. - OFDM-based WUS/WUR in NR is considered as the starting point |
| Qualcomm |  | We support to study the power consumption model of UE energy-efficient mode in RRC connected state. The power model in TR38.869 for NR OFDM-based LP-WUS operation can be taken as the starting point and merged into the UE power model. We propose to not define a separate receiver (WUR) and avoid discussing architecture. At this stage, we propose to not go into the detail of this mode. Proposal 3.2.2-2: Study the definition and power consumption model for 6G OFDM-based WUR energy-efficient modem mode, considering at least the following aspects: RX antenna setting, CFO characteristics, restrictions in processing capability (e.g., 3/5MHz, sequence matching only), etc. |
| Huawei, HiSilicon | Almost | Generally OK. Another aspect to be considered is whether it is time domain processing or frequency domain processing. Proposal 3.2.2-2-Huawei: Study the definition and power consumption model for 6G OFDM-based WUR, considering at least the following aspects: RX antenna setting, CFO characteristics, restrictions in processing capability (e.g., 3/5MHz, sequence matching only, time or frequency processing), etc. |
| Lenovo |  | Ok |
| ZTE, Sanechips | Y | This aspect also is discussed in idle mode. |
| CEWiT |  | The proposal is not clear, whether its for UE. If yes, then we agree with the comment of spreadtrum to clarify “Study the definition and power consumption model for 6G OFDM-based WUR for UE, considering at least the following aspects: RX antenna setting, CFO characteristics, restrictions in processing capability (e.g., 3/5MHz, sequence matching only), etc.” |
| Samsung |  | Proposal 3.2.2-2: Study the definition use-cases, architecture and power consumption model of UE receiver for 6G OFDM-based DL WUS WUR, considering at least the following aspects: RX antenna setting, CFO characteristics, restrictions in processing capability (e.g., 3/5MHz, sequence matching only), etc. |
| Fraunhofer | Y |  |
| TCL | Y |  |
| OPPO | Y | The term of OFDM-based WUR/WUS is used in both FL summary of idle mode and connected mode. It is important to reach a common understanding. In FL of idle mode, we understand that it refers to a signal transmitted from BS to UE. We assume this is the same in connected mode, even the related procedures are different. |
| DCM | Y | We are fine with the first part “Study the definition and power consumption model for 6G OFDM-based WUR”. But we think it is required to first determine what WURs can do, whether they can do RRM measurement or they are limited to receive only WUS? Depending on the functions that WURs possess, the necessary power consumption model can be changed. We propose following Proposal 3.2.2-2: Study the definition and power consumption model for 6G OFDM-based WUR |
| Tejas | Y | We support |
| AT&T |  | A similar discussion is taking place in idle mode FL summary. Prefer the WUR/WUS discussion to be handled there |

Moderator thanks companies’ valuable feedback. On this topic, the following are identified issues and suggested proposal revision for further consideration (next meeting):

Issues Identified:

- Scope Clarification: General agreement that this proposal is for the UE side, with [vivo], [Qualcomm], [Ericsson], [Apple], and [CEWiT] emphasizing:

  - Use NR Rel-19 OFDM-based WUR as starting point

  - Avoid fixing architecture prematurely

  - Focus on UE low-power receiver functions

- Terminology Preferences: [Qualcomm] preferred "energy-efficient modem mode" instead of "WUR" to avoid architectural discussions

- Level of Detail: [Ericsson] and [Qualcomm] suggested removing detailed aspects like CFO characteristics at this early stage

- Functionality Scope: [DCM] and [Nokia] raised important questions about what functions the WUR should support (e.g., RRM measurements, wake-up functions)

Revised Proposal:

Study UE low-power receiver functions and corresponding power consumption model for 6G OFDM-based WUR, considering:

- RX antenna setting, CFO characteristics, and processing constraints (e.g., 3/5 MHz, sequence matching)

- Supported functionalities beyond wake-up (e.g., RRM measurements)

- Whether/how to reuse NR Rel-19 OFDM-based LP-WUS/WUR designs

- Architecture specifics are FFS

Other BS/UE power consumption model requirements

[FUTUREWEI] proposes studying energy consumption evaluation methodologies for AI/ML-based approaches. [NEC] emphasizes accounting for power consumption of real-time model inference. Therefore, the following is suggested:

Proposal 3.2.3-1: Study whether/how to model/evaluate power consumption for AI/ML-based processing in 6G

| Company | Support? (Y/N) | Comments/Revisions |
| --- | --- | --- |
| Ofinno |  | We should take a common view across this whole agenda. In the Idle mode FL summary it is indicated that AI/ML will be discussed in 11.6 directly. If that is agreeable for companies then we should take the same approach here. |
| LG Electronics |  | We are open to discuss power consumption modeling for AI/ML. However, we may need to first discuss in which agenda item this modeling can be studied. |
| CMCC | N | Should be discussed in AI 11.6. |
| Fujitsu |  | Clarification is needed regarding WUR. Is it equipped on UE side only or can be equipped on both BS and UE sides. |
| CATT |  | Similar view with Ofinno and CMCC, Proposal 3.2.3-1 can be discussed in AI 11.6 |
| vivo |  | We support to study this in AI ML agenda |
| Nokia |  | We agree this should be studied, but in the AI/ML agenda item. |
| Ericsson |  | We should focus on communication-related power consumption to avoid complication. AI/ML aspects can be discussed in AI 11.6 and 11.2. |
| Google | Y | We believe this is critical. Power consumption for AI/ML processing is quite different from other processing. Since we image more AI-related processing would be introduced in 6GR, we support this proposal. |
| InterDigital |  | In our view, power consumption model can be defined within this agenda, but details on the model structures should be agreed before the discussion in AI 11.6 |
| Apple |  | The topic was discussed in R18 AI/ML study. The general view was that the power consumption of AI/ML-based processing is very much implementation dependent even for the same amount of processing complexity, and it did not seem possible to have a common power consumption model. It was decided to use FLOPs as the metric. We would like to understand if the situation has changed before we agree to study it again. |
| Qualcomm | N | Not in this agenda item. How to evaluate the complexity and power impact of AI/ML processing in 6G should be discussed the AI/ML agenda item if needed. |
| Huawei, HiSilicon | Unsure | To be discussed later. |
| Lenovo |  | Can be studied as part of AI/ML A.I |
| ZTE, Sanechips |  | OK to discuss in AI/ML agenda |
| CEWiT |  | We’re open to discuss power consumption modeling for AI/ML based energy efficiency in 6G, but the right agenda item needs to be decided first. |
| Samsung |  | OK – it can be done in AI/ML |
| Fraunhofer |  | While we consider this to be an important aspect, it might not fit in this agenda item. |
| TCL |  | We are open to discuss but at least do not need to discuss in this topic at this stage. |
| OPPO | Y |  |
| Vodafone | Y | Important to have this evaluation, the power modelling can be done in this agenda item and its usage can be done in the AI/ML agenda item. |
| Futurewei | Y | While we would prefer to treat this problem here, we think that the most important for RAN1 is to make sure that the AI/ML energy problem is covered. We are OK if the majority wants it to be treated on the AI/ML agenda. |
| DCM | Y | Support. Are we going to discuss this in this agenda? |
| Tejas |  | Can be studied as part of the AI/ML agenda. |
| NEC | Y | With the anticipated proliferation of AI/ML applications in 6G, the conventional assumption that RF (Tx/Rx) operations are the primary power consumers is no longer tenable. We must therefore account for the power consumed by inference operations, both at the UE and within the network (with a particular focus on models running at the DU). The need to do so is further amplified when AI/ML applications, such as those for RS overhead reduction, are themselves intended to achieve greater power savings for the network and UE. |
| AT&T |  | Agree EE for AI/ML-based operation is a crucial issue. Further guidance from the chair is needed on the agenda in which this is handled |

Moderator thanks companies’ valuable feedback. On this topic, the following are identified issues:

Issues Identified (Merged):

- Agenda Item Alignment: Overwhelming consensus from [Ofinno], [CMCC], [CATT], [vivo], [Nokia], [Ericsson], [Qualcomm], [Lenovo], [ZTE, Sanechips], [Samsung], [Fraunhofer], [TCL], [Futurewei], [Tejas], [Google], [OPPO], [Vodafone], and [Apple] that AI/ML power consumption discussion belongs in the dedicated AI/ML agenda item (11.6)

- Modeling Challenges: [Apple] and [CMCC] raised valid points about:

  - Difficulty of creating common power consumption models for AI/ML due to high implementation dependency

  - Use of FLOPs as metrics in past studies

  - Need for agreed complexity measures as primary proxy

Given the expertise required for the AI/ML power consumption model lies in AI/ML network implementations and complexity-performance trade-off, instead of existing communication operation, moderator suggests that 11.6 will be the proper agenda for discussing this power consumption model.

Spatial-Domain Enhancement Directions

CSI Related Enhancements

[Ericsson], [CMCC], [Lenovo], [InterDigital], [NTT DOCOMO], [Fujitsu], [vivo], [ZTE], et. al, jointly support enabling spatial-/power-domain adaptation with reduced reference signals/feedback and related CSI enhancements, including study multi-CSI reporting for different on-off patterns/power levels with overhead reduction. [AT&T] specifically proposes to investigate DFT-based precoding framework for straightforward sub-precoder derivation. [NEC] proposes to study CSI framework supporting dynamic antenna adaptation across multiple TRPs. With the above CSI related considerations, moderator would like to suggest the following proposal:

Proposal 3.3.1-1: Identify CSI related enhancement direction(s)/solution(s) for 6G EE improvement, considering

- Reduced CSI-RS overhead (e.g. scaling with cell load)

- Reduced CSI calculation complexity and feedback overhead

- Support of coordinated adaptation across multiple TRPs

| Company | Support? (Y/N) | Comments/Revisions |
| --- | --- | --- |
| Ofinno |  | In generally to study but are we already identifying the sub-bullets as directions? Maybe we can say considering potential direction(s)/solution(s): … . Same comment may apply to most of the FL proposals in our view. By “reduced” do we mean adaptation of CSI-RS in spatial domain? |
| LG Electronics | Y |  |
| CMCC | Y | Okay for further study |
| ETRI |  | We are okay with the main sentence and first two bullet points, but for the last bullet, clarification seems needed to better understand the scheme. |
| Fujitsu | Y |  |
| Spreadtrum |  | We should focus on high-level technologies, e.g., - Enhancements on CSI-RS (re)configuration - Enhancements on CSI feedback - Coordinated adaptation across multiple TRPs |
| CATT |  | We think Spreadtrum’s version is better to make the proposal more general. |
| Xiaomi |  | We are fine with the direction of studying CSI related enhancement for 6G EE. However, we doubt to the necessity to including multi-TRP at this stage as it highly depends on MIMO discussion whether multi-TRP is supported or not in Day 1. |
| vivo |  | Each technique for EE study should be clear about the target, this EE target is mainly for BS or for UE Before we agree to study; otherwise it is not clear what and how to study. |
| Nokia |  | We believe that MIMO experts could add more improvements later. Thus, we propose to add “…. considering at least”. In addition, we would like to add the following bullet points: - Investigate also the SRS-based operation for 6G EE. - Potential power adaptation for CSI-RS operations. - Enhanced CSI triggering mechanisms |
| Ericsson | Y |  |
| Google |  | We are fine with the first two bullets. However, regarding the third bullet, we are reluctant to support it. We should study from single TRP scenario, which should be the basic scenario. |
| InterDigital |  | We would like to propose more general study scopes and propose the following based on the Spreadtrum’s proposal. - Enhancements on CSI-RS - Enhancements on CSI feedback - Enhancements on multi-TRP operation |
| Apple |  | A general comment for this proposal and the subsequent proposals is that the language should be open at this stage in terms of what direction(s)/solution(s) are studied. The meaning of each sub-bullet should also be clear. Proposal 3.3.1-1: Identify potential CSI related enhancement direction(s)/solution(s) for 6G EE improvement, considering e.g. - Reduced CSI-RS overhead (e.g. scaling with cell load) - Reduced CSI calculation complexity and feedback overhead - Support of coordinated adaptation across multiple TRPs |
| Qualcomm | N | We should discuss the energy enhancement mechanism before identifying and discussing its CSI impact. |
| Huawei, HiSilicon | Almost | We consider certain NR techniques are baseline, e.g. the CSI enhancements in Rel-18. |
| Lenovo |  | ok |
| ZTE, Sanechips |  | We think the second subbullet is for multi-CSI report case. So to avoid confusion, it is updated accordingly. Proposal 3.3.1-1: Identify CSI related enhancement direction(s)/solution(s) for 6G EE improvement, considering - Reduced CSI-RS overhead (e.g. scaling with cell load) - Reduced multi-CSI calculation complexity and feedback overhead - Support of coordinated adaptation across multiple TRPs |
| CEWiT |  | Similar view with CATT, that Spreadtrum’s version of the proposal is more generic and suitable for this meeting. |
| Samsung |  | Proposal 3.3.1-1: IdentifyStudy CSI related enhancement direction(s)/solution(s) for 6G EE improvement, considering - Reduced CSI-RS overhead (e.g. scaling with cell load) - Reduced CSI calculation complexity and feedback overhead Support of coordinated adaptation across multiple TRPs |
| TCL | Y |  |
| Fainity | Y | Does the first bullet mean (1) “avoid scaling CSI-RS overhead when the cell load becomes higher” or (2) “reduce CSI-RS overhead, but the overhead scales when cell load becomes higher”? |
| OPPO | Y | Suggest to use study instead of identify. |
| Vodafone |  | Propose to study either here or in the AI/ML agenda whether CSI compression can be used to reduce the CSI-RS overhead for spatial port adapation |
| Futurewei |  | We want to clarify that this does not refer to AI/ML techniques to improve CSI and CSI reporting. Otherwise, the question should have the same answer as the question above on AI/ML energy efficiency. |
| DCM | Y | Identifying in what CSI related enhancement is required should considered. |
| Tejas | Y |  |
| NEC | Y |  |
| AT&T |  | Fine in general, yet prefer prioritizing sTRP scenarios for now, due to lack of clarity on mTRP modes to be supported in 6G, if any |

Moderator thanks companies’ valuable feedback. On this topic, the following are identified issues and suggested proposal revision for further consideration (next meeting):

Issues Identified:

- Generality Requirements: [Ofinno], [Apple], [Samsung], [Google], [Spreadtrum], and [InterDigital] suggested making the proposal more general at this stage, using "study" instead of "identify" and removing specific examples

- Scope Expansion: [Nokia] proposed adding important aspects:

  - SRS-based operation

  - Power adaptation for CSI-RS operations

  - Enhanced CSI triggering mechanisms

- Multi-TRP Considerations: [Xiaomi] and [Google] questioned the inclusion of multi-TRP at this stage, preferring to focus on single-TRP scenarios first as baseline

- Enhancement Prioritization: [Qualcomm] suggested that energy enhancement mechanisms should be discussed before their CSI impact

Revised Proposal:

Study CSI-related enhancement directions for 6G EE improvement, considering at least the following aspects:

- Reduced CSI-RS overhead (e.g., scalable with load)

- Reduced CSI/multi-CSI calculation and feedback overhead

- SRS-based operation

- Power adaptation for CSI-RS operations

- Enhanced CSI triggering mechanisms

- FFS: Support of coordinated adaptation across multiple TRPs

BM Related Enhancements

[ETRI], [NEC], [ZTE], [CATT]  propose dynamic TRP-level on-off operation as part of beam management framework, and [Rakuten] suggests beam-level and antenna-port-level muting with interaction to CSI reporting and PDCCH scheduling. With the above, the following is proposed for further study

Proposal 3.3.2-1: Identify BM related enhancement direction(s)/solution(s) for 6G EE improvement, considering

- Inclusion of TRP-level on-off operation as part of beam management framework

- Beam-level antenna muting and DTX/DRX

| Company | Support? (Y/N) | Comments/Revisions |
| --- | --- | --- |
| Ofinno |  | Mixing DTX/DRX which overlaps with other proposals in both this summary and the idle mode FL summary may create duplicated discussion. We are not sure that beam-level muting is necessary to be included at this point. We could focus on TRP level first. |
| LG Electronics |  | In our view, TRP on/off operation is higher level issue than beam management enhancement. So, we propose to promote TRP on/off operation to the main bullet while putting BM related enhancement into the sub-bullet. |
| CMCC | Y | Okay for further study |
| ETRI | Y | For proper agenda item to discuss TRP-level on-off operation, we basically think that it can be treated under TCI framework in MIMO agenda in the future. However, at this stage, we can focus on identifying issues/directions and the wording “as part of BM framework” may be removed for now. |
| Fujitsu |  | The relationship between DTX/DRX and beam management is unclear. Besides, it needs to be clarified whether the DTX/DRX refers to the cell DTX/DRX or the UE DTX/DRX. |
| Spreadtrum |  | For first sub-bullet, the relationship between it and beam management framework is unclear. It can be revised as “TRP-level on-off operation”. For second sub-bullet, “DTX/DRX” is unclear for us. It can be revised as “Beam-level antenna muting” |
| Panasonic | Y |  |
| CATT | Y | Agree with the proposal. |
| xiaomi |  | Same comments as Proposal 3.3.1-1. We are open to discuss TRP based solution but want to first check whether multi-TRP is Day 1 feature or not. |
| vivo | N | Each technique for EE study should be clear about the target, this EE target is mainly for BS or for UE before we agree to study; otherwise it is not clear what and how to study. For example, we are not sure “Beam-level antenna muting and DTX/DRX” is BS side or UE side or both. |
| Nokia |  | We prefer to add “at least” to the first line. |
| Ericsson | Y | In our understanding, TRP-Level on-off inclusion means that the UE only measures the Beams corresponding to the TRPs that are on and do not measure the beams that are off. In 5G, the number of beams (and the corresponding channel measurement resources) the UE shall measure are indicated to the UE as part of the CSI report configuration. It is already possible to configure multiple CSI report configs with different beams in the 5G NR to support this use-case. More efficient ways to support TRP-level on-off concept and beam-level antenna muting and DTX/DRX can be studied in 6G. Having said that we like to emphasize that similar to 5G, TRP should be transparent to the UE. |
| Google |  | On TRP-level on-off operation, again, we believe that 6GR should consider S-TRP scenario as basis. |
| InterDigital |  | We would like to have a more general proposal without specific study scopes as shown in the below. Study BM related enhancements for 6G EE improvement, |
| Apple |  | It is probably sufficient to list “TRP on-off operation” as one potential direction. Whether it is part of beam management framework or not can be further discussed when we get down to the next level of details. “Beam-level antenna muting” and “DTX/DRX” in the 2nd sub-bullet are not clear. Need clarification. |
| Qualcomm | N | As in 4.1, there should be a discussion on BM and energy savings before identifying and discussing candidates. |
| Huawei, HiSilicon | Y |  |
| Lenovo | Y |  |
| ZTE, Sanechips | Y |  |
| CEWiT |  | For second sub-bullet, the relationship between DTX/DRX and beam management is unclear. |
| Samsung |  | Proposal 3.3.2-1: IdentifyStudy BM related enhancement direction(s)/solution(s) for 6G EE improvement, considering - Inclusion of TRP-level on-off operation as part of beam management framework Beam-level antenna muting and DTX/DRX |
| Fraunhofer | Y |  |
| TCL | Y |  |
| Fainity | Y | UL-only TRP deployment could be an efficient way to save UE’s transmission power. Asymmetric TRP scenario is suggested to be considered under this study. |
| OPPO | Y | Suggest to use study instead of identify. |
| Vodafone |  | Propose to study either here or in the AI/ML agenda whether beam management AI/ML framework can be used for the Beam-level antenna muting |
| Futurewei |  | Which BM solutions will be identified/? Are they part of the MIMO agenda item? Should we wait until these BM solutions are identified? |
| Tejas | Y |  |
| NEC |  | We agree to support TRP-level on-off however the “Beam-level antenna muting” is not clear to us. |
| AT&T |  | Prefer prioritizing sTRP scenarios + cell DTX enhancements to be discussed prior to BM framework compliance to cell DTX |

Moderator thanks companies’ valuable feedback. On this topic, the following are identified issues and suggested proposal revision for further consideration (next meeting):

Issues Identified:

- Clarity and Scope Issues: Multiple companies found confusion regarding:

  - [Ofinno], [Fujitsu], and [CEWiT] found inclusion of DTX/DRX confusing and overlapping with other proposals

  - [Apple] and [NEC] were unclear about "beam-level antenna muting"

  - [ETRI] suggested removing "as part of BM framework" for now

  - [LG Electronics] and [Ericsson] proposed treating TRP on/off as a higher-level issue than BM enhancement

- Multi-TRP Focus: [Xiaomi] and [Google] questioned the focus on multi-TRP, similar to CSI proposal

- Generality Preference: [InterDigital] and [Samsung] preferred a more general proposal without specific solutions listed

- UE Transparency: [Ericsson] and [Ofinno] emphasized preserving UE transparency to TRP operations

Revised Proposal:

Study BM-related enhancements for 6G EE improvement, taking into account:

- TRP-level on/off operation, considering UE transparency

- Optional beam-level muting mechanisms

- Avoiding overlap with separate DTX/DRX discussions

Other Spatial-Domain Enhancements

If the above proposals cannot include your spatial-domain EE improving schemes, companies are encouraged to provide comments/references for other companies’ investigation/evaluation.

| Company | Comments for additional enhancement(s) |
| --- | --- |
| Ericsson | We think that spatial domain adaptations for PRACH should also be included. |
| Qualcomm | We propose to study support of monitoring PDCCH with a reduced number of antennas. |
| Lenovo | Study Low resolution ADC/DAC based transmitter and receiver chain and quantify the benefit in terms of NES, OOB leakage etc., |
| CEWiT | Applicability of the adaptation to different signals and channels including SSB can be considered. Further, it should be discussed if AI/ML based enhancements for spatial-domain adaptation can be studied in this Agenda. |

Moderator thanks companies’ valuable feedback and would like to suggest companies’ further consideration of the above techniques for potential 6G EE improvement.

Frequency-Domain Enhancement Directions

Multi-Carrier/CA Related Enhancements

[Samsung], [Qualcomm], [NTT DOCOMO], [OPPO], [vivo], [TCL], [Ericsson], [ZTE], [Lenovo], [InterDigital], [Rakuten] support multi-carrier and CA enhancements. Companies emphasize SSB-less SCell, on-demand activation, and single-cell multi-carrier approaches. In this regard, moderator would like to suggest the following proposal:

Proposal 3.4.1-1:  Identify multi-carrier/CA related enhancement direction(s)/solution(s) for 6G EE improvement, considering the following aspects

- Anchor/beacon carrier and data carrier architecture

  - Anchor carrier provides AO SSB/OD SIB1/PRACH/paging on demand or at low periodicity

  - Data carriers remain dormant by default; (de)activation by network or UE demand

- Single cell of multi carriers

- SSB less SCell operations

| Company | Support? (Y/N) | Comments/Revisions |
| --- | --- | --- |
| Ofinno |  | Generally okay to study. This proposal may have some impact to/dependency on other agenda items as well. Some comments: - We would like to clarify if this is only for single cell or also for multi-cell. If for both we suggest to modify the main bullet to say multi-carrier/cell/CA. - We think we could discuss further the definition/behavior of anchor carrier and data carrier. Perhaps we can just say “study definition/behavior of anchor and data carriers considering always on and on-demand signals/channels” - Suggest to just use one term for now (i.e., remove “beacon”) - Suggest to also say Single cell with multi carriers |
| LG Electronics |  | We prefer to make sub-bullets more general, e.g., Proposal 3.4.1-1: Identify multi-carrier/CA related enhancement direction(s)/solution(s) for 6G EE improvement, considering the following aspects - Anchor/beacon carrier and data carrier architecture - FFS: definition/functionality of anchor/beacon/data carrier - Anchor carrier provides AO SSB/OD SIB1/PRACH/paging on demand or at low periodicity - Data carriers remain dormant by default; (de)activation by network or UE demand - Single cell consisting of multi carriers - SSB less SCell operations |
| CMCC | Y | Okay for further study |
| ETRI | Y | We support the proposal and also support the LG’s revised version. One question for clarification: Does this proposal cover both idle mode and connected mode operations, or focus only on connected mode operation? We think that the multi-cell operation with functionally separated cells is also applicable for idle mode. |
| Fujitsu |  | Regarding the 1st bullet, the definition of anchor carrier and data carrier as well as the default state (e.g., data carriers remain dormant by default), requires further discussion. Updates from LG are fine with us. |
| Spreadtrum |  | For first sub-bullet, we think it is a scenario or use case of multi-carriers. Whether data carriers is only used for data transmission is unclear. For third sub-bullet, besides SSB less for SCell, fast activation/deactivation for SCell should also be considered. |
| Panasonic | Y | To clarify, the second bullet is not clear to us. Does it mean single cell with multiple BWP/frequency fragments? |
| CATT | Y | Okay for further study |
| vivo |  | Not sure the Anchor/beacon carrier and data carrier architecture related to connected mode operation and it is for UE/NW power saving. Same comments for Single cell of multi carriers. In our view, for connected mode power saving solutions in frequency domain, fast SCell activation/deactivation needs to be studied. In case the fast activation of SCell can be done, then the necessity of Scell dormancy can be re-evaluated. We suggest following change Proposal 3.4.1-1: Identify multi-carrier/CA related enhancement direction(s)/solution(s) for 6G EE improvement for BS and UE, considering the following aspects - Anchor/beacon carrier and data carrier architecture - Anchor carrier provides AO SSB/OD SIB1/PRACH/paging on demand or at low periodicity - Data carriers remain dormant by default; (de)activation by network or UE demand - Single cell of multi carriers - Fast Scell activation and deactivation - FFS the necessity of SCell dormancy - SSB less SCell operations |
| Nokia |  | Add “at least” to the first line. OK to study “Control-Plane-less” SCell (e.g. SSB/SIB/paging-less) |
| Ericsson | Y (in principle) | It should be part of the study to define what role that the anchor carrier resp. data carrier should take, what dependency there is between them. |
| Google |  | Structure wise, it seems the first bullet is assumed based on the second bullet? Perhaps we should consider to move the first bullet under the second bullet. Regarding anchor carrier, we also suggest OD-SSB, which is also observed in some other contributions. |
| InterDigital |  | We prefer LGE’s version. |
| Apple |  | It is not clear what “Anchor/beam carrier” means for connected mode. This seems more related to idle mode operation. Or the intention is to say PCell and SCell? Maybe the 2nd sub-bullet can be replaced by “fast SCell activation/deactivation”? It is not clear to us how “single cell of multi carriers” is related to energy saving. |
| Qualcomm | N | We prefer to not use “identify” in this meeting and to study instead. In order to discuss potential solution(s), we suggest to set the existing NR CA with SCell activation/deactivation/dormancy as the baseline. Further enhancements and performance benefits should be analyzed with the baseline as the reference. For this proposal specifically, the list is incomplete. For example, we proposed to study fast cell activation/deactivation and to study UL-only/DL-only/UL-DL cell activation, indicating of maximum number of scheduled active carrier, and PDCCH monitoring. Regarding the first bullet, we wonder whether the carriers here are PCell and SCells of a CA. It looks like CA with PCell having on-demand or low periodicity cell-specific signals + SSB-less SCells, or another form of multi carrier operation beyond CA. Further clarifications are necessary. Given “SSB less SCell operations” is listed, we suggest to delete this bullet for now. Proponent can further clarify this. Regarding the second bullet (single cell of multi carriers), this has not been defined, and may result in substantial changes on CA framework. Therefore, we suggest to have more discussion before agreeing to study energy efficiency benefit of this proposal. We think it might be early to list sub-bullets and potential solutions and our first preference is to remove them. Alternatively, we suggest following updates for the proposal. Proposal (Alt 1): Study multi-carrier/CA related enhancement direction(s)/solution(s) for 6G EE improvement, considering the following aspects Proposal (Alt 2): Study multi-carrier/CA related enhancement direction(s)/solution(s) for 6G EE improvement, considering at least the following aspects. - Anchor/beacon carrier and data carrier architecture - Anchor carrier provides AO SSB/OD SIB1/PRACH/paging on demand or at low periodicity - Data carriers remain dormant by default; (de)activation by network or UE demand - FFS: Single cell of multi carriers - SSB less SCell operations - Fast cell activation/deactivation - UL-only/DL-only/UL-DL cell activation - RF and baseband adaptation for PDCCH monitoring NR CA with SCell activation/deactivation and SCell dormant BWP should be the baseline for further discussion. |
| Huawei, HiSilicon |  | The scope of the proposal is unclear on whether/which UE state is relevant. For example, OD-SIB1 is for IDLE UEs, then data carriers cannot be dormant (which is an operation for connected UEs in CA). Another example for IDLE UEs, the data carrier may also provide PRACH resource for RACH offloading purpose. “low periodicity” is unclear – is the intention longer periodicity? In general we can simplify the proposal without the details: Identify multi-carrier/CA related enhancement direction(s)/solution(s) for 6G EE improvement, considering the following aspects - Anchor/beacon carrier and data carrier architecture - Anchor carrier provides AO SSB/OD SIB1/PRACH/paging on demand or at low periodicity - Data carriers remain dormant by default; (de)activation by network or UE demand - FFS associated signals/functions as well as signaling for respective carrier - Note: the name of the carrier is for discussion purpose - Single cell of multi carriers - SSB less SCell operations |
| Lenovo |  | Ok |
| ZTE, Sanechips |  | Suggest some updates with following reasons - Whether anchor carrier could have OD-SIB1, paging, PRACH should be further discussed. Therefore, at current stage, we can just say at least includes AO SSB. - Data carrier implies it is used for data transmission. We should make it more general to avoid confusion - Should not limit to the only case, Single cell of multi carriers. Proposal 3.4.1-1: Identify multi-carrier/CA related enhancement direction(s)/solution(s) for 6G EE improvement, considering the following aspects - Anchor/beacon carrier and data carrier architecture - Anchor carrier at least provides AO SSB/OD SIB1/PRACH/paging on demand or at low periodicity - Non-anchor Data carriers remain dormant by default; (de)activation by network or UE demand - Single cell of multi carriers associates to multi carriers/cells/TRPs - SSB less SCell operations |
| CEWiT | Y | Open to further discussYCEWiT |
| Samsung |  | Proposal 3.4.1-1: IdentifyStudy multi-carrier/CA related enhancement direction(s)/solution(s) for 6G EE improvement, considering the following aspects - Anchor/beacon carrier and data carrier architecture - Anchor carrier provides basic/common signals or channels (e.g., AO SSB or OD SSB/OD SIB1/PRACH/paging) on demand or at low adaptable periodicity - Data carriers remain dormant by default; (de)activation by network or UE demand - Single cell of with multi carriers - SSB less SCell operations across multiple cells - Sharing or offloading of signals or channels across multiple cells - Other aspects are not precluded |
| Fraunhofer | Y |  |
| TCL |  | Same views like LG. |
| Fainity | Y | Generally fine with listed aspects but wonder whether they be discussed separately or jointly since we believe they are mutually impacted by each other. |
| OPPO | Y | We support this proposal. Although we think that part of the study scope covers both idle and connected mode. Suggest to use study instead of identify. |
| Vodafone | Y |  |
| Futurewei |  | OK in principle, but more discussions are necessary to clarify the role of the anchor/beacon carriers. How is the impact on the idle captured? Let’s define the scenarios and UE (IDLE and connected) behavior first. |
| DCM | Y | We are fine with the direction to consider multi-carrier operation for both RRC Connected and RRC IDLE MODE. |
| Tejas | Y | Functionality of anchor carrier and data carrier can be FFS. |
| NEC |  | The concept of a "single cell of multi-carriers" should be discussed as part of the broader 6G overall procedure (11.1), as its implications extend beyond energy efficiency to other areas of improvement, such as scheduling. Therefore, from our perspective, our work under this agenda item can focus on studying the power-saving advantages of this approach and defining the requirements for such a framework to improve power efficiency. However, the design of the framework itself should be excluded from the scope of this particular item. |

Moderator thanks companies’ valuable feedback. On this topic, the following are identified issues and suggested proposal revision for further consideration (next meeting):

Issues Identified (Merged):

- Scope and Clarity Issues: Multiple companies requested clarification:

  - [Ofinno] and [ETRI] asked whether this applies to single-cell or multi-cell and idle or connected mode

  - [Apple] and [Panasonic] were unclear about "single cell of multi carriers" and its EE benefit

  - [Huawei, HiSilicon] pointed out unclarity regarding UE states

- Generality vs. Specificity: [LG Electronics], [Qualcomm], [Huawei, HiSilicon], and [vivo] preferred more general proposals:

  - [vivo] suggested focusing on fast SCell activation/deactivation

  - [Qualcomm] provided comprehensive alternative aspects to study

- Baseline Requirements: Need for clear baseline using NR CA with SCell activation/deactivation and dormancy

- Terminology: [Ofinno] suggested using single term instead of "anchor/beacon"

Revised Proposal:

Study multi-carrier/CA-related enhancements for 6G EE improvement, considering:

- NR CA with SCell activation/deactivation and dormancy as baseline

- Definition/behavior of anchor and data carriers (including always-on/periodic and/or on-demand/aperiodic cell signals)

- Fast SCell activation/deactivation

- SSB-less SCell operations

- RF/baseband adaptation for PDCCH monitoring

- Applicability to both single-cell and multi-cell deployments, and to both idle and connected modes

BWP Related Enhancements

[Qualcomm], [vivo], [Xiaomi], [Ericsson], [OPPO], [CMCC], [ZTE], [InterDigital], [MediaTek], et. al, support BWP enhancements. Companies emphasize need for simplified BWP switching, reduced parameters, and improved reliability. By the above, the following proposal is suggested:

Proposal 3.4.2-1: Identify BWP related enhancement direction(s)/solution(s) for 6G EE improvement, considering

- Reduced parameter set (e.g., bandwidth, rank, SSSG periodicity, min scheduling offset))

- Reduced data interruption impact by BWP switching (e.g., by group/common BWP switching)

- Improved BWP switching reliability

| Company | Support? (Y/N) | Comments/Revisions |
| --- | --- | --- |
| Ofinno | Y | We prefer to remove the (e.g.,)’s in the two bullets. See also our comment in 4.1. |
| LG Electronics |  | We suggest to remove detailed examples in the sub-bullets, as follows. Additionally, coordination in terms of which agenda item will treat BWP-related issue might be necessary. Proposal 3.4.2-1: Identify BWP related enhancement direction(s)/solution(s) for 6G EE improvement, considering - Reduced parameter set (e.g., bandwidth, rank, SSSG periodicity, min scheduling offset)) - Reduced data interruption impact by BWP switching (e.g., by group/common BWP switching) - Improved BWP switching reliability |
| CMCC | Y | Okay for further study |
| ETRI | Y | We prefer to remove examples at this stage. |
| Fujitsu | Y with comment | We suggest to remove the “e.g.,” in brackets. At the current stage, general directions would be sufficient. |
| Spreadtrum |  | For three sub-bullets, it is unclear whether they are beneficial for energy saving. And it is also unclear whether group/common BWP switching is beneficial for data interruption. |
| Panasonic | Y |  |
| CATT | Y | Okay for further study |
| xiaomi | Y with comment | We agree the direction of BWP simplification/enhancement. On the other hand, we think we don’t need to touch many details at this stage. The detailed mechanisms can be discussed in later phase. Proposal 3.4.2-1: Identify BWP related enhancement direction(s)/solution(s) for 6G EE improvement, considering - Reduced parameter set (e.g., bandwidth, rank, SSSG periodicity, min scheduling offset)) - Reduced data interruption impact by BWP switching (e.g., by group/common BWP switching) - Improved BWP switching reliability |
| vivo |  | We believe the key motivation for BWP switching is for UE power saving. This is true for 6G as well. About the example of (e.g., by group/common BWP switching), we wondered by group/common BWP switching, how the data interruption impact by BWP switching can be reduced? In addition, as we discussed in our paper, we see EE for DL BWP switching from wider BW to a narrow BW. But for UL, UL BWP switching from wider BW to a narrow BW does not bring much power saving given PA is the dominant factor of the power consumption rather than the BW. At this stage, the signalling design for BWP switching can be discussed later, once we agree by dynamic BWP switching , EE can be achieved. Suggest following Proposal 3.4.2-1: Identify BWP related enhancement direction(s)/solution(s) for 6G UE EE improvement, considering - Reduced parameter set configured per BWP (e.g., bandwidth, rank, SSSG periodicity, min scheduling offset)) - Identify the parameters contributing to the power consumption - Reduced data interruption impact by BWP switching (e.g., by group/common BWP switching) - Improved BWP switching reliability - FFS the need of UL BWP switching for UE power saving |
| Nokia |  | It is being proposed to study the UE operating bandwidth framework in 11.1, thus the considered schemes should eventually be subjected to the operating BW framework (e.g. improved NR BWP). The parameter set reduction etc. may be covered in considerations in 11.1 to simplify the operation. Improving the reliability of operating BW adaptation and reducing the delay are important elements to enhance, but these are not only related to EE, but they are also more generic. In summary, while these are important aspects to study, we are not sure they should be covered under EE agenda. |
| Ericsson |  | Add: “BWP switching reliability and delay” |
| Google | Yes with comment | We think BWP related enhancement is needed, which is a bit complicated in 5G. However, we have one clarification question on the first bullet. Is the intention to say “reduced parameter set to be updated during BWP switching” or “reduced parameter set in a BWP configuration”. |
| InterDigital |  | Prefer to not take 5G BWP as starting point for the discussion on dynamic BW adaptation. Suggested rewording: Study control of UE operating bandwidth for 6GR: Minimize data interruption and maximize reliability when switching between narrowband and wideband operation Reduced parameter set when in narrowband operation […] Note: there seems to be overlap between this proposal and FL proposals in AI 11.1 and 11.3.2. |
| Apple |  | It is not clear to us how “group/common BWP switching” can reduce the data interruption impact. |
| Qualcomm |  | In general, we agree to study the adaptation mechanisms and address issues raised above. However, the solution might not replace the BWP framework, but be within it, similar to how SSSG switching works in NR. Therefore, we propose to study an adaptation mechanism and separate we can discuss BWP related issues. Proposal 3.4.2-1: Identify Study BWP UE adaptation related enhancement direction(s)/solution(s) for 6G EE improvement, considering - Reduced parameter set (e.g., bandwidth, rank, SSSG periodicity, min scheduling offset)) - Reduced data interruption impact by BWP switching (e.g., by group/common BWP switching) - Improved BWP switching reliability Generally, we think it would be best to separate the discussion about how to improve the BWP framework and about what needs to be adapted for power savings. |
| Huawei, HiSilicon |  | First, it is a little bit confusing how to ‘reduce’ the parameter set. We understand the intention is to have limited number of parameter set or limited number of candidate values. Thus we suggest to use ‘restricted’. Second, reducing the power consumption of PDCCH itself (e.g., studying the suitable PDCCH CORESET size) is also important for PDCCH power reduction. Third, depending on UE implementation, UE power consumption may not scale linearly along with BW. Therefore, it is important to study the BWP size granularity. A coarser BWP granularity can be considered. Based on the above, we have the following suggestion. Proposal 3.4.2-1-Huawei: Identify BWP related enhancement direction(s)/solution(s) for 6G EE improvement, considering - Restrictedduced parameter set or values (e.g., CORESET size, bandwidth, rank, SS/SSSG periodicity, min scheduling offset)) - Reduced data interruption impact by BWP switching (e.g., by group/common BWP switching) - Improved BWP switching reliability - BWP granularity |
| Lenovo | Y |  |
| ZTE, Sanechips |  | The first subbullet is a solution, but the third subbullet is a direction. The subbullets should be in parallel. We should make the proposal more general, instead of touching the detailed schemes currently Proposal 3.4.2-1: Identify BWP related enhancement direction(s)/solution(s) for 6G EE improvement, considering - Reduced power consumption parameter set (e.g., bandwidth, rank, SSSG periodicity, min scheduling offset)) - Reduced switching delay data interruption impact by BWP switching (e.g., by group/common BWP switching) - Improved BWP switching reliability Or we can list all the candidate solutions. |
| Samsung | N | “Study” instead of “identify” First discuss whether/when configuration of multiple BWPs is needed before discussing switching. No need to include specific examples in the parameter set although “bandwidth” is acceptable. Same for the “reduced data interruption impact” which is also related to “reduced parameter set”. We do not identify any issue with BWP switching reliability – also, discussion, if any, this will need to be for any indication in general (e.g. cell DTX/DRX) and not only for BWP switching. |
| TCL | Y |  |
| Fainity |  | Dormant BWP and further enhancements should be included in the study as well, especially RRM behaviors. |
| OPPO | Y | The name of BWP could be generalized as “TX/RX parameter adaptation”. It could be BWP or non-BWP. Also we would be happy to decide earlier if the BWP mechanism is kept in 6G. |
| Futurewei |  | OK to study |
| DCM |  | We are OK with first and second sub-bullet. For first sub-bullet, we consider that BWP changes parameter as only bandwidth. For third sub-bullet, what is the meaning of “Improved BWP switching reliability” ? Does this mean improving BWP switching delay or other aspect? |
| Tejas | Y |  |

Moderator thanks companies’ valuable feedback. On this topic, the following are identified issues and suggested proposal revision for further consideration (next meeting):

Issues Identified:

- Generality vs. Specificity: [LG Electronics], [Xiaomi], [Fujitsu], [InterDigital], and [Ericsson] suggested removing examples and keeping the proposal general

- Clarity and Scope Concerns:

  - [Spreadtrum] and [vivo] questioned the EE benefit of some proposed enhancements

  - [vivo] questioned the need for UL BWP switching for power saving

  - [Nokia] and [InterDigital] pointed out overlap with other agenda items

  - [Qualcomm] proposed separating BWP framework discussion from power savings adaptation

- Terminology Evolution: [OPPO] suggested generalizing "BWP" to "TX/RX parameter adaptation" or "UE operating bandwidth"

- Enhancement Focus: [Huawei, HiSilicon], [vivo], and [Samsung] emphasized minimizing interruption, maximizing reliability, and considering BWP granularity

Revised Proposal:

Study control of UE operating bandwidth for 6G to improve UE EE, considering at least the following aspects:

- Minimize data interruption and maximize reliability when switching between narrowband and wideband

- Reduce parameter set

- Consider BWP granularity and switching procedures

- UL specifics and necessity for UE power saving are FFS

Other Frequency-Domain Enhancements

If the above proposals cannot include your frequency-domain EE improving schemes, companies are encouraged to provide comments/references for other companies’ investigation/evaluation.

| Company | Comments for additional enhancement(s) |
| --- | --- |
| Qualcomm | We propose to study decoupling, or independent adaptation, of RF and baseband power state. This mechanism enables the UE to more efficiently receive wideband transmission when the full throughput of such a bandwidth will not be sustained. This is a mechanism that balances UE and network energy by enabling the network to move to the more efficient wideband scheduling and at the same time allows the UE baseband to operate slower in an energy-efficient state. |

Moderator thanks companies’ valuable feedback and would like to suggest companies’ further consideration of the above techniques for potential 6G EE improvement.

Time-Domain Enhancement Directions

PDCCH Monitoring Adaptation

[Apple], [Ericsson], [LG], [Qualcomm], [ZTE], [OPPO], [InterDigital], et al, support to harmonize C-DRX, PDCCH skipping, SSSG switching, LP-WUS, toward a unified PDCCH monitoring adaptation design. The following are therefore proposed for further study:

Proposal 3.5.1-1: Identify enhancement direction(s)/solution(s) related to PDCCH monitoring adaptation for 6G EE improvement, considering

- Achieving reduced PDCCH monitoring power consumption with minimum impact to BS scheduling, by e.g., default min scheduling offset > 0 and reset to 0 after 1st data scheduling, two-stage control indications: low-power pre-indication before full control decode

- Unified design of C-DRX, PDCCH skipping, SSSG switching, and 6G WUS

- Mechanism(s) to indicate PDCCH monitoring occasion(s) or blind decoding information

| Company | Support? (Y/N) | Comments/Revisions |
| --- | --- | --- |
| Ofinno |  | First bullet may be too detailed for this stage. Suggest to make it more general (e.g., reduced PDCCH monitoring power consumption considering BS impact) Second bullet seems to imply all the features are studied in detail. Maybe better to say considering minimized number of features for similar functionality |
| LG Electronics |  | We suggest to delete detailed examples in the first sub-bullet, as follows. Proposal 3.5.1-1: Identify enhancement direction(s)/solution(s) related to PDCCH monitoring adaptation for 6G EE improvement, considering - Achieving reduced PDCCH monitoring power consumption with minimum impact to BS scheduling, by e.g., default min scheduling offset > 0 and reset to 0 after 1st data scheduling, two-stage control indications: low-power pre-indication before full control decode - Unified design of C-DRX, PDCCH skipping, SSSG switching, and 6G WUS - Mechanism(s) to indicate PDCCH monitoring occasion(s) or blind decoding information |
| CMCC | Y | Okay for further study |
| ETRI |  | We support the main sentence. However, bullet points seem too details at this stage. |
| Fujitsu | Y with comment | Same view as proposal 3.4.2-1, we prefer to remove the “e.g.,” in the first bullet. |
| Spreadtrum |  | For first sub-bullet, it seems that “by e.g., default min scheduling offset > 0 and reset to 0 after 1st data scheduling, two-stage control indications: low-power pre-indication before full control decode” are detail technologies. It is too early to discuss and should be removed. For second sub-bullet, it is unclear what the “unified design” means. For third sub-bullet, its function is to reduce PDCCH monitoring power consumption, which maybe overlap with first sub-bullet We suggest to revise the proposal as follow Proposal 3.5.1-1: Identify enhancement direction(s)/solution(s) related to PDCCH monitoring adaptation for 6G EE improvement, considering - Achieving reduced PDCCH monitoring power consumption with minimum impact to BS scheduling, by e.g., default min scheduling offset > 0 and reset to 0 after 1st data scheduling, two-stage control indications: low-power pre-indication before full control decode - Unified design of C-DRX, PDCCH skipping, SSSG switching, and 6G WUS - Mechanism(s) to indicate PDCCH monitoring occasion(s) or blind decoding information |
| Panasonic | In general Y | On the first bullet, we think to adapt the minimum scheduling gap has merit but the example of “default min scheduling offset > 0 and reset to 0 after 1st data scheduling” is too specific to agree on at this stage. |
| CATT |  | We prefer LG’s version without more details at this stage. |
| Xiaomi | Y with comment | - Agree with LGE that we don’t need mention details - For ‘Unified design of C-DRX, PDCCH skipping, SSSG switching, and 6G WUS’, it is too early to say we can achieve unified design for C-DRX, PDCCH skipping, SSSG switching and 6G WUS. We suggest to have generic wording like ‘unified design to reduce PDCCH monitoring’ Hence, we have the following suggestion: Proposal 3.5.1-1: Identify enhancement direction(s)/solution(s) related to PDCCH monitoring adaptation for 6G EE improvement, considering - Achieving reduced PDCCH monitoring power consumption with minimum impact to BS scheduling, by e.g., default min scheduling offset > 0 and reset to 0 after 1st data scheduling, two-stage control indications: low-power pre-indication before full control decode - Unified design of C-DRX, PDCCH skipping, SSSG switching, and 6G WUS reducing PDCCH monitoring - Mechanism(s) to indicate PDCCH monitoring occasion(s) or blind decoding information |
| vivo |  | There are many solutions to adapt PDCCH monitoring for UE power saving, we think the existing solutions that bring the most power saving gain and less impact on the performance should be prioritized. Any new solutions should be evaluated carefully and compare with existing solutions. The original proposal provide too many details. Therefore, suggest following Proposal 3.5.1-1: For PDCCH monitoring adaptation for 6G UE EE improvement, study existing solution(s) and potential enhancements that bring the most power saving gain with minimum impact to BS scheduling, including the following - C-DRX, - PDCCH skipping, - SSSG switching, - Cross-slot scheduling - 6G WUS |
| Nokia |  | Add “at least” to first line. Remove eg example – too detailed for a high level proposal Final bullet can be deleted, seems unnecessary given earlier bullet |
| Ericsson | OK, comment | ‘default min scheduling offset > 0 and reset to 0 after 1st data scheduling’ will lead to multiple k0 values in the scheduler. |
| Google |  | As mentioned by some companies, we also think first bullet is too detailed. We can remove the “e.g.” part. On the second bullet, we suggest adding “Strive for” to soften the wordings. Hence, we suggest the following revisions. Proposal 3.5.1-1: Identify enhancement direction(s)/solution(s) related to PDCCH monitoring adaptation for 6G EE improvement, considering - Achieving reduced PDCCH monitoring power consumption with minimum impact to BS scheduling, by e.g., default min scheduling offset > 0 and reset to 0 after 1st data scheduling, two-stage control indications: low-power pre-indication before full control decode - Strive for Uunified design of C-DRX, PDCCH skipping, SSSG switching, and 6G WUS - Mechanism(s) to indicate PDCCH monitoring occasion(s) or blind decoding information |
| InterDigital |  | We support Xiaomi’s version. |
| Apple |  | The first bullet can be replaced by “cross-slot scheduling enhancement”, if the intention is to cover QC’s proposal. However, this does not seem to belong to PDCCH monitoring adaptation. Proposal 3.5.1-1: Identify potential enhancement direction(s)/solution(s) related to reduced power consumption for PDCCH monitoring adaptation for 6G EE improvement, considering e.g. - Achieving reduced PDCCH monitoring power consumption with minimum impact to BS scheduling, by e.g., default min scheduling offset > 0 and reset to 0 after 1st data scheduling, two-stage control indications: low-power pre-indication before full control decode - Unified design for PDCCH monitoring adaptation, taking into account of C-DRX, PDCCH skipping, SSSG switching, and 6G WUS - Mechanism(s) to indicate PDCCH monitoring occasion(s) or blind decoding information |
| Qualcomm |  | Similar to other cases, we propose to replace “identify” with “study” and the list of items could be removed or extended to cover more cases. |
| Huawei, HiSilicon |  | First, the ‘Achieving reduced PDCCH monitoring power consumption with minimum impact to BS scheduling, by e.g., default min scheduling offset > 0 and reset to 0 after 1st data scheduling’ is more related to power saving for data transmission, instead of PDCCH monitoring. Also, it is better not to have such a detailed description at the very beginning. Second, PDCCH monitoring triggered by drx-InactivityTimer due to UL pre-scheduling is also a serious issue, which should be studied. Proposal 3.5.1-1-Huawei: Identify enhancement direction(s)/solution(s) related to PDCCH monitoring adaptation for 6G EE improvement, considering - Achieving reduced PDCCH monitoring power consumption with minimum impact to BS scheduling, by e.g., default min scheduling offset > 0 and reset to 0 after 1st data scheduling, two-stage control indications: low-power pre-indication before full control decode - Unified design of C-DRX, PDCCH skipping, SSSG switching, and 6G WUS - Mechanism(s) to indicate PDCCH monitoring occasion(s) or blind decoding information - Mechanism(s) to avoid unnecessary inactivitytimer extension for reducing PDCCH monitoring occasions |
| Lenovo |  | Ok |
| ZTE, Sanechips |  | Open to study. But want to clarify if LP-WUS is introduced, do we really need to enhance the above techniques? I guess the first thing we need to consider is the necessity of the legacy features and the necessity of the enhancement. |
| CEWiT |  | Okay for further study |
| Samsung |  | “Study” instead of “identify” Proposal 3.5.1-1: The specifics/sub-bullets for consideration should be removed as there are many other mechanisms to be considered and all need discussion. |
| Fraunhofer | Y |  |
| TCL |  | Suggest to delete detailed information at this stage. |
| Fainity | Y | Two-stage control indications vs. LP-WUS may require further justification on what the difference is. |
| OPPO | Y | We agree the proposal |
| Futurewei |  | OK to study |
| DCM |  | For first sub-bullet, we are OK to discuss cross-slot scheduling but current wording is too detail. We propose as following Proposal 3.5.1-1: Identify enhancement direction(s)/solution(s) related to PDCCH monitoring adaptation for 6G EE improvement, considering - Unified design of C-DRX, PDCCH skipping, SSSG switching, and 6G WUS - Mechanism(s) to indicate PDCCH monitoring occasion(s) or blind decoding information |
| Tejas | Y | There should not be any restriction on the type of scheme under first bullet. Hence, we suggest the following updated proposal Identify enhancement direction(s)/solution(s) related to PDCCH monitoring adaptation for 6G EE improvement, considering - Achieving reduced PDCCH monitoring power consumption with minimum impact to BS scheduling, by e.g., default min scheduling offset > 0 and reset to 0 after 1st data scheduling, two-stage control indications: low-power pre-indication before full control decode - Unified design of C-DRX, PDCCH skipping, SSSG switching, and 6G WUS - Mechanism(s) to indicate PDCCH monitoring occasion(s) or blind decoding information |
| NEC |  | Joint design of cell DTX/DRX and UE C-DRX should also be included in this part |

Moderator thanks companies’ valuable feedback. On this topic, the following are identified issues and suggested proposal revision for further consideration (next meeting):

Issues Identified:

- Generality vs. Specificity: [LG Electronics], [ETRI], [Fujitsu], [Nokia], [Apple], and [Samsung] suggested removing detailed examples and keeping the proposal general

- Unified Design Clarity: [Spreadtrum] and [Xiaomi] were unclear about "unified design" meaning, suggesting more generic wording for "harmonized and simplified design"

- Scope and Overlap: [Spreadtrum] pointed out potential overlap between bullet points, while [vivo] suggested studying existing solutions and their enhancements

- Additional Mechanisms: [Huawei, HiSilicon] proposed adding mechanisms to avoid unnecessary inactivity timer extension due to UL pre-scheduling

- Cross-Feature Integration: Need to strive for unified design across related features (C-DRX, PDCCH skipping, SSSG switching, LP-WUS)

Revised Proposal:

Study enhancements for PDCCH monitoring adaptation for 6G EE, aiming to:

- Reduce PDCCH monitoring power consumption with minimal BS scheduler impact

- Strive for harmonized and simplified design across related features (C-DRX, PDCCH skipping, SSSG switching, LP-WUS)

- Provide mechanisms to indicate PDCCH monitoring occasions and blind-decoding information

- Avoid unnecessary inactivity timer extensions due to UL pre-scheduling

Connected Mode WUS/WUR Operation

[vivo], [CMCC], [NTT DOCOMO], [Ericsson], [Samsung], [Apple], [FUTUREWEI], [Nokia], [Huawei], [CATT], [Qualcomm], [ZTE], [InterDigital], [Vodafone], [LG], [NEC], [MediaTek], et. al, strongly support 6G WUS/WUR for connected mode. Companies emphasize OFDM-based design for better coverage.

Proposal 3.5.2-1: Identify enhancement direction(s)/solution(s) related to connected mode WUS/WUR operations for 6G EE improvement, considering

- Performance (coverage) and signal overhead trade-off

- Reliable DRX-less operation

- Integration with PDCCH monitoring adaptation mechanism(s)

- Implementation flexibility without requiring separate receiver

- Support small payload in WUS

| Company | Support? (Y/N) | Comments/Revisions |
| --- | --- | --- |
| Ofinno |  | Prefer not to list all the techniques at this stage and with limited time to discuss on each one. Maybe we can just agree to the main bullet for now. |
| LG Electronics | N | Several sub-bullets (e.g., reliable DRX-less operation and small payload in WUS) don’t seem to be suitable under the main bullet, in addition, Proposal 3.5.1-1 and Proposal 3.5.2-1 seem to be overlapped. It would be preferrable to combine two proposals at once. |
| CMCC | Y | Okay for further study |
| ETRI |  | We support the main sentence. However, bullet points seem too details at this stage. |
| Spreadtrum |  | “Reliable DRX-less operation” is unclear for us. It is too early to support small payload in WUS. We suggest to revise the proposal as follow Proposal 3.5.2-1: Identify enhancement direction(s)/solution(s) related to connected mode WUS/WUR operations for 6G EE improvement, considering - Performance (coverage) and signal overhead trade-off - Reliable DRX-less operation - Integration with PDCCH monitoring adaptation mechanism(s) - WUS/WUR based PDCCH monitoring adaptation schemes - Implementation flexibility without requiring separate receiver - Support small payload in WUS |
| CATT |  | Seams that there are some overlap between Proposal 3.5.1-1 and 3.5.2-1, e.g. Unified design of C-DRX, PDCCH skipping, SSSG switching, and 6G WUS in Proposal 3.5.1-1 and Integration with PDCCH monitoring adaptation mechanism(s) in Proposal 3.5.2-1, both are trying to say how the WUS/WUR operation effect the PDCCH monitoring. |
| Xiaomi |  | We suggest we first try to identify what need to enhance for LP-WUS/WUR, instead of diving detailed solutions. With this in mind, we support the version from Spreadtrum. |
| vivo |  | This proposal the main target is UE power saving in our understanding. From our reading, many companies proposed following enhancements for connected mode WUS/WUR operations: better coverage, robustness, and efficiency by reducing the overhead and carry more information bits. In addition about whether separate receiver is needed or not is more like implementation issue, even in 5G, the LP-WUS feature does not necessarily require the UE to implement a separate receiver . Therefore, following update is proposed Proposal 3.5.2-1: Identify enhancement direction(s)/solution(s) related to connected mode WUS/WUR operations for 6G UE EE improvement, considering - Performance (coverage) and signal overhead trade-off - Reliable C-DRX-less operation - Integration with PDCCH monitoring adaptation mechanism(s) - Implementation flexibility without requiring separate receiver - Study the information Support small payload in WUS - Study further use cases besides waking-up indication |
| Nokia |  | Add “at least” to first line Remove reliable DRX-less operation Add “Support measurements” – a key potential benefit of the WUR/Efficient power mode of operation, is to perform measurements without having to fully power up the whole device. |
| Ericsson |  | - First bullet, not clear on the meaning of 'performance and signal overhead tradeoff'. If we apply only OFDM-based WUR, both performance and overhead aspects are improved. Or we can directly propose considering OFDM-based WUR as starting point as in Idle mode. - To add NW energy consumption and gNB complexity - “without requiring separate receiver” this cannot be ensured as it depends on the implementation. - To remove “Support small payload in WUS”. Instead, WUR/WUS functionalities in addition to wake-up indication (e.g., RRM). Early to discuss payload as it depends on other aspects. - Add another bullet for RRM measurement. |
| Google |  | We also think DRX-less operation is unclear. We can support other bullets. |
| InterDigital |  | As we commented before, we prefer not to list all the candidate options. To focus on the principle, we propose the following update. Identify enhancement direction(s)/solution(s) related to connected mode WUS/WUR operations for 6G EE improvement, considering - Performance (coverage) and signal overhead trade-off - Reliable DRX-less operation - Integration with PDCCH monitoring adaptation mechanism(s) - Implementation flexibility without requiring separate receiver - Support small payload in WUS |
| Apple |  | Suggest replacing “Performance (coverage) and signal overhead trade-off” with “Tradeoff between power consumption and performance (coverage)/overhead” It is not clear what “DRX-less operation” is. Is it LP-WUS without C-DRX? “Integration with PDCCH monitoring adaptation mechanism(s)” is already covered in the previous proposal. We would like some clarification on “Implementation flexibility without requiring separate receiver” and its implication. Suggest replacing the last bullet with “other payload/signaling that can be received by the low power receiver” |
| Qualcomm |  | We suggest to rephrase “Identify” with “Study”, same as our comments to the other proposals. Similar to our comment to Proposal 3.2-2, we propose not to define/assume a separate receiver for energy-efficient mode; rather, it should be treated as a UE state that saves energy consumption while operates a certain type of communication (e.g., WUS or WUS-like signal reception). On the bullets under Proposal 3.5.2-1, we have not yet common understanding what these are. We suggest to delete these for now. Having said that, we propose to update the main bullet as follows: Study energy saving benefits and performance/overhead trade-off of energy efficient UE modem operation in connected mode. NR OFDM-based LP-WUS signal design and the corresponding UE procedures can be taken as the starting point. Companies to provide necessary changes/enhancements to the NR OFDM-based LP-WUS design and procedures. |
| Huawei, HiSilicon |  | For the last bullet, ‘small payload’ is not clear to us. Thanks to the OFDM based waveform, more information bits can be supported, thus more functionality (other than simple wakeup) can be supported. Proposal 3.5.1-1-Huawei: Identify enhancement direction(s)/solution(s) related to connected mode WUS/WUR operations for 6G EE improvement, considering at least - Performance (coverage) and signalling overhead trade-off - Reliable DRX-less operation - Integration with PDCCH monitoring adaptation mechanism(s) - Implementation flexibility without requiring separate receiver - Supported functionalities small payload in of WUS |
| Lenovo |  | Add potential impact to NW energy savings in the bullet. |
| ZTE, Sanechips |  | Also should include - Function extension, e.g., WUS carrying control information. Additionally, for the subbullet ‘Implementation flexibility without requiring separate receiver’, if it is implementation issue, we do not need to consider here. |
| TCL |  | We are fine with main bullet but for five bullets, it seems like no unified direction to discuss, e.g., we are confused what is implementation flexibility for WUS/WUR operations without requiring separate receiver. We thus suggest to simplify the bullet explanations. |
| Fainity | Y | The payload size carried by WUS should be carefully designed to reduce unnecessary UE wake-ups. |
| OPPO | Y |  |
| Futurewei |  | OK with the main bullet. The subbulets require more discussions. |
| DCM |  | For third sub-bullet, there are possibility of coexistence between LP-WUS and PDCCH monitoring adaptation mechanism(s). It should be more general word. For fifth sub-bullet, there are possibility of larger payload for larger subgroups. This also should be more general word. We propose as following Proposal 3.5.2-1: Identify enhancement direction(s)/solution(s) related to connected mode WUS/WUR operations for 6G EE improvement, considering - Performance (coverage) and signal overhead trade-off - Reliable DRX-less operation - Integration/coexistence with PDCCH monitoring adaptation mechanism(s) - Implementation flexibility without requiring separate receiver - Updating payload in WUS |
| NEC | Y |  |

Moderator thanks companies’ valuable feedback. On this topic, the following are identified issues and suggested proposal revision for further consideration (next meeting):

Issues Identified:

- Overlap with Previous Proposal: [LG Electronics] and [CATT] pointed out overlap with Proposal 6.1-1, with LG suggesting to combine them

- Clarity and Scope Issues:

  - [Spreadtrum] and [Google] were unclear about "reliable DRX-less operation"

  - [Ericsson] questioned "performance and signal overhead tradeoff"

  - [vivo] and [Huawei, HiSilicon] suggested studying information payload and further use cases

- Implementation Flexibility: [Ericsson], [ZTE, Sanechips], [Nokia], [Apple], and [TCL] noted that avoiding separate receiver is implementation issue and should not be mandated

- Generality Preference: [Ofinno], [InterDigital], [Qualcomm] preferred more high-level proposal, with [Qualcomm] suggesting "energy-efficient UE modem operation" starting from NR LP-WUS

- Additional Functions: [Nokia] and [Apple] emphasized considering additional functions beyond wake-up (e.g., RRM measurements)

Revised Proposal:

Study energy-efficient UE modem operation in CONNECTED mode, considering OFDM-based WUR and the following aspects:

- Power/performance/overhead trade-offs for both UE and network

- Integration with PDCCH monitoring adaptation mechanisms

- Implementation flexibility (no architectural mandate for separate receiver)

- Additional functions beyond wake-up (e.g., RRM measurements)

- Coverage, latency, and reliability considerations

Other Time-Domain Enhancements

It is noticed that cell-DTX/DRX and on-demand SSB/SIB1 operation will be discussed and handled by FL (Ericsson/Magnus) of idle-mode energy efficiency enhancements. If your timing-domain EE improving scheme(s) can still not be properly included, companies are encouraged to provide comments/references for other companies’ investigation/evaluation.

| Company | Comments for additional enhancement(s) |
| --- | --- |
| vivo | We think following time domain enhancements are also important for 6G UE EE - Relax UE L1 and L3 measurement and reporting and its appropriate application scenario(s) - Study UE uplink DTX to reduce the transmission duration and increase sleep duration for UE in 6GR, details in our contribution R1-2505420 |
| Nokia | Cell-DTX/DRX and on-demand SSB/SIB1 operations can be applied to the UE in RRC connected-mode as well. Is it going to be considered in idle-mode energy efficiency enhancements as well? This point is unclear to us. Please clarify it. |
| Google | In 5G, Cell DTX/DRX operation is typically governed by a semi-static pattern. This framework lacks a dynamic mechanism to notify UE that a sleeping cell is awakened for urgent communications or an active cell is opportunistically put into sleep mode during sudden drops in traffic load. Consequently, the network cannot efficiently adapt to real-time load variations. Hence, we propose to also study dynamic switch between active and non-active duration. |
| Apple | We do not think cell DTX/DRX should be discussed in idle mode agenda. In fact the impact is probably much bigger for connected mode. It is better to discuss it here at least for connected mode. |
| Qualcomm | While not strictly a time-domain enhancement, baseband power state could be driven by uplink instead of downlink this is due to late changes to an uplink transmission and could be prevented by studying methods to guarantee no lates changes to uplink transmissions. Proposal: Study how to guarantee no late changes to UL transmissions, to save UE baseband energy. We propose to study autonomous PUSCH transmission as a method to reduce PDCCH monitoring at the UE and PDCCH transmission at the base station. Proposal: Study a mode for PUSCH transmission without a grant and with UE selected parameters. |
| Huawei, HiSilicon | In 5G, most of the power saving gain comes from reducing unnecessary PDCCH monitoring, since the percentage of PDCCH-only is high. However, in 6G, it is expected that the data volume will be highly increased than 5G. Therefore, energy saving for data transmission should be studied for both gNB and UE. In our tdoc, we proposed ways to reduce the energy consumption for data transmission, by e.g., aggregated transmission, or avoid transmitting padding bits (e.g., for UL pre-scheduling scenario). Also, as we commented under 6.2, cross-slot scheduling can be discussed. Furthermore, cell DTRX discussed in another FL Summary seems to focus on idle mode, which is against the original cell DTRX and even UEDRX operation. Therefore, we have the following suggestion. Proposal 6.3-1: Identify enhancement direction(s)/solution(s) related to data transmission for 6G EE improvement, considering - Aggregated transmission in one-shot - Enhancements to cell DTRX operation, e.g. dynamic cell DTRX pattern adapatation - Reduce the power consumption of transmitting padding bits - Cross-slot scheduling for reducing the power consumption of data transmission |
| Samsung | cell-DTX/DRX and on-demand SSB/SIB1 should be studied for CONNECTED mode as well. |
| OPPO | Cell-DTX/DRX for 6G should also study the following aspects which we believe are not in the scope of idle-mode energy efficiency. From our contribution R1-2505761, we see the following aspects and suggest to be studied: -Study to improve the connected UE scheduling delay -Study to further reduce the TRS transmission |
| DCM | As vivo’s commnet, we also consider direction of relax UE L1 and L3 measurement and reporting and its appropriate application scenario(s). High level part should be discussed in RAN1. |

Moderator thanks companies’ valuable feedback and would like to suggest companies’ further consideration of the above techniques for potential 6G EE improvement.

Power-Domain Enhancement Directions

For power domain enhancements, low-PAPR waveform is proposed by [TCL], [NEC], [Lenovo], [Rakuten], et. al, yet waveform agenda looks a more reasonable place for comprehensive and deep study. From companies contributions, it is possible to further study the following aspects: Adaptive emission masks and context-based EVM targets [TCL], Multi-CSI based adaptation in power domain [CATT], [ZTE], [CMCC], and granular DL power control framework [Vodafone]:

Proposal 7-1: Identify enhancement direction(s)/solution(s) related to power-domain adaptation for 6G EE improvement, considering

- Efficient CSI feedback for power-domain adaptation

- Granular DL power control framework

- Adaptive emission masks and context-based EVM targets

| Company | Comments for additional enhancement(s) |
| --- | --- |
| Ofinno | Okay with main bullet. See our comment in 4.1. |
| LG Electronics | We fail to understand what “granular DL power control framework” means and the last bullet seems to be RAN4’s scope. With this regard, we would suggest to delete the last two sub-bullets. |
| CMCC | Okay for further study the main bullets and first sub-bullet which is already discussed in Rel-18. |
| ETRI | Same view as LG. Okay with main sentence and first bullet point. |
| Fujitsu | We are fine with the main bullet and the first sub-bullet. |
| CATT | OK with the first sub-bullet. More clarification is needed for the second sub-bullet |
| Xiaomi | Agree with LGE. |
| vivo | Each technique for EE study should be clear about the target, this EE target is mainly for BS or for UE before we agree to study; otherwise it is not clear what and how to study. For example, we suspect the first and 2nd may be for BS EE, the third Adaptive emission masks and context-based EVM targets may be for UE and may also be related to waveform. |
| Nokia | We may consider also the following aspects: - Reduced CSI-RS overhead (e.g. scaling with cell load) - similarly to 4.1 - Reduced CSI calculation complexity and feedback overhead - similarly to 4.1 - Efficient SSB power ramp-down for cell shutdown |
| Ericsson | Same view as LG, CMCC,… |
| InterDigital | Fine with the main bullet and the first sub-bullet. |
| Apple | It is not clear to us what “Granular DL power control framework” means. The last bullet does not seem to belong to RAN1. |
| Qualcomm | Similar to the discussion in Section 4, we think this proposal is too detailed for this stage of discussion and propose to keep only a high-level main proposal. Identify Study enhancement direction(s)/solution(s) related to power-domain adaptation for 6G EE improvement, considering • Efficient CSI feedback for power-domain adaptation • Granular DL power control framework • Adaptive emission masks and context-based EVM targets |
| Huawei, HiSilicon | We suggest to add “UL wideband transmission with lower PAPR”. |
| ZTE, Sanechips | Also feel the third bullet is out of RAN1 scope. |
| CEWiT | Okay for further study the main bullets and first sub-bullet which is already discussed in Rel-18. Second sub-bullet needs more clarity. |
| Samsung | “Study” instead of “identify” The specifics/sub-bullets for consideration should be removed as there are many other mechanisms to be considered and all need discussion. |
| TCL | Okay with the first bullet. |
| OPPO | We suggest to use study instead of identify since it is too early to identify the enhancement at this moment. Suggested updates Proposal 7-1: Study Identify enhancement direction(s)/solution(s) power-domain related enhancements to power-domain adaptation for 6G EE improvement, considering - Efficient CSI feedback for power-domain adaptation - Granular DL power control framework - Adaptive emission masks and context-based EVM targets |
| Vodafone | Support the proposal. To add more detail on granular DL power control framework: We have identified two specific scenarios where dynamic DL power control could be beneficial from an energy saving perspective, particularly in cases of average to high instantaneous PRB load, i.e., when most of the PRBs during a time slot are allocated for DL data transmission: - Single-user slot: When a single heavy user is engaged in significant data transfer that occupies most of or all the available PRBs. - If the user is in good radio condition, important reductions in DL power might result in insignificant decrease in throughput. Here, power control would involve uniformly reducing transmitted power across all subcarriers, maintaining a flat Power Spectral Density (PSD) of the transmitted DL signal, which will not pose any challenge for the PA implementation at Base Station side. - Multi-user slot: When multiple users are allocated PRBs within the same time domain slot. - As different UEs experience varying radio conditions, or due to small data buffer requirements leading to over-dimensioned PRB allocations for a given UE (the UE gets more PRBs than really needed), adjusting the PSD across different PRB allocations may be advantageous. Specifically, reducing the PSD for users in good radio conditions or with overbooked PRB allocations compared to the maximum PSD applied by default in 4G and 5G could be beneficial from the power consumption perspective. However, this approach presents complexities and challenges related to Power Amplifier (PA) efficiency, as the PSD of the DL signal, typically constant in 4G/5G, would vary across subcarriers/PRBs allocated to different users. This study may need to have feedback from RAN4. |
| Futurewei | OK to study, the third bullet is not in the scope. Make clear that other solutions may be further considered. |
| NEC | Support |
| AT&T | Second bullet is not clear and last bullet should not be RAN1 led. |

If the above proposal cannot include your power-domain EE improving schemes, companies are encouraged to provide comments/references for other companies’ investigation/evaluation.

| Company | Comments for additional enhancement(s) |
| --- | --- |

Moderator thanks companies’ valuable feedback. On this topic, the following are identified issues and suggested proposal revision for further consideration (next meeting):

Issues Identified:

- RAN1 Scope Concerns: [LG Electronics], [Apple], [ZTE, Sanechips], and [AT&T] raised concerns that "adaptive emission masks and context-based EVM targets" are outside RAN1 scope

- Clarity Issues: [LG Electronics] and [Apple] were unclear about "granular DL power control framework" meaning

- Generality Preference: [Qualcomm] and [Samsung] preferred more high-level proposal without detailed sub-bullets

- Additional Topics: [Nokia] suggested adding SSB power ramp-down, and [Huawei, HiSilicon] proposed adding "UL wideband transmission with lower PAPR"

Revised Proposal:

Study enhancement direction(s)/solution(s) related to power-domain adaptation for 6G EE improvement, considering at least the following aspects:

- Efficient CSI feedback for power-domain adaptation

- Dynamic and granular DL power control framework

- UL wideband transmission with lower PAPR

- Efficient SSB power ramp-down for cell shutdown

Considerations on how to make native and joint energy saving for network and UE

Companies have common understanding to make native and joint energy saving for both network and UE. It is very useful for companies to share view on how to make the energy saving features be native and favourable for both network and UE, which helps to form the principles and converge the technical directions for energy saving.

| Company | Views |
| --- | --- |
| Huawei, HiSilicon | It would be useful to discuss how to make native and joint energy saving for both network and UE, in addition to discuss energy saving directions/techniques in separate domains. For example, as proposed by several companies, a power saving mode (or a basic capability set, or a minimum kernel) with power saving techniques from multiple domains can be considered. |

Moderator thanks companies’ valuable feedback and would like to suggest companies’ further consideration of the above comment for joint network and UE EE optimization.

Contacts

Below is a contact list for companies’ delegates following the energy efficiency topic in the 6G Radio SI:

| Company | Delegates | Email address(es) |
| --- | --- | --- |
| Apple | Sigen Ye; Dan Wu; Hong He; Seunghee Han | sigen_ye@apple.com; dan_wu4@apple.com; hhe5@apple.com; seunghee.han@apple.com |
| CATT | Shupeng Li; Miaomiao Liu | lsp@catt.cn; liumiaomiao@catt.cn |
| CEWiT | Deepak Agarwal; Deepak PM | deepak@cewit.org.in; deepakpm@cewit.org.in |
| CMCC | Xiaodong Shen; Minghan Jiao | shenxiaodong@chinamobile.com; jiaominghan@chinamobile.com |
| DCM | Takashi Ikeuchi; Taichi Shichijo; Naoya Shibaike; Shinya Kumagai | takashi.ikeuchi.gs@nttdocomo.com; taichi.shichijou.ma@nttdocomo.com; naoya.shibaike.eg@nttdocomo.com; shinya.kumagai.yw@nttdocomo.com |
| Ericsson | Magnus Åström; Gustav Lindmark; Mohammad Mozaffari; Yanpeng Yang | magnus.astrom@ericsson.com; gustav.lindmark@ericsson.com; mohammad.mozaffari@ericsson.com; yanpeng.yang@ericsson.com |
| ETRI | Sunghyun Moon; Junghoon Lee | sh.moon@etri.re.kr; jh.lee@etri.re.kr |
| Fainity | ChieMing | chieming@fainnov.com |
| Fraunhofer | Geordie George; Gustavo Costa; Nazanin Vatanian; Elke Roth-Mandutz | geordie.george@iis.fraunhofer.de; gustavo.wagner.oliveira.da.costa@iis.fraunhofer.de; nazanin.vatanian@iis.fraunhofer.de; elke.roth-mandutz@iis.fraunhofer.de |
| Fujitsu | Lei Zhang | zhanglei@fujitsu.com |
| Futurewei | George Calcev; Hussain Elkotby | gcalcev@futurewei.com; helkotby@futurewei.com |
| Google | Alex Liou | alexliou@google.com |
| Huawei, HiSilicon | Yi Wang; Yifan Xue; Xiaolei Tie; Yan Cheng; Matthew Webb | wangyi6@huawei.com; xueyifan1@huawei.com; tiexiaolei@hisilicon.com; chengyan.cheng@huawei.com; matthew.webb@huawei.com |
| IIT Kanpur | Dheeraj Naidu Amudala; Jyotirmay Saini | dheeraja@iitk.ac.in; jsaini@iitk.ac.in |
| Lenovo | Karthikeyan Ganesan; Ali Ramadan Ali | kganesan@lenovo.com; aali@lenovo.com |
| LG Electronics | Seonwook Kim; Suckchel Yang; Sechang Myung; Youngdae Lee | Sseonwook.kim@lgepartner.com; suckchel.yang@lge.com; sechang.myung@lge.com; youngdae.lee@lge.com |
| MediaTek | Chiou-Wei Tsai; Weide Wu | cw.tsai@mediatek.com; weide.wu@mediatek.com |
| NEC | Pravjyot Singh Deogun | pravjyot.deogun@emea.nec.com |
| Nokia | Naizheng Zheng; David Bhatoolaul; Cássio Ribeiro | naizheng.zheng@nokia-sbell.com; david.bhatoolaul@nokia.com; cassio.ribeiro@nokia.com |
| Ofinno | Ryan Keating; Yunjung Yi | rkeating@ofinno.com; yyi@ofinno.com |
| OPPO | Zhisong Zuo; Hao Lin; Zhenshan Zhao | zuozhisong@oppo.com; lin.hao@oppo.com; zhaozhenshan@oppo.com |
| Panasonic | Hongchao Li; Suzuki Hidetoshi; Iwata Ayako; Kuruvatti, Nandish; Horiike, Naoto | Hongchao.Li@eu.panasonic.com; suzuki.hidetoshi@jp.panasonic.com; iwata.ayako@jp.panasonic.com; Nandish.Kuruvatti@eu.panasonic.com; Naoto.Horiike@eu.panasonic.com |
| Qualcomm | Gabi Sarkis; Hung Ly | gsarkis@qti.qualcomm.com; hdly@qti.qualcomm.com |
| Samsung | Youngbum Kim; Hongbo Si; Emad Farag; Qi Xiong | youngbum.kim@samsung.com; hongbo.si@samsung.com; e.farag@samsung.com; q1005.xiong@samsung.com |
| Sharp | Hiroki Takahashi; Juan Liu; Emily Lai | takahashi.hiroki@mail.sharp; juan.liu@cn.sharp-world.com; emily.ch.lai@sharp-world.com.tw |
| Spreadtrum | Yu Ding; Huan Zhou; Zhenzhu Lei; Hualei Wang; Lei Gu | Yu.Ding@unisoc.com; Huan.Zhou@unisoc.com; Reven.Lei@unisoc.com; Hualei.Wang@unisoc.com; Lei.gu@unisoc.com |
| TCL | Rongling Jian; Wenwen Huang; Yuanqing Yang | rongling.jian@tcl.com; wenwen5.huang@tcl.com; yuanqing4.yang@tcl.com |
| vivo | Xin Qu; Lihui Wang; Gen Li; Huan Wang; Xueming Pan | quxin@vivo.com; wanglihui@vivo.com; reagan.li@vivo.com; wanghuan@vivo.com; panxueming@vivo.com |
| Xiaomi | Lei Wang; Sicong Zhao | wanglei25@xiaomi.com; zhaosicong@xiaomi.com |
| ZTE, Sanechips | Youjun Hu; Xuan Ma; Mengzhu Chen | hu.youjun1@zte.com.cn; ma.xuan1@zte.com.cn; chen.mengzhu@zte.com.cn |

All email addresses (separated by “;”)

magnus.astrom@ericsson.com; gustav.lindmark@ericsson.com; mohammad.mozaffari@ericsson.com; yanpeng.yang@ericsson.com; alexliou@google.com; rongling.jian@tcl.com; wenwen5.huang@tcl.com; yuanqing4.yang@tcl.com; Hongchao.Li@eu.panasonic.com; suzuki.hidetoshi@jp.panasonic.com; iwata.ayako@jp.panasonic.com; Nandish.Kuruvatti@eu.panasonic.com; Naoto.Horiike@eu.panasonic.com; gsarkis@qti.qualcomm.com; hdly@qti.qualcomm.com; zhanglei@fujitsu.com; chieming@fainnov.com; naizheng.zheng@nokia-sbell.com; david.bhatoolaul@nokia.com; cassio.ribeiro@nokia.com; takahashi.hiroki@mail.sharp; juan.liu@cn.sharp-world.com; emily.ch.lai@sharp-world.com.tw; wangyi6@huawei.com; xueyifan1@huawei.com; tiexiaolei@hisilicon.com; chengyan.cheng@huawei.com; matthew.webb@huawei.com; shenxiaodong@chinamobile.com; jiaominghan@chinamobile.com; lsp@catt.cn; liumiaomiao@catt.cn; sh.moon@etri.re.kr; jh.lee@etri.re.kr; pravjyot.deogun@emea.nec.com; wanglei25@xiaomi.com; zhaosicong@xiaomi.com; quxin@vivo.com; wanglihui@vivo.com; reagan.li@vivo.com; wanghuan@vivo.com; panxueming@vivo.com; hu.youjun1@zte.com.cn; ma.xuan1@zte.com.cn; chen.mengzhu@zte.com.cn; youngbum.kim@samsung.com; hongbo.si@samsung.com; e.farag@samsung.com; q1005.xiong@samsung.com; dheeraja@iitk.ac.in; sigen_ye@apple.com; dan_wu4@apple.com; hhe5@apple.com; seunghee.han@apple.com; kganesan@lenovo.com; aali@lenovo.com; geordie.george@iis.fraunhofer.de; gustavo.wagner.oliveira.da.costa@iis.fraunhofer.de; nazanin.vatanian@iis.fraunhofer.de; elke.roth-mandutz@iis.fraunhofer.de; gcalcev@futurewei.com; helkotby@futurewei.com; takashi.ikeuchi.gs@nttdocomo.com; taichi.shichijou.ma@nttdocomo.com; naoya.shibaike.eg@nttdocomo.com; shinya.kumagai.yw@nttdocomo.com; deepak@cewit.org.in; deepakpm@cewit.org.in; rkeating@ofinno.com; yyi@ofinno.com; cw.tsai@mediatek.com; weide.wu@mediatek.com; Yu.Ding@unisoc.com; Huan.Zhou@unisoc.com; Reven.Lei@unisoc.com; Hualei.Wang@unisoc.com; Lei.gu@unisoc.com; Sseonwook.kim@lgepartner.com; suckchel.yang@lge.com; sechang.myung@lge.com; youngdae.lee@lge.com; zuozhisong@oppo.com; lin.hao@oppo.com; zhaozhenshan@oppo.com; jsaini@iitk.ac.in

Summary of Agreements in RAN1#122 Meeting

Agreement

Study how to reuse and update reference configurations in TR 38.864 for 6G BS.

Agreement

Study how/whether to reuse or update the power model in TR 38.864 for evaluating BS power consumption for 6G BS.

Agreement

- Study metric(s) for UE energy efficiency.

- Study metric(s) for BS energy efficiency.

Agreement

Study reference configurations and power consumption model for 6G UE, considering but not restricted to the following:

- TR 38.840 (UEPS), TR38.875 (RedCap), TR38.865 (eRedCap), and TR38.869 (LP-WUS/WUR) for reference configurations

- TR 38.840 (UEPS), TR38.875 (RedCap), and TR38.869 (LP-WUS/WUR) for power consumption models

Agreement

- Study baseline BS setting(s) for evaluating 6G BS EE improvement/impact, considering NR features and 6G BS reference configuration(s)

- Study baseline UE setting(s) for evaluating 6G UE EE improvement/impact, considering NR features and 6G UE reference configuration(s)

References

RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator), RAN #108, June 2025.

R1-2506303, RAN1 workplan for Rel-20 Study of 6GR, NTT DOCOMO, China Mobile, AT&T, Vodafone, RAN1 #122, August 2025.

R1-2505131, Energy Efficiency in 6G Radio, Nokia, RAN1 #122, August 2025.

R1-2505145, Discussion on 6G energy efficiency techniques, FUTUREWEI, RAN1 #122, August 2025.

R1-2505176, Discussion on energy efficiency for 6GR, Spreadtrum (UNISOC), RAN1 #122, August 2025.

R1-2505187, Views on energy saving for 6GR, Huawei, RAN1 #122, August 2025.

R1-2505291, Consideration on 6GR Energy Efficiency, Sony, RAN1 #122, August 2025.

R1-2505297, Discussion on energy efficiency of 6GR, CATT, RAN1 #122, August 2025.

R1-2505420, Discussion on UE and network energy efficiency, vivo, RAN1 #122, August 2025.

R1-2505467, Discussion on energy efficiency for 6GR, Xiaomi, RAN1 #122, August 2025.

R1-2505589, Discussion on energy efficiency for 6GR, Samsung, RAN1 #122, August 2025.

R1-2505607, Discussion on energy efficiency for 6GR, ZTE Corporation, Sanechips, RAN1 #122, August 2025.

R1-2505625, On 6G energy efficiency, Ericsson, RAN1 #122, August 2025.

R1-2505631, Energy Efficiency, Tejas Networks Ltd., RAN1 #122, August 2025.

R1-2505641, Discussion on Physical Layer Design for Energy Efficiency in 6G, NEC, RAN1 #122, August 2025.

R1-2505677, Initial Views on 6GR Energy Efficiency, Ofinno, RAN1 #122, August 2025.

R1-2505698, Discussion on 6G energy efficiency aspects, TCL, RAN1 #122, August 2025.

R1-2505761, Discussion on energy saving consideration for 6GR, OPPO, RAN1 #122, August 2025.

R1-2505769, Discussion on Energy Efficiency for 6GR, Quectel, RAN1 #122, August 2025.

R1-2505789, On 6GR design for energy efficiency, Panasonic, RAN1 #122, August 2025.

R1-2505834, 6G Study on Energy Savings, Fraunhofer IIS, Fraunhofer HHI, RAN1 #122, August 2025.

R1-2505858, Discussion on energy efficiency for 6GR, LG Electronics, RAN1 #122, August 2025.

R1-2505917, Views on 6G energy efficiency, Apple, RAN1 #122, August 2025.

R1-2505972, Discussion on energy efficiency for 6GR, Fujitsu, RAN1 #122, August 2025.

R1-2505991, Considerations for 6G energy efficiency, KT Corp., RAN1 #122, August 2025.

R1-2505995, Discussion on 6GR Energy Efficient design, Lenovo, RAN1 #122, August 2025.

R1-2506003, Discussion on energy efficiency, HONOR, RAN1 #122, August 2025.

R1-2506005, Discussion on energy efficiency and energy saving, CAICT, RAN1 #122, August 2025.

R1-2506014, Study on energy efficiency for 6GR, Sharp, RAN1 #122, August 2025.

R1-2506024, Energy efficiency, MediaTek Inc., RAN1 #122, August 2025.

R1-2506069, High-level view on energy efficiency aspects in 6GR, ETRI, RAN1 #122, August 2025.

R1-2506101, Discussion on Energy Efficiency of 6GR interface, CMCC, RAN1 #122, August 2025.

R1-2506134, On 6GR energy efficiency, Vodafone, Bouygues Telecom, Deutsche Telekom, RAN1 #122, August 2025.

R1-2506146, Energy Efficiency in 6GR air interface, InterDigital, Inc., RAN1 #122, August 2025.

R1-2506152, Views on 6G energy efficiency, SK Telecom, RAN1 #122, August 2025.

R1-2506222, Energy Efficiency in 6GR, Qualcomm Incorporated, RAN1 #122, August 2025.

R1-2506237, Views on Energy Efficiency for 6GR Interface, AT&T, RAN1 #122, August 2025.

R1-2506310, Discussion on Energy Efficiency, NTT DOCOMO, INC., RAN1 #122, August 2025.

R1-2506324, Discussion on Energy Efficiency for 6G Radio, WILUS Inc., RAN1 #122, August 2025.

R1-2506346, Network Energy Savings Use Cases in 6GR, Rakuten Mobile, Inc., RAN1 #122, August 2025.

R1-2506352, Discussion on 6G energy efficiency, Google, RAN1 #122, August 2025.

R1-2506363, Energy Efficiency in 6G networks - NW and UE energy saving, CEWiT, RAN1 #122, August 2025.

R1-2506392, Considerations for 6GR Energy Efficiency, IIT Kanpur, RAN1 #122, August 2025.