3GPP TSG-RAN WG1 Meeting # 122	R1-2506683

Bengaluru, India, 25th – 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0684 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TEI19 Carrier-Switching SRS with UL Tx Switching [SRSCS_ULTxSwitch] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-09-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TEI19 agreement in RAN1#120bis: Agreement - To resolve ambiguities with concurrent configuration of SRS-CS and ulTxswitch in a case where a UE configured with SRS CS on target CC and its “switch-from” CC on CC2 and configured with UL Tx switching operation for UL CC2 and at least one UL CC1 - Confirm that the prioritization rules in 38.214 Sec. 6.2.1.3 are applied between target and CC1, regardless of SRS-AS antenna port configuration on target CC, if UE indicates based on srs-SwitchingAffectedBandsListNR-r17 that SRS-CS on target impacts CC1, where CC1 is one of the CC(s) which may share Tx chains with source CC. No spec change is needed. - Note: if UE does not indicate srs-SwitchingAffectedBandsListNR-r17, UE can only perform simultaneous transmission when the total number of involved Tx chains for simultaneous transmission on all the bands/carriers is not greater than number of Tx chains supported by the UE for simultaneous UL transmission. No spec change is needed. - If the UE is under the operation state in which all Tx chains are available at the source CC, the required switching time before the beginning of SRS-CS transmission on target CC is SRS-SwitchingTimeNR. Otherwise, the required switching time before the beginning of SRS-CS transmission on target CC is indicated by a UE capability. Details about UE capability will be discussed in UE feature session. - The existing scheduling restriction of maximum one switching per reference slot for UL Tx switching is also taking into account the triggered SRS CS After SRS transmissions within an SRS resource set is done, if UE is indicated to transmit on CC1, the required switching time between the end of SRS-CS transmission on target CC and start of UL transmission on CC1 is indicated by the UE capability defined above. Otherwise, it is assumed for the determination of any future switching time that all Tx chains are returned to the source CC, and the RF tuning time to switch from target to source will be SRS-SwitchingTimeN RAN1#121 agreed on a text proposal reflected in this CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specifying that the UE indicating the new capability either uses the maximum of the SRS-CS switching time or the UL Tx Switching time, or a sum of the two switching times in the respective carrier swtiching. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The agreed TEI item is not supported by the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6, 6.2.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

### 6.1.6	Uplink switching

The UE may omit uplink transmission during the uplink switching gap NTx1-Tx2 if the conditions defined in this clause are met and the UE is configured with uplinkTxSwitching or uplinkTxSwitchingMoreBands. The switching gap NTx1-Tx2 is indicated by UE capability uplinkTxSwitchingPeriod2T2T if uplinkTxSwitching-2T-Mode is configured, and uplinkTxSwitchingPeriod otherwise in clauses 6.1.6.1, 6.1.6.2.0, 6.1.6.3, and is determined based on higher layer parameter switchingPeriodConfigForBandPair in clause 6.1.6.2.2 for uplink switching configured with 2, 3 or 4 uplink bands if uplinkTxSwitchingMoreBands is configured:

-	If a UE indicated a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination

-	Configured with a MCG using E-UTRA radio access and with a SCG using NR radio access (EN-DC), or

-	Configured with uplink carrier aggregation, or

-	Configured in a serving cell with two uplink carriers with higher layer parameter supplementaryUplink.

The conditions under which the switching gap may be present are defined for each of the cases in clauses 6.1.6.1, 6.1.6.2, and 6.1.6.3 respectively.

If an uplink switching is triggered for an uplink transmission starting at T0, after T0-Toffset, the UE is not expected to cancel the uplink switching, or to trigger any other new uplink switching occurring before T0 for any other uplink transmission that is scheduled after T0-Toffset, where Toffset is the UE processing procedure time defined for the uplink transmission(s) triggering the switch given in clause 5.3, clause 5.4, clause 6.1, clause 6.2.1, clause 6.4 and in clause 9 of [6, TS 38.213].

The UE does not expect to perform more than one uplink switching in a slot with µUL = max(µUL, 1, µUL, 2), where the µUL, 1 corresponds to the subcarrier spacing of the active UL BWP of one uplink carrier before the switching gap and the µUL, 2 corresponds to the subcarrier spacing of the active UL BWP of the other uplink carrier after the switching gap.

For uplink switching configured with 3 or 4 uplink bands

-	If two contiguous intra-band uplink carriers are configured to a UE, the UE may assume that the active UL BWPs of the two carriers are configured with the same subcarrier spacing.

-	The UE does not expect to perform more than one uplink switching in a reference slot with µUL, where the µUL corresponds to the maximum subcarrier spacing of the active UL BWPs of all the configured uplink carriers.

-	If 500 µs is determined by the UE capability uplinkTxSwitchingMinimumSeparationTime, when within any two consecutive reference slots corresponding to numerology µUL,

-	the UE first performs one uplink switch and later performs another uplink switch and

-	at least three bands are involved in the transmissions before the first switch, between the first switch and the second switch, and after the second switch,

the separation time between the start of all transmission(s) after the first switch and the start of all transmission(s) after the second switch is not expected to be less than 500 µs. If other than 500 µs is determined by the UE capability uplinkTxSwitchingMinimumSeparationTime, no additional restrictions apply.

-	If an uplink switching is triggered for uplink transmission(s) with a gap between the start of the first uplink transmission(s) and the end of the last preceding uplink transmission(s) that is smaller than the determined switching gap NTx1-Tx2, the UE determines the band of the switching period location, defined in [8, TS 38.101-1] based on the priority of the bands configured by uplinkTxSwitchingBandList. Among the bands either in switch-from or switch-to bands but not both, the switch is located on either,

-	the switch-from band(s) if the highest priority band is a switch-to band, or

-	the switch-to band(s) if the highest priority band is a switch-from band.

If an uplink carrier is configured as a switch-from carrier for a UE with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier as defined in clause 6.2.1.3 and the uplink carrier is also configured with uplinkTxSwitching or uplinkTxSwitchingMoreBands, then

-	if an uplink switching is triggered by an uplink transmission on a band other than the band where the switch-from carrier is configured and the preceding uplink transmission is an SRS transmission as defined in clause 6.2.1.3,

-	if the uplink transmission is not associated with a DCI, or it is based on a DCI ending at least N2  symbols before the end of the SRS transmission, then the uplink switching gap NTx1-Tx2  in this clause is determined as maxswitchingTimeUL,switchingTimeDL+ NTx1-Tx2 if the UE indicates [SRSCS_ULTxSwitch] set to ‘sum’ for the band combination and maxswitchingTimeUL,switchingTimeDL,NTx1-Tx2 otherwise,

-	Otherwise, if the DCI triggering the uplink Tx switching does not end in the time interval T-N2, T+maxswitchingTimeUL,switchingTimeDL,NTx1-Tx2-N2, where T is the end of the SRS transmission, the uplink switching gap NTx1-Tx2  in this clause is determined as

-	NTx1-Tx2 if the UE indicates [SRSCS_ULTxSwitch] set to ‘sum’ for the band combination,

-	maxswitchingTimeUL,switchingTimeDL,NTx1-Tx2 if the UE indicates [SRSCS_ULTxSwitch] set to ‘max’ for the band combination;

-	UE does not expect to receive the DCI triggering uplink Tx switching ending in the time interval T-N2, T+maxswitchingTimeUL,switchingTimeDL,NTx1-Tx2-N2, where T is the end of the SRS transmission,

-	where NTx1-Tx2 is the switching gap determined in this clause assuming that one uplink channel or signal were transmitted on the switch-from carrier and uplink switching would be triggered.

-	the UE does not expect to perform both uplink switching and the switching defined in clause 6.2.1.3 in a reference slot with µUL, where the µUL corresponds to the maximum subcarrier spacing of the active UL BWPs of all the configured uplink carriers.

-	if an uplink transmission is triggered on the band where the switch-from carrier is configured and the preceding uplink transmission is an SRS transmission as defined in clause 6.2.1.3, no uplink switching is triggered by this clause.

<omitted text>

#### 6.2.1.3	UE sounding procedure between component carriers

For a carrier of a serving cell c1 with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, denote c2 as the corresponding carrier of a serving cell whose UL transmissions are temporarily suspended as signalled by higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier. Define the set Sc2={c2, s1(c2)…sN-1(c2)} as the set of carriers of serving cells that each carrier meets one of the following conditions:

-	si(c2) is in the same band and same TAG as c2;

-	si(c2) is a carrier of inter-band CA with c2 and si(c2) is indicated through the capability signalling srs-SwitchingAffectedBandsListNR-r17 to be affected by the SRS switch from c2 to c1;

where 1≤i≤N-1.

A UE can be configured with SRS resource(s) on a carrier c1 with slot formats comprised of DL and UL symbols and not configured for PUSCH/PUCCH transmission. For carrier c1, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier the switching from carrier c2 which is configured for PUSCH/PUCCH transmission. During SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR or as defined by TSRSCS below), the UE temporarily suspends the uplink transmission on carriers in the set S(c2).

For an SRS transmission starting in symbol Nc1 of carrier c1 and a conflicting transmission in any carrier within the set Sc2  starting in symbol NS, the UE shall apply the prioritization / dropping rules in the remainder of this clause taking into account:

-	DCI(s) for which the time interval between the last symbol of PDCCH and Nc1 is at least N2 symbols and an additional time duration TSRSCS,  and the time interval between the last symbol of PDCCH and NS is at least   N2 symbols; and

-	semi-persistent CSI reports or SRS considered active at least N2 symbols and an additional time duration TSRSCS before Nc1, and considered active at least N2 symbols before NS.

where

-	if the switch-from carrier is not configured with Uplink switching as defined in clause 6.1.6, 
 TSRSCS=max⁡{switchingTimeUL,switchingTimeDL},

-	if the switch-from carrier is configured with Uplink switching as defined in clause 6.1.6

-	if the UE indicates [SRSCS_ULTxSwitch] set to ‘max’ for the band combination, 
TSRSCS=max⁡{switchingTimeUL,switchingTimeDL, NTx1-Tx2 },

-	if the UE indicates [SRSCS_ULTxSwitch] set to ‘sum’ for the band combination, 
TSRSCS=maxswitchingTimeUL,switchingTimeDL, + NTx1-Tx2,

where NTx1-Tx2 is determined in clause 6.1.6 assuming that one uplink channel or signal were transmitted on the switch-from carrier and uplink switching would be triggered. NTx1-Tx2=0, if no uplink switching were triggered.

and the time interval unit of OFDM symbol is counted based on the smaller subcarrier spacing across any carrier within the set Sc2, c1 and their corresponding scheduling cells.

The following prioritization rules shall be applied in case of collision between a transmission of SRS over carrier c1  and transmission of a physical signal/channel over a carrier of a serving cell in set Sc2

-	the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH on a carrier of a serving cell in set Sc2 happen to overlap in the same symbol

-	the UE shall not transmit a periodic/semi-persistent SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 and PUSCH transmission carrying aperiodic CSI on a carrier of a serving cell in set Sc2 happen to overlap in the same symbol

-	the UE shall drop PUCCH/PUSCH transmission carrying periodic/semi-persistent CSI comprising only CQI/PMI/L1-RSRP/L1-SINR, and/or SRS transmission on a carrier of a serving cell in set Sc2 configured for PUSCH/PUCCH transmission whenever the transmission and SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 happen to overlap in the same symbol

-	the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR on a carrier of a serving cell in set Sc2 whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133]) as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 happen to overlap in the same symbol.

For an aperiodic SRS triggered in DCI format 2_3 and if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeA', and given by SRS-CarrierSwitching, without PUSCH/PUCCH transmission, the UE in each serving cell transmits the configured one or two SRS resource set(s) from srs-ResourceSetToAddModList with higher layer parameter usage set to 'antennaSwitching' and higher layer parameter resourceType in SRS-ResourceSet set to 'aperiodic'.

For an aperiodic SRS triggered in DCI format 2_3 and if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeB' without PUSCH/PUCCH transmission, the UE in each serving cell transmits the configured one or two SRS resource set(s) from srs-ResourceSetToAddModList with higher layer parameter usage set to 'antennaSwitching' and higher layer parameter resourceType in SRS-ResourceSet set to 'aperiodic'.

For an aperiodic SRS triggered in DCI format 1_1 or 1_2, if the UE is configured by SRS-CarrierSwitching, it transmits SRS on one serving cell not configured for PUSCH/PUCCH transmission scheduled by the DCI and the UE in the serving cell transmits the configured one or two SRS resource set(s) with higher layer parameter usage set to 'antennaSwitching' and higher layer parameter resourceType in SRS-ResourceSet set to 'aperiodic'.

For an aperiodic SRS triggered in DCI format 1_3, if the UE is configured by SRS-CarrierSwitching, for an SRS transmission in a scheduled cell not configured for PUSCH/PUCCH transmission, the UE transmits the configured one or two SRS resource set(s) with higher layer parameter usage set to 'antennaSwitching' and higher layer parameter resourceType in SRS-ResourceSet set to 'aperiodic'.

If the UE is not configured for PUSCH/PUCCH transmission on carrier c1 with slot formats comprised of DL and UL symbols, and if the UE is not capable of simultaneous reception and transmission on carrier c1 and serving cell c2, the UE is not expected to be configured or indicated with SRS resource(s) such that SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) would collide with the REs corresponding to the SS/PBCH blocks configured for the UE or the slots belonging to a control resource set indicated by MIB or SIB1 on serving cell c2.

For n-th (n ≥ 1) aperiodic SRS transmission on a cell c, upon detection of a positive SRS request on a grant, the UE shall commence this SRS transmission on the configured symbol and slot provided

-	it is no earlier than the summation of

-	the maximum time duration between the two durations spanned by N OFDM symbols of the numerology of cell c and the cell carrying the grant respectively, and

-	the UL or DL RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR,

-	it does not collide with any previous SRS transmissions, or interruption due to UL or DL RF retuning time, except if the previous SRS transmission is in the same cell c and the UE reports stayOnTargetCC-SRS-CarrierSwitch for the corresponding band combination.

otherwise, n-th SRS transmission is dropped, where N is the reported capability as the minimum time interval in unit of symbols, between the DCI triggering and aperiodic SRS transmission.

In case of inter-band carrier aggregation, a UE can simultaneously transmit SRS and PUCCH/PUSCH across component carriers in different bands subject to the UE's capability.

In case of inter-band carrier aggregation, a UE can simultaneously transmit PRACH and SRS across component carriers in different bands subject to UE's capability.

If the UE is not configured for PUSCH/PUCCH transmission for at least one serving cell configured with slot formats comprised of DL and UL symbols, and if the UE is not capable of simultaneous reception and transmission on serving cell c1 and serving cell s(c2), and if a UE

-	is configured with multiple serving cells and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the configured multiple serving cells including serving cell c1 and s(c2), and

-	indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and

-	is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells,

the UE shall apply first the prioritization/dropping rules described above for sounding procedure between component carriers and then apply the procedures for directional collision handling in clause 11.1 of [6, TS 38.213].

<omitted text>