3GPP TSG RAN WG1 Meeting #122bis			R1-2506777

Prague, Czech, Oct 13th – 17th, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on 6G frame structure

Agenda item:	11.3.2

Document for:	Discussion and Decision

Introduction

In RAN#108, a new SID of 6GR was agreed and one of the objectives is frame structure design as shown below [1].

- Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1]

In RAN1#122, the following agreements were achieved on frame structure and numerology [2].

Agreement - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - Whether to discuss the FFS will be subject to RANP decision. - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz - FFS: For around 15GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz, 120kHz - Whether to discuss it will be subject to RANP decision - For between 24.25GHz - 52.6GHz - Subcarrier spacing 120kHz is supported - FFS whether to allow using additional subcarrier spacing for SSB - FFS subcarrier spacing for PRACH and up to initial access discussion. Conclusion - Numerologies for sensing is up to sensing agenda discussion. Agreement - 6GR supports normal cyclic prefix, i.e., same as the normal CP defined in NR. - FFS potential need for other CP

In this contribution, our considerations on frame structure design including numerology and channel bandwidth for 6GR are provided.

Numerology and bandwidth

Considerations in numerology and channel bandwidth design

- Channel bandwidth and numerology in NR

In 5G NR, seven distinct numerologies are defined with SCS values ranging from 15kHz to 960kHz, as shown in Table 1. 15-60 kHz SCS are supported for FR1 and 60-960 kHz are supported for FR2. The duration of an OFDM symbol inversely correlates with the SCS. Notably, NCP caters to all SCS options, whereas ECP is supported only for 60kHz. These scalable numerologies are particularly designed to accommodate wider bandwidth requirements, higher frequency bands and lower latency applications. For channel bandwidth, NR supports up to 100MHz in FR1, up to 400MHz in FR2-1, and 2000MHz in FR2-2.

Table 1 Numerologies in NR

|  |  | Cyclic prefix |
| --- | --- | --- |
| 0 | 15 | Normal |
| 1 | 30 | Normal |
| 2 | 60 | Normal, Extended |
| 3 | 120 | Normal |
| 4 | 240 | Normal |
| 5 | 480 | Normal |
| 6 | 960 | Normal |

Observation 1: NR supports scalable numerologies, and the channel bandwidth is up to 100MHz for FR1 and 400MHz for FR2-1.

- KPI of 6GR

The usage scenarios of IMT-2030 are envisaged to expand on those of IMT-2020 into broader use requiring evolved and new capabilities, including immersive communication (IC), hyper reliable and low-latency communication (HRLLC), and massive communication (MC). In addition to expanded IMT-2020 usage scenarios, IMT-2030 is envisaged to enable new usage scenarios arising from capabilities, which previous generations of IMT were not designed to support. The new usage scenarios include integrated sensing and communication (ISAC), integrated AI and communication, and ubiquitous connectivity. The new usage scenarios IC, HRLLC, and MC are the expansion of IMT-2020 usage scenarios eMBB, URLLC, and mMTC respectively. Accordingly, the capabilities are also enhanced as shown in the Figure 1.

Figure 1 Capabilities of IMT-2030

- For example, the latency is expected to be further reduced to 0.1-1ms for HRLLC. The latency over the air interface refers to the contribution by the radio network to the time from when the source sends a packet of a certain size to when the destination receives it. In RAN#108, it was agreed that the target latency can be 1ms or even less.

- For mobility, the relative maximum speed between the radio nodes is defined, where the QoS requirements should be guaranteed at the defined speed.  The research target of mobility could be 500-1000 km/h. With a such high speed, the doppler shift is large, especially in the higher frequency.

- In addition, the new capability of coverage is a new KPI for 6G. Coverage refers to the ability to provide access to communication services for users in a desired service area. It is defined as the cell edge distance of a single cell through link budget analysis. It can be expected that 6G should be able to support a large coverage.

All these are related to frame structure and numerology. In other words, both numerology and frame structure are significant key points to achieve the targets. Therefore, they should be considered when designing numerology and frame structure.

Proposal 1: The defined capabilities of 6GR (e.g., TPR) should be considered in numerology and frame structure, including at least latency, mobility and coverage.

- MRSS

For 5G-6G migration, multi-RAT spectrum sharing (MRSS) is a good way to improve spectrum utilization efficiency, reduce operational and deployment costs, and encourage innovation and diverse services. 6G is expected to have a higher spectrum efficiency. Therefore, 5G-6G MRSS should be considered in the first release of 6G for compatibility with 5G NR with spectrum re-farming, which is similar to 4G-5G DSS.

One example of 5G-6G MRSS is shown in Figure 2. 5G and 6G dynamically share the same carrier frequency by rate matching (RM) around the signal of the other RAT. From UE perspective, 6G UE only connects to 6G xNB. If the NR signal is transmitted on some REs, then the 6G signal should not use such REs. To achieve a perfect efficiency of resource sharing, 6G design should consider backward compatibility in terms of numerology and frame structure. For example, SCS of 5G and 6G should be the same at least for the shared spectrum.

Figure 2 An example of 5G-6G MRSS

Proposal 2: MRSS between 5G and 6G should be considered in 6G numerology design, e.g. compatible numerology.

Numerology

NR has scalable SCS with the basis of 15 kHz. The higher frequency band is, the larger SCS is, since the delay spread gets less and the doppler shift gets larger with the increase of carrier frequency. 6G may have a wider frequency bands than NR. Therefore, the scalable SCS should also be supported in 6G. In addition, considering backward compatibility in MRSS for better spectrum efficiency, 6G SCS should be aligned with that of NR, especially in the same frequency spectrum. Therefore, the 5G design with SCS of 2n*15 kHz can also be considered in 6G as the benchmark.

Proposal 3: 6GR design should take 5G SCS 2n*15 kHz as the benchmark (n≥0).

In NR, two types of CP are supported, i.e., Normal CP and extended CP. The extended CP is only supported for 60kHz SCS. It was agreed to at least support normal CP in last RAN1 meeting. Even though, these two CP types have been supported since LTE, extended CP is not deployed in the commercial network before. Considering there is no additional requirement for communication, the other CP should not be supported for communication.

Proposal 4: The other types of CP (e.g., shorter CP or longer CP than the normal CP) should not be supported.

Numerical analysis

For around 7GHz, it was agreed to study two SCS, i.e., 30kHz and 60kHz.  A smaller SCS can better cope with inter-symbol interference caused by delay spread. Analysis on inter-symbol interference for 30kHz SCS and 60kHz SCS at 7GHz is shown in Table 2. The channel model CDL-A is used. The maximum delay is given by TR38.901. The clusters without ISI are the clusters with the delay relative to the shortest cluster covered by the CP, i.e., the delay is smaller than the CP. For the other clusters with the delay relative to the shortest cluster not covered by the CP, they will cause ISI. The percentage of Tx power causing ISI is the ratio of total power of the clusters with ISI to the total power of all the clusters. Based on numerical analysis, some clusters may cause ISI when the wanted delay DSdesired is larger than 100. Compared with 30kHz SCS, 60kHz SCS has more clusters causing ISI and higher percentage of Tx power causing ISI due to the short CP. In the severest case, 18.73% transmission power cause the ISI. Therefore, performance may get worse for 60kHz SCS.

Observation 2: Based on numerical analysis, compared with 30kHz SCS, transmission with 60kHz SCS causes more ISI due to transmission power leakage.

- 2.45%, 9.4% and 18.73% transmission power of 60kHz SCS are leaked to the next OFDM symbol in terms of 300ns, 500ns and 1000ns delay spread, respectively.

Table 2 The analysis of the inter-symbol interference

| Wanted delay DSdesired (ns) | Maximum Delay(ns) | SCS=30kHz, NCP= 2346(ns) |  | SCS=60kHz, NCP= 1171(ns) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | The number of clusters without ISI | Percentage of Tx power causing ISI (%) | The number of clusters without ISI | Percentage of Tx power causing ISI (%) |
| 30 | 290 | 23 | 0 | 23 | 0 |
| 100 | 966 | 23 | 0 | 23 | 0 |
| 300 | 2898 | 22 | 0.03% | 17 | 2.45% |
| 500 | 4829 | 19 | 1.33% | 13 | 9.4% |
| 1000 | 9659 | 13 | 9.4% | 9 | 18.73% |
| NOTE: The total number of clusters is 23. |  |  |  |  |  |

As discussed above, coverage is an important KPI for 6GR. It can be predicted that coverage is a pain point for around 7GHz due to high propagation loss in high frequency.  30kHz SCS has a better coverage than 60kHz SCS due to the longer symbol duration.

Observation 3: Based on the numerical analysis,

- for delay spread in around 7GHz, 30kHz SCS can handle it better than 60kHz SCS.

- 30kHz SCS has a better coverage than 60kHz SCS

Evaluation of 30kHz vs 60kHz

To compare the performance, a PDSCH LLS is conducted. In the evaluation, the same resource size and the same TBS are used. The simulation assumption is shown in the appendix. The simulation results in different delay spread are shown in Figure 3-4.

Figure 3 PDSCH performance in DS=100ns

Figure 4 PDSCH performance in DS=300ns

Based on the evaluation, when DS=100ns, 30kHz SCS gets 1 dB performance gain compared with 60kHz SCS.  In 30kHz SCS, the frequency interval of DMRS is smaller, which can cope with a smaller coherence bandwidth better. When ns=300ns, 30kHz SCS has a better performance since ISI affects the PDSCH performance for 60kHz SCS.

Observation 4: Based on the simulation results, 30kHz SCS has a better performance than 60kHz SCS with about 1 dB gain.

In addition, the channel estimation performance is also evaluated, including the difference between the estimated channel based on the DMRS and the real channel in different delay spread. The standard deviation of the estimated channel is calculated by δ=(Hest-Hrea)2, where Hest is the estimated channel, and Hrea is the real channel. The simulation results are shown in Figure 5. It shows that the channel estimation performances are close between 30kHz SCS and 60kHz SCS in the case of 100ns delay spread. When the delay spread increases, 30kHz SCS has a better performance than 60kHz SCS and the performance gap gets larger. The reason is the ISI gets worse, which may impact the channel estimation performance.

Figure 5 The standard deviation of the estimated channel

Observation 5: 30kHz SCS has a better channel estimation performance than 60kHz SCS when the delay spread is larger than 100ns.

Based on the numerical analysis and simulation results, 30kHz SCS is preferred for around 7GHz due to the performance gain.

Proposal 5: Regarding SCS, for frequency bands around 7GHz, 30kHz SCS is adopted.

Channel bandwidth

Considering various frequency ranges in 6G, different channel bandwidths in different carrier frequencies are expected. NR has expanded the maximum channel bandwidth from 20MHz to 100MHz. It is expected that 6G will further increase the channel bandwidth, especially in new frequency bands.

- NR has supported at most 100MHz channel bandwidth for around 7GHz (i.e., 6425-7125 MHz, band n104). In addition, around 7GHz is expected to be allocated for commercial wireless communication as 6G spectrum adopted in many parts of the world. In this case, 6G should support the maximum channel bandwidth larger than NR, e.g., 200MHz/400MHz.  Therefore, 200Hz/400MHz should be supported from the commercial requirement perspective. Between them, 400MHz should be considered since it has higher resource efficiency, e.g., only one common signal (SSB, SIB, paging) is only transmitted while they should be transmitted twice in the CA and the RRC configuration signalling overhead can be reduced.

- For sub-6GHz, NR supports at most 100MHz channel bandwidth. 6G should also support 100MHz channel bandwidth. The operator may have also requirement of deploying single carrier with larger bandwidth. Therefore,200MHz channel bandwidth should also be supported for 6G.

- In FR2-1, since the mmWave spectrum resource is abundant, a lot of spectrum are allocated for IMT. It is pretty common for a single operator to have more than 200 MHz of continuous bandwidth. Due to the benefits of the single carrier with larger bandwidth as discussed above, 800MHz can be supported for FR2-1.

Proposal 6: 6G should support a larger maximum channel bandwidth.

- 400MHz for around 7GHz

- 100MHz (for FDD) and 200MHz (for TDD) for sub-6GHz

- 800MHz for 24.25GHz - 52.6GHz.

Summary of numerology and channel bandwidth

To sum up, we have the following proposal for the numerology and channel bandwidth.

Proposal 7: The following numerology and maximum channel bandwidth should be considered in 6G

|  | Sub 6GHz (FDD) | Sub 6GHz (TDD) | Around 7GHz | 24.25GHz - 52.6GHz |
| --- | --- | --- | --- | --- |
| Maximum CBW (MHz) | 100 | 200 | 400 | 800 |
| SCS (kHz) | 15/[30] | 30 | 30 | 120 |
| FFT size | 8k | 8k | 16k | 8k |

In addition, based on the above discussion, we propose to update the following RAN1 agreements agreed in RAN1#122 to align with the agreements reached in RAN#109 meeting. As a noted agreed by the RAN#109 meeting, carrier frequency of 15GHz has not been confirmed by ITU-R/WRC [3].

- Include carrier frequency of 15 GHz for the 6G scenarios (other than rural scenario) as part of 6G study. - Note: Carrier frequency of 15GHz has not been confirmed by ITU-R/WRC

We prefer to firstly focus on other carrier frequencies that have been confirmed by ITU-R/WRC. For example, in between upper bound of around 7GHz and FR2-1, 10-10.5GHz has been identified for IMT usage for some countries in region 2 in WRC-23.   However, if major companies prefer to study 15GHz from now, we would like to make it more general to include both around 10GHz and around 15GHz and add the note from RAN#109 meeting as well in the following agreements.

Proposal 8: The agreement reached in RAN1#122 should be updated as below.

Agreement - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - Whether to discuss the FFS will be subject to RANP decision. - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz - FFS: For around 15GHz frequency range between upper bound of around 7GHz and FR2-1 (e.g. around 10GHz and around 15GHz) - The following subcarrier spacing options can be studied - 30kHz, 60kHz, 120kHz - Whether to discuss it will be subject to RANP decision - Note: Carrier frequency of 15GHz has not been confirmed by ITU-R/WRC - For between 24.25GHz - 52.6GHz - Subcarrier spacing 120kHz is supported - FFS whether to allow using additional subcarrier spacing for SSB - FFS subcarrier spacing for PRACH and up to initial access discussion.

Regarding the potential application of 30kHz SCS for FDD for around e.g., 1-2.5GHz, this should be studied and evaluated at least considering the deployment scenarios, MRSS requirements and benefits of 30kHz SCS. For operators with MRSS need, it would be better to apply 15kHz SCS for FDD for 6G to ensure compatibility and avoid mutual interference between 5G and 6G. However, we may also consider the case where clean refarming to 6G without MRSS.  So, for operators without MRSS need, it would be possible to apply 30kHz SCS for FDD for 6G if good justification is observed.

Observation 6: Regarding the potential application of 30kHz SCS for FDD for around e.g., 1-2.5GHz, this should be studied and evaluated at least considering the deployment scenarios, MRSS requirements and benefits of 30kHz SCS.

Frame structure

Frame structure in NR

In NR frame structure, the length of a radio frame is always 10ms and the length of a subframe is 1ms. A frame/sub-frame has different number of slots depending on the numerology as shown in Table 3 and Table 4.

Table 3 Number of OFDM symbols per slot, slots per frame, and slots per subframe for NCP

| μ | Nsymbslot | Nslotframe,μ | Nslotsubframe,μ |
| --- | --- | --- | --- |
| 0 | 14 | 10 | 1 |
| 1 | 14 | 20 | 2 |
| 2 | 14 | 40 | 4 |
| 3 | 14 | 80 | 8 |
| 4 | 14 | 160 | 16 |
| 5 | 14 | 320 | 32 |
| 6 | 14 | 640 | 64 |

Table 4 Number of OFDM symbols per slot, slots per frame, and slots per subframe for ECP

| μ | Nsymbslot | Nslotframe,μ | Nslotsubframe,μ |
| --- | --- | --- | --- |
| 2 | 12 | 40 | 4 |

In NR, the network can configure the TDD configuration for the UE, including the semi-static configured slot format, and the dynamic slot format. The semi-static configured slot format further includes cell-specific slot format configuration and UE-specific configuration. They are both configured by the higher layer parameters. For the dynamic slot format, the common DCI can indicate one slot format from multiple slot formats configured by the RRC signaling. Therefore, NR supports a pretty flexible TDD configuration. In our views, the NR frame structure design can be the starting point for 6G.

Observation 7: NR supports flexible TDD configuration.

Proposal 9: NR frame structure design should be the starting point for 6G.

Considerations in the frame structure design for 6GR

- Native SBFD

In RAN1#122, it was agreed to support gNB semi-static SBFD and dynamic TDD in 6GR study [2]. In addition, the considerations of the gNB dynamic SBFD, UE SBFD, and gNB FD are further studied. Anyway, the full duplex operation requires the new symbol type, such as SBFD symbol in 5G-Advanced. Therefore, they may have the impact to the frame structure and thus should be considered in the frame structure design.

Proposal 10: Native SBFD should be considered in frame structure design from day1.

- ISAC

IASC is a key technology for 6G systems to combine communication and sensing functionalities into a unified system. It allows the network to simultaneously transmit data and extract environment-related information/the signal for sensing through the same infrastructure. It can bring advantages such as cost reduction, power consumption reduction, and optimization of resource utilization compared with two independent systems. As shown in Figure 6, 6G ISAC may expand the typical positioning use cases, such as UAV detection and tracking, vehicle and pedestrian detection and so on. In addition, it also includes more use cases such as ship detection and classification, micro-deformation detection, infrastructure collapse monitoring, prediction for future state, etc.

| UAV detection and tracking | Vehicle and pedestrian detection | Ship detection and classification |
| --- | --- | --- |
| Micro-deformation detection | Infrastructure collapse monitoring | Prediction for future state |

Figure 6 ISAC use cases

For ISAC, the network or the UE should transmit sensing RS depending on the sensing modes. For example, in the positioning in 5G, the network may transmit the positioning RS to the UE. The sensing RS and the communication signal may share the same radio resource. Therefore, the frame structure design should also consider the potential sensing RS for ISAC.

Proposal 11: ISAC should be be considered in frame structure design.

- Diverse emerging traffics

In the duplex discussion, some companies proposed to remove the flexible symbol in 6G and apply fixed TDD pattern. However, this is not aligned with the development trend of mobile communication system, i.e., diverse emerging traffics. From this perspective, designing the 6G duplex should not only consider the lessons learned from the 5G deployment, but also consider the future diverse emerging traffics.

Proposal 12: Emerging diverse traffic should be be considered in frame structure design, e.g., via supporting flexible symbols.

Frame structure for 6GR

As discussed above, 6G should consider the enhanced capabilities and the new usage scenarios in the frame structure design.

- SBFD

The first release of 6G system may inherently incorporate duplex technologies, including at least SBFD, and consider potential dynamic SBFD, IFBD. In 5G NR, the time domain TDD frame structure is defined over the entire carrier bandwidth starting from Rel-15. In Rel-19 NR SBFD, the SBFD subband is configured on the DL/flexible symbols configured by the TDD frame structure. That is, the TDD frame structure is the same across different subbands. In the context of 6G system, the restriction on implementing SBFD/IBFD configurations exclusively within D-symbol and F-symbol is unnecessary, especially considering the new usage scenarios in 6G. For the first release of 6G system, the frame structure design should comprehensively address both time and frequency domains. To facilitate this, both of semi-static and dynamic frame structure can be configured per subband as shown in Figure 7 for example. This kind of frame structure should be considered in 6G as it can provide a more flexible configuration and have a good forward compatibility.

Figure 7 An example of slot format in consideration with SBFD

In addition to three distinct symbol types: D symbol, U symbol and F symbol defined in 5G NR, a new symbol type, referred to as mixed DL and UL symbol, can also be introduced to represent SBFD symbol. An example is shown in Figure 8. This hybrid symbol is employed for SBFD transmission, and potential overlapping full duplex communication, in order to effectively integrate both DL and UL transmissions.

Figure 8 An example of slot format in consideration with SBFD

Proposal 13: RAN1 considers the following enhancement for frame structure in 6GR.

- Subband based frame structure configuration

- A new symbol type for SBFD

- ISAC

The ISAC RS and some of communication signals may share the radio resource. The ISAC RS and the communication signal can be transmitted at least in TDM manner. In this case, some symbols or slots may be used for only ISAC RS transmitted. These symbols or slots can be explicitly configured in the frame structure. Another alternative is the ISAC RS and the communication signal can be transmitted in FDM manner. They can be transmitted on the same slot or OFDM symbol. In this case, the frame structure can be configured in one of the subband.

Proposal 14: Time domain resources e.g. symbols or slots configured only for ISAC should be considered in 6G.

Conclusion

In this contribution, numerology, channel bandwidth, and frame structure are discussed. We have the following observations and proposals.

Numerology and channel bandwidth

Observation 1: NR supports scalable numerologies, and the channel bandwidth is up to 100MHz for FR1 and 400MHz for FR2-1.

Proposal 1: The defined capabilities of 6GR (e.g., TPR) should be considered in numerology and frame structure, including at least latency, mobility and coverage.

Proposal 2: MRSS between 5G and 6G should be considered in numerology design, e.g. compatible numerology.

Proposal 3: 6GR design should take 5G SCS 2n*15 kHz as the benchmark (n≥0).

Proposal 4: The other types of CP (e.g., shorter CP or longer CP than the normal CP) should not be supported.

Observation 2: Based on numerical analysis, compared with 30kHz SCS, transmission with 60kHz SCS causes more ISI due to transmission power leakage.

- 2.45%, 9.4% and 18.73% transmission power of 60kHz SCS are leaked to the next OFDM symbol in terms of 300ns, 500ns and 1000ns delay spread, respectively.

Observation 3: Based on the numerical analysis,

- for delay spread in around 7GHz, 30kHz SCS can handle it better than 60kHz SCS.

- 30kHz SCS has a better coverage than 60kHz SCS

Observation 4: Based on the simulation results, 30kHz SCS has a better performance than 60kHz SCS with about 1 dB gain.

Observation 5: 30kHz SCS has a better channel estimation performance than 60kHz SCS when the delay spread is larger than 100ns.

Proposal 5: Regarding SCS, 30kHz SCS is adopted for frequency bands around 7GHz.

Proposal 6: 6G should support a larger maximum channel bandwidth.

- 400MHz for around 7GHz

- 100MHz (for FDD) and 200MHz (for TDD) for sub-6GHz

- 800MHz for 24.25GHz - 52.6GHz.

Proposal 7: The following numerology and maximum channel bandwidth should be considered in 6G

|  | Sub 6GHz (FDD) | Sub 6GHz (TDD) | Around 7GHz | 24.25GHz - 52.6GHz |
| --- | --- | --- | --- | --- |
| Maximum CBW (MHz) | 100 | 200 | 400 | 800 |
| SCS (kHz) | 15/[30] | 30 | 30 | 120 |
| FFT size | 8k | 8k | 16k | 8k |

Proposal 8: The agreement reached in RAN1#122 should be updated as below.

Agreement - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - Whether to discuss the FFS will be subject to RANP decision. - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz - FFS: For around 15GHz frequency range between upper bound of around 7GHz and FR2-1 (e.g. around 10GHz and around 15GHz) - The following subcarrier spacing options can be studied - 30kHz, 60kHz, 120kHz - Whether to discuss it will be subject to RANP decision - Note: Carrier frequency of 15GHz has not been confirmed by ITU-R/WRC - For between 24.25GHz - 52.6GHz - Subcarrier spacing 120kHz is supported - FFS whether to allow using additional subcarrier spacing for SSB - FFS subcarrier spacing for PRACH and up to initial access discussion.

Observation 6: Regarding the potential application of 30kHz SCS for FDD for around e.g., 1-2.5GHz, this should be studied and evaluated at least considering the deployment scenarios, MRSS requirements and benefits of 30kHz SCS.

Frame structure

Observation 7: NR supports flexible TDD configuration.

Proposal 9: NR frame structure design should be the starting point for 6G.

Proposal 10: Native SBFD should be considered in frame structure design from day1.

Proposal 11: ISAC should be be considered in frame structure design.

Proposal 12: Emerging diverse traffic should be be considered in frame structure design, e.g., via supporting flexible symbols.

Proposal 13: RAN1 considers the following enhancement for frame structure in 6GR.

- Subband based frame structure configuration

- A new symbol type for SBFD

Proposal 14: Time domain resources e.g.  symbols or slots configured only for ISAC should be considered in 6G.

References

3GPP RAN#108, RP-251881, New SID: Study on 6G Radio

3GPP RAN1#122, Chair’s notes

3GPP RAN#108, RP-252914, Moderator's summary for RAN led 6G SI: Deployment scenarios

Appendix

Field test

For numerology design, a field test is conducted to measure the channel delay spread of 7GHz. The scenario of the field test is shown in the Figure 9 below. The base station is set up on top of a building. The channel delay spread is measured in two different scenarios. In Scenario 1, the UE moves along a main road in the city with tall trees on the both sides. These trees may lead to rich NLOS propagation paths. In scenario 2, the UE moves around an urban core area including many tall buildings. Due to the blocking by buildings, there may be no LOS path sometimes. Some key settings of the field test are summarized in Table 4.

Figure 9 Field test scenarios for delay spread

Table 4 Key settings of the field test

| Setting | Value |
| --- | --- |
| Frequency | 7 GHz |
| Tx number for UE | 2 |
| UE Tx power | 26dBm |
| Channel quality measurement | SRS based UL Channel quality measurement |
| Scenario#1 | Road with tall trees, largest distance is around 2~3 km |
| Scenario#2 | Tall buildings, largest distance is around 700m |
| Note1: No Rx beam forming is applied for the above measurement. Note2: Different data processing methods may have some impacts on the result. For the above results, we applied 3 dB above the noise floor as a threshold to determine paths. |  |

The field test results are shown in Figure 10. It should be noted that for normal CP the CP length for 30kHz SCS is 288 time units and the CP length for 60kHz SCS is 144 time units. In the most cases, the delay spread is less than the CP length of 60kHz SCS, i.e., 91.92% in Scenario 1 and 88.52% in Scenario 2. However, in a significant portion of samples, the delay spreads exceed the CP length of 60kHz SCS, i.e., 8.08 % in Scenario 1 and 11.48% in Scenario 2. Only very few delay spreads exceed the CP length of 30kHz SCS, i.e., 0.04% in Scenario 1 and 0.01% in Scenario 2. In general, Scenario 2 has a longer delay spread than Scenario 1 since the propagation environment is more complicated. In summary, around 8.08 % ~ 11.48% samples of the channel delay spreads are out of the range of CP for 60kHz subcarrier spacing, which will seriously impact system performance.

Figure 10 Field test results

Simulation assumptions

The LLS assumptions for PDSCH is shown in Table 6.

Table 6 Simulation assumptions

| Parameters | Values |
| --- | --- |
| SCS | 30kHz, 60kHz |
| Channel Type | PDSCH |
| Carrier Frequency | 7GHz |
| MCS | QPSK 1/2 |
| Channel Model | CDL-A |
| Channel Estimation | Ideal PMI, RI=1 |
| DMRS Demodulation | 2DMMSE |
| Delay Spread | 30ns, 100ns, 300ns |
| UE speed | 3km/h |
| Transmission Times | 1 |
| PRB | 20 PRBs for 60kHz SCS, 40 PRBs for 30kHz SCS |
| Antenna Config | 8Tx / 2Rx |
| Layer Num | 1 |
| CP type | NCP |
| CSI Feedback Delay | 1 slot delay |

The DMRS pattern is shown in Figure 11.

Figure 11 DMRS pattern in the simulation