3GPP TSG RAN WG1 #122b                                                                R1-2506814

Prague, Czech Republic, October 13th – 17th, 2025

Agenda Item:	11.2

Source:	Spreadtrum, UNISOC

Title:	Evaluation assumption for 6GR

Document for:	Discussion and decision

Introduction

The SID of Study on 6G Radio was approved in RAN#108 [1].

Based on RAN1 chair agenda, following aspects will be discussed in this agenda item:

Discussions on models, scenarios, parameters, and methodology, metrics/criteria, as well as traffic model that can be commonly used for evaluating technology proposals.

In this contribution, we focus on the UE antenna models used for different frequency bands.

UE antenna models

For UE, there are currently two candidate antenna models [2]. Alt.1 is the cross-polarized panel array antenna model, which is similar to the antenna model used on the gNB. Alt. 2 is a newly introduced antenna model for handheld terminal devices. The brief introduction of the two models is as follows:

Alt 1: Cross-polarized panel array antenna model

The antenna configuration is indicated as (M, N, P, Mg,Ng; Mp, Np), where M and N are the number of vertical, horizontal antenna elements within a panel, P is number of polarizations, Mg is the number of panels in a column, Ng is the number of panels in row; and Mp and Np are the number of vertical, horizontal TXRUs within a panel and polarization.

Figure 1 Cross-polarized panel array antenna model

Alt 2: Handheld UE antenna placement candidate locations relative to centre of device

The UE antenna can be modelled by placing the antenna element/array at the placement candidate locations relative to the centre of the UE. The size of the device can be set to (X cm, Y cm, Z cm), in depth, width, and height respectively. For handheld UE devices, the Device dimensions for UE antenna modelling are (15 cm, 7 cm, 0 cm).

Figure 2 Handheld UE antenna placement candidate locations relative to centre of device

In our view, for the evaluation of 7GHz and below frequency bands, only Alt. 1 needs to be considered and for the evaluation of 15GHz and above frequency bands, both Alt. 1 and Alt. 2 can be taken into account.

Furthermore, considering the size of UE, practical assumptions are more valuable for the evaluation. Therefore, in our opinion, the number of antenna elements of UE should not be too large, such as no more than 8, especially for handheld terminals.

Proposal 1: The assumption for the number of UE antenna elements should not exceed 8, especially for handheld terminals.

Based on the above discussion, we propose the antenna models for different frequency bands as shown in the following table.

Table 1 UE antenna model for different frequency bands

|  | ~700MHz | ~2GHz | ~4GHz | ~7GHz | ~15GHz | ~30GHz |
| --- | --- | --- | --- | --- | --- | --- |
| Alt1 | 1T2R (1,1,1,1,1;1,1) (1,2,1,1,1;1,1,2) | 1T2R (1,1,1,1,1;1,1) (1,2,1,1,1;1,1,2) | 2T4R (1,2,1,1,1;1,2) (2,2,1,1,1;1,2,2) | 2T4R (1,2,1,1,1;1,2) (2,2,1,1,1;1,2,2) | 4T8R (2,2,1,1,1;1,2,2) (2,4,1,1,1;1,2,4) | 8T8R (2,2,2,1,1;1,2,2) (2,2,2,1,1;1,2,2) |
| Alt2 | / | / | / | / | [1,2,…,8] | [1,2,…8] |

Proposal 2: The following assumption in Table 1 for UE antenna model can be baseline for 6GR evaluation.

Conclusion

In this contribution we discuss UE antenna models and make the following proposals:

Proposal 1: The assumption for the number of UE antenna elements should not exceed 8, especially for handheld terminals.

Proposal 2: The following assumption in Table 1 for UE antenna model can be baseline for 6GR evaluation.

References

- RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO

- TR 38.901 V19.0.0, “Study on channel model for frequency from 0.5 to 100 GHz”, 2025-06