3GPP TSG RAN WG1 #122bis		R1-2507159

Prague, Czech Republic, October 13th – 17th, 2025

Source:	OPPO

Title:	Remaining Issues of NR MIMO Phase 5

Agenda Item:	8.2

Document for:	Discussion and Decision

- Introduction

In this contribution, we will discuss a few remaining issues of NR MIMO phase 5, and then provide the corresponding solution.

- UE-initiated BM

When time window is configured, the UE triggers the event-driven reporting according to the number of event instances. As specified in the current spec, the UE transmits UEIRI in the first PUCCH when the number of event instances is greater than or equal to the configured number threshold. After the UE transmits the UEIRI, the UE shall reset the counter of the event instance. That is necessary to prevent UE from repeating the transmission of UEIRI for the same event.

Proposal 1: The UE reset the number of event instances after transmitting UEIRI and adopt the following TP for 38.214:

5.2.1.5.4.1 UE Initiated CSI reporting <omitted text> If the number of event instances determined by the counter for such reference signal is greater than or equal to eventInstanceCount, the UE transmits UEIRI on a PUCCH format 0 or format 1 in the PUCCH resource (in the CC provided by pucchCell, if configured, in the CSI-ReportConfig) configured by PUCCHResource in the CSI-ReportConfig and reset the counter of the event instances. <omitted text>

The specification of Event 7 in the current rel-19 spec does not work due to the following reasons. We can consider the issue for two different cases:

- When time window is not configured, the reason for not working is the event 7 reporting contents. In this case, the event 7 is triggered when the valueofQ highest L1-RSRP of the CSI-RSs or SSBs of activated TCI states is a threshold greater than the L1-RSRP of ‘current beam’. However, the beam reporting cannot carry the information of TCI state corresponding to the valueofQ highest L1-RSRP. That means no matter what is the configured valueofQ, the gNB does not receive valid information. In other word, allowing configuring valueofQ does not change the system behavior.

- When time window is configured, besides the issue of beam reporting content mentioned above, another issue is that the activated TCI state corresponding to the valueofQ highest L1-RSRP could vary from event instance to event instance according to the current spec. That behavior could totally mess up the procedure of Event 7.

To resolve the above issues so that the event 7 works, we can consider the following alternative solutions:

- For Event-7, the gNB only supports the following configurations (1) If time window is not configured, valueofQ can be configured to any value. (2) If time window is configured, the valueofQ can only be 1.

- In the UEI beam reporting for Event 7, the UE additionally reports the indicators of activated TCI states with L1-RSRP equal or worse than the valueofQ highest L1-RSRP.

- When time window and M > 1 are configured, the valueofQ highest L1-RSRP should correspond to the same CSI-RS or SSB of activated TCI state. This solution is not preferred because it would complicate both specification description and UE implementation.

Therefore, we propose the following to resolve the issues of Event 7:

Proposal 2: To make Event 7 of Rel19 workable, support one of the following solutions:

- The gNB can only provide the following configurations for Event 7: (1) If time window is not configured or M =1, the valueofQ can be configured with any value. (2) If time window and M > 1 are configured, the valueofQ can only be 1.

- The UE additionally reports indicators of activated TCI states with L1-RSRP being equal or lower than the valueofQ highest L1-RSRP.

The UE determines the event based on the counter of event instances. When the accumulated number of event instances archives the configured threshold, the UE transmits the UEIRI on the first PUCCH resource, as described in the spec. The UE resets the counter of the event instances when the RS corresponding to the ‘current beam’ is changed, which is also specified in the current spec. There is generally a time gap between the moment of the satisfied condition and the transmission occasion of the first PUCCH, the transmission occasion of the corresponding PUSCH. If the indicated TCI state is changed before the UE transmits UEIRI in the first PUCCH or beam reporting in the PUSCH, the UE shall cancel the transmission because the beam reporting does not provide valid information any more. The event is triggered based on the previous indicated TCI state, but not the current indicated TCI state at the moment of UEIRI transmission or the beam reporting transmission in the PUSCH.

Proposal 3: When the CSI-RS or SSB corresponding to the indicated TCI state is updated, the UE shall cancel the transmission in the first PUCCH resource or beam reporting in the 2nd PUSCH.

A time gap between the 1st PUCCH transmission and the corresponding PUSCH for Mode A shall be introduced to avoid UE repeating the UEIRI transmission. In Mode A, the UE transmit UEIRI in the configured PUCCH resource when the trigger condition is met. After that, the UE shall expect a PUSCH for CSI reporting would be scheduled. There shall be a time gap before the UE can transmit another UEIRI in PUCCH so that the gNB has time to schedule the PUSCH transmission.

Proposal 4: For Mode A, after one UEIRI transmission in a first PUCCH, the UE does not transmit another UEIRI in a first PUCCH within a time interval, which can be predefined or configured.

In current spec, the counter of event instances is reset when the CSI-RS or SSB corresponding to the ‘current beam’ is updated. Besides that, when any of the parameters used in determining an event instance is updated, the counter of event instances shall be reset. The reason is when some parameter(s) used in determining event instance is updated, the event instance determined after that would be different from the event instances determined before that. Thus, they cannot be counted together. For both Event 2 and Event 7, when the value of eventThreshold is updated, the counter shall be reset. For Event 1, when the value of  is eventThreshold-Event1 updated, the counter shall be reset. For Ecvent 7, when the value of valueofQ isupdated, the counter shall be reset.

Proposal 5: The counter for event instances shall be reset if the eventThreshold, eventThreshold-Event1 or valueofQ is updated and adopt the following TP for 38.214:

5.2.1.5.4.1 UE Initiated CSI reporting <omitted text> The counter of the event instances for such reference signal is reset: - if the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is updated, or - if the value of eventThreshold, eventThreshold-Event1 or valueOfQ is updated. The UE does not expect that a CSI trigger state associated with CSI report configuration(s) configured with the higher layer parameter eventType is further associated with other CSI report configurations that are not configured with the higher layer parameter eventType. <omitted text>

- Conclusions

In this contribution, we discussed a few remaining issues of NR MIMO phase 5 and the following proposals are made:

Proposal 1: The UE reset the number of event instances after transmitting UEIRI and adopt the following TP for 38.214:

5.2.1.5.4.1 UE Initiated CSI reporting <omitted text> If the number of event instances determined by the counter for such reference signal is greater than or equal to eventInstanceCount, the UE transmits UEIRI on a PUCCH format 0 or format 1 in the PUCCH resource (in the CC provided by pucchCell, if configured, in the CSI-ReportConfig) configured by PUCCHResource in the CSI-ReportConfig and reset the counter of the event instances. <omitted text>

Proposal 2: To make Event 7 of Rel19 workable, support one of the following solutions:

- The gNB can only provide the following configurations for Event 7: (1) If time window is not configured or M =1, the valueofQ can be configured with any value. (2) If time window and M > 1 are configured, the valueofQ can only be 1.

- The UE additionally reports indicators of activated TCI states with L1-RSRP being equal or lower than the valueofQ highest L1-RSRP.

Proposal 3: When the CSI-RS or SSB corresponding to the indicated TCI state is updated, the UE shall cancel the transmission in the first PUCCH resource or beam reporting in the 2nd PUSCH.

Proposal 4: For Mode A, after one UEIRI transmission in a first PUCCH, the UE does not transmit another UEIRI in a first PUCCH within a time interval, which can be predefined or configured.

Proposal 5: The counter for event instances shall be reset if the eventThreshold, eventThreshold-Event1 or valueofQ is updated and adopt the following TP for 38.214:

5.2.1.5.4.1 UE Initiated CSI reporting <omitted text> The counter of the event instances for such reference signal is reset: - if the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is updated, or - if the value of eventThreshold, eventThreshold-Event1 or valueOfQ is updated. The UE does not expect that a CSI trigger state associated with CSI report configuration(s) configured with the higher layer parameter eventType is further associated with other CSI report configurations that are not configured with the higher layer parameter eventType. <omitted text>