3GPP TSG RAN WG1 #122bis			R1- 2507204

Prague, Czech, Oct 13th – 17th, 2025

Source: 	Tejas Networks

Title:	Associating a detected object with its true counterpart in ISAC

Agenda Item:	10.5

Document for:	Discussion & Decision

- Introduction

According to Chair Notes RAN1#122 v14, section 10.5 [Study on Integrated Sensing and Communication (ISAC) for NR], companies are required to report the methods used for associating detected objects with true targets.

R1-2506479 Summary #4 on evaluations for NR ISAC Moderator (Xiaomi) Agreement For the purpose of performance metric calculation, association of the detected object(s) and the true target(s) should fulfil at least the following conditions: - One true target is associated with at most one detected object - One detected object is associated with at most one true target - The same association applies to miss detection, false alarm probability Type 2 (if defined) and positioning/velocity accuracy - Companies to report the method used for association of the detected object(s) and the true target(s) - Observation: - If the circle (threshold) increases - False alarm rate is reduced, hence BETTER performance - Miss detection is reduced, hence BETTER performance - Accuracy is reduced, hence BAD performance

- Measurements

Monostatic sensing in ISAC leverages a co-located transceiver for both transmission and reception, enabling simultaneous communication and environmental perception from the same node. The advantages of monostatic sensing are

- Compact Hardware Integration

- No need for inter-node time or frequency alignment

- Lower system complexity - No need for distributed coordination or multi-node signalling

The following figure shows a diagrammatic representation of a typical Monostatic sensing

.

Observation 1: In MIMO-based monostatic sensing, significant self-interference is expected.

Proposal 1: Employing distinct subpanels for transmission and reception reduces self-interference during simultaneous operation in monostatic sensing scenarios.

Given that signal power attenuates proportionally to the square of the distance, and the echoed return path experiences a similar decay, beamforming becomes essential to achieve an extended sensing range

Also, while beamforming extends the detection range, it inherently requires multiple beams to span the full 120-degree sector. This involves extracting the range, angle, beam index, and Doppler.

Observation 2: Beamforming is essential for extending the sensing range; however, it necessitates the transmission of multiple sequentially swept beams to ensure full sector coverage.

Proposal 2: Multiple beam-sounding involves extracting range, angle, beam index, and Doppler metrics from successive beams, followed by data collation and association of range-angle pairs for each detected target.

  - Paring target ranges and angles

This step involves the spatial and temporal association of echoes. Each beam may detect multiple targets, and to estimate the precise angle of arrival, algorithms like MUSIC or MVDR are used. But this also means that the estimated distances and the estimated angles have to be paired.

While ESPRIT provides only the angles, MUSIC or MVDR provides a spectrum, where the height of peaks corresponds to the covariance or the signal strength. This may be compared with the correlation strength in the delay estimation plot.

Observation 3: A single transmitted beam may encounter more than one target, thereby producing multiple range and angle-of-arrival measurements

Proposal 3: The correlation peaks of time of arrival and that of angle-of-arrival spectra, derived from MUSIC or MVDR algorithms, are sorted by magnitude and matched based on corresponding peak heights.

  - Beam-ID-based target segregation

Since targets reflect signals only when properly illuminated, not all are visible at once. This staggered visibility helps in separating targets and matching detections to actual positions.

Observation 4: Selective visibility of targets because of beamforming inherently helps distinguish targets and align detected reflections with their true counterparts

Proposal 4: By tagging detected targets with their respective beam identifiers, the pool of contending candidates is reduced, aiding in the reliable association of detections with actual targets.

  - Merging readings from multiple Beams

Due to beam overlap, duplicate detections from adjacent beams must be fused, and measurements from all beams aggregated to obtain a unified estimate of range, angle, and Doppler for each target within the sector.

In the illustrated scenario, target 2 is observed by beams 0 and 1. Such redundant detections need to be appropriately merged to obtain unified target representations.

There may be targets (like target 5 in the figure) outside the range.

Observation 5: A single target may be detected multiple times by adjacent overlapping beams

Proposal 5: Redundant detections from overlapping beams are appropriately merged to obtain a unified target representation

  - Marking border around true targets for detected target association

For each true target, an imaginary border with a radius less than half the minimum target separation is drawn.

Observation 6: Tighter thresholds may be possible for nearby targets as they experience higher SNR, and have a smaller Euclidean error from the same angle of arrival estimation error.

Proposal 6: To assess missed or false detections, a threshold region is defined around each true target, with a radius smaller than half the minimum inter-target Euclidean distance

  - Finding the missed detection / false detection

If a target is detected outside of this range, or if multiple targets are detected for a true target, then such detections are considered false detections.

If no target is detected within this border, then it is declared a missed detection.

Proposal 7: In the absence of any detected targets within the threshold vicinity of a true target, the instance is classified as a missed detection.

Proposal 8: A detection is classified as false if it lies outside the threshold region of a true target, or if multiple detections occur within the threshold zone, where only the nearest one is accepted and the rest are considered false.

  - Accepting only one detected target within the marked border

If multiple targets are detected within the border, then the target nearest the true target is considered the estimation, and the other targets are considered false alarms

Proposal 9: For assessing positional accuracy, only one estimated target is considered valid per true target.

  - Finding the estimation error of the detected targets

Estimation error is quantified by computing the Euclidean distance between the estimated and actual target positions, considering x and y coordinates, and optionally z. The estimated location is derived using the base station’s orientation and position, in conjunction with the target’s range, angle of arrival, and associated beam indices. True velocity is inferred from successive measurements of the target over time.

Proposal 10: The estimation error is computed as the Euclidean distance between the true target position and the nearest estimated location within its threshold region.

- Conclusion

In this contribution, we report the following methods we considered for associating detected objects with true targets

Observation 1: In MIMO-based monostatic sensing, significant self-interference is expected.

Proposal 1: Employing distinct subpanels for transmission and reception reduces self-interference during simultaneous operation in monostatic sensing scenarios.

Observation 2: Beamforming is essential for extending the sensing range; however, it necessitates the transmission of multiple sequentially swept beams to ensure full sector coverage.

Proposal 2: Multiple beam-sounding involves extracting range, angle, beam index, and Doppler metrics from successive beams, followed by data collation and association of range-angle pairs for each detected target.

Observation 3: A single transmitted beam may encounter more than one target, thereby producing multiple range and angle-of-arrival measurements

Proposal 3: The correlation peaks of time of arrival and that of angle-of-arrival spectra, derived from MUSIC or MVDR algorithms, are sorted by magnitude and matched based on corresponding peak heights.

Observation 4: Selective visibility of targets because of beamforming inherently helps distinguish targets and align detected reflections with their true counterparts

Proposal 4: By tagging detected targets with their respective beam identifiers, the pool of contending candidates is reduced, aiding in the reliable association of detections with actual targets.

Observation 5: A single target may be detected multiple times by adjacent overlapping beams

Proposal 5: Redundant detections from overlapping beams are appropriately merged to obtain a unified target representation

Observation 6: Tighter thresholds may be possible for nearby targets as they experience higher SNR, and have a smaller Euclidean error from the same angle of arrival estimation error.

Proposal 6: To assess missed or false detections, a threshold region is defined around each true target, with a radius smaller than half the minimum inter-target Euclidean distance

Proposal 7: In the absence of any detected targets within the threshold vicinity of a true target, the instance is classified as a missed detection.

Proposal 8: A detection is classified as false if it lies outside the threshold region of a true target, or if multiple detections occur within the threshold zone, where only the nearest one is accepted and the rest are considered false.

Proposal 9: For assessing positional accuracy, only one estimated target is considered valid per true target.

Proposal 10: The estimation error is computed as the Euclidean distance between the true target position and the nearest estimated location within its threshold region.

- References

[1] Chair Notes RAN1#122 v14, Draft Agenda, Bengaluru, India, Aug 25th-29th 2025, section 10.5

[2] R1-2506479, Summary #4 on evaluations for NR ISAC, Moderator (Xiaomi), Agenda item 10.5.1, Bengaluru, India, Aug 25th – 29th 2025.