3GPP TSG RAN WG1 #122bis 	R1- 2507294

Prague, Czech, Oct 13th – 17th, 2025

Agenda item:		8.7.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Maintenance for Rel19 IoT-NTN

Document for:		Discussion and Decision

# Introduction

In RANP # 107 meeting, WI for Rel19 IoT NTN [1] was approved with objective in RAN1 as:

- Support of Capacity enhancements for uplink

  - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 [RAN1, RAN2, RAN4]

    - Multi-tone support for 15 kHz SCS should also be considered

    - Specify necessary signalling, if needed

    - Update RF requirements accordingly, if needed

Note: Impact of impairment shall be taken into account

At meeting of RAN1 #116bis to RAN1 #122, many of IoT NTN UL enhancement issues have been discussed and enhanced.

While, considering the limited time, still there are some issues not discussed but needed, or discussed but no agreement. We will discuss these issues and provide our view in this contribution.

# Discussion

## OCC in UL segments in IoT NTN

In normative phase, there has been already some discussion on OCC in scenarios with UL gaps. For UL gaps for DL synchronization, as UE will postpone the NPUSCH, there seems no additional impact when OCC is applied.

While in RAN1 #122 meeting, the impact from UL gap for NPRACH to OCC has been discussed. RAN1 observation is if there is UL gap for NPRACH, then if part of OCC codeword is dropped, then the orthogonality of OCC will be destroyed, thus RAN1 spec is updated to avoid the partial dropping of OCC codeword

While for UL gaps corresponding to segmented transmission in NTN scenario, there will be similar issue.

Observation 1: There is issue for OCC applied together with segment based gap in IoT NTN.

In legacy IoT the link budget is enhanced by use of repetitions and this also includes repetitions of NPUSCH transmissions. In release 17, the 3GPP defined repetitions, when used in NTN, are divided into segments. This enables time/frequency adjustment at each segment to handle the synchronization drift caused by the satellite movement. Between the segments the UE need apply a gap, which may cause dropping of one or more symbols of the next segment.

Specifically, for NB-IoT the segment gap and duration are:

- uplinkSegmentedPrecompensationGap-r17 {sym1,sl1,sl2}

- npusch-TxDuration-r17 {ms2, ms4, ms8, ms16, ms32, ms64, ms128, ms256}

Observation 2: Legacy IoT NTN rely on segmented repetition periods to facilitate time/frequency synchronization.

One open issue is thus how the UE shall apply OCC when one or more symbols are dropped as part of a segmented transmission in a repetition period. For example, if the UE drops one symbol when an OCC length of two is used (i.e. spreading the single symbol to two symbols), it may cause challenges in the eNB receiver due to the orthogonality of the OCC is not completely preserved.

For symbol level OCC of NPUSCH with 3.75kHz SCS, TDM DMRS for OCC will be used with 4 slots for DMRS orthogonality. For slot level segment, if only 1 DMRS symbol remained for 4 slots after the segment based dropping, the demodulation performance will be significant degraded for that UE.

For slot level OCC, CDM DMRS for OCC will be used. For slot level segment, if one DMRS symbol is dropped together with the slot, the orthogonality of the DMRS for OCC will be destroyed, and the performance will degrade.

While for symbol level segment gap, similarly, the orthogonality of the OCC will be destroyed, resulting in performance degradation.

Additionally, it can not be solved by network scheduling as dropping part of OCC codeword will always impact OCC orthogonality, especially for segment gap during the NPUSCH trasnmission.

Observation 3: The dropping of symbols as part of segmented transmissions may impact the orthogonality of the OCC or DMRS for OCC.

In order to ensure the eNB decoding performance one simple way could be that the UE drops an entire OCC codeword, i.e. the symbols that are spread according to the OCC code length, when at least one symbol of the OCC codeword is impacted by the segment gap. Referring to the above example, the UE should thus drop not only the symbol in the gap, but also the corresponding symbol outside the gap that belongs to the same OCC codeword (length two).

Proposal 1: RAN1 to support that the UE shall drop all symbols that are spread according to the OCC length, when at least one symbol is part of the segmentat gap.

Based on above discussion, we propose the following TP1 for 36.211

| Reason for change: | The OCC codeword should be complete and fully aligned between UEs, but segment based gap will make the OCC codeword incomplete and impact the orthogonality of OCC or DMRS for OCC. |
| --- | --- |
| Summary of change: | Ngapprecompensation is extended to be a multiple of OCC length when OCC is applied. |
| Consequences if not approved: | When Ngapprecompensation is not multiple of OCC length, the OCC orthogonality will be destroyed for transmission just after the segment based gap. |

- 10.1.3.6

<omitted text>

For a UE communicating over NTN, after transmissions (and/or postponements due to NPRACH) of Nsegmentprecompensation time units, for frame structure type 1, a transmission gap of Ngapprecompensation time units shall be counted for the NPUSCH resource mapping but not used for transmission of the NPUSCH according to the UE capability ntn-SegmentedPrecompensationGaps-r17 as specified in 3GPP TS 36.331 [9]. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity of Ngapprecompensation is configured by higher layers based on the UE capability if signalled when OCC is not applied, or the quantity of Ngapprecompensation is configured by higher layers based on the UE capability if signalled and extended to the minimum value that is a multiple of the OCC length when OCC is applied.

<omitted text>

Considering some comments in RAN1 #122 meeting to consider both inter slot and inter symbol OCC, then one way is only the pairs of data symbols (as mentioned in 36.211 10.1.3.6, inter slot for slot level OCC or intra slot for symbol level OCC) with OCC corresponding to the transmission gap are dropped. Then the below TP2 can be applied for 36.211.

| Reason for change: | The OCC codeword should be complete and fully aligned between UEs, but segment based gap will make the OCC codeword incomplete and impact the orthogonality of OCC or DMRS for OCC. |
| --- | --- |
| Summary of change: | The pairs of data symbol with OCC corresponding to the transmission gap are dropped when OCC is applied, |
| Consequences if not approved: | When Ngapprecompensation is not multiple of OCC length, the OCC orthogonality will be destroyed for transmission just after the segment based gap. |

- 10.1.3.6

<omitted text>

For a UE communicating over NTN, after transmissions (and/or postponements due to NPRACH) of Nsegmentprecompensation time units, for frame structure type 1, a transmission gap of Ngapprecompensation time units shall be counted for the NPUSCH resource mapping but not used for transmission of the NPUSCH according to the UE capability ntn-SegmentedPrecompensationGaps-r17 as specified in 3GPP TS 36.331 [9]. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity of Ngapprecompensation is configured by higher layers based on the UE capability if signalled when OCC is not applied, or the quantity of Ngapprecompensation is configured by higher layers based on the UE capability if signalled and the pairs of data symbol corresponding to the transmission gap shall be counted for the NPUSCH resource mapping but not used for transmission of the NPUSCH when OCC is applied.

<omitted text>

Proposal 2: RAN1 should adopt the proposed TP1 or TP2 for update of the segment based gap when OCC is applied.

## OCC with TA adjustment in IoT NTN

36.213, section 16.1.2

When the UE's uplink NPUSCH transmissions in NB-IoT uplink slot n and NB-IoT uplink slot n+1 are overlapped due to the timing adjustment, the UE shall complete transmission of NB-IoT uplink slot n and not transmit the overlapped part of NB-IoT uplink slot n+1.

For IoT TN and NTN, there is description on how UE will adjust TA and adjustment of the NPUSCH transmission as in 36.213 section 16.1.2,

When OCC is enabled and an overlapped part of one OCC codeword is not transmitted according to the legacy NB-IoT processing for time adjustment, the orthogonality of the OCC codeword will be destroyed, for both SCS 3.75kHz and SCS 15kHz.

Additionally, there are cases where only one UE of the OCC paired UEs received the TA command and adjust the TA in the OCC codeword. Then the OCC orthogonality will be destroyed for the paired UE.

Observation 4: When one of the UE in OCC pairing do not transmit overlapped part because of the TA adjustment, the OCC orthogonality will be destroyed for both SCS 3.75kHz and SCS 15kHz.

To keep the orthogonality of OCC between the paired UEs, when one part of an OCC codeword is not transmitted because of TA adjustment, one simple way is the OCC codeword(s) corresponding to the not transmitted part is also not transmitted.

Proposal 3: When part of one OCC codeword is not transmitted because of the TA adjustment, the OCC codeword should not be transmitted, to guarantee the orthogonality of the OCC.

Based on above discussion, we propose the following TP for 36.213

| Reason for change: | The OCC codeword should be complete and fully aligned between UEs, but overlapped part not transmitted will make the OCC codeword incomplete and impact the orthogonality of OCC. |
| --- | --- |
| Summary of change: | ForΔf=15 kHz, the overlapped part and the repeated of it are not be transmitted. For Δf=3.75 kHz, the the pairs of data symbols in the overlapped part are not transmitted. |
| Consequences if not approved: | The OCC orthogonality will be destroyed if only overlapped part is not transmitted. |

- 16.1.2

When the UE's uplink NPUSCH transmissions in NB-IoT uplink slot n and NB-IoT uplink slot n+1 are overlapped due to the timing adjustment, the UE shall complete transmission of NB-IoT uplink slot n and not transmit the overlapped part of NB-IoT uplink slot n+1 when OCC is not applied, otherwise if OCC is applied

- for Δf=15 kHz, the UE shall complete transmission of NB-IoT uplink slot n and not transmit the overlapped part of NB-IoT uplink slot n+1 and the repeated overlapped part in NB-IoT uplink slot n+2,

- for Δf=3.75 kHz, the UE shall complete transmission of NB-IoT uplink slot n and not transmit the pairs of data symbols in the overlapped part of NB-IoT uplink slot n+1.

Proposal 4: RAN1 should adopt the above proposed TP for update of the TA adjustment processing when OCC is applied.

# Conclusion

In this contribution, we discussed our view on remaining issue of OCC for IoT NTN. Our observations and proposals are as follows:

Observation 1: There is issue for OCC applied together with segment based gap in IoT NTN.

Observation 2: Legacy IoT NTN rely on segmented repetition periods to facilitate time/frequency synchronization.

Observation 3: The dropping of symbols as part of segmented transmissions may impact the orthogonality of the OCC or DMRS for OCC.

Observation 4: When one of the UE in OCC pairing do not transmit overlapped part because of the TA adjustment, the OCC orthogonality will be destroyed for both SCS 3.75kHz and SCS 15kHz.

Proposal 1: RAN1 to support that the UE shall drop all symbols that are spread according to the OCC length, when at least one symbol is part of the segmentat gap.

Proposal 2: RAN1 should adopt the proposed TP1 or TP2 for update of the segment based gap when OCC is applied.

Proposal 3: When part of one OCC codeword is not transmitted because of the TA adjustment, the OCC codeword should not be transmitted, to guarantee the orthogonality of the OCC.

Proposal 4: RAN1 should adopt the above proposed TP for update of the TA adjustment processing when OCC is applied.

# References

- RP-250472, “Revised WI: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, March 2025