| 3GPP TSG RAN WG1 #122bis | R1- 2507613 |
| --- | --- |
| Prague, Czech, Oct 13th – 17th, 2025 |  |
| Agenda item: 11.6 Title: AI/ML Framework and Use Cases for 6GR Air Interface Source: MediaTek Inc. Document for: Discussion and Decision |  |

# Summary of R20 SI and RAN1#122 Meeting

The R20 study item (SI): “Study on 6G Radio” was approved in RAN#108 [1] . Among the various objectives of the study, the following pertains to AI/ML for 6GR and Radio Access Network.

Captured from RP-251881 [1] - AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In the previous RAN1#122 meeting, the following agreement on information regarding the AI/ML use cases was reached [3].

Agreement For 6GR AI/ML use cases identification/categorization, for each (sub-)use case proposed, proponent companies are encouraged to study and report the following: - Definition of each (sub-)use case, including at least AI/ML model input/output - The evaluation assumption, methodology, KPIs, benchmark, and preliminary simulation results - Assumption on training types, e.g., - offline training, online training/finetuning - Label construction (if applicable), including whether/how to obtain label data for model training - Assumption on model location for inference, e.g., UE-sided model, NW-sided model, and two-sided model - Collaboration/interaction between UE and NW, e.g., - no collaboration/interaction - UE/Network collaboration targeting at separate or joint ML operation - High level potential specification impact

The following guidance was provided by the Moderator in RAN1#122 [3] regarding AI/ML use cases and framework/evaluation aspects.

#1 For use cases Companies are encouraged to provide evaluations and analysis on each proposed use cases based on the agreement in RAN 1#122. We will do clarification on each use case, and categorization based on the clarification, hopefully with a full list. - Priority of each use case discussion will base on the one with more positive simulation results and solid analysis on the aspects identified in the agreement. - The one without simulation results will not be treated, except for 5GA use cases and extension.

#2 for frame structure/evaluation methodology If time is allowed, we can consider some discussion starting from the following proposals. Proposal 1.2A: For 6G LCM framework for AI/ML for air interface, consider the 5G NR AI/ML LCM framework (see section 4 “General AI/ML framework” of TR 38.843 v18.0.0) as a starting point. - Study the necessity of potential enhancements for LCM, and if justified, the enhancement details. The examples to study include: - Data and model management - Handling of additional conditions - Enablers for continuous (online) on-device model training/finetuning - framework for AI/ML processing and memory [Note: the study strived to provide a unified LCM across use cases] Proposal 1.1A : For evaluation of AI/ML use cases in 6GR, consider - Performance related metrics, including intermediate (model) performance KPIs, link level KPIs (e.g., BLER) and system level KPIs (e.g., throughput, overhead) - Where the overhead including the overhead associated with inferencing, performance monitoring and (re-)training - AI/ML Model related metrics, including model/computational complexity, inference latency, training latency (when applicable), - FFS on whether/how to measure power consumption - Inter-vendor collaboration when applicable - Generalization performance under a wide range of conditions - FFS on whether and how to consider realistic deployment scenarios Note: Detailed metrics to be discussed per use case.

In this contribution, we identify use cases for study in RAN1 and a unified AI/ML framework for handling multiple use cases.

# AI/ML Framework for 6GR Air Interface

In 5G, AI/ML has been explored as powerful tools for optimizing certain components of the physical layer separately, such as CSI prediction, CSI compression, positioning, and beam management [2]. However, 6G, in contrast, envisions a native integration of AI/ML into the physical layer, moving from isolated applications to end-to-end intelligent processing; requiring a more adaptive and comprehensive framework to pave the way for emerging applications. In our view, a general/unified AI/ML framework designed for the 6GR air interface, should focus on the following aspects:

- Extensibility for emerging applications: The AI/ML frameworks developed during 5G were closely tied to a limited set of well-defined use cases. While such use case-specific frameworks are successful in handling the life cycle of AI/ML models in their specific context, they are too narrowly designed to handle the broader and more complex landscape of applications and training paradigms expected in 6G. It will be crucial to consider the potential wide range of applications and diverse training paradigms that 6G encompasses when it comes to designing a framework.

- Leveraging synergy between use cases: During 5G studies, each use case was studied separately with its own requirements for training, inference, and monitoring. Despite making considerable efforts to satisfy these needs, the synergy between use cases was completely overlooked. Also, AI/ML models that can handle multiple tasks were ignored. We believe a successful framework for 6G should identify synergies between selected use cases and incorporate those in both AI/ML model design and LCM procedures. By identifying and leveraging these overlaps, the framework can ease deployment, lower system complexity, and support a more scalable and efficient integration of AI/ML into the 6G physical layer.

- Facilitating both offline and online model (re-)training: 5G AI/ML use cases have solely targeted offline training where AI/ML models are fully developed before deployment without any possibility of update within the specified LCM. 6G framework for AI/ML is expected to have flexibility in enabling the user equipment (UE) and network (NW) to update/re-train their AI/ML models wherever needed based on their fresh knowledge and observations with minimal interruption in their functionality.

- Avoiding unreasonable signaling overhead for monitoring and training/inference (when applicable): The 6G framework should be designed around shared needs and common design elements across potential AI/ML use cases. Signaling procedures for data collection, training, inference, and monitoring mechanisms should be built with reusability and extendibility in mind, allowing components developed for one task to be extended or adapted to others. For instance, monitoring results from one use case could inform the performance of another, or data collected for one use case could help another use case too. Such an approach reduces redundancy at a signaling level.

- Facilitating separate/partial training of AI/ML models: In 5G, AI/ML models were discussed as single end-to-end module; disregarding any modular design of AI/ML models. So, in the context of 5G AI/ML, it is not possible to update/modify a part of AI/ML model, even one-sided ones. This problem is more acute for two-sided AI/ML models where UE and NW vendors undergo strict collaboration efforts for any change in their AI/ML models. In our view, 6G framework should provide new solutions for training and modification of both one-sided and two-sided AI/ML models.

## Example and feasibility study of a unified architecture

Consider Figure 2-1, which illustrates a unified AI/ML architecture for multiple applications using CSI-related data, e.g., CSI compression, link adaptation, positioning, etc. It consists of two neural network (NN) components:

- a common “backbone” NN model which performs training/inference on raw channel information/measurements and produces a lower-dimensional latent space representation, and

- one or more smaller "head" networks, which perform training/inference with the latent space representation and produces data tailored for one or more different use cases/applications (apps).

Since the same backbone NN can be used across applications/use cases which make use of CSI-related data, the parameters could be reused, thus reducing memory requirements. Further, once the backbone network is trained, only the “heads” need to be trained for each application, thus reducing the training requirements.

Figure 2-1 Example of unified architecture for AI/ML use cases related to channel information.

Head model-2

Head model-1

Head model-N

App-1

App-2

App-N

Channel information

Backbone model

Latent space

We present a feasibility study on the above unified architecture for two use cases: CSI compression and positioning. Consider the architecture shown in Figure-2 which uses a CSI autoencoder structure with the same CSI encoder backbone model, but different head models for CSI recovery and positioning.

We train the overall model in two stages:

      - The CSI autoencoder is trained first, and the encoder backbone is fixed thereafter.

Figure 2-2 Structure of unified architecture for CSI compression and positioning.

CSI Decoder head (Transformer)

Positioning head (MLP)

App-1: Positioning

App-2: CSI Compression

Channel information

(32x26 real matrix)

CSI Encoder backbone model (Transformer)

Latent space

Further, CSI information with position tags is input to the encoder to generate latent space representations. The latent space outputs paired with the ground truth positions, are used to train the positioning head model, which is a multilayer perceptron (MLP).

The CSI information used for training is obtained from the DIstributed CHAnnel Sounder by University of Stuttgart (DICHASUS) [4]. The CSI data is in the form: [4 antenna arrays × 8 antennas × 13 complex delay taps, position (x,y)]. 75586 CSI data samples are used for training the CSI autoencoder with 90%/10% training/validation split, while 3,000 samples having position labels are used to train the position head with 90%/10% training/validation split. The performance of CSI compression and positioning accuracy are evaluated using 8399 data samples.

The following evaluation metrics are considered:

- Normalized mean square error (NMSE) for CSI compression performance.

- Mean absolute error (MAE) for positioning performance.

The evaluation results are provided in Table 2-1. From Table 2-1, although the common backbone model is trained only once in combination with the CSI decoder, the combination of the common CSI Encoder backbone and the Positioning MLP can perform as good or even better than an NN consisting of the CSI Encoder and the Positioning MLP trained together specifically for positioning. Similarly, for CSI compression, the model based on the unified architecture can perform better than a benchmark model from the literature. This demonstrates the advantage of such a unified model in training and life cycle management (LCM). Since the backbone model is common for both use cases, it exploits the synergy between the use cases to reduce the training requirements on the overall model. Further, since the backbone needs to be trained/updated only once or less infrequently compared to the head networks, it enables partial (re-)training (of only the head networks), while relaxing the model update requirements for all the use cases.

| Compression Ratio | CSI Compression NMSE |  | Positioning MAE |  |
| --- | --- | --- | --- | --- |
|  | TransNet [5] | Unified architecture | Dedicated NN | Unified architecture |
| 8 | 0.0676 | 0.0457 | 0.245 m | 0.240 m |
| 16 | 0.1228 | 0.0765 | 0.245 m | 0.246 m |
| 32 | 0.2185 | 0.1216 | 0.245 m | 0.280 m |

Table 2-1 Performance of unified architecture for two different use cases: CSI compression and positioning based on CSI.

Observation 1: A unified model architecture comprising a common backbone model and multiple head models corresponding to multiple use cases is studied for the use cases of CSI compression and positioning. It is observed to have better or at least comparable performance to using dedicated models for each use case. It also enables more efficient LCM by allowing partial (re-)training and less frequent updates to the common backbone model.

Proposal 1: Study a general/unified AI/ML framework for 6GR Air Interface considering the following aspects:

- Extensibility for emerging applications

- Leveraging synergy between use cases

- Facilitating both offline and online model (re-)training

- Avoiding unreasonable signalling overhead for monitoring and training/inference (when applicable)

- Facilitating separate/partial training of AI/ML models

# Use Cases Overview

The use cases studied during NR Rel-18 and Rel-19 could be a starting point for 6GR AI/ML study. In the following subsections, we highlight new use cases motivated by signaling overhead reduction and link performance improvements. The use cases are related to DMRS overhead reduction, CSI inference/prediction, and CSI feedback.

## Uplink DMRS Overhead Reduction

Demodulation reference signals (DMRS) are required for coherent demodulation of physical uplink shared channel (PUSCH) data in the UL. However, they introduce additional overhead since the resource elements (REs) used for transmitting DMRS cannot be used to transmit data. E.g., with a 1-symbol Type-1 DMRS with four orthogonal ports, all the 12 REs in a PRB would be used for DMRS, and if these REs were instead used to transmit data, approximately 7% throughput gain could be obtained. However, if DMRS are not transmitted, it is not possible to perform coherent demodulation at the receiver using classical methods. In this situation AI/ML-based methods can be leveraged to get gains from DMRS-less transmissions, while still performing better than classical methods in terms of block error rate (BLER) and throughput. Another aspect of using AI/ML-based methods is the performance vs. complexity trade-off which needs to be carefully evaluated. The NW generally has more capabilities to perform complex AI/ML-based algorithms and hence, it is preferable to study such schemes for UL transmission first.

## CSI Inference/Prediction

Accurate channel information is essential for MIMO transmissions. With the expected use of larger antenna arrays at the NW/gNB in 6G and the increased number of antenna ports, more resources are needed for downlink (DL) CSI- reference signal (RS) transmission and feedback, which decreases average system spectral efficiency over the system bandwidth, increases power consumption (especially during low-load data traffic) and processing delay. For time division duplex (TDD) systems, uplink (UL) sounding reference signals (SRS) can be used for DL CSI acquisition based on channel reciprocity to reduce overhead. However, UL SRS has limited coverage compared to DL CSI-RS. Thus, reducing CSI-RS overhead for large-scale MIMO systems to maximize their spectral efficiency, coverage and reliability will be critical.

CSI-RS overhead reduction can be done in spatial, frequency, and time domains. It is challenging to maintain accurate channel estimation with only a few RSs. Classical methods are typically based on specific mathematical/statistical models which may not match with real environments well, leading to performance degradation. As a powerful tool, AI/ML models have shown their potential to learn complex channel characteristics from data, enabling them to infer unobserved channel information accurately and efficiently [3]. We believe AI/ML based channel inference based on reduced CSI-RS overhead, i.e., less-frequent CSI-RS in time, sparser CSI-RS in frequency and fewer CSI-RS antenna ports in the spatial domain, could further improve KPIs of the system, such as spectral efficiency and power consumption, and thus, should be studied by taking complexity, energy efficiency and latency into account.

## CSI Feedback/Compression

DL CSI codebook design has been evolved to accommodate different types of MIMO use cases, i.e., multi-user MIMO, multi-TRP coherent/non-coherent transmission, etc. The various configured codebooks, increase CSI computation complexity and feedback overhead, which could decrease the energy and spectral efficiencies. Hence for 6G, we should establish a unified DL CSI feedback framework to efficiently handle various use cases and potential extensions without sacrificing energy and spectral efficiency while trying to address challenges from multi-vendor operation and life cycle management.

In the 5G AI/ML study spatial-frequency domain CSI compression was studied using a two-sided AI model, where CSI reconstruction at the NW produces the precoding vectors per each frequency unit as output. However, compressing and reconstructing the raw channel estimate instead of the precoding vectors could provide advantages. Moreover, in 6G, higher order MIMO dimensions are expected, which increase the importance of efficient CSI compression and feedback.

# Use Cases Details

## Uplink DMRS Overhead Reduction with AI-Transceiver/Receiver

Some prior research work has shown the theoretical feasibility of an end-to-end (E2E) AI/ML-based transceiver in the physical layer [9], while several others propose various neural network (NN) architectures for MIMO/MIMO-OFDM receivers [10][11][12]. An E2E AI/ML-based MIMO-OFDM transceiver which did not require DMRS transmissions was considered by Korpi et al. [13], and it showed performance closely matching classical Quadrature Amplitude Modulation (QAM) transmission coupled with DMRS-based channel estimation and the “K-best” MIMO decoder at the receiver.

Our own simulations reveal that an AI/ML-based E2E transceiver is beneficial for improving throughput and bit/block error rate (BER/BLER) in the uplink (UL) for a SIMO/MIMO-OFDM link. The improvement in throughput is enabled by avoiding DMRS transmissions in the UL, while still achieving better BER and BLER due to the use of a NN-based receiver at the NW and a learned constellation at the UE. We also find that for a SIMO-OFDM UL with reduced-DMRS and classical QAM transmission, a NN-based receiver at the NW can still offer BLER and throughput gains over a conventional receiver.

Definition

Figure 4-1 MIMO-OFDM AI-Transceiver: Architecture for Training.

UL DMRS-less transmission for SIMO/MIMO-OFDM is enabled by employing a trainable constellation at the UE and a NN-based receiver at the NW. Figure 4-1 shows the transmitter and receiver architecture for the proposed AI-transceiver.

- UL reduced-DMRS transmission for SIMO-OFDM is enabled by employing a NN-based AI-receiver at the NW and conventional QAM transmissions from the UE.

Figure 4-2 SIMO-OFDM with AI-Receiver: Architecture for training.

Model input: For sub-use case (a), the model input consists of uncoded transmit bits. For sub-use case (b), the model input consists of the received complex symbols Y.

Model output: For both sub-use cases (a) and (b) the model output is the log likelihood ratios (LLR) of each transmitted bit at the receiver which can be subsequently fed into a classical channel decoder.

Evaluation

Assumptions: Uplink transmission from a single UE over the CDL-C channel model [15] with reception by a single gNB is assumed, with other parameters as provided in Table 4-1.

| Parameter | SIMO | MIMO |
| --- | --- | --- |
| Number of antennas at the gNB | 8 | 4 |
| Number of antennas at the UE | 1 | 2 |
| Carrier frequency | 3.5 GHz | 3.5 GHz |
| Subcarrier spacing | 30 kHz | 30 kHz |
| Number of PRBs | 24 | 6 |
| DMRS configuration for non-AI methods | 2 symbols per slot | 2 symbols per slot |
| Number of bits per symbol | 6 | 6 |
| Channel coding | LDPC | LDPC |
| Code rate | 0.5 | 0.5 |
| Channel model | CDL-C 100 ns | CDL-C 100 ns |
| UE speed | 0 ∼ 25 km/h | 0 ∼ 20 km/h |

Table 4-1 Parameters for Evaluation of AI transceiver/receiver.

Methodology: The E2E AI-transceiver is trained on 3GPP channel models with parameters as given in Table 4-1. The loss function is the binary cross entropy (BCE) between the transmitted and decoded bits, which is given by

LBCE=- 1Ni=1NbilogLLRi+1-bilog1-LLRi,

where  bi is the ith information bit, LLRi is the LLR of the ith decoded bit, and N is the total number of transmitted bits during training. The other parameters related to training are provided in

Key performance indicators (KPIs): The evaluation is provided in terms of the block error rate (BLER) and throughput assuming transmission of a single slot of 14 OFDM symbols over each channel realization.

Benchmarks for SIMO-OFDM UL: For SIMO-OFDM, 5G NR-like transmission with 64-QAM and low-density parity check (LDPC) coding, with 2-symbol DMRS in OFDM symbols 2 and 11 within the 14-symbol slot is assumed for all the benchmark schemes. At the receiver, the following different schemes are assumed:

      - Perfect channel estimation (CE) with LMMSE equalization (EQ) and maximum a posteriori probability (MAP) soft demapping

      - LMMSE CE with LMMSE EQ and MAP soft demapping

      - Least squares (LS) CE with LMMSE EQ and MAP soft demapping

Simulation Results for SIMO-OFDM UL:

For sub-use cases (a) and (b), Figure 4-4 and Figure 4-3 show the BLER and throughput performance respectively of both the AI-transceiver and AI-receiver for the SIMO-OFDM UL. For sub-use case (a), DMRS-less transmission is assumed, while for sub-use case (b) only 1-DMRS transmission in a slot is assumed, compared to 2-DMRS for the benchmark schemes.

Figure 4-3 Throughput vs. SNR performance for SIMO-OFDM UL.

Figure 4-4 BLER vs. SNR performance for SIMO-OFDM UL.

From Figure 4-4, we observe that the proposed AI-transceiver provides a SNR gain of 1.2 dB compared to the benchmark 2 (QAM+LMMSE-CE+LMMSE-EQ) at 10% BLER, while the AI-receiver alone with 1-DMRS can provide a gain of 0.9 dB compared to the same benchmark.

From Figure 4-3, we observe that the proposed AI transceiver provides a throughput gain of 39% compared to benchmark 2 at the SNR where benchmark 2 attains 10% BLER (3.32 dB), while the AI receiver with 1-DMRS provides a throughput gain of 28%.

Benchmarks for MIMO-OFDM UL: There are two benchmark schemes at the receiver used for comparison with the proposed AI-transceiver, assuming 64-QAM transmission for both:

- Perfect CE and maximum likelihood (ML) MIMO detection.

- LS CE followed by K-best MIMO detection.

Simulation Results for MIMO-OFDM UL:

For sub-use case (a), Figure 4-6 Error! Reference source not found. and Figure 4-5 show the BLER and throughput performance respectively of the AI-transceiver for the MIMO-OFDM UL.

From Figure 4-6 the proposed scheme reduces the SNR required to achieve a 10% BLER by 2.2 dB compared to benchmark scheme 2. Further, from Figure 4-5, the proposed scheme achieves a throughput gain of 24% compared to benchmark 2 at the SNR of 8.9 dB where benchmark 2 achieves 10% BLER.

Figure 4-5 Throughput vs. SNR performance for MIMO-OFDM.

Figure 4-6 BLER vs. SNR performance for MIMO-OFDM.

With the encouraging initial results as shown above, we believe further studies are required on a common evaluation framework for thoroughly investigating the performance, training and inference complexity, and inference latency of an E2E AI/ML-based transceiver for the SU-MIMO UL compared to a traditional link, to evaluate its feasibility for real time operation.

Observation 2: For both SIMO and MIMO OFDM UL, an AI-transceiver with DMRS-less transmission offers significant gains in BLER and throughput over conventional transceivers that use DMRS.

Observation 3: For SIMO OFDM UL, the use of an AI-receiver at the NW with reduced-DMRS conventional QAM transmission also offers BLER and throughput gain over conventional receivers that use DMRS.

Training and Inference

Training is performed offline, with labelled data generated via link level simulation with the channel model in Table 4-1. Additionally, offline fine-tuning of the model is possible at the NW to tailor it to specific site deployments. During UL reception, inference is performed online by the gNB NN receiver, which feeds the resulting LLRs to a classical channel decoder. For MIMO-OFDM, a transformer model is used at the NW receiver, while for SIMO-OFDM the architecture consists of CNN and Residual Network (ResNet) layers enhanced Attention Features obtained from an SNR Estimator NN.

For analyzing the inference complexity of the AI-transceiver, we consider floating-point operations (FLOPs) and assume that one addition, subtraction, multiplication, or division of a real number constitutes 1 FLOP. We ignore the computational complexity at the transmitter because during inference it is only required to map the coded bits to symbols, which does not require significant computation.  For the NN-receiver in the AI-Transceiver for MIMO-OFDM, the inference complexity for a 2T4R system with 6 PRBs and one slot is 10,856 M FLOPs with 3.2 M parameters, while that for the traditional receiver in benchmark 2 is 146 M flops. Further, the variation of the number of FLOPs for different MIMO configurations and a fixed modulation order of 8 is shown in Figure 4-8. Similarly, the variation of complexity with the modulation order for an 8T32R system is shown in Figure 4-7. We observe that the complexity of the AI-receiver is significantly higher than benchmark 2, especially for relatively smaller number of transmit spatial layers and modulation order. Although benchmark 2 uses a simplified exhaustive search, it still exhibits a steeper increase in complexity with increase in the number of transmitted spatial layers or the modulation order compared to the AI-receiver. The proposed AI-receiver only requires slight adjustments to some model parameters (e.g., the number of transformer layers and embedding size) with the increase in number of spatial layers or modulation order,

Figure 4-7 Complexity in number of M FLOPs vs. the number of bits per symbol for an 8T32R MIMO-OFDM system with 6 PRBs.

Figure 4-8 Complexity in number of M FLOPS vs. the number of transmit streams with 8 bits per symbol for a MIMO-OFDM system with 6 PRBs .

which results in a milder complexity scaling.

Observation 4: The complexity of the AI-receiver is significantly higher than the conventional receiver, although the gap reduces with the increase in number of transmitted spatial layers or modulation order. Therefore, it is challenging to implement an AI-receiver at the UE for DL operation. On the other hand, it could have a more favourable performance-complexity trade-off when implemented in the NW for UL operation.

Proposal 2: Deprioritize study of AI-receiver at the UE-side for DL reception due to the high inference complexity which would translate to high implementation complexity at the UE. Any 6G study related to AI-receivers should be restricted to inference at the NW-side for UL transmission only.

Collaboration/Interaction between UE and NW

According to the collaboration levels defined in the 5G AI/ML study [2], both sub-use cases entail a collaboration Level y, i.e., signaling-based collaboration without model transfer. The following interactions between UE and NW are anticipated:

      - Sub-use case (a): for the AI-transceiver, after offline training of the model, the NW signals the transmit constellation(s) to be used by UE in the form of complex symbol set(s) and the bits-to-symbol mapping(s) to be used.

      - Sub-use cases (a) and (b): The NW would signal to the UE any changes in the DMRS configuration (e.g., legacy to DMRS-less).

      - Sub-use case (a): During performance monitoring for the AI-transceiver, the NW could indicate to the UE to use the classical QAM constellation for a certain number of slots, for performance comparison with the learned constellation.

Potential Specification Impact

Both the sub-use cases (a) and (b) require a data collection framework for offline training of the AI-transceiver/AI-receiver model.

Further, for both sub-use cases (a) and (b) a DMRS configuration update needs to be sent whenever a DMRS-less transmission is activated by the NW in the uplink.

For the AI-transceiver in sub-use case (a), signaling is required for conveying the trained constellation to the UE by the NW and for monitoring the performance of the AI-transceiver.

Proposal 3: Study the feasibility of an AI-transceiver and/or AI-receiver at the NW-side to enable DMRS-less communication in the UL, for SIMO OFDM, with the following details. The study should include a suitable framework for evaluating its performance, training and inference complexity, and inference latency compared to a reference transceiver/receiver architecture that does not use AI.

| Use case | Sub-use case (a): UL DMRS Overhead Reduction with AI-Transceiver | Sub-use case (b): UL DMRS Overhead Reduction with AI-Receiver |
| --- | --- | --- |
| Model input | Uncoded transmit data bits | Received complex symbols in frequency domain |
| Model output | LLRs of transmit bits to be fed into channel decoder | LLRs of transmit bits to be fed into channel decoder |
| Label | Transmit bit labels (0 or 1) | Transmit bit labels (0 or 1) |
| Training types | Offline training | Offline training |
| KPI | BLER, throughput | BLER, throughput |
| Benchmark | SIMO-OFDM UL: LMMSE CE with LMMSE EQ and MAP soft demapping MIMO-OFDM UL: LS CE and K-best MIMO detection | SIMO-OFDM UL: LMMSE CE with LMMSE EQ and MAP soft demapping |
| Preliminary result | SIMO-OFDM UL: - BLER: 1.2 dB gain in SNR at 10% BLER - Throughput: 39% gain MIMO-OFDM UL: - BLER: 2.2 dB gain in SNR at 10% BLER - Throughput: 24% gain | SIMO-OFDM UL: - BLER: 0.9 dB gain in SNR at 10% BLER - Throughput: 28% gain |
| Model location for inference | NW (UL only) | NW (UL only) |
| Collaboration/interaction between UE and NW | - NW signals the transmit constellation(s) to be used by UE. - NW indicates to the UE to use classical QAM transmission for a certain number of slots for performance monitoring. - NW indicates to the UE any changes in the DMRS configuration. | NW indicates to the UE any changes in the DMRS configuration. |
| Potential specification impact | - Data collection framework for offline training - Signalling for indicating transmit constellation(s) to be used by UE, performance monitoring, and change(s) in DMRS configuration | - Data collection framework for offline training. - Signalling for indicating change(s) in DMRS configuration |
| Model backbone | SIMO-OFDM: CNN+ResNet+Attention Features, MIMO-OFDM: Transformer | SIMO-OFDM: CNN+ResNet+Attention Features |
| Model complexity | SIMO-OFDM (1T8R, 6 PRBs): 128 M FLOPs MIMO-OFDM (2T4R, 6 PRBs): 10,856 M FLOPs | SIMO-OFDM (1T8R, 6 PRBs): 128 M FLOPs |

## CSI Inference in Spatial and/or Frequency Domain

In this section we highlight the details of the following sub-use cases for CSI overhead reduction over antenna ports and/or sub-channels via AI-based CSI inference/prediction in the spatial and/or frequency domain.

Definition

- CSI overhead reduction across antenna ports and/or sub-channels via AI-based CSI inference in the spatial and/or frequency domain. The general processing flow is shown in Figure 4-10.

Figure 4-9 AI/ML based joint CSI-RS pattern and channel inference.

Figure 4-10 AI/ML based channel inference at UE side.

AI-based joint CSI-RS design and CSI inference in spatial and/or frequency domain, as shown in Figure 4-9.

Figure 4-11 Example of spatial channel extrapolation with reduced CSI-RS antenna ports.

One example of spatial domain CSI overhead reduction is shown in Figure 4-11. It is possible to reconstruct the full spatial channel at the UE side with fewer spatial CSI-RS from the NW based on an AI/ML model. Conventional methods e.g., based on compressive sensing are another option. Further evaluation is need for this direction.

Model input:

- Estimated channel with reduced RS ports/ sub-channels.

- Channel for the whole antenna panel/ sub-band.

Model output:

- Estimated channel for the whole antenna panel/ sub-band.

- Spatial/frequency domain CSI-RS pattern and estimated channel based on the spatial/frequency domain CSI-RS pattern.

Training and Inference

The training can be done offline for both sub-use cases (a) and (b). Inference for both sub-use cases (a) and (b) will be at the UE, provided the possible CSI-RS-patterns are fixed after training in sub-use case (b). However, if the CSI-RS-pattern is dynamically inferred, inference of the CSI-RS pattern would need to be performed at the NW-side, while inference for the CSI is performed at the UE-side.

Collaboration/Interaction between UE and NW

The interaction is required mainly for the NW to indicate to the UE about the spatial and/or frequency domain CSI-RS pattern/location, and this falls under collaboration level y as per the classification in the 5G AI/ML study [2].

Potential Specification Impact

The specification impact is mainly in defining the spatial and/or frequency domain CSI-RS pattern, the signaling required for indicating the CSI-RS pattern/configuration and the data collection and monitoring aspects.

Proposal 4: Study the feasibility of AI/ML-based channel inference for CSI-RS overhead reduction in spatial and/or frequency domains including the following sub-use cases and related details.

| Use case | Sub-use case (a): AI-based CSI inference in the spatial and/or frequency domain with given sparse CSI-RS pattern. | Sub-use case (b): AI-based joint CSI-RS design and CSI inference in spatial and/or frequency domain |
| --- | --- | --- |
| Model input | Estimated channel with reduced RS ports/ sub-channels. | Channel for the whole antenna panel/sub-band. |
| Model output | Estimated channel for the whole antenna panel/ sub-band. | Spatial/frequency domain CSI-RS pattern and estimated channel based on the spatial/frequency domain CSI-RS pattern. |
| Training types | Offline training | Offline training |
| KPI | Intermediate: SGCS/NMSE System level: Throughput | Intermediate: SGCS/NMSE System level: Throughput |
| Model location for inference | CSI inference at UE | RS generation at NW (could be fixed after training), CSI inference at UE |
| Collaboration/interaction between UE and NW | Signaling to indicate CSI-RS pattern/configuration | Signaling to indicate CSI-RS pattern/configuration |
| Potential specification impact | - Defining spatial and/or frequency domain CSI-RS pattern - Signaling required for indicating the CSI-RS pattern/configuration. - Data collection and monitoring aspects | • Defining spatial and/or frequency domain CSI-RS pattern • Signaling required for indicating the CSI-RS pattern/configuration. • Data collection and monitoring aspects |

## CSI Prediction in Time Domain

To reduce the overhead of reference signal transmission in the downlink, one additional dimension to explore is the time domain. In dynamic environments with high UE mobility, sending frequent CSI-RS with short periodicity causes excessive overhead given the density of reference signals in time, frequency, and spatial domains. While the previous section aims at dealing with this issue in spatial and frequency domain, here we describe a complementary solution at time domain.

Our objective is to increase the periodicity of CSI-RS transmission without sacrificing the accuracy of PMI used for downlink transmission much. In our approach, the AI/ML model observes the historical CSI obtained from CSI-RS transmissions with long periodicity, and predict future CSI at much finer time granularity, which can be as small as every slot. In this sense, while overhead of CSI-RS transmission is significantly reduced, the finer time resolution at prediction horizon mitigates the aging effect.

It may be argued that 5G AI/ML CSI prediction is also serving the same purpose; however, 5G AI CSI prediction uses the same time resolution in observation and prediction horizon. Therefore, with increasing periodicity of CSI-RS transmissions as observations to AI/ML model results longer intervals between predicted instances. Assuming unchanged CSI between predicted instances inevitably causes significant error. As such, benefit of 5G AI/ML CSI compression diminishes with longer periodicity of CSI-RS transmission. Figure 4-12 depicts the key difference between our proposed approach in this contribution and what have been studied in 5G AI/ML CSI prediction.

| - 5G AI/ML CSI Prediction | - Temporal-domain CSI Prediction in this contribution |
| --- | --- |

Figure 4-12 Difference between approach proposed by this contribution and 5G AI/ML CSI Prediction

Definition

As shown in Figure 4-13, the past CSI samples obtained from CSI-RS transmissions during the observation window serves as the input sequence to the AI/ML model to learn the evolution of CSI at long time steps, and based on these observations, the AI/ML model predicts the future CSI instances at the shorter time resolutions/steps (which can be as low as every slots, or once per multiple slots).  The AI/ML model can reside either at the NW and work based on the reported CSI from UE or reside at the UE and work based on the UE’s measurements to perform prediction of future CSI instances. However, like 5G AI CSI prediction, a UE-side model is preferred as the NW may not have access to the CSI in the raw channel form and the UE’s historical reports may carry different number of layers per observed rank.

Model input: Historical CSI samples observed from CSI-RS measurements within observation window.

Model output: Future CSI instances at time granularity much lower than CSI-RS periodicity (e.g., every slot)

Figure 4-13 Illustration of timing and I/O of AI/ML model for temporal domain CSI prediction

Evaluation

We have evaluated the proposed scheme in a scenario with 32 Tx port at BS, 1 Rx, UE with 3m/s speed, UMa channel model, SCS of 15kHz, 52 PRB (13 SB with 4 PRB per SB), and CSI-RS periodicity of 10ms. The AI/ML model observes 5 past CSI obtained from CSI-RS and predicts CSI for next 9 slots each of which has 1ms duration. We have compared AI performance with sample and hold as a benchmark used during Rel 18 &19 studies. As shown in Figure 4-14, AI/ML-based prediction significantly outperforms non-AI baseline, especially at instances which are far in prediction horizon. On average over 9 predicted slots, AI/ML-based results 7.8 dB (138%) better than non-AI baseline in terms of NMSE. Also, the performance difference can reach up to 10.62 dB in terms of NMSE.

Figure 4-14 Evaluation of temporal-domain AI/ML CSI prediction compared to non-AI benchmark.

Training and Inference

The training can be carried out offline with the model input and output indicated above. For inference, the model is either located at the UE-side or NW side. If AI/ML model is located at UE side (which is the preferred case) it predicts the CSI/PMI for desired slots and reports them to NW. To address the overhead issue of reporting CSI/PMI for multiple predicted instances, a practical consideration is to avoid reporting per every slot. Since the channel evolution within few slots are smoother compared to periodicity of CSI-RS, we can rely on an interpolation technique at the NW side at the slots for which there is no CSI/PMI report from UE, and NW intends to use those slots for data transmission in the downlink. In this sense, the AI-based CSI prediction at the UE can provide few reliable anchor instances for the NW to enable it to use interpolation techniques even for long periodicity of CSI-RS. It should be noted that this overhead issue does not exist if the AI/ML model reside at the NW side.

Collaboration/Interaction between UE and NW

Interaction between the UE and NW is required for the following:

- To send predicted CSI/PMIs from the UE to the NW.

- To initiate data collection, the UE could signal an indication to the NW.

Since the training is conducted offline and the inference is at the UE-side, the only collaboration involved is signaling as outlined above, and therefore would be classified as level y, following the 5G AI/ML study.

Potential Specification Impact

New signaling would be required to meet the signaling requirements outlined in Section 4.3.4. Further specification changes may be required for suitable ground truth data collection pertaining to the CSI corresponding to prediction window with desired time resolution.

Proposal 5: Study temporal-domain AI/ML CSI prediction with different time resolution at prediction and observation window to reduce reference signaling overhead.

| Use case | Time domain CSI prediction combining CSI-RS and DMRS measurements |
| --- | --- |
| Model input | Past CSI samples obtained from CSI-RS measurements |
| Model output | CSI samples corresponding to future slots |
| Label | CSI samples corresponding to future slots (needs enhancement in the spec for fine time resolution) |
| Training types | Offline |
| KPI | NMSE, SGCS, Throughput |
| Benchmark | Sample and hold |
| Model location for inference | Either NW or UE |
| Collaboration/interaction between UE and NW | UE to initiate data collection and report prediction if AI/ML model resides at UE |
| Potential specification impact | Signaling for data collection and inference |

## CSI Feedback/Compression

A potential approach for CSI feedback in 6G is to use explicit channel feedback (or, direct channel feedback), which provides more comprehensive channel information to the NW for further processing instead of codebook-based CSI feedback. Essentially, the aim is to provide via feedback, high-resolution channel information just like what may be obtained via UL SRS-based channel acquisition. Although classical compression methods can be applied for direct channel feedback, AI/ML models have the capability to better capture complicated channel features of dynamic environments. Models can directly compress the channel into a latent space for feedback and recovery and have shown superior compression ratios with maintained accuracy in prior literature [7]. To apply AI/ML models in real systems, further comprehensive studies are needed to consider, including but not limited to, training and inference complexity, inference latency, and model generalization to unseen scenarios.

Impact of Latent Message Errors on 5G NR CSI Reconstruction Performance

In the 5G NR CSI feedback framework as shown in Figure 4-15, the errors in the latent message fed back from the UE to the NW are ignored. However, these can have a significant impact on the CSI reconstruction performance, as shown in Figure 4-16, where 20,000 CSI data samples are used for testing by uniformly randomly selecting the positions of the error bits in the latent message. The performance loss becomes more tolerable with larger overhead (latent message size).

Figure 4-15 Illustration of inference stage in 5G NR CSI feedback use case.

Figure 4-16 5G NR CSI reconstruction performance with bit errors in the latent message.

The reconstruction performance in the presence of errors in the latent message can be improved by training a robust model which is exposed to such bit errors in the training stage, as shown in Figure 4-17.

Figure 4-17 Training a robust model for CSI compression.

The performance of the robust model in Figure 4-17 is then evaluated and compared with the traditional model training, with the results shown in Figure 4-18. The performance can be significantly improved by training robust models, especially for smaller latent message sizes.

Figure 4-18 Performance comparison of robust model with traditional model for CSI feedback.

Observation 5: Errors in the latent message can cause performance degradation in the 5G NR CSI feedback and reconstruction scheme. These can be mitigated by training robust models.

In RAN1#122 [3] several companies have proposed joint source channel coding (and modulation) (JSCC(M)) for CSI feedback. In our view, the impact of the errors in the latent message for CSI feedback should be studied in detail first, before studying more sophisticated schemes like JSCC(M), which would have a huge impact on the UE modem architecture.

Proposal 6: RAN1 should study the impact of bit errors in the latent message for CSI feedback. Sophisticated schemes like JSCCM for CSI feedback have large impact on UE modem architecture and should be considered only if other techniques to handle errors in the latent message, e.g., by training robust models, are not found to be sufficient.

Study of CSI Compression Based on Unified Architecture

As outlined in Section 2.1, the unified architecture can be used for CSI compression. The compression can be based on either (a) the raw channel or (b) the precoder, which are the sub-use cases.

Definition

Model input:

- Estimated channel based on CSI-RS

- Estimated precoder based on CSI-RS

Model output:

- Reconstructed channel

- Reconstructed precoder

Figure 4-19 CSI compression with different compression ratios based on the unified architecture.

CSI compression head 2

App-2: CSI Compression CR=16

Channel information

(32x26 real matrix)

Backbone model

(CSI Encoder CR=8)

Latent space

CSI compression head 3

App-2: CSI Compression CR=32

CSI compression head 1

App-2: CSI Compression CR=8

Evaluation

For sub-use case (a), we present evaluations using the unified architecture in Section 2.1. The model structure for the CSI compression part is shown in Figure 4-19. With the common backbone, different CSI compression head targeting different compression ratio can be considered. E.g., the common backbone provides initial feature extraction with lower compression ration so that the latent space can keep as much information as possible for different downstream applications, i.e., different compression ratios.

The CSI dataset is from DICHASUS [4], where each data sample consists of the CSI of the shape [4 antenna arrays × 8 antennas × 13 complex delay taps]. 75,586 CSI samples are used with a 90%/10% training/validation split, and the CSI compression performance is evaluated with 8399 data samples.

The evaluation results are shown in Table 4-2 and compared with a scheme in the literature [5] in terms of the NMSE. It can be observed that the scheme based on the proposed architecture performs better than the benchmark.

| Compression ratio | TransNet [5] | with CR=8 backbone |
| --- | --- | --- |
| 8 | 0.0676 / -11.7021dB | 0.0457 |
| 16 | 0.1228/-9.10913dB | 0.084815 |
| 32 | 0.2185/-6.6061dB | 0.1498 |

Table 4-2 Evaluation results for CSI compression based on unified architecture.

Training and Inference

The training can be done offline. If autoencoder type two-side model is used, for both (a) and (b) inference for the encoder will be performed at the UE-side while inference for the reconstruction is performed at the NW-side.

Collaboration/Interaction Between UE and NW

As a two-side AI/ML use case, UE and NW naturally need to collaborate during data collection, training, inference, and monitoring. While the detailed signaling and information exchange between UE and NW may be different from 5G CSI compression, the same principals can be re-used:

- Data Collection: For the NW-side data collection, as UE is the entity that measures CSI, NW should rely on the UE to report the data required for training the NW’s respective part of AI/ML model.  For UE-side data collection, UE may request to configure the reference signals for CSI measurement without any need to report.

- Training: To ensure inter-operability of UE’s encoder and NW’s decoder, UE and NW can leverage the existing inter-vendor collaboration schemes defined for 5G AI CSI compression, joint training, or new techniques. It guarantees the intermediate representation of CSI, i.e., CSI feedback generated by UE’s encoder is interpretable for NW’s decoder.

- Inference: To complete the inference cycle, UE needs to compress CSI using its encoder and send it to NW; and NW does the decompression using its corresponding decoder.

- Monitoring: Since the UE is the data generation point and NW is KPI measurement point, to identify whether encoder-decoder pair is functioning properly, NW and UE need to collaborate on providing sufficient information to each other to enable a proper monitoring mechanism.

Potential Specification Impact

The potential specification impact involves the framework for data collection, joint training of the two-sided model, and monitoring mechanisms.

Proposal 7: Study the feasibility of AI/ML-based unified downlink CSI feedback framework, including but not limited to, its performance, training and inference complexity, inference latency, model generalization issues.

| Use case | Sub-use case (a): Unified AI-based CSI feedback of channel | Sub-use case (a): Unified AI-based CSI feedback of precoder |
| --- | --- | --- |
| Model input | Estimated channel based on CSI-RS | Estimated precoder based on CSI-RS |
| Model output | Reconstructed channel | Reconstructed precoder |
| Training types | Offline | Offline |
| KPI | Intermediate KPI: NMSE System level KPI: throughput | Intermediate KPI: SGCS System level KPI: throughput |
| Model location for inference | Two-sided: compression at UE and reconstruction at NW | Two-sided: compression at UE and reconstruction at NW |
| Collaboration/interaction between UE and NW | Data collection, training, inference, monitoring | Data collection, training, inference, monitoring |
| Potential specification impact | Data collection, training, inference, monitoring | Data collection, training, inference, monitoring |

# Conclusion

In the following, we highlight the observations and proposals within this contribution:

## AI/ML Framework

Observation 1: A unified model architecture comprising a common backbone model and multiple head models corresponding to multiple use cases is studied for the use cases of CSI compression and positioning. It is observed to have better or at least comparable performance to using dedicated models for each use case. It also enables more efficient LCM by allowing partial (re-)training and less frequent updates to the common backbone model.

Proposal 1: Study a general/unified AI/ML framework for 6GR Air Interface considering the following aspects:

- Extensibility for emerging applications

- Leveraging synergy between use cases

- Facilitating both offline and online model (re-)training

- Avoiding unreasonable signalling overhead for monitoring and training/inference (when applicable)

- Facilitating separate/partial training of AI/ML models.

## Use Case: UL DMRS Overhead Reduction with AI-transceiver/AI-receiver

Observation 2: For both SIMO and MIMO OFDM UL, an AI-transceiver with DMRS-less transmission offers significant gains in BLER and throughput over conventional transceivers that use DMRS.

Observation 3: For SIMO OFDM UL, the use of an AI-receiver at the NW with reduced-DMRS conventional QAM transmission also offers BLER and throughput gain over conventional receivers that use DMRS.

Observation 9: The complexity of the AI-receiver is significantly higher than the conventional receiver, although the gap reduces with the increase in number of system dimensions. Therefore, it is challenging to implement an AI-receiver at the UE for DL operation. On the other hand, it could have a more favourable performance-complexity trade-off when implemented in the NW for UL operation.

Proposal 2: Deprioritize study of AI-receiver at the UE-side for DL reception due to the high inference complexity which would translate to high implementation complexity at the UE. Any 6G study related to AI-receivers should be restricted to inference at the NW-side for UL transmission only.

Proposal 3: Study the feasibility of an AI-transceiver and/or AI-receiver at the NW-side to enable DMRS-less communication in the UL, for SIMO OFDM, with the following details. The study should include a suitable framework for evaluating its performance, training and inference complexity, and inference latency compared to a reference transceiver/receiver architecture that does not use AI.

| Use case | Sub-use case (a): UL DMRS Overhead Reduction with AI-Transceiver | Sub-use case (b): UL DMRS Overhead Reduction with AI-Receiver |
| --- | --- | --- |
| Model input | Uncoded transmit data bits | Received complex symbols in frequency domain |
| Model output | LLRs of transmit bits to be fed into channel decoder | LLRs of transmit bits to be fed into channel decoder |
| Label | Transmit bit labels (0 or 1) | Transmit bit labels (0 or 1) |
| Training types | Offline training | Offline training |
| KPI | BLER, throughput | BLER, throughput |
| Benchmark | SIMO-OFDM UL: LMMSE CE with LMMSE EQ and MAP soft demapping MIMO-OFDM UL: LS CE and K-best MIMO detection | SIMO-OFDM UL: LMMSE CE with LMMSE EQ and MAP soft demapping |
| Preliminary result | SIMO-OFDM UL: - BLER: 1.2 dB gain in SNR at 10% BLER - Throughput: 39% gain MIMO-OFDM UL: - BLER: 2.2 dB gain in SNR at 10% BLER - Throughput: 24% gain | SIMO-OFDM UL: - BLER: 0.9 dB gain in SNR at 10% BLER - Throughput: 28% gain |
| Model location for inference | NW (UL only) | NW (UL only) |
| Collaboration/interaction between UE and NW | - NW signals the transmit constellation(s) to be used by UE. - NW indicates to the UE to use classical QAM transmission for a certain number of slots for performance monitoring. - NW indicates to the UE any changes in the DMRS configuration. | NW indicates to the UE any changes in the DMRS configuration. |
| Potential specification impact | - Data collection framework for offline training - Signalling for indicating transmit constellation(s) to be used by UE, performance monitoring, and change(s) in DMRS configuration | - Data collection framework for offline training. - Signalling for indicating change(s) in DMRS configuration |
| Model backbone | SIMO-OFDM: CNN+ResNet+Attention Features, MIMO-OFDM: Transformer | SIMO-OFDM: CNN+ResNet+Attention Features |
| Model complexity | SIMO-OFDM (1T8R, 6 PRBs): 128 M FLOPs MIMO-OFDM (2T4R, 6 PRBs): 10,856 M FLOPs | SIMO-OFDM (1T8R, 6 PRBs): 128 M FLOPs |

## Use Case: CSI Inference in Spatial and/or Frequency Domain

Proposal 4: Study the feasibility of AI/ML-based channel inference for CSI-RS overhead reduction in spatial and/or frequency domains including the following sub-use cases and related details.

| Use case | Sub-use case (a): AI-based CSI inference in the spatial and/or frequency domain with given sparse CSI-RS pattern. | Sub-use case (b): AI-based joint CSI-RS design and CSI inference in spatial and/or frequency domain |
| --- | --- | --- |
| Model input | Estimated channel with reduced RS ports/ sub-channels. | Channel for the whole antenna panel/sub-band. |
| Model output | Estimated channel for the whole antenna panel/ sub-band. | Spatial/frequency domain CSI-RS pattern and estimated channel based on the spatial/frequency domain CSI-RS pattern. |
| Training types | Offline training | Offline training |
| KPI | Intermediate: SGCS/NMSE System level: Throughput | Intermediate: SGCS/NMSE System level: Throughput |
| Model location for inference | CSI inference at UE | RS generation at NW (could be fixed after training), CSI inference at UE |
| Collaboration/interaction between UE and NW | Signaling to indicate CSI-RS pattern/configuration | Signaling to indicate CSI-RS pattern/configuration |
| Potential specification impact | - Defining spatial and/or frequency domain CSI-RS pattern - Signaling required for indicating the CSI-RS pattern/configuration. - Data collection and monitoring aspects | • Defining spatial and/or frequency domain CSI-RS pattern • Signaling required for indicating the CSI-RS pattern/configuration. • Data collection and monitoring aspects |

## Use Case: CSI Prediction in Time Domain

Proposal 5: Study temporal-domain AI/ML CSI prediction with different time resolution at prediction and observation window to reduce reference signaling overhead.

| Use case | Time domain CSI prediction combining CSI-RS and DMRS measurements |
| --- | --- |
| Model input | Past CSI samples obtained from CSI-RS measurements |
| Model output | CSI samples corresponding to future slots |
| Label | CSI samples corresponding to future slots (needs enhancement in the spec for fine time resolution) |
| Training types | Offline |
| KPI | NMSE, SGCS, Throughput |
| Benchmark | Sample and hold |
| Model location for inference | Either NW or UE |
| Collaboration/interaction between UE and NW | UE to initiate data collection and report prediction if AI/ML model resides at UE |
| Potential specification impact | Signaling for data collection and inference |

## Use Case: CSI Feedback/Compression

Observation 5: Errors in the latent message can cause performance degradation in the 5G NR CSI feedback and reconstruction scheme. These can be mitigated by training robust models.

Proposal 6: RAN1 should study the impact of bit errors in the latent message for CSI feedback. Sophisticated schemes like JSCCM for CSI feedback have large impact on UE modem architecture and should be considered only if other techniques to handle errors in the latent message, e.g., by training robust models, are not found to be sufficient.

Proposal 7: Study the feasibility of AI/ML-based unified downlink CSI feedback framework, including but not limited to, its performance, training and inference complexity, inference latency, model generalization issues.

| Use case | Sub-use case (a): Unified AI-based CSI feedback of channel | Sub-use case (a): Unified AI-based CSI feedback of precoder |
| --- | --- | --- |
| Model input | Estimated channel based on CSI-RS | Estimated precoder based on CSI-RS |
| Model output | Reconstructed channel | Reconstructed precoder |
| Training types | Offline | Offline |
| KPI | Intermediate KPI: NMSE System level KPI: throughput | Intermediate KPI: SGCS System level KPI: throughput |
| Model location for inference | Two-sided: compression at UE and reconstruction at NW | Two-sided: compression at UE and reconstruction at NW |
| Collaboration/interaction between UE and NW | Data collection, training, inference, monitoring | Data collection, training, inference, monitoring |
| Potential specification impact | Data collection, training, inference, monitoring | Data collection, training, inference, monitoring |

# References

- 3GPP RP-251881, “New SID: Study on 6G Radio”, RAN#108, Jun 2025.

- 3GPP TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”, Jun 2025.

- 3GPP R1-2506456, “Moderator summary #2 on AI/ML for 6GR”, RAN1#122, Aug 2025.

- dichasus-cf0x Dataset: Distributed Antenna Setup in Industrial Environment, Day 1, https://dichasus.inue.uni-stuttgart.de/datasets/data/dichasus-cf0x/

- Y. Cui, A. Guo and C. Song, "TransNet: Full Attention Network for CSI Feedback in FDD Massive MIMO System," in IEEE Wireless Commun. Lett., vol. 11, no. 5, pp. 903-907, May 2022.

- Z. Zhang, J. Zhang, Y. Zhang, L. Yu and G. Liu, "AI-Based Time-, Frequency-, and Space-Domain Channel Extrapolation for 6G: Opportunities and Challenges," in IEEE Veh. Technol. Mag., vol. 18, no. 1, pp. 29-39, Mar 2023, doi: 10.1109/MVT.2023.3234169.

- J. Chen, W. J. Hillery, K. R. Mestav, C. Nuzman, I. Saniee and Y. Xing, "CSI Compression for Massive MIMO: Model-Based or Data-Driven?," in IEEE Wireless Commun., vol. 32, no. 1, pp. 22-27, Feb 2025, doi: 10.1109/MWC.010.2400078.

- Z. Wei et al., "Multiple Reference Signals Collaborative Sensing for Integrated Sensing and Communication System Towards 5G-A and 6G," in IEEE Trans. Veh. Technol., vol. 73, no. 10, pp. 15185-15199, Oct. 2024, doi: 10.1109/TVT.2024.3410352

- S. Cammerer, et al., "Trainable Communication Systems: Concepts and Prototype," in IEEE Trans. Commun., vol. 68, no. 9, pp. 5489-5503, Sep 2020, doi: 10.1109/TCOMM.2020.3002915.

- R. Wiesmayr, et al., “Design of a Standard-Compliant Real-Time Neural Receiver for 5G NR”, Sep 2024, [Online] https://arxiv.org/abs/2409.02912

- S. Cammerer, et al., “A Neural Receiver for 5G NR Multi-user MIMO”, Proc. IEEE Globecom, Dec 2023, [Online] https://arxiv.org/abs/2312.02601

- D. Korpi, et al., “DeepRx MIMO: Convolutional MIMO Detection with Learned Multiplicative Transformations”, Proc. IEEE ICC, Jun 2021, [Online] https://ieeexplore.ieee.org/abstract/document/9500518

- D. Korpi, et al., “Deep Learning-Based Pilotless Spatial Multiplexing”, Proc. IEEE Asilomar Conf. Sig. Sys. Comput., Oct 2023, [Online] https://arxiv.org/abs/2312.05158

- M. Honkala, D. Korpi and J. M. J. Huttunen, "DeepRx: Fully Convolutional Deep Learning Receiver," in IEEE Trans. Wireless Commun., vol. 20, no. 6, pp. 3925-3940, Jun 2021, doi: 10.1109/TWC.2021.3054520.

- 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz (Release 19)”, Jun 2025.