3GPP TSG RAN WG1 #122bis		                                               	R1-2507795

Prague, Czech, Oct 13th – 17th, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on UE features for AI/ML for NR Air Interface

Agenda Item:	9.1

Document for: 	Discussion and Decision

- Introduction

At the RAN#107 meeting, WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved with the objective as follows [1]:

Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2 - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025 - Core requirements for the above three use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. NOTE: offline training is assumed for the purpose of this project. NOTE: Coordination with SA/SA WGs of the ongoing work as it may relate to their required work.

In this contribution, we present our view on UE features for AI/ML for NR Air Interface.

- Discussion on FG for all CSI-related use cases

At RAN1 #122 meeting, the issue about the terminology of APU or CPU,2/CPU,3 was discussed. Based on the discussions, there is no need to change the CPU,2/CPU,3 in the current specification. It is better to use the aligned terminology with the specification for the UE feature discussions. Therefore, we propose changing all APUs occurring in FG 58 to the term aligned with the specification.

Proposal 1: Revise the following FGs about the usage of APU and change the APU and corresponding items to align with the term CPU,2/CPU,3.

- FG 58-0-1, FG 58-1-2, FG 58-1-4, FG 58-3-1, FG 58-3-2.

Besides the APU issue, there are two remaining issues for FG 58-0-1 after RAN1 #122 meeting.

- The first issue is about the prerequisite FG. Since this FG focuses on the AI/ML dedicated resource pools for the CSI report, the FG on CSI report framework (FG 2-35) should be the prerequisite FG.

- The second issue is about the note for UEs that do not have the dedicated processing resources. We believe that this note is necessary to ensure the UE correctly reports its capability. Therefore, it can be further clarified as shown in our proposals.

Proposal 2: Update FG 58-0-1 as shown in the following table.

| 58. NR_AIML_air | 58-0-1 | CSI report framework for UE-side inference | 1. Number of APU CPU,x pools N 2. Maximum number of APU CPU,x in each APU CPU,x pool of UE-sided inference for CSI report(s) for simultaneously in a CC 3. Maximum number of APU CPU,x in each APU CPU,x pool of UE-sided inference for CSI report(s) simultaneously across all CCs | FFS 2-35 | yes | n/a | Maximum number of APUs for UE-sided inference is unknown to the network | Per UE | N/A | N/A | N/A | Component 1 candidate values: {1,2} Component 12 candidate values: {1…8} Component 23 candidate values: {1…32} Note: Component 2 and 3 candidate values are signalled separately for each pool [A UE that does not support this FG reuses the CPU should not report non-zero occupied CPU,2 or CPU,3 values in any dependency FG.] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Discussion on FGs for AI/ML in beam management

  - NW side beam prediction

The only remaining issue for FG 58-1-1 is the prerequisite FG. Since this FG is for the NW-sided model and UEs only need to report their beam measurements based on the enhanced beam reporting, we suggest FG 2-24 as the prerequisite FG.

Proposal 3: Update FG 58-1-1 as shown in the following table.

| 58. NR_AIML_air | 58-1-1 | Increased number of reported RSs for beam management | 1. Support of reporting format for L1-RSRP measurements not including CRI/SSBRI other than one for the largest measured L1-RSRP in a reporting instance, if the number of reported L1-RSRPs is equal to the size of the measurement resource set. 2. Support of reporting format for L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB, if the number of reported L1-RSRPs is smaller than the size of the measurement resource set 3. Maximum number of M reported RSs, M>4 | FFS 2-24 | yes | n/a | Increased number of reported beams for beam management is not supported | Per UE | N/A | N/A | N/A | Component 3 candidate values: {6,8} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - UE inference result reporting

Besides the APU issues, the prerequisite FGs for FG 58-1-2 and 58-1-4 have not been decided yet. Similar to FG 58-1-1, the basic beam management framework (FG 2-24) should be one of the prerequisite FGs. One note is also added to clarify the UE behaviors further if FG 58-0-1 is not signalled based on Proposal 2.

For FG 58-1-2, we also propose the candidate values for components 7a, 7b, 12, 13, 14, and 17. For component 14, we suggest considering a limited number of candidate values to reduce the NW operation burden in managing UE resources. Besides the value 0, we propose three candidates to reflect the UE choice on small, medium, and large-scale models. The same proposals are also applied to FG 58-1-4.

Proposal 4: Update FG 58-1-2 and FG 58-1-4 as shown in the following table.

| 58. NR_AIML_air | 58-1-2 | UE-side beam prediction for BM Case1 for inference] | 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 [for inference] with UE-side model 3. Maximum number of inference report(s) configured for BM-Case1 per BWP 3a. Maximum number of inference report(s) configured for BM-Case1 across all CCs 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A 7a: Supported maximum number of resources for Set B 7b: Supported maximum number of resources for Set A 8. Supported CSI-RS resource types 9. Supported inference report types 11. Supported BM-Case 1 sub-usecase(s): {setB-subset-of-setA, setB-different-from-setA, both} 12. Supported maximum number of predicted beams in each reporting instanceFFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection 13. Supported number of occupied CPU 14. Supported number of occupied APU CPU,2/CPU,3 15. Supported value of d for the relaxation of Z3 timeline 16. Supported value of d’ for the relaxation of Z’3 timeline 17. Occupied resource pool between CPU,2 and CPU,3APU pool | FFS 2-24 | yes | n/a | UE-sided beam prediction for BM Case 1 [for inference] is not supported | Per UE | No | No | No | Component 8 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS} Component 9 candidate values: {Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report} FFS: candidate values for components Component 7a candidate values: {4, 8, 16, 32} Component 7b candidate values: {16, 32, 64, 128} Component 12 candidate values: {4, 6, 8} Component 13 candidate values: {0, 1, 8} Component 14 candidate values: {0, 2, 4, 8} Component 17 candidate values: {CPU,2 or CPU,3} Note: UE should not report non-zero value for Component 14 if FG 58-0-1 is not signalled. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-1-4 | UE-side beam prediction for BM Case2 [for inference] | 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 for inference with UE-side model 3. Maximum number of inference report(s) configured for BM-Case2 per BWP 3a. Maximum number of inference report(s) configured for BM-Case2 across all CCs 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A 7a: Supported maximum number of resources for Set B 7b: Supported maximum number of resources for Set A 8. Supported CSI-RS resource types 9. Supported inference report types 11. Supported maximum number of predicted beams in each predicted time instance 12. Supported maximum number of predicted time instances 13. Supported maximum total number of reported predicted beams for predicted time instances in one report 14. Supported combinations of supported value(s) of valid time duration for each predicted time instance and number of predicted beams for each value of valid time duration 21. supported number of occupied CPU 22. Supported number of occupied APU CPU,2/CPU,3 23. supported value of d for the relaxation of Z3 timeline 24. supported value of d’ for the relaxation of Z’3 timeline 25. Occupied resource pool between CPU,2 and CPU,3APU pool | FFS 2-24 | yes | n/a | UE-side beam prediction for BM-Case2 [for inference] is not supported | Per UE | No | No | No | Component 8 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS} Component 9 candidate values: {Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report} FFS: candidate values for components Component 7a candidate values: {4, 8, 16, 32} Component 7b candidate values: {16, 32, 64, 128} Component 11 candidate values: {16, 32, 64, 128} Component 12 candidate values: {1,2,4} Component 13 candidate values: {6, 8} Component 21 candidate values: {0, 1, 8} Component 22 candidate values: {0, 2, 4, 8} Component 25 candidate values: {CPU,2 or CPU,3} Note: UE should not report non-zero value for Component 22 if FG 58-0-1 is not signalled. | Optional with capability signalling |

  - Performance monitoring

FG 58-1-6 is defined for the performance monitoring for BM-Case 1 and BM-Case 2. Therefore, its prerequisite FG should be at least one of FG 58-1-2 or FG 58-1-4. The granularity of the performance monitoring should be the same as the other ones for the same use case, which is per UE. We also propose the candidate values for the components.

Proposal 5: Update FG 58-1-6 as shown in the following table.

| 58. NR_AIML_air | 58-1-6 | Performance monitoring for UE-sided model | 1. Support of performance monitoring with RS-PAI of AI/ML model for beam prediction. 2. Maximum total number of the configured CSI-RS resources for monitoring RS resource set 3. Maximum number of monitoring report(s) configured per BWP 4. Maximum number of monitoring report(s) configured across all CCs 5. Maximum number of monitoring occasions for RS-PAI calculation | FFS Either one of 58-1-2 or 58-1-4 | yes | n/a | Performance monitoring for UE-sided model is not supported | FFS Per UE | FFS No | FFS No | FFS No | FFS: component candidate values Component 2 candidate values: {16, 32, 64, 128} Component 3 candidate values: {1, 2} Component 4 candidate values: {1,2,4,8} Component 5 candidate values: {1,2,3,4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - Data collection

The only one remaining issue for FG 58-1-7 is the prerequisite FG. Because the data collection is based on the legacy CSI framework and it does not require UE to support any AI/ML feature, the prerequisite FG should be the legacy CSI framework, i.e., FG 2-35 only.

Proposal 6: Update FG 58-1-7 as shown in the following table.

| 58. NR_AIML_air | 58-1-7 | Data collection for UE-side beam prediction | 1. Support of data collection for UE-side beam prediction 2. Support of SS/PBCH block and 1-port CSI-RS based RSRP measurements for measurement RS resource sets (Set B and Set A) for data collection 3. Supported sub-use cases 6: Supported maximum number of resources for Set B 7: Supported maximum number of resources for Set A 8. Support of SSB as RS type for Set B 9. Support of CSI-RS as RS type for Set B 10. Support of SSB as RS type for Set A 11. Support of CSI-RS as RS type for Set A | FFS 2-35 | yes | n/a | Data collection for UE-side beam prediction is not supported | Per UE | N/A | N/A | N/A | Component 3 candidate values: {‘Set B equal to Set A’, ‘Set B subset of Set A’, ’Set B not a subset of Set A’} Component 6 candidate values: {4, 8, 16, 32, 64} Component 7 candidate values: {8, 16, 32, 64} Note: it is up to RAN2 whether this FG is merged into data collection FG defined by RAN2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Discussion on FGs for AI/ML for positioning accuracy enhancement

  - FG58-2-2: Support reception of AD for UE-based positioning Case 1

Regarding the support of reception of AD for UE-based positioning Case 1, the note can be removed because RAN1 does not have any agreements for specific capability signalling for AD components.

Proposal 7: FG 58-2-2 should be modified as follows.

| 58-2-2 | Support reception of AD for UE-based positioning Case 1 | Indicates support of reception of AD for UE-based positioning Case 1 | 58-2-1 | N/A | N/A | Reception of AD for UE-based positioning Case 1 is not supported | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. Note: For specific ADs, UE indicate supported AD that can be received as in [FFS: FGs] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - FG58-2-3/3a/3b: DL PRS Resources for UE-based positioning Case 1 (on a band/BC)

In Rel-19 discussion, there is no agreement to enhance DL PRS Resources for AI/ML based positioning. Therefore, we think it is straightforward to follow the elements of existing FGs 13-2/3/4 series.  We also think that the relationship between FG 58-2-4 and 58-2-3 series is similar to the relationship between FG13-1 and FG 13-3 series of basic capabilities for legacy DL-TDOA of L1 UE features. Based on them, prerequisite feature group is FG 58-2-4.

Proposal 8: FG 58-2-3, 58-2-3a, and 58-2-3b should follow the elements of existing FGs 13-2/3/4 series. The FGs should be modified as follows.

| 58-2-3 | DL PRS Resources for UE-based positioning Case 1 | 1. Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE 2. Max number of TRPs across all positioning frequency layers per UE 3. Max number of positioning frequency layers UE supports | [13-1] 58-2-4 | N/A | N/A |  | Per UE | n/a | n/a | n/a | Component 1 candidate values: {1, 2} Component 2 candidate values: {4, 6, 12, 16, 24, 32, 64, 128, 256} Component 3 candidate values: {1, 2, 3, 4} Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58-2-3a | DL PRS Resources for UE-based positioning Case 1 on a band | 1. Max number of DL PRS Resources per DL PRS Resource Set 2. Max number of DL PRS Resources per positioning frequency layer | [13-1] 58-2-4 | N/A | N/A |  | Per Band | n/a | n/a | n/a | Component 1 candidate values: {1, 2, 4, 8, 16, 32, 64} Component 2 candidate values: {6, 24, 32, 64, 96, 128, 256, 512, 1024} Note: For component 1, the values 16, 32, 64 are only applicable to FR2 bands Note: For component 2, the value 6 is only applicable to FR1 bands Need for location server to know if the feature is supported. Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |
| 58-2-3b | DL PRS Resources for UE-based positioning Case 1 on a band combination | 1. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1-only 2. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2-only 3. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation 4. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2 in FR1/FR2 mixed operation | [13-1] 58-2-4 | N/A | N/A |  | Per BC | n/a | n/a | n/a | Component 1 candidate values: {6, 24, 64, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR1 only BC Component 2 candidate values: {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR2 only BC Component 3 candidate values: {6, 24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands Component 4 candidate values: {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands] Need for location server to know if the feature is supported. Note: the reported value is the total number across all bands in the corresponding BC Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |

  - FG58-2-4: DL PRS Processing Capability for UE-based positioning Case 1

Regarding DL PRS Processing Capability for UE-based positioning Case 1, we think the last note can be removed. At the last meeting, it was agreed that the prerequisite feature group for FG 58-2-1 is only FG 58-2-4, which means UE always reports FG 58-2-4 when UE can use AI/ML based positioning for Case 1. Therefore, FG 58-2-4 can be always referred to and the note can be removed.

Proposal 9: FG 58-2-4 should be modified as follows.

| 58-2-4 | DL PRS Processing Capability for UE-based positioning Case 1 | 1. Maximum DL PRS bandwidth in MHz, which is supported and reported by UE. 2. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 3. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. 4. Max number of DL PRS resources that UE can process in a slot under it Note: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) of no more than 30%. |  | No | N/A |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Component 1 candidate values: -FR1 bands values {5, 10, 20, 40, 50, 80, 100} -FR2 bands values {50, 100, 200, 400} Component 3 candidate values: -T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms -N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Component 4 candidate values: -FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz -FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Notes for component 3: a. UE reports one combination of (N, T) values per band, where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth (B) in MHz supported by UE b. UE is not expected to support DL PRS bandwidth that exceeds the reported DL PRS bandwidth value c. UE DL PRS processing capability is defined for a single positioning frequency layer. UE capability for simultaneous DL PRS processing across positioning frequency layers is not supported in Rel.16 (i.e. for a UE supporting multiple positioning frequency layers, a UE is expected to process one frequency layer at a time) d. UE DL PRS processing capability is agnostic to DL PRS comb factor configuration e. The reporting of (N, T) values for maximum BW in MHz is not dependent on SCS Note: if the UE does not indicate this capability for a band or band combination, the UE does not support PRS processing in this band or band combination. Note: If UE does not provide [this FG] but the UE supports Case 1, FG 13-1 indicates the DL PRS processing capabilities common across all positioning methods including UE-based positioning Case 1.] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - FG58-2-5: Support of SSB from neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1

For FG58-2-5, following the legacy capability “Support of SSB from neighbour cell as QCL source of a DL PRS,” prerequisite feature group should be DL PRS Processing Capability. Following this principle, prerequisite feature group should be FG 58-2-4. FG 13-1 is not necessary because FG 58-2-4 can be always provided when UE can use UE-based positioning Case 1.

Proposal 10: FG 58-2-5 should be modified as follows.

| 58-2-5 | Support of SSB from neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 | 1. Support of SSB from neighbour cell as QCL source of a DL PRS 2. Support of reuse SSB measurement from RRM for receiving PRS Note: Refers to Type-C for FR1 and Type-C & Type-D support for FR2 | [58-2-4; otherwise 13-1] | No | N/A | SSB from neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 is not supported | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - FG58-2-6: Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1

Same as FG 58-2-5, FG 58-2-6 follows the legacy capability “Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS,” i.e., prerequisite feature group is FG 58-2-4. The same as FG 58-2-5, FG 13-1 is not necessary.

Proposal 11: FG 58-2-6 should be modified as follows.

| 58-2-6 | Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 | 1. Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS Note 1: Refers to Type-D support for FR2 Note 2: A PRS from a PRS-only TP is treated as PRS from a non-serving cell | [58-2-4; otherwise 13-1] | No | N/A | DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 is not supported | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. DL PRSs are in the same band | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - UE feature for RRC_INACTIVE state and RRC_IDLE state for Case 1

At the last meeting, Rel-17/18 features for RRC_CONNECTED state for Case 1 were introduced (e.g., FG 58-2-15, 58-2-17) and UE features RRC_INACTIVE state and RRC_IDLE state for Case 1 were proposed. In our understanding, there is no agreement to support AI/ML based positioning for Case 1 only in RRC_CONNECTED state. Therefore, UE features for other RRC states for Case 1 should be introduced.

Proposal 12: The following UE feature group for RRC_INACTIVE state and RRC_IDLE state should be supported.

- FG 58-2-12: Support of UE-based positioning Case 1 in RRC_INACTIVE state

- FG 58-2-13: DL PRS processing capabilities in RRC inactive state for UE-based positioning Case 1

- FG 58-2-14: Support of UE-based positioning Case 1 in RRC_IDLE

- FG 58-2-15b: DL PRS processing capabilities for aggregated PRS processing of 2 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE - UE-based positioning Case 1

- FG 58-2-15c: DL PRS processing capabilities for aggregated PRS processing of 3 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE - UE-based positioning Case 1

- FG 58-2-18: PRS bandwidth aggregation in RRC_ INACTIVE UE-based positioning Case 1

- FG 58-2-19: PRS bandwidth aggregation in RRC_IDLE - UE-based positioning Case 1

| 58-2-12 | Support of UE-based positioning Case 1 in RRC_INACTIVE state | UE-based positioning Case 1 in RRC_INACTIVE state is supported | [58-2-3 and 58-2-13] | N/A | N/A | UE-based positioning Case 1 in RRC_INACTIVE state is not supported | Per Band | n/a | n/a | n/a | Support of UE-based positioning Case 1 in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58-2-13 | DL PRS processing capabilities in RRC inactive state for UE-based positioning Case 1 | 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE 3. Max number of DL PRS resources that UE can process in a slot | [58-2-4] | N/A | N/A |  | Per band | n/a | n/a | n/a | Component 1 candidate values: {Type 1, Type 2} Component 2 candidate values: T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Component 3 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Need for location server to know if the feature is supported Note: Having the PRS processing capabilities in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state Note: if UE supports same values for one or more components as in FG 27-6, then the UE can skip indicating these components in this FG and the values in corresponding FG 27-6 components indicate supported PRS processing capabilities in RRC inactive state for Case 1 | Optional with capability signalling |
| 58-2-14 | Support of UE-based positioning Case 1 in RRC_IDLE | Support of UE-based positioning Case 1 in RRC_IDLE | [ 58-2-4, 58-2-12, 58-2-13] | N/A | N/A | UE-based positioning Case 1 in RRC_IDLE is not supported | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signalling |
| 58-2-15b | DL PRS processing capabilities for aggregated PRS processing of 2 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE - UE-based positioning Case 1 | 1. Maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE 2. Maximum DL PRS bandwidth in MHz, per PFL 3. DL PRS buffering capability 4. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE. 5. Maximum number of aggregated DL PRS resources across aggregated PFLs that UE can process in a slot | [58-2-13] | N/A | N/A | DL PRS processing capabilities for aggregated PRS processing of 2 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE - UE-based positioning Case 1 is not supported | Per Band | n/a | n/a | n/a | Component 1 candidate values: a) FR1 bands: {10, 20, 40, 50, 80, 100, 160, 200} b) FR2 bands: {100, 200, 400, 800} Component 2 candidate values:a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} Note: Component 3 in [FG 58-2-15b (this FG)] follows buffering capability type reported in [FG 58-2-4] Component 4 candidate values: a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note: this value N should be equal or smaller than the value N reported by [FG 58-2-13] or this value T should be equal or larger than the value T reported by [FG 58-2-13] Component 5 candidate values: a. FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b. FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: each two linked PRS resources are counted as 1 resource Note: this value should be equal or smaller than the value reported by [FG 58-2-13] Note: if UE supports same values for one or more components as in FG 41-4-1b, then the UE can skip indicating these components in this FG and the values in corresponding FG 41-4-1b components indicate supported for aggregated PRS processing of 2 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE capabilities for Case 1 | Optional with capability signalling |
| 58-2-15c | DL PRS processing capabilities for aggregated PRS processing of 3 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE - UE-based positioning Case 1 | 1. Maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE 2. Maximum DL PRS bandwidth in MHz, per PFL 3. DL PRS buffering capability 4. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE. 5. Max number of aggregated DL PRS resources across aggregated PFLs that UE can process in a slot under it | [58-2-15b] | N/A | N/A | DL PRS processing capabilities for aggregated PRS processing of 3 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE - UE-based positioning Case 1 is not suported | Per Band | n/a | n/a | n/a | Component 1 candidate values: a) FR1 bands: {15, 20, 30, 40, 50, 60, 80, 100, 120, 140, 150, 160, 180, 200, 240, 300} b) FR2 bands: {150, 200, 300, 400, 600, 800, 1000, 1200} Component 2 candidate values: a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} Note: Component 3 in [FG 58-2-15c (this FG)] follows buffering capability type reported in [FG 58-2-4] Component 4 candidate values: a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note: this value N should be equal or smaller than the value N reported by [FG 58-2-13] or this value T should be equal or larger than the value T reported by [FG 58-2-13] Component 5 candidate values: a. FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b. FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: each three linked PRS resources are counted as 1 resource Note: this value should be equal or smaller than the value reported by [FG 58-2-13] Note: if UE supports same values for one or more components as in FG 41-4-1c, then the UE can skip indicating these components in this FG and the values in corresponding FG 41-4-1c components indicate supported aggregated PRS processing of 3 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE for Case 1 | Optional with capability signalling |
| 58-2-18 | PRS bandwidth aggregation in RRC_ INACTIVE UE-based positioning Case 1 | Support of PRS bandwidth aggregation in RRC_ INACTIVE for UE-based positioning Case 1 | [58-2-12, 58-2-15b] | N/A | N/A | PRS bandwidth aggregation in RRC_ INACTIVE for UE-based positioning Case 1 is not supported | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |
| 58-2-19 | PRS bandwidth aggregation in RRC_IDLE - UE-based positioning Case 1 | Support of PRS bandwidth aggregation in RRC_IDLE for UE-based positioning Case 1 | [58-2-14, 58-2-15b] | N/A | N/A | PRS bandwidth aggregation in RRC_IDLE for UE-based positioning Case 1 is not supported | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |

- Discussion on FGs for AI/ML for CSI prediction

  - FGs for inference result report

As proposed in Proposal 1, the APU issue should also be solved for this FG. According to the discussions of the last meeting, FG 58-0-1 will not be signalled if UE has no processing resource pool dedicated to AI/ML. Therefore, FG 58-0-1 cannot be the prerequisite of FG 58-3-1. Instead, FG 2-35 (CSI framework) can be used.

Proposal 13: Update FG 58-3-1 and FG 58-3-2 as shown in the following table.

| 58. NR_AIML_Air | 58-3-1 | CSI prediction for UE-sided inference when N4=1 | 1. Support of CSI prediction for UE-sided inference when N4=1 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 5. Support parameter combinations with L=2,4 6. Support for rank = 1,2 7. Support for the size of DD-basis, N4=1 8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 11. Scaling factor for active resource counting Kp 12. supported value of t for the relaxation of Z and Z’ timeline 13. supported number of occupied CPU 14. Supported number of occupied APU CPU,2/CPU,3 15. Occupied resource pool between CPU,2 and CPU,3APU pool | 2-35 58-0-1 | yes | n/a | CSI prediction for N4=1 for inference is not supported | Per band and Per BC | n/a | n/a | n/a | Component 3 candidate values: a. {4,8,12,16,24,32} b. {2,3,4 … 64} c. {4, …, 256} Component 14 candidate values: {0, 2, 4, 8} Component 15 candidate values: {CPU,2 or CPU,3} Note: UE should not report non-zero value for Component 14 if FG 58-0-1 is not signalled. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_Air | 58-3-2 | CSI prediction for UE-sided inference when N4>1 | 1. Support of CSI prediction for UE-sided inference when N4>1 2. Support for reporting predicted PMI with N4>1 3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR 5. Support for the size of DD-basis, N4>1 6. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting 7. Supported number of occupied CPU 8. Supported number of occupied APU CPU,2/CPU,3 9. Occupied resource pool between CPU,2 and CPU,3APU pool | 58-3-1 | yes | n/a | CSI prediction for N4>1 for inference is not supported | Per band and Per BC | N/A | N/A | N/A | Component 3 candidate values: a. {1,2,4,8} b. {4,8,12,16,24,32} c. {2,3,4 … 64} d. {4, …, 256} Component 6 candidate values: a. {1,2,4,8} b. {4,8,12,16,24,32} c. {4,8,12} d.{4, …, 256} Component 8 candidate values: {0, 2, 4, 8} Component 9 candidate values: {CPU,2 or CPU,3} Note: UE should not report non-zero value for Component 8 if FG 58-0-1 is not signalled | Optional with capability signalling |

  - FGs for data collection

Like FG 58-1-6, the prerequisite FG for data collection should be FG 2-35, which resolves the only one remaining issue of this FG.

Proposal 14: Update FG 58-3-4 as shown in the following table.

| 58. NR_AIML_Air | 58-3-4 | UE side data collection for CSI prediction | 1. Support of data collection for CSI prediction | FFS 2-35 | yes | n/a | UE side data collection for CSI prediction is not supported | Per band and Per BC | N/A | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - FGs for performance monitoring

This FG has no remaining FFS part. However, there is one maintenance agreement during RAN1 #122 that should be reflected in this FG [2].

Agreement: For CSI-PAI report, support - OCPU =X, X is reported by UE capability - For AP monitoring report, CPU occupancy starts from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the CSI-PAI report - For SP monitoring report, CPU occupancy starts from the first symbol of the latest periodic/semi-persistent CSI-RS occasion of the inference report not later than CSI reference resource of the inference report, until the last symbol of the configured PUSCH carrying the CSI-PAI report

Based on the online discussions about this agreement, the intention to allow UE to report occupied CPU by UE capability is to ensure that the UE can support both inference and performance monitoring simultaneously. The value of X can be 0, 1, or 2 based on the online discussions. According to the agreement and the corresponding discussions, we added Component 5 into this FG, and its candidate values can be 0, 1, or 2. To reflect the intention of introducing this UE capability, a note is added to restrict that the UE must report a valid O_CPU combination for inference and performance monitoring.

Proposal 15: Update FG 58-3-5 as shown in the following table.

| 58. NR_AIML_Air | 58-3-5 | Performance monitoring for CSI prediction model | 1. Supported of two performance metric SGCS 2. Support of one wideband frequency granularity SGCS per layer 3. Support of one configured time instance for N4>1 4. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 5. Supported number of occupied CPU | 58-3-1 | Yes | n/a | Performance monitoring for CSI prediction model is not supported | Per Band and Per BC | N/A | N/A | N/A | Component 4 candidate values: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} Component 5 candidate values: {0, 1, 2} Note: The summation of the value reported by Component 5 and the one reported by Component 13 of FG58-3-1 (or Component 7 of FG58-3-2) should not significant than the N_CPU UE reported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Conclusion

In this contribution, we discussed UE features for AI/ML for NR Air Interface. Based on the discussion, we made the following proposals.

FGs for all CSI-related use cases

Proposal 1: Revise the following FGs about the usage of APU and change the APU and corresponding items to align with the term CPU,2/CPU,3.

- FG 58-0-1, FG 58-1-2, FG 58-1-4, FG 58-3-1, FG 58-3-2.

Proposal 2: Update FG 58-0-1 as shown in the following table.

| 58. NR_AIML_air | 58-0-1 | CSI report framework for UE-side inference | 1. Number of APU CPU,x pools N 2. Maximum number of APU CPU,x in each APU CPU,x pool of UE-sided inference for CSI report(s) for simultaneously in a CC 3. Maximum number of APU CPU,x in each APU CPU,x pool of UE-sided inference for CSI report(s) simultaneously across all CCs | FFS 2-35 | yes | n/a | Maximum number of APUs for UE-sided inference is unknown to the network | Per UE | N/A | N/A | N/A | Component 1 candidate values: {1,2} Component 12 candidate values: {1…8} Component 23 candidate values: {1…32} Note: Component 2 and 3 candidate values are signalled separately for each pool [A UE that does not support this FG reuses the CPU should not report non-zero occupied CPU,2 or CPU,3 values in any dependency FG.] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FGs for AI/ML in beam management

Proposal 3: Update FG 58-1-1 as shown in the following table.

| 58. NR_AIML_air | 58-1-1 | Increased number of reported RSs for beam management | 1. Support of reporting format for L1-RSRP measurements not including CRI/SSBRI other than one for the largest measured L1-RSRP in a reporting instance, if the number of reported L1-RSRPs is equal to the size of the measurement resource set. 2. Support of reporting format for L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB, if the number of reported L1-RSRPs is smaller than the size of the measurement resource set 3. Maximum number of M reported RSs, M>4 | FFS 2-24 | yes | n/a | Increased number of reported beams for beam management is not supported | Per UE | N/A | N/A | N/A | Component 3 candidate values: {6,8} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4: Update FG 58-1-2 and FG 58-1-4 as shown in the following table.

| 58. NR_AIML_air | 58-1-2 | UE-side beam prediction for BM Case1 for inference] | 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 [for inference] with UE-side model 3. Maximum number of inference report(s) configured for BM-Case1 per BWP 3a. Maximum number of inference report(s) configured for BM-Case1 across all CCs 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A 7a: Supported maximum number of resources for Set B 7b: Supported maximum number of resources for Set A 8. Supported CSI-RS resource types 9. Supported inference report types 11. Supported BM-Case 1 sub-usecase(s): {setB-subset-of-setA, setB-different-from-setA, both} 12. Supported maximum number of predicted beams in each reporting instanceFFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection 13. Supported number of occupied CPU 14. Supported number of occupied APU CPU,2/CPU,3 15. Supported value of d for the relaxation of Z3 timeline 16. Supported value of d’ for the relaxation of Z’3 timeline 17. Occupied resource pool between CPU,2 and CPU,3APU pool | FFS 2-24 | yes | n/a | UE-sided beam prediction for BM Case 1 [for inference] is not supported | Per UE | No | No | No | Component 8 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS} Component 9 candidate values: {Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report} FFS: candidate values for components Component 7a candidate values: {4, 8, 16, 32} Component 7b candidate values: {16, 32, 64, 128} Component 12 candidate values: {4, 6, 8} Component 13 candidate values: {0, 1, 8} Component 14 candidate values: {0, 2, 4, 8} Component 17 candidate values: {CPU,2 or CPU,3} Note: UE should not report non-zero value for Component 14 if FG 58-0-1 is not signalled. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-1-4 | UE-side beam prediction for BM Case2 [for inference] | 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 for inference with UE-side model 3. Maximum number of inference report(s) configured for BM-Case2 per BWP 3a. Maximum number of inference report(s) configured for BM-Case2 across all CCs 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A 7a: Supported maximum number of resources for Set B 7b: Supported maximum number of resources for Set A 8. Supported CSI-RS resource types 9. Supported inference report types 11. Supported maximum number of predicted beams in each predicted time instance 12. Supported maximum number of predicted time instances 13. Supported maximum total number of reported predicted beams for predicted time instances in one report 14. Supported combinations of supported value(s) of valid time duration for each predicted time instance and number of predicted beams for each value of valid time duration 21. supported number of occupied CPU 22. Supported number of occupied APU CPU,2/CPU,3 23. supported value of d for the relaxation of Z3 timeline 24. supported value of d’ for the relaxation of Z’3 timeline 25. Occupied resource pool between CPU,2 and CPU,3APU pool | FFS 2-24 | yes | n/a | UE-side beam prediction for BM-Case2 [for inference] is not supported | Per UE | No | No | No | Component 8 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS} Component 9 candidate values: {Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report} FFS: candidate values for components Component 7a candidate values: {4, 8, 16, 32} Component 7b candidate values: {16, 32, 64, 128} Component 11 candidate values: {16, 32, 64, 128} Component 12 candidate values: {1,2,4} Component 13 candidate values: {6, 8} Component 21 candidate values: {0, 1, 8} Component 22 candidate values: {0, 2, 4, 8} Component 25 candidate values: {CPU,2 or CPU,3} Note: UE should not report non-zero value for Component 22 if FG 58-0-1 is not signalled. | Optional with capability signalling |

Proposal 5: Update FG 58-1-6 as shown in the following table.

| 58. NR_AIML_air | 58-1-6 | Performance monitoring for UE-sided model | 1. Support of performance monitoring with RS-PAI of AI/ML model for beam prediction. 2. Maximum total number of the configured CSI-RS resources for monitoring RS resource set 3. Maximum number of monitoring report(s) configured per BWP 4. Maximum number of monitoring report(s) configured across all CCs 5. Maximum number of monitoring occasions for RS-PAI calculation | FFS Either one of 58-1-2 or 58-1-4 | yes | n/a | Performance monitoring for UE-sided model is not supported | FFS Per UE | FFS No | FFS No | FFS No | FFS: component candidate values Component 2 candidate values: {16, 32, 64, 128} Component 3 candidate values: {1, 2} Component 4 candidate values: {1,2,4,8} Component 5 candidate values: {1,2,3,4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 6: Update FG 58-1-7 as shown in the following table.

| 58. NR_AIML_air | 58-1-7 | Data collection for UE-side beam prediction | 1. Support of data collection for UE-side beam prediction 2. Support of SS/PBCH block and 1-port CSI-RS based RSRP measurements for measurement RS resource sets (Set B and Set A) for data collection 3. Supported sub-use cases 6: Supported maximum number of resources for Set B 7: Supported maximum number of resources for Set A 8. Support of SSB as RS type for Set B 9. Support of CSI-RS as RS type for Set B 10. Support of SSB as RS type for Set A 11. Support of CSI-RS as RS type for Set A | FFS 2-35 | yes | n/a | Data collection for UE-side beam prediction is not supported | Per UE | N/A | N/A | N/A | Component 3 candidate values: {‘Set B equal to Set A’, ‘Set B subset of Set A’, ’Set B not a subset of Set A’} Component 6 candidate values: {4, 8, 16, 32, 64} Component 7 candidate values: {8, 16, 32, 64} Note: it is up to RAN2 whether this FG is merged into data collection FG defined by RAN2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FGs for AI/ML for positioning accuracy enhancement

Proposal 7: FG 58-2-2 should be modified as follows.

| 58-2-2 | Support reception of AD for UE-based positioning Case 1 | Indicates support of reception of AD for UE-based positioning Case 1 | 58-2-1 | N/A | N/A | Reception of AD for UE-based positioning Case 1 is not supported | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. Note: For specific ADs, UE indicate supported AD that can be received as in [FFS: FGs] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 8: FG 58-2-3, 58-2-3a, and 58-2-3b should follow the elements of existing FGs 13-2/3/4 series. The FGs should be modified as follows.

| 58-2-3 | DL PRS Resources for UE-based positioning Case 1 | 1. Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE 2. Max number of TRPs across all positioning frequency layers per UE 3. Max number of positioning frequency layers UE supports | [13-1] 58-2-4 | N/A | N/A |  | Per UE | n/a | n/a | n/a | Component 1 candidate values: {1, 2} Component 2 candidate values: {4, 6, 12, 16, 24, 32, 64, 128, 256} Component 3 candidate values: {1, 2, 3, 4} Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58-2-3a | DL PRS Resources for UE-based positioning Case 1 on a band | 1. Max number of DL PRS Resources per DL PRS Resource Set 2. Max number of DL PRS Resources per positioning frequency layer | [13-1] 58-2-4 | N/A | N/A |  | Per Band | n/a | n/a | n/a | Component 1 candidate values: {1, 2, 4, 8, 16, 32, 64} Component 2 candidate values: {6, 24, 32, 64, 96, 128, 256, 512, 1024} Note: For component 1, the values 16, 32, 64 are only applicable to FR2 bands Note: For component 2, the value 6 is only applicable to FR1 bands Need for location server to know if the feature is supported. Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |
| 58-2-3b | DL PRS Resources for UE-based positioning Case 1 on a band combination | 1. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1-only 2. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2-only 3. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation 4. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2 in FR1/FR2 mixed operation | [13-1] 58-2-4 | N/A | N/A |  | Per BC | n/a | n/a | n/a | Component 1 candidate values: {6, 24, 64, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR1 only BC Component 2 candidate values: {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR2 only BC Component 3 candidate values: {6, 24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands Component 4 candidate values: {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands] Need for location server to know if the feature is supported. Note: the reported value is the total number across all bands in the corresponding BC Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |

Proposal 9: FG 58-2-4 should be modified as follows.

| 58-2-4 | DL PRS Processing Capability for UE-based positioning Case 1 | 1. Maximum DL PRS bandwidth in MHz, which is supported and reported by UE. 2. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 3. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. 4. Max number of DL PRS resources that UE can process in a slot under it Note: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) of no more than 30%. |  | No | N/A |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Component 1 candidate values: -FR1 bands values {5, 10, 20, 40, 50, 80, 100} -FR2 bands values {50, 100, 200, 400} Component 3 candidate values: -T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms -N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Component 4 candidate values: -FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz -FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Notes for component 3: a. UE reports one combination of (N, T) values per band, where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth (B) in MHz supported by UE b. UE is not expected to support DL PRS bandwidth that exceeds the reported DL PRS bandwidth value c. UE DL PRS processing capability is defined for a single positioning frequency layer. UE capability for simultaneous DL PRS processing across positioning frequency layers is not supported in Rel.16 (i.e. for a UE supporting multiple positioning frequency layers, a UE is expected to process one frequency layer at a time) d. UE DL PRS processing capability is agnostic to DL PRS comb factor configuration e. The reporting of (N, T) values for maximum BW in MHz is not dependent on SCS Note: if the UE does not indicate this capability for a band or band combination, the UE does not support PRS processing in this band or band combination. Note: If UE does not provide [this FG] but the UE supports Case 1, FG 13-1 indicates the DL PRS processing capabilities common across all positioning methods including UE-based positioning Case 1.] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 10: FG 58-2-5 should be modified as follows.

| 58-2-5 | Support of SSB from neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 | 1. Support of SSB from neighbour cell as QCL source of a DL PRS 2. Support of reuse SSB measurement from RRM for receiving PRS Note: Refers to Type-C for FR1 and Type-C & Type-D support for FR2 | [58-2-4; otherwise 13-1] | No | N/A | SSB from neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 is not supported | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 11: FG 58-2-6 dhould be modified as follows.

| 58-2-6 | Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 | 1. Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS Note 1: Refers to Type-D support for FR2 Note 2: A PRS from a PRS-only TP is treated as PRS from a non-serving cell | [58-2-4; otherwise 13-1] | No | N/A | DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 is not supported | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. DL PRSs are in the same band | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 12: The following UE feature group for RRC_INACTIVE state and RRC_IDLE state should be supported.

- FG 58-2-12: Support of UE-based positioning Case 1 in RRC_INACTIVE state

- FG 58-2-13: DL PRS processing capabilities in RRC inactive state for UE-based positioning Case 1

- FG 58-2-14: Support of UE-based positioning Case 1 in RRC_IDLE

- FG 58-2-15b: DL PRS processing capabilities for aggregated PRS processing of 2 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE - UE-based positioning Case 1

- FG 58-2-15c: DL PRS processing capabilities for aggregated PRS processing of 3 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE - UE-based positioning Case 1

- FG 58-2-18: PRS bandwidth aggregation in RRC_ INACTIVE UE-based positioning Case 1

- FG 58-2-19: PRS bandwidth aggregation in RRC_IDLE - UE-based positioning Case 1

| 58-2-12 | Support of UE-based positioning Case 1 in RRC_INACTIVE state | UE-based positioning Case 1 in RRC_INACTIVE state is supported | [58-2-3 and 58-2-13] | N/A | N/A | UE-based positioning Case 1 in RRC_INACTIVE state is not supported | Per Band | n/a | n/a | n/a | Support of UE-based positioning Case 1 in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58-2-13 | DL PRS processing capabilities in RRC inactive state for UE-based positioning Case 1 | 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE 3. Max number of DL PRS resources that UE can process in a slot | [58-2-4] | N/A | N/A |  | Per band | n/a | n/a | n/a | Component 1 candidate values: {Type 1, Type 2} Component 2 candidate values: T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Component 3 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Need for location server to know if the feature is supported Note: Having the PRS processing capabilities in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state Note: if UE supports same values for one or more components as in FG 27-6, then the UE can skip indicating these components in this FG and the values in corresponding FG 27-6 components indicate supported PRS processing capabilities in RRC inactive state for Case 1 | Optional with capability signalling |
| 58-2-14 | Support of UE-based positioning Case 1 in RRC_IDLE | Support of UE-based positioning Case 1 in RRC_IDLE | [ 58-2-4, 58-2-12, 58-2-13] | N/A | N/A | UE-based positioning Case 1 in RRC_IDLE is not supported | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signalling |
| 58-2-15b | DL PRS processing capabilities for aggregated PRS processing of 2 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE - UE-based positioning Case 1 | 1. Maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE 2. Maximum DL PRS bandwidth in MHz, per PFL 3. DL PRS buffering capability 4. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE. 5. Maximum number of aggregated DL PRS resources across aggregated PFLs that UE can process in a slot | [58-2-13] | N/A | N/A | DL PRS processing capabilities for aggregated PRS processing of 2 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE - UE-based positioning Case 1 is not supported | Per Band | n/a | n/a | n/a | Component 1 candidate values: a) FR1 bands: {10, 20, 40, 50, 80, 100, 160, 200} b) FR2 bands: {100, 200, 400, 800} Component 2 candidate values:a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} Note: Component 3 in [FG 58-2-15b (this FG)] follows buffering capability type reported in [FG 58-2-4] Component 4 candidate values: a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note: this value N should be equal or smaller than the value N reported by [FG 58-2-13] or this value T should be equal or larger than the value T reported by [FG 58-2-13] Component 5 candidate values: a. FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b. FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: each two linked PRS resources are counted as 1 resource Note: this value should be equal or smaller than the value reported by [FG 58-2-13] Note: if UE supports same values for one or more components as in FG 41-4-1b, then the UE can skip indicating these components in this FG and the values in corresponding FG 41-4-1b components indicate supported for aggregated PRS processing of 2 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE capabilities for Case 1 | Optional with capability signalling |
| 58-2-15c | DL PRS processing capabilities for aggregated PRS processing of 3 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE - UE-based positioning Case 1 | 1. Maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE 2. Maximum DL PRS bandwidth in MHz, per PFL 3. DL PRS buffering capability 4. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE. 5. Max number of aggregated DL PRS resources across aggregated PFLs that UE can process in a slot under it | [58-2-15b] | N/A | N/A | DL PRS processing capabilities for aggregated PRS processing of 3 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE - UE-based positioning Case 1 is not suported | Per Band | n/a | n/a | n/a | Component 1 candidate values: a) FR1 bands: {15, 20, 30, 40, 50, 60, 80, 100, 120, 140, 150, 160, 180, 200, 240, 300} b) FR2 bands: {150, 200, 300, 400, 600, 800, 1000, 1200} Component 2 candidate values: a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} Note: Component 3 in [FG 58-2-15c (this FG)] follows buffering capability type reported in [FG 58-2-4] Component 4 candidate values: a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note: this value N should be equal or smaller than the value N reported by [FG 58-2-13] or this value T should be equal or larger than the value T reported by [FG 58-2-13] Component 5 candidate values: a. FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b. FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: each three linked PRS resources are counted as 1 resource Note: this value should be equal or smaller than the value reported by [FG 58-2-13] Note: if UE supports same values for one or more components as in FG 41-4-1c, then the UE can skip indicating these components in this FG and the values in corresponding FG 41-4-1c components indicate supported aggregated PRS processing of 3 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE for Case 1 | Optional with capability signalling |
| 58-2-18 | PRS bandwidth aggregation in RRC_ INACTIVE UE-based positioning Case 1 | Support of PRS bandwidth aggregation in RRC_ INACTIVE for UE-based positioning Case 1 | [58-2-12, 58-2-15b] | N/A | N/A | PRS bandwidth aggregation in RRC_ INACTIVE for UE-based positioning Case 1 is not supported | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |
| 58-2-19 | PRS bandwidth aggregation in RRC_IDLE - UE-based positioning Case 1 | Support of PRS bandwidth aggregation in RRC_IDLE for UE-based positioning Case 1 | [58-2-14, 58-2-15b] | N/A | N/A | PRS bandwidth aggregation in RRC_IDLE for UE-based positioning Case 1 is not supported | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |

FGs for AI/ML for CSI prediction

Proposal 13: Update FG 58-3-1 and FG 58-3-2 as shown in the following table.

| 58. NR_AIML_Air | 58-3-1 | CSI prediction for UE-sided inference when N4=1 | 1. Support of CSI prediction for UE-sided inference when N4=1 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 5. Support parameter combinations with L=2,4 6. Support for rank = 1,2 7. Support for the size of DD-basis, N4=1 8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 11. Scaling factor for active resource counting Kp 12. supported value of t for the relaxation of Z and Z’ timeline 13. supported number of occupied CPU 14. Supported number of occupied APU CPU,2/CPU,3 15. Occupied resource pool between CPU,2 and CPU,3APU pool | 2-35 58-0-1 | yes | n/a | CSI prediction for N4=1 for inference is not supported | Per band and Per BC | n/a | n/a | n/a | Component 3 candidate values: a. {4,8,12,16,24,32} b. {2,3,4 … 64} c. {4, …, 256} Component 14 candidate values: {0, 2, 4, 8} Component 15 candidate values: {CPU,2 or CPU,3} Note: UE should not report non-zero value for Component 14 if FG 58-0-1 is not signalled. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_Air | 58-3-2 | CSI prediction for UE-sided inference when N4>1 | 1. Support of CSI prediction for UE-sided inference when N4>1 2. Support for reporting predicted PMI with N4>1 3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR 5. Support for the size of DD-basis, N4>1 6. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting 7. Supported number of occupied CPU 8. Supported number of occupied APU CPU,2/CPU,3 9. Occupied resource pool between CPU,2 and CPU,3APU pool | 58-3-1 | yes | n/a | CSI prediction for N4>1 for inference is not supported | Per band and Per BC | N/A | N/A | N/A | Component 3 candidate values: a. {1,2,4,8} b. {4,8,12,16,24,32} c. {2,3,4 … 64} d. {4, …, 256} Component 6 candidate values: a. {1,2,4,8} b. {4,8,12,16,24,32} c. {4,8,12} d.{4, …, 256} Component 8 candidate values: {0, 2, 4, 8} Component 9 candidate values: {CPU,2 or CPU,3} Note: UE should not report non-zero value for Component 8 if FG 58-0-1 is not signalled | Optional with capability signalling |

Proposal 14: Update FG 58-3-4 as shown in the following table.

| 58. NR_AIML_Air | 58-3-4 | UE side data collection for CSI prediction | 1. Support of data collection for CSI prediction | FFS 2-35 | yes | n/a | UE side data collection for CSI prediction is not supported | Per band and Per BC | N/A | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 15: Update FG 58-3-5 as shown in the following table.

| 58. NR_AIML_Air | 58-3-5 | Performance monitoring for CSI prediction model | 1. Supported of two performance metric SGCS 2. Support of one wideband frequency granularity SGCS per layer 3. Support of one configured time instance for N4>1 4. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 5. Supported number of occupied CPU | 58-3-1 | Yes | n/a | Performance monitoring for CSI prediction model is not supported | Per Band and Per BC | N/A | N/A | N/A | Component 4 candidate values: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} Component 5 candidate values: {0, 1, 2} Note: The summation of the value reported by Component 5 and the one reported by Component 13 of FG58-3-1 (or Component 7 of FG58-3-2) should not significant than the N_CPU UE reported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Reference

[1] RP-250792, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, Mar. 2025.

[2] RAN1 chair (CMCC), “Draft Report of 3GPP TSG RAN WG1 #122”, Aug. 2025.