3GPP TSG RAN WG1 #122bis	R1-2507873

Prague, Czech, Oct 13th – 17th, 2025

Source:	Sharp

Title:	Maintenance on measurements related enhancements for LTM

Agenda item:	8.8

Document for: 	Discussion

Introduction

In this contribution, remaining issues related to the determination of subcarrier spacing for timing advance in CLTM and timing advance command related to LTM Candidate Timing Advance Command MAC CE are further analyzed.

Discussion

How to determine subcarrier spacing of timing advance for CLTM

LTM Candidate Timing Advance Command MAC CE is introduced for CLTM (conditional LTM), which comprises a Timing Advance Command field. This field indicates the index value TA used to control the amount of timing adjustment that MAC entity has to apply when UE switches to the candidate cell during CLTM. This TA is associated with the time alignment amount NTA, where NTA=TA∙16∙642μ.

To determine NTA for conditional LTM, the value TA will be indicated by the timing advance command field in the LTM Candidate Timing Advance Command MAC CE. However, how to determine the subcarrier spacing (i.e., μ) for NTA is not determined yet.

According to current TS38.213 [1] specification, NTA is defined as follows: “A timing advance command  in case of random access response or in an absolute timing advance command MAC CE or in a cell switch command, TA, for a TAG indicates NTA values by index values of TA = 0, 1, 2, ..., 3846, where an amount of the time alignment for the TAG with SCS of 2μ∙15 kHz is NTA=TA∙16∙642μ. NTAis relative to the SCS of the first uplink transmission from the UE after the reception of the random access response or absolute timing advance command MAC CE or the cell switch command.”

From our perspective, this method － where NTA is relative to the SCS of the first uplink transmission from the UE after the reception of the random access response or absolute timing advance command MAC CE or the cell switch command － cannot be directly applied to the case of the LTM Candidate Timing Advance Command MAC CE. Under CLTM, the UE may not apply the TA in the LTM Candidate Timing Advance Command MAC CE immediately after receiving it. Instead, the UE may store the TAvalue and only determine NTA after it decides which candidate cell to switch to. Therefore, the existing method for determining the SCS for NTA－based on the first uplink transmission following the LTM Candidate Timing Advance Command MAC CE － cannot be reused.

As a result, a new method for determining the subcarrier spacing for NTA may require further discussion. For example, NTA is relative to the SCS of the first uplink transmission from the UE after the UE switches to the target cell during CLTM.

How to determine the subcarrier spacing for an amount of time alignment, NTA=TA∙16∙642μ, may require further discussion in the case that the timing advance command is received in the LTM Candidate Timing Advance Command MAC CE for conditional LTM. Adopt TP#1 for TS 38.213.

  - For example, NTA is relative to the SCS of the first uplink transmission from the UE after the UE switches to the target cell during CLTM.

TP#1:

Reason for change:

The existing method for determining the SCS for NTA－based on the first uplink transmission following the LTM Candidate Timing Advance Command MAC CE － cannot be reused for CLTM. As under CLTM, the UE may not apply the TA in the LTM Candidate Timing Advance Command MAC CE immediately after receiving it. Instead, the UE may store the TAvalue and only determine NTA after it decides which candidate cell to switch to. Clarification of the subcarrier spacing determination for timing advance indicated in the LTM Candidate Timing Advance Command MAC CE for CLTM may be needed.

Summary of change:

Add “, or after the UE switches to the target cell during CLTM” to section 4.2 in TS 38.213.

Consequence if not approved:

The specification for NTA determination in CLTM is incomplete, as the SCS determination for NTA is unclear.

##### 4.2 UE procedure for determining physical downlink control channel assignment < Unchanged parts are omitted > A timing advance command [11, TS 38.321] in case of random access response or in an absolute timing advance command MAC CE or in a cell switch command, TA, for a TAG indicates NTA values by index values of TA = 0, 1, 2, ..., 3846, where an amount of the time alignment for the TAG with SCS of 2μ∙15 kHz is NTA=TA∙16∙642μ. NTA is defined in [4, TS 38.211] and is relative to the SCS of the first uplink transmission from the UE after the reception of the random access response or absolute timing advance command MAC CE or the cell switch command, or after the UE switches to the target cell during CLTM. < Unchanged parts are omitted >

Timing advance command related to LTM Candidate Timing Advance Command MAC CE

LTM Candidate Timing Advance Command MAC CE  is introduced for CLTM, which comprises a Timing Advance Command field. For the timing advance command description in section 4.2 in current TS 38.213 specification, the LTM Candidate Timing Advance Command MAC CE case is missing.

LTM Candidate Timing Advance Command MAC CE should be included in the timing advance command description in section 4.2 in TS 38.213. Adopt TP#2 for TS 38.213.

TP#2:

Reason for change:

In the existing description of the timing advance command in section 4.2 in current TS 38.213 specification, the case for the LTM Candidate Timing Advance Command MAC CE case is missing.

Summary of change:

Add “or in a LTM Candidate Timing Advance Command MAC CE” to section 4.2 in TS 38.213.

Consequence if not approved:

The specification for the timing advance command description is incomplete, as the LTM Candidate Timing Advance Command MAC CE case is missing.

##### 4.2 UE procedure for determining physical downlink control channel assignment < Unchanged parts are omitted > A timing advance command [11, TS 38.321] in case of random access response or in an absolute timing advance command MAC CE or in a cell switch command or in a LTM Candidate Timing Advance Command MAC CE, TA, for a TAG indicates NTA values by index values of TA = 0, 1, 2, ..., 3846, where an amount of the time alignment for the TAG with SCS of 2μ∙15 kHz is NTA=TA∙16∙642μ. NTA is defined in [4, TS 38.211] and is relative to the SCS of the first uplink transmission from the UE after the reception of the random access response or absolute timing advance command MAC CE or the cell switch command. < Unchanged parts are omitted >

Conclusions

In this contribution, we discussed the issues regarding enhancement of measurements to support LTM which includes CSI acquisition for candidate cell and Conditional intra-CU LTM. And we have proposals as follows.

- How to determine the subcarrier spacing for an amount of time alignment, NTA=TA∙16∙642μ, may require further discussion in the case that the timing advance command is received in  the LTM Candidate Timing Advance Command MAC CE for conditional LTM. Adopt TP#1 for TS 38.213.

  - For example, NTA is relative to the SCS of the first uplink transmission from the UE after the UE switches to the target cell during CLTM.

LTM Candidate Timing Advance Command MAC CE should be included in the timing advance command description in section 4.2 in TS 38.213. Adopt TP#2 for TS 38.213.

References

5G; NR; Physical layer procedures for control (3GPP TS 38.213 version 19.1.0 Release 19).