3GPP TSG RAN WG1#122bis	R1-2508092

Prague, Czech, October 13th – 17th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.901 | CR |  | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to incorporate new agreements on ISAC CM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Sensing_NR |  |  |  |  |  | Date: |  |  | 2025-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN1 #122bis, multiple agreements were made on the correction to TR 38.901 v19.1.0. The reasons for the changes are as follows. - For monostatic sensing, the existing generation of RCS component σS,n',m',n,mk,p,σS,n,m, n',m'k,p breaks channel reciprocity - Confusion the Table 7.9.4.2-2 Part-1 and Part-2 for the applicable UE type(s) for UT monostatic sensing - The consideration of ST as transmitter/receiver is to determine the reference Technical Report for channel generation in Clause 7.9.0 - ‘terrestrial UE’ is occasionally captured by ‘normal UE’ in Clause 7.9.3 - The definition is for bisector angle in Clause 7.9.2.1 is confusing - Multiple typos need correction. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes to TR 38.901 v19.1.0 are: - For monostatic sensing, enforces the relation between RCS component σS,n',m',n,mk,p,σS,n,m, n',m'k,p for channel reciprocity in step 10) in Clause 7.9.4.1 - Clarify the applicable UE type(s) for Table 7.9.4.2-2 Part-1 and Part-2 for UT monostatic sensing - Capture that the consideration of ST as transmitter/receiver is to determine the reference Technical Report for ISAC channel generation in Clause 7.9.0 - Change all ‘normal UE’ to ‘terrestrial UE’ in Clause 7.9.3 - Revise the definition is for bisector angle in Clause 7.9.2.1 - Multiple typos corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion on the implementation of the channel model for ISAC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.9.0, 7.9.2, 7.9.3, 7.9.4.1, 7.9.4.2, 7.9.5.2, 7.9.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

7.9	Channel model(s) for ISAC

7.9.0	Introduction

< Unchanged text omitted >

The channel model for ISAC in Clause 7.9 is designed based on the channel model defined in the previous clauses within Clause 7 taking into account the known properties, e.g., location, Radar Cross-Section (RCS), polarization and etc. of one or more physical objects. A physical object is categorized as a sensing target (ST) or an environment object (EO). A ST is an object of interest for sensing. An EO is a non-target object with known location. Two types of EO are supported in the ISAC channel model. A first type of EO (type-1 EO) has similar characteristic as a ST and is modelled in the same way as a ST. In the following descriptions in Clause 7.9, only the related details on ST are described, which is also applicable to type-1 EO. A second type of EO (type-2 EO, Clause 7.9.5.2) is of large size and is modelled differently from a ST.

The large scale and small scale parameters of the channel between any two of sensing transmitter (STX), ST and sensing receiver (SRX) in a sensing scenario are obtained from the Technical Reports for the same communication scenario unless updates on the parameter values are specially described. The ST is considered as a receiver or transmitter, respectively, in the determination of a reference Technical Report to generate the channel model for a STX-ST link or a ST-SRX link.

< Unchanged text omitted >

7.9.2	Physical object model

7.9.2.0	Introduction

A ST is modelled with one or multiple scattering points. Each scattering point of a ST (SPST) is used to model the total scattering effects of some adjacent scattering centres at the ST. The impact of a SPST to the channel includes at least two aspects, i.e., the RCS (Radar Cross Section, Clause 7.9.2.1) and the polarization matrix (Clause 7.9.2.2).

7.9.2.1	RCS of a sensing target

The RCS of a SPST is a scalar value defined in LCS of the ST and is dependent on both the incident angle and the scattered angle. The RCS values with same incident/scattered angles can be referred as monostatic RCS values.

The RCS related coefficient σRCS of a SPST for a pair of incident/scattered angles is composed of a first component σM which is included in the large-scale parameters (described in Step 15 in Clause 7.9.4.1), and a second component σD and third component σS which are both included in the small-scale parameters (described in Step 10 in Clause 7.9.4.1), i.e., σRCS=σMσDσS. σM is a deterministic value for the SPST. σD can be fixed to 1 or can be angular dependent. σS follows log-normal distribution. The mean μσS_dB and standard deviation σσS_dB used to characterize 10lgσS satisfy a fixed relation.

< Unchanged text omitted >

For UAV of large size with single SPST, human with RCS model 2 with single SPST, vehicle with single/multiple SPSTs, and AGV with single/multiple SPSTs, the values/pattern 10lgσMσD, denoted as σMD_dBθi,ϕi,θs,ϕs, of the RCS for a SPST is deterministic based on the incident angle (θi,ϕi,) and the scattered angle (θs,ϕs).

σMD_dB(θi,ϕi,θs,ϕs)=maxGmax-min-σVdBθ+σHdBϕ,σmax-k1sink2β2+5log10cosβ2,  Gmax-σmax,σFS(θi,ϕi,θs,ϕs) 	(7.9.2-3)

With σVdBθ, σHdBϕ defined by,

σVdBθ=-min12θ-θcenterθ3dB2, σmax

σHdBϕ=-min12ϕ-ϕcenterϕ3dB2, σmax

where,

-	(θ,ϕ) are zenith angle and azimuth angle of the bisector between the incident ray with zenith angle and azimuth angle (θi,ϕi), and scattered ray with zenith angle and azimuth angle (θs,ϕs)θi,ϕi,θs,ϕs.

-	β∈0°,180°. β is the bistatic angle between the incident ray and scattering ray within the plane defined by incident angle (θi,ϕi,) and scattering angle (θs,ϕs).

-	k1= 6.05, k2=1.33 for UAV with large size.

-	k1=0.5714, k2=0.1 for human with RCS model 2.

-	k1= 6, k2=1.65 for vehicle with single/multiple SPSTs.

-	k1= 12, k2=1.45 for AGV with single/multiple SPSTs.

< Unchanged text omitted >

7.9.3	Reference channel models and required updates

A transmitter or receiver in the sensing operation can be TRP, terrestrial UE, vehicle UE, aerial UE, AGV UE or RSU-type UE. The reference TR(s) to generate the channel for each combination of transmitter and receiver for each sensing scenario are provided in Table 7.9.3-1, where the terrestrial UE, AGV are referred as terrestrial UE.

Table 7.9.3-1: Reference TRs to generate channel for ISAC

| Case | Tx/Rx | Rx/Tx | Reference TR to define the channel model |
| --- | --- | --- | --- |
| 1 | TRP | TRP | For sensing scenario UMi/UMi-AV, UMa/UMa-AV, RMa/RMa-AV, InH, InF - TRP-TRP link of scenario UMi, UMa, InH, and InF following the option based on TR 38.901 defined in Clause A.3 of TR 38.858 - For InF, hUE is changed to the same height as the BS - TRP-UE link of scenario RMa defined in Clause 7 of TR 38.901 by setting hUE=35m (see note 1) For sensing scenario Highway - TRP-UE link of scenario RMa in Clause 7 of TR 38.901 by setting hUE=35m for FR1(see note 1) - TRP-TRP link of scenario UMa following the option based on TR 38.901 defined in Clause A.3 of TR 38.858 For sensing scenario Urban grid - TRP-TRP link of scenario UMa following the option based on TR 38.901 defined in Clause A.3 of TR 38.858 For sensing scenario HST - TRP-UE link of scenario RMa in Clause 7 of TR 38.901 by setting hUE=35m for FR1 (see note 1) - TRP-TRP link of scenario UMa following the option based on TR 38.901 defined in Clause A.3 of TR 38.858 for FR2 |
| 2 | TRP | terrestrial UE | For sensing scenario UMi/UMi-AV, UMa/UMa-AV, RMa/RMa-AV, InH, InF - TRP-UE link of scenario UMi, UMa, RMa, InH, and InF in Clause 7 of TR 38.901 For sensing scenario Highway and Urban grid - P2B link of scenario Highway and Urban grid in Clause 6 of TR 37.885 For sensing scenario HST - TRP-UE link of scenario RMa in Clause 7 of TR 38.901 for FR1 and TRP-UE link of scenario UMa in Clause 7 of TR 38.901 for FR2 |
| 3 | TRP | vehicle UE | For sensing scenario Highway and Urban grid - V2B link of scenario Highway and Urban grid in Clause 6 of TR 37.885 For sensing scenario UMi, UMa, and RMa - TRP-UE link of scenario UMi, UMa, and RMa in Clause 7 of TR 38.901 |
| 4 | TRP | aerial UE | For sensing scenario UMa-AV, UMi-AV, and RMa-AV - TRP-aerial UE link of scenario UMa-AV, UMi-AV, and RMa-AV in Clause Annex A and B of TR 36.777 for FR1 - Reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 |
| 5 | terrestrial UE | terrestrial UE | For sensing scenario UMi/UMi-AV, UMa/UMa-AV, RMa/RMa-AV, InH, InF - UE-UE link of scenario UMi, UMa, InH, and InF following the option based on TR 38.901 defined in Clause A.3 of TR 38.858 - TRP-UE link of scenario RMa defined in Clause 7 of TR 38.901 by setting hBS =1.5m (see note 2) For sensing scenario Highway and Urban grid - P2P link in Clause 6 of TR 37.885 For sensing scenario HST - TRP-UE link of scenario RMa in Clause 7 of TR 38.901 for FR1, e.g., hBS=1.5m, - UE-UE link of scenario UMa following the option based on TR 38.901 defined in Clause A.3 of TR 38.858 for FR2 |
| 6 | terrestrial UE | vehicle UE | For sensing scenario UMi, UMa, RMa - UE-UE link of scenario UMi, UMa following the option based on TR 38.901 defined in Clause A.3 of TR 38.858 - TRP-UE link of scenario RMa defined in Clause 7 of TR 38.901 by setting hBS =1.5m For sensing scenario Highway and Urban grid - V2P link in Clause 6 of TR 37.885 |
| 7 | terrestrial UE | aerial UE | For sensing scenario UMi-AV, UMa-AV, and RMa-AV - TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting hBS =1.5m for FR1 - Reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 - The corresponding parameter values in FR2 are used |
| 8 | vehicle UE | vehicle UE | For sensing scenario Highway and Urban grid - V2V link of scenario Highway and Urban grid in Clause 6 of TR 37.885 For sensing scenario UMi, UMa, and RMa - UE-UE link of scenario UMi, UMa following the option based on TR 38.901 defined in Clause A.3 of TR 38.858 - TRP-UE link of scenario RMa defined in Clause 7 of TR 38.901 by setting hBS =1.5m |
| 9 | aerial UE | aerial UE | For sensing scenario UMi-AV, UMa-AV, RMa-AV - TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting height of TRP equal to the height of the first aerial UE for FR1 (see note 4) - Reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 - The corresponding parameter values in FR2 are used |
| 10 | TRP | RSU-type UE | Highway and Urban grid - B2R link in Clause 6 of TR 37.885 |
| 11 | RSU-type UE | terrestrial UE | Highway and Urban grid - V2V link in Clause 6 of TR 37.885, with antenna height at RSU is 5m |
| 12 | RSU-type UE | RSU-type UE | Highway and Urban grid - V2V link in Clause 6 of TR 37.885, with antenna height at RSU is 5m |
| 13 | RSU-type UE | vehicle UE | Highway and Urban grid - V2V link in Clause 6 of TR 37.885, with antenna height at RSU is 5m |
| NOTE 1: ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 NOTE 2: ASD and ZSD statistics updated to be the same as ASA and ZSA NOTE 3: Indoor office scenario can be categorized into 5 sub-indoor scenarios defined in TR38.808. NOTE 4: First aerial UE height is hUT1, second aerial UE height is hUT2, where abshUT1-hBS≤abshUT2-hBS. |  |  |  |

The large scale and small scale parameters of the STX-ST link and the ST-SRX link for a sensing scenario are determined according to Table 7.9.3-1 assuming the same sensing scenario. The targets of UAV and vehicle are respectively considered as an aerial UE and vehicle UE. Other targets are considered as terrestrial UE. The proper case for each combination of STX/STX and ST are provided in Table 7.9.3-2.

Table 7.9.3-2: Channel model for STX-ST and ST-SRX link

| STX/SRX | Target | Cases in Table 7.9.3-1 assuming the same sensing scenario |
| --- | --- | --- |
| TRP | UAV | Case 4: TRP-Aerial UE link |
|  | Human | Case 2: TRP- terrestrial UE link |
|  | Vehicle | Case 3: TRP-Vehicle UE link |
|  | AGV | Case 2: TRP- terrestrial UE link |
|  | Object (Hazard) | Case 2: TRP- terrestrial UE link |
| Terrestrial UE | UAV | Case 7: terrestrial UE - Aerial UE link |
|  | Human | Case 5: terrestrial UE- terrestrial UE link |
|  | Vehicle | Case 6: terrestrial UE - Vehicle UE link |
|  | AGV | Case 5: terrestrial UE- terrestrial UE link |
|  | Object (Hazard) | Case 5: terrestrial UE- terrestrial UE link |
| Vehicle UE | Human | Case 6: terrestrial UE - Vehicle UE link |
|  | Vehicle | Case 8: Vehicle UE - Vehicle UE link |
|  | Object (Hazard) | Case 6: terrestrial UE - Vehicle UE link |
| Aerial UE | UAV | Case 9: Aerial UE - Aerial UE link |
| AGV UE | AGV | Case 5: terrestrial UE - terrestrial UE link |
| RSU-type UE | Human | Case 11: RSU-type UE - terrestrial UE link |
|  | Vehicle | Case 13: RSU-type UE - vehicle UE link |

The large scale and small scale parameters of the STX-SRX link, i.e., background channel for a sensing scenario are determined according to Table 7.9.3-1 assuming the same sensing scenario. The proper case for each combination of STX and SRX are provided in Table 7.9.3-3. For TRP monostatic sensing mode, a reference point (RP) is considered as a terrestrial UE. For UT monostatic sensing mode, a RP is considered as a TRP and an aerial UE is considered as a terrestrial UE.

Table 7.9.3-3: Channel model for background channel

| STX/SRX | SRX/STX | Cases in Table 7.9.3-1 assuming the same sensing scenario |
| --- | --- | --- |
| TRP | TRP | Case 1: TRP-TRP link |
|  | Terrestrial UE | Case 2: TRP- terrestrial UE link |
|  | Vehicle UE | Case 3: TRP-Vehicle UE link |
|  | Aerial UE | Case 4: TRP-Aerial UE link |
|  | AGV UE | Case 2: TRP- terrestrial UE link |
|  | RSU-type UE | Case 10: TRP - RSU-type UE link |
| Terrestrial UE | Terrestrial UE | Case 5: terrestrial UE- terrestrial UE link |
|  | Vehicle UE | Case 6: terrestrial UE - Vehicle UE link |
|  | Aerial UE | Case 7: terrestrial UE - Aerial UE link |
|  | AGV UE | Case 5: terrestrial UE - terrestrial UE link |
|  | RSU-type UE | Case 11: RSU-type UE - terrestrial UE link |
| Vehicle UE | Vehicle UE | Case 8: Vehicle UE - Vehicle UE link |
|  | RSU-type UE | Case 13: RSU-type UE - vehicle UE link |
| Aerial UE | Aerial UE | Case 9: Aerial UE - Aerial UE link |
| AGV UE | AGV UE | Case 5: terrestrial UE - terrestrial UE link |
| RSU-type UE | RSU-type UE | Case 12: RSU-type UE – RSU-type UE link |

The following updates compared to the reference TRs are identified to generate ISAC channel.

-	For human as a sensing target with a single scattering point, the height of the scattering point is 1.5 m.

-	For sensing scenario RMa, UMi-AV, UMa-AV and RMa-AV, the height of a scattering point of a target is used to calculate the LOS probability and pathloss, regardless of the lower bound of hUT in the reference TRs that are used to generate ISAC channel.

-	For sensing scenario UMi, UMa, the height of a scattering point of a target is used to calculate the LOS probability and pathloss, regardless of the lower bound of hUT in the reference TRs that are used to generate ISAC channel, except that hUT=1.5 m is assumed for breakpoint distance (dBP) calculation.

-	For sensing scenario Urban grid, the absolute delay model of scenarios UMa in Table 7.6.9-1 are reused

-	For sensing scenario highway and HST, the absolute delay model of scenarios RMa and UMa are reused for FR1 and FR2 respectively

-	For sensing scenarios UMi-AV, UMa-AV and RMa-AV,

-	For the TRP-TRP link and TRP-terrestrial UE link, the absolute delay model of scenarios UMi, UMa and RMa are respectively reused.

-	For the terrestrial UE- terrestrial UE link, the terrestrial UE - aerial UE link and the aerial UE - aerial UE link, the absolute delay model of scenario UMi is reused.

-	For the TRP- aerial UE link, the absolute delay model of scenarios UMi, UMa and RMa are respectively reused.

-	On Case 7 in Table 7.9.3-1, the LOS probability between an aerial UE and a terrestrial UE is generated by Table 7.9.3-4. hUT is the height of the aerial UE.

-	On Case 9 in Table 7.9.3-1, the LOS probability between two aerial UEs is generated by Table 7.9.3-5. hUT1,hUT2 are the height of the two aerial UEs.

< Unchanged text omitted >

7.9.4	Fast fading model

7.9.4.1	Target channel

< Unchanged text omitted >

Step 10: Obtain the power for all generated paths

The power of a path (k,p,n',m',n,m) is given by

Pn',m',n,mk,p=σD, n',m',n,mk,pminσS,n',m',n,mk,p,10μσσS_dB+3σσS_dB10Prx,n',m'k,pPtx,n,mk,p	(7.9.4-1)

where, σD, n',m',n,mk,p, σS,n',m',n,mk,p are the second and third component of RCS at the SPST p for a path are derived by the incident angle, i.e., AOA (ϕtx,n,m,AOAk,p), ZOA (θtx,n,m,ZOAk,p) of the ray in the STX-SPST link and the scattered angle, i.e., AOD (ϕrx,n',m',AODk,p), ZOD (θrx,n',m',ZODk,p) of the ray in the SPST-SRX link, referring to the RCS model defined in Clause 7.9.2.1 for the SPST. σS,n',m',n,mk,p is independently determined for each path of a SPST of the same ST and of the different STs. σS,n',m',n,mk,p of a path of a SPST keeps unchanged until it is updated. For monostatic sensing mode, σS,n',m',n,mk,p is equal to σS,n,m, n',m'k,p if n=n' and m=m'.

NOTE:	The transformation between GCS and LCS for the incident/scattered angles is necessary to determine the RCS at the SPST p. Prx,n',m'k,p,Ptx,n,mk,p are respectively the ray power in the SPST-SRX link and the STX-SPST link.

< Unchanged text omitted >

7.9.4.2	Background channel

< Unchanged text omitted >

Step 2: Generate 3 reference points (RPs) for the STX/SRX.

Draw the 2D distance between the STX/SRX and each RP and the height of each RP respectively Gαd,βd+cd and Gαh,βh+ch with parameters defined in Table 7.9.4.2-1/2, where the UT in Table 7.9.4.2-2 Part-1 refers to any of terrestrial UE, vehicle UE, AGV UE and RSU-type UE, and the UT in Table 7.9.4.2-2 Part-2 refers to an aerial UE. Note that Gαd,βd and Gαh,βh are random values generated by the shape-rate parameterized Gamma distribution fx|α,β=βαΓ(α)xα-1e-βx. With uniform distribution within range -π,π, draw the LOS AOD (ϕLOS,AOD) between the STX/SRX and the first RP. The LOS AOD is further rotated by 23π and 43π to respectively derive the LOS AOD from the STX/SRX to the second and third RPs. Consequently, the 3D location of each RP can be calculated.

< Unchanged text omitted >

7.9.5	Additional modelling components

7.9.5.2	Type-2 environment object

< Unchanged text omitted >

3.	Between Step 8 and 9 in Clause 7.9.4.1, insert one step to generate NLOS rays specularly reflected by type-2 EO, if present.

In a STX-SPST link, the general procedure to model a type-2 EO as described above is executed by substitute Tx with the STX and Rx with the SPST. A NLOS ray specularly reflected by a type-2 EO, if present, is represented by n=0,m>0. dtx,EO,mk,p=dEO,θtx,0,m,ZOAk,p=θEO, ZOA,ϕtx,0,m,AOAk,p=ϕEO, AOA,θtx,0,m,ZODk,p=θEO, ZOD,ϕtx,0,m,AODk,p=ϕEO, AOD,  CPMtx,0n, mk,p=CPMEO.

In a SPST-SRX link, the general procedure to model a type-2 EO as described above is executed by substitute Tx with the SPST and Rx with the SRX. A NLOS ray specularly reflected by a type-2 EO, if present, is represented by n'=0,m'>0. drx,EO,m'k,p=dEO,θrx,0,m',ZOAk,p=θEO, ZOA,ϕrx,0,m',AOAk,p=ϕEO, AOA,θrx,0,m',ZODk,p=θEO, ZOD,ϕrx,0,m',AODk,p=ϕEO, AOD,  CPMrx,0n, m'k,p=CPMEO.

< Unchanged text omitted >

7.9.5.4	Doppler of mobile scatterers

To support scatterer mobility, the Doppler frequency component fD,n',m',n,mk,pt in the channel coefficient generation in step 14 in clause 7.9.4.1 should be updated as follows.

fD,n',m',n,mk,pt=rrx,k,p,n',m'Ttvrxt+rk,p,n',m'Ttvk,pt+2αrx,n',m'k,pDrx,n',m'k,pλ0+rtx,k,p,n,mTtvtxt+rk,p,n,mTtvk,pt+2αtx,n,mk,pDtx,n,mk,pλ0	(7.9.5-14)

where,

-	Drx,n',m'k,p is a random variable from -vscatt to vscatt, and vscatt is the maximum speed of the clutter. αrx,n',m'k,p is a random variable of Bernoulli distribution with mean p' if n'>0, otherwise αrx,n',m'k,p=0. Parameter p' determines the proportion of mobile scatterers and can thus be selected to appropriately model statistically larger number of mobile scatterers (higher p') or statistically smaller number of mobile scatterers (e.g. in case of a completely static environment: p'=0 results in all scatterers having zero speed).

-	Dtx,n,mk,p is a random variable from -vscatt to vscatt, and vscatt is the maximum speed of the clutter. αtx,n,mk,p is a random variable of Bernoulli distribution with mean p if n>0, otherwise αtx,n,mk,p=0. Parameter p determines the proportion of mobile scatterers and can thus be selected to appropriately model statistically larger number of mobile scatterers (higher p) or statistically smaller number of mobile scatterers (e.g. in case of a completely static environment: p=0 results in all scatterers having zero speed).

< Unchanged text omitted >