3GPP TSG RAN WG1 #122bis			R1-250XXXX

Prague, Czech, Oct 13th – 17th, 2025

Agenda Item:	8.5

Source:	Ad-Hoc Chair (Ericsson)

Title:	Session notes for AI 8.5 (Maintenance on Enhancements of NES for NR)

Document for:	Discussion, Decision

## Maintenance on Enhancements of network energy savings for NR

Note: Maximum one contribution. For efficient review, please use the following sections in your contribution corresponding to the maintenance issues, if any:

- On-demand SSB SCell operation

- On-demand SIB1 for idle/inactive mode UEs

- Adaptation of common signal/channel transmissions

[122bis-R19-NES] Email discussion on NES– Ajit (Ericsson)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2506824	Discussion on remaining issues of Rel-19 NES	ZTE Corporation, Sanechips

R1-2506875	Maintenance on enhancements of network energy savings for NR	vivo

R1-2506950	Maintenance on Rel-19 Network Energy Savings	Huawei, HiSilicon

R1-2506965	Maintenance on Enhancements of network energy savings for NR	Xiaomi

R1-2507002	Discussion on maintenance issue on Enhancements of network energy savings for NR	CMCC

R1-2507122	Maintenance on enhancements of network energy savings for NR	CATT

R1-2507134	Maintenance on enhancements of network energy savings for NR	OPPO

R1-2507231	Maintenance on Enhancements of network energy savings for NR	Samsung

R1-2507293	Maintenance on Enhancements of network energy savings	Nokia, Nokia Shanghai Bell

R1-2507353	Remaining issues on enhancements of NES for NR	LG Electronics

R1-2507456	Discussion on on-demand SSB SCell operation	Ofinno

R1-2507494	A remaining issue on SSB time domain adaptation	ETRI

R1-2507516	Maintenance for Network Energy Saving	Google

R1-2507556	Maintenance on enhancements of network energy savings for NR	DENSO CORPORATION

R1-2507566	Maintenance on enhancements of network energy savings for NR	Sharp

R1-2507621	Maintenance on Enhancements of network energy savings for NR	MediaTek Inc.

R1-2507639	Maintenance for R19 NES	Ericsson

R1-2507649	Remaining issues on Rel-19 NES	Apple

R1-2507698	Maintenance on network energy savings for NR	Qualcomm Incorporated

R1-2507875	Correction on OD-SIB1	ASUSTeK

### On-demand SSB SCell operation

### On-demand SIB1 for idle/inactive mode UEs

### Adaptation of common signal/channel transmissions