3GPP TSG RAN WG1 #123                              			                   R1-2508358

Dallas, USA, Nov 17th – 21st, 2025

Agenda item:   11.4.1

Source: 	    Lekha Wireless Solutions

Title:	                Channel coding

Document for:   Discussion and decision

1. Introduction

In RAN1#122bis meeting, 3GPP RAN1 has discussed the channel coding schemes for 6GR and the following agreement was made [2]:

Agreement - For 6G channel coding, LDPC is used for data (including SIBs) and Polar code is used for L1 control information (larger than 11 bits, including PBCH) - For 6G LDPC - Working assumption: For data rate within NR range, reuse of NR LDPC design is supported - For data rate beyond NR range, study LDPC extension with acceptable performance-complexity tradeoff for both NW side and UE side - Note: Applicability of the potential LDPC extension to data rate within NR range will be further discussed - For 6G Polar code - Working assumption: For control information within NR range (larger than 11 bits), reuse of NR Polar code design is supported - For control information beyond NR range, study Polar code extension with acceptable performance-complexity tradeoff for both NW side and UE side - Note: Necessity for control information beyond NR range is to be further discussed - Polar code maximum mother code length is kept as 1024. - FFS: further motivation(s) for potential extension/enhancement until RAN1#123

This contribution presents our views on channel coding for 6GR interface.

2. Discussion

Agreement

- For 6G channel coding, LDPC is used for data (including SIBs) and Polar code is used for L1 control information (larger than 11 bits, including PBCH)

- For 6G LDPC

  - Working assumption: For data rate within NR range, reuse of NR LDPC design is supported

  - For data rate beyond NR range, study LDPC extension with acceptable performance-complexity tradeoff for both NW side and UE side

    - Note: Applicability of the potential LDPC extension to data rate within NR range will be further discussed

- For 6G Polar code

  - Working assumption: For control information within NR range (larger than 11 bits), reuse of NR Polar code design is supported

  - For control information beyond NR range, study Polar code extension with acceptable performance-complexity tradeoff for both NW side and UE side

    - Note: Necessity for control information beyond NR range is to be further discussed

    - Polar code maximum mother code length is kept as 1024.

  - FFS: further motivation(s) for potential extension/enhancement until RAN1#123

In this document, we discuss about the various enhancements for LDPC and polar codes in 6GR.

2.1 Further Improvements for LDPC Codes in 6GR

Traditional decoding algorithms for LDPC codes, such as belief propagation (BP) and sum-product algorithm (SPA), have been extensively studied and optimized to achieve near-optimal decoding performance. These algorithms exploit the graphical structure of LDPC code graphs and iteratively exchange messages between variable and check nodes to estimate the transmitted codeword. While BP and SPA offer competitive performance for moderate block lengths and code rates, their computational complexity scales unfavourably with increasing code length and complexity.

ML assisted LDPC Decoding: In response to the scalability challenges of traditional decoding algorithms, researchers have explored alternative approaches to LDPC decoding, including machine learning-assisted techniques. Machine learning algorithms offer the potential to learn complex mappings between received signals and decoded information, thereby circumventing the need for explicit probabilistic inference. Results indicated that the machine learning-assisted LDPC decoding scheme exhibited resilience to noise, interference, and fading effects, thereby ensuring reliable communication in challenging environments. Additionally, the decoding scheme demonstrated robustness to hardware imperfections and adversarial attacks, highlighting its suitability for practical deployment in 5G networks.

Observation 1: Machine learning models can adaptively optimize decoding strategies for LDPC codes based on observed channel conditions, error patterns, and historical data.

Proposal 1: Recent advances and promising directions in existing coding schemes, are expected to meet the various requirements of 6G networks. The enhancements for LDPC codes including machine learning-based LDPC decoders and autoencoders using end-to-end learning approaches present promising advancements.

2.2 Further Improvements for Polar Codes in 6GR

2.2.1 Learning-based Polar Code Design

Machine learning can be used to optimize the construction of polar codes, replacing traditional analytic or heuristic methods. A neural network learns bit-channel reliabilities directly from decoding performance, enabling data-driven code design. The approach generalizes to various channel models, code lengths, and decoding algorithms (e.g., SC, SCL). Results show improved error-rate performance compared to conventional Arıkan or Gaussian Approximation constructions. This demonstrates a flexible, learning-based framework for future adaptive and robust code design in communications systems.

2.2.2 Learning-based Successive Cancellation Decoder

A neural network–assisted successive cancellation (SC) decoder is proposed for polar codes to enhance decoding performance. The model learns to correct error-prone decision patterns within the SC process using a data driven approach. By integrating neural corrections into traditional decoding, it achieves lower error rates without significant complexity increase. The method is scalable and compatible with existing hardware, enabling practical deployment. Hence, deep learning can intelligently refine classical decoding algorithms for improved reliability.

2.2.3 Learning-based Adaptive Polar Coding

A learning based adaptive rate selection for precoded polar codes is proposed under SCL decoding. It introduces a neural network that predicts throughput or frame error rate (FER) based on parameters such as SNR, code rate, and list size, eliminating the need for exhaustive simulations. Using this predictor, the system dynamically selects the optimal code rate to maximize throughput while satisfying reliability constraints. The approach leverages the 5G NR reliability sequence for compact frozen-bit specification, allowing practical online adaptation. Simulation results show consistent throughput gains over fixed-rate 5G polar codes. The method effectively bridges ML-based prediction with classical coding theory for real-time adaptation. Hence, it demonstrates a scalable, standard-compatible way to improve polar code performance in time-varying channels.

Observation 2: A learning based polar code design learns optimal polar code constructions from decoding performance, achieving improved error rates and adaptability across channels and decoders.

Observation 3: A learning–assisted SC decoder learns to correct error-prone decisions in the decoding process, improving reliability without major complexity increases. This hybrid approach integrates deep learning with classical decoding for practical, scalable performance gains.

Observation 4: Learning based adaptive polar coding predicts optimal code rates for precoded polar codes based on SNR, rate, and list size, enabling adaptive throughput optimization without exhaustive simulations. This ML-based approach allows for real-time, scalable, and reliability-aware polar code adaptation.

Proposal 2: Machine learning enhances polar code design, decoding, and adaptation by learning bit reliabilities, correcting SC decoding errors, and predicting optimal code rates for varying conditions.  These data-driven methods offer scalable, efficient, and robust improvements in communication system performance.

3. Conclusions

This contribution has presented Lekha Wireless Solution’s views on ‘Channel coding for 6GR interface’. The following observations and proposals are therefore made:

Observation 1: Machine learning models can adaptively optimize decoding strategies for LDPC codes based on observed channel conditions, error patterns, and historical data.

Proposal 1: Recent advances and promising directions in existing coding schemes, are expected to meet the various requirements of 6G networks. The enhancements for LDPC codes including machine learning-based LDPC decoders and autoencoders using end-to-end learning approaches present promising advancements.

Observation 2: A learning based polar code design learns optimal polar code constructions from decoding performance, achieving improved error rates and adaptability across channels and decoders.

Observation 3: A learning–assisted SC decoder learns to correct error-prone decisions in the decoding process, improving reliability without major complexity increases. This hybrid approach integrates deep learning with classical decoding for practical, scalable performance gains.

Observation 4: Learning based adaptive polar coding predicts optimal code rates for precoded polar codes based on SNR, rate, and list size, enabling adaptive throughput optimization without exhaustive simulations. This ML-based approach allows for real-time, scalable, and reliability-aware polar code adaptation.

Proposal 2: Machine learning enhances polar code design, decoding, and adaptation by learning bit reliabilities, correcting SC decoding errors, and predicting optimal code rates for varying conditions.  These data-driven methods offer scalable, efficient, and robust improvements in communication system performance.

4. References

[1] R1-250XXXX, Draft Agenda, 3GPP TSG RAN WG1 #123, Dallas, USA, Nov 17th – 21st, 2025

[2] R1-2508012, FL summary#3 for 6G channel coding, 3GPP TSG RAN WG1 Meeting #122bis, Prague, Czech, October 13th – 17th, 2025