3GPP TSG-RAN WG1 Meeting #123	R1-2508736

Dallas, USA, November 17 – 21, 2025

Agenda Item:	11.3.2

Source:	Huawei, HiSilicon

Title:	Numerology and Frame Structure for 6GR Air Interface

Document for:	Discussion and Decision

Introduction

In RAN1#122b meetings, the following agreements were achieved.

Agreement For communication, 6GR considers NR frame structure used as a starting point for the study item, - Resource defined by one subcarrier and one symbol is called as resource element (RE). - Resource block (RB) is defined where the number of consecutive subcarriers per RB is the same for all numerologies and the number of subcarriers per RB is 12 - Radio Frame length is 10ms - Each radio frame is split into 10 subframes, each with a duration of 1 ms - For given SCS and for given symbol, the symbol duration, normal CP length and boundary is same as NR design. - A slot is defined as supporting 14 consecutive symbols for normal CP case and all subcarrier spacings. Agreement 6GR study assumes same SCS between 6GR Sync signals and other channels/signals (except PRACH) for a given band. - FFS: same/different SCS between 6GR sync signal and other channels/signals (except PRACH) for FR2-1. - Note: ISAC is separately discussed in ISAC session. Agreement - RAN1 assumes 400MHz maximum channel bandwidth at network side and 30kHz SCS around 7GHz - Study whether and how to enable UE to support 400MHz bandwidth

In this contribution, scenarios and design criteria for 6GR numerology and frame structure are discussed, aiming to support the agreed existing and new services, and to satisfy RAN requirements defined in the SID [1].

Numerology

For the new 6G spectrum, candidate numerologies shall be investigated, including remaining issues in maximum channel bandwidth and subcarrier spacing.

Channel bandwidth

- Channel bandwidth for around 7GHz

Last meeting, RAN1 agreed to assume a maximum Channel BandWidth (CBW) of 400MHz at network side for around 7GHz, and agreed to study whether/how to support 400MHz channel bandwidth at UE side. From our point of view, with a continuous 400MHz CBW at network side, at least the following options can be used at UE side:

- Option#1: UE has maximum CBW equal to 400MHz and operates with a single 400MHz carrier.

- Option#2: UE has a maximum CBW smaller than 400MHz (e.g., 200MHz) and may perform carrier aggregation (CA) to support 400MHz;

A thorough examination of both options should consider, at a minimum, the following aspects:

- Performance

RAN1 could evaluate UPT performance and QoS satisfaction for each option, considering the associated power consumption. Additional performance metrics can also be discussed and analysed as long as a fair comparation is made.

- NW/UE energy efficiency

Energy efficiency is a key consideration for 6G. In the evaluation, power consumption for different load scenarios should be considered for each option. Energy evaluation related details can be found in our companion paper [2].

- NW/UE complexity:

System overhead at the network side is one of the significant factors affecting complexity. Specifically, the overhead of PDCCH for scheduling, and common channel & signal overhead, e.g., the overhead of SSB and system information transmitted within a periodicity. At UE side, hardware implementation complexity and corresponding cost are essential. The evaluation could include required number of RF chains, FFT size, PA transmission efficiency and flexibility of UE capability reporting.

- From RAN1 perspective, at least the following aspects need to be considered in the study of the UE maximum channel bandwidth

- Performance (e.g, UPT gains, QoS satisfaction ratios, and NW/UE energy efficiency gains) and cost (e.g., NW/UE complexity) of the different UE maximum CBW schemes.

Subcarrier spacing

- SCS for 6GR Sync signals

RAN1 agreed that 6GR study assumes same SCS between 6GR Sync signals and other channels/signals (except PRACH) for a given band. However, for FR2-1 same/different SCS between 6GR sync signal and other channels/signals (except PRACH) is still for further study. Single SCS for 6GR sync signal in FR2-1 i.e., 120kHz is useful to reduce UE blind search during initial cell search and simplify both implementation and specification. Hence, we propose:

RAN1 assumes same SCS between 6GR Sync signals and other channels/signals (except PRACH) in FR2-1, i.e., only 120kHz in FR2-1.

Frame Structure

When designing 6GR frame structure, the following key aspects, including duplex modes for frame structure configuration, time-domain structure and frequency-domain structure, should be considered.

Duplex and frame structure configuration

As described in our paper [3], 6GR duplex and frame structure configuration should meet the following requirements:

- Support high throughput and low latency for immersive communication to enable new services

- Enhanced uplink coverage to achieve comparable coverage to 5G mid-band at around 7GHz

- Enhanced cell-edge performance, including both DL and UL, to provide uniform user experience

The existing duplex and frame structure configuration might not fully meet these more stringent 6GR requirements. Thus, in next section we explore the flexibility of TDD, and Sub-Band Full Duplex (SBFD), and flexible pairing of DL and UL carrier.

- Flexibility of TDD

To optimize spectrum usage, dynamic TDD was proposed in 5G, allowing for dynamic adjustment of DL and UL resources based on traffic demands e.g., using group-common slot format indicator (SFI) in DCI format 2_0. However, the excessive flexibility in direction adjustment introduces CLI that are hard to be timely and properly treated by neighbouring cells, which is the primary challenge in practical deployment.

From forward compatibility perspective, DL symbols, UL symbols, flexible symbols and SBFD symbols can be re-used in 6GR.  However, the use cases for flexible symbols can be discussed and studied. 5G NR also support both cell and UE specific RRC configuration for frame structure configuration. Cell-specific RRC configuration is a robust approach that maintains a common frame structure to the fundamental functionality and coordination between operators. UE specific RRC configuration is also significant to enable gNB adjusting UEs’ available DL/UL resources according to their DL/UL traffic requirements, which provides more flexibility for gNB scheduling than cell specific RRC configuration only. So, in our view, both cell-specific RRC configuration and UE-specific RRC configuration are needed in 6GR to offer a better balance between flexibility and implementation simplicity. Note that RRC signalling in general is slow allowing enough time for Neighbour cells to coordinate among themselves to properly handle the CLI.

DL symbols, UL symbols, flexible symbols and SBFD symbols should be kept. FFS the use cases of flexible symbols.

Cell-specific RRC configuration and UE-specific RRC configuration should be used in 6GR to configure the frame structure of the UEs.

For the SFI indication, simplification can be considered. In NR, the SFI is used to indicate the formats of a number of slots in advance, allowing the UE to determine the slot format of a specific slot in advance. The maximum number of slots is 320, with their formats indicated in advance. The implementation complexity arises from the various slot format options. A large number of predefined slot formats are listed in a table. One possibility is to avoid slot-level SFI and instead consider a few long-term slot formats that better match long-term traffic characteristics. Another possibility is to reuse the SFI mechanism framework while simplifying the SFI table.

Transmission direction indication in TDD should be studied in 6GR taking into consideration the realistic variation in DL/UL traffic. E.g., RAN1 study and design a simplified SFI indication framework.

- SBFD

In NR, semi-static SBFD is specified to provide enhanced UL coverage for TDD spectrum. From the perspective of frame structure, time-frequency locations of SBFD DL/UL subbands in NR are configured to UEs in addition to tdd-UL-DL-ConfigurationCommon. Then the DL symbols or flexible symbols configured by tdd-UL-DL-ConfigurationCommon are overridden into SBFD symbols. The signalling procedure seems complicated and could be improved. For 6GR, the frame structure configuration should support SBFD natively. For example, the time and frequency domains configurations, e.g., subband-level TDD UL/DL configuration, can be studied.

Semi-static BS side SBFD should natively be supported.

- Time-frequency domain configuration, e.g., subband-level TDD UL/DL configuration, shall be further considered.

In NR, Semi-static SBFD is configured using SIB1 messages, which include an uplink subband configuration in the downlink symbols. This configuration is specific to each cell and is not easily changeable. From our perspective, different uplink subband widths can adapt to varying levels of uplink throughput, capacity and uplink coverage of different target data rates. Specifically, wider uplink subbands can facilitate the extension of coverage for higher uplink data rate, such as 50Mbps or 100Mbps, which is necessary for mobile AI and other advanced services. However, wider uplink subbands may also result in increased downlink loss due to the orthogonality of uplink and downlink in the frequency domain. To achieve a balance between uplink and downlink experiences, the uplink subband width can be adjusted, as illustrated in Figure 1. One possible way is making this adjustment through DCI or MAC CE mechanisms to change SBFD configurations, allowing the network to reconfigure the uplink subband width according to the specific uplink requirements and minimize downlink performance loss. Another way is to introduce  a flexible subband mechanism where in addition to DL/UL subband(s), gNB can explicitly configure flexible subband(s) to the UEs, in which either DL transmissions or UL transmissions  are allowed up to gNB scheduling. The entire mechanism works similarly to flexible symbol mechanism in NR, except that it is restricted within a subband of a symbol.

Figure 1 : An example of switching between two SBFD patterns (flexible SBFD)

Flexible SBFD change in terms of subband bandwidth can be further studied in 6GR.

- Flexible pairing of DL and UL

Existing duplex modes (including FDD, TDD, and SBFD) are restricted by a fixed association between DL carrier and UL carrier in the same band. To enable flexible spectrum deployment for NW operator and improve UE experience, flexible pairing of DL and UL could be considered, allowing DL and UL operation on same or different frequency bands. The advantages for both UE and NW operator can be found in [4]. RAN2 has already agreed to study how to use UL and DL spectrum more efficiently (e.g., decoupling of UL and DL, etc).

From the perspective of frame structure configuration, DL and UL configuration/indication is important to enable that flexibility by paired carrier cooperation. In Figure 2, the two carriers (e.g., Downlink CC1@High Band A, Uplink CC2@Low Band B) with different TDD configurations are deployed at NW side to guarantee the UL coverage. From UE perspective, DL resources can be configured in CC1 while UL resources can be configured in CC2. In Slot#2 and Slot#7, both DL and UL resource are available in CC1 and CC2. UE-specific directions in these slots should be further configured or indicated to UE in 6GR. Thus, 6G frame structure configuration should further consider and study how to support the flexible uplink and downlink pairing.

Figure 2: An example of flexible DL/UL pairing

6GR frame structure configuration considering flexible uplink and downlink paring should be studied.

Timing unit for scheduling

14-symbol slot has been agreed to be defined in 6GR. It can be seen as the time reference for physical channel/signal, including HARQ timing, PDSCH/PUSCH timing, the periods and etc, and time unit for scheduling. However, the association between slot and timing unit for scheduling needs to be discussed separately in 6GR.

- Timing unit for scheduling

Short duration and flexible location

To better accommodate diverse services including the agreed existing and new services, 6GR scheduling schemes also need to consider their diverse requirements. As described above, new services, including immersive communications (such as XR) and HRLLC, have more stringent requirements on reliability and latency. Thus, short duration and flexible location are also needed in 6G.

Achieving higher energy efficiency in 6GR is a fundamental design principle in RAN1. 6GR supports larger bandwidth (400MHz), allowing compact data to be transmitted quickly in a shorter period of time (i.e., few OFDM symbols) to enable ultra-fast data transmission and nearly zero-latency feedback, thereby minimizing wake-up time and maximizing sleep opportunities for both the network and UEs. For example, to maximize the sleep duration, time domain compact transmission (e.g., aggregated transmission in a short duration as shown in Figure 3) for DL/UL data can be considered in energy saving .

Figure 3: An example of DL short duration and flexible location

Besides, in NR slot-based scheduling, i.e., PxSCH mapping type A, the starting positions of data transmission are fixed at the beginning of each slot, i.e., the starting position of PDSCH is symbol index {0, 1, 2, 3} while that of PUSCH is symbol index#0 as illustrated in  Figure 4. In the scenario of cross-slot scheduling introduced in 5G to avoid the need for excessive data buffering, UE always transmits data from the starting position of allocated slot (e.g., slot#n+1). When scheduling multiple UEs from different directions in slot#n+1, their data transmissions are required to use FDM in the frequency domain and therefore occupy a longer duration in the time domain. Consequently, this extended long duration results in increased processing time for the UE (or less sleeping duration), which in turn raises the energy consumption. Thus, 6GR should allow flexible mapping type (similar to NR PxSCH mapping type B without its existing limitations) where the allocation of PxSCH could start at any symbol within a slot, thereby enhancing UE energy efficiency.

Figure 4: An example of flexible location of PxSCH

Long duration

Additionally, achieving further enhanced spectral efficiency, enhanced coverage and improved cell-edge performance is a key objective in 6GR, enabling the reuse of existing 3.5 GHz sites for 6GR deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. As the carrier frequency increases, both downlink and uplink coverage decrease. Thus, studying long-length data transmission for enhancing the coverage might be required.  Furthermore, multi-slot operations, e.g., DMRS bundling, and DMRS overhead reduction and DCI overhead reduction, could also be considered for enhanced spectral efficiency.

Last but not least, a unified design, including flexible duration and flexible location, in 6GR could be considered to avoid multiple options for the same functionality since the multiple types of scheduling unit are specified in different release and their similar schemes are not unified. For example, mapping type A and mapping type B all support flexible scheduled length, e.g., the length of the PDSCH L can be {2, 3, 4, …, 13} symbols in type B while that can be {3, 4, …, 13} symbols in type A. 6G can simplify them by adopting a unified mapping type.

Time-domain scheduling in 6GR with flexible duration and flexible location should be further studied.

Frequency-domain physical resources

In RAN1#122b, it has been agreed that “Resource block (RB) is defined where the number of consecutive subcarriers per RB is the same for all numerologies and the number of subcarriers per RB is 12”.  PRB and CRB are used in NR and are related with the definition of RB. We do not see that 6GR frame structure  requires a change in their definitions and usages. Hence, we propose:

6GR defines CRB and PRB similar to NR.

Conclusion

According to the above discussions, we have the following observations and proposals:

Observations

- From RAN1 perspective, at least the following aspects need to be considered in the study of the UE maximum channel bandwidth

- Performance (e.g, UPT gains, QoS satisfaction ratios, and NW/UE energy efficiency gains) and cost (e.g., NW/UE complexity) of the different UE maximum CBW schemes.

Proposals

- RAN1 assumes same SCS between 6GR Sync signals and other channels/signals (except PRACH) in FR2-1, i.e., only 120kHz in FR2-1.

- DL symbols, UL symbols, flexible symbols and SBFD symbols should be kept. FFS the use cases of flexible symbols.

- Cell-specific RRC configuration and UE-specific RRC configuration should be used in 6GR to configure the frame structure of the UEs.

- Transmission direction indication in TDD should be studied in 6GR taking into consideration the realistic variation in DL/UL traffic. E.g., RAN1 should study and design a simplified SFI indication framework.

- Semi-static BS side SBFD should natively be supported.

- Time-frequency domain configuration, e.g., subband-level TDD UL/DL configuration, shall be further considered.

- Flexible SBFD change in terms of subband bandwidth can be further studied in 6GR.

- 6GR frame structure configuration considering flexible uplink and downlink paring should be studied.

- Time-domain scheduling in 6GR with flexible duration and flexible location should be further studied.

- 6GR defines CRB and PRB similar to NR.

- References

RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO, Prague, Czech Republic, June 9-13, 2025

R1-2508739, “Energy Saving Consideration for 6G”, Huawei, HiSilicon, 3GPP TSG RAN1#123, Dallas, USA, Nov. 17 – 21, 2025

R1-2505184, “Numerology and frame structure for 6GR air interface”, Huawei, HiSilicon, 3GPP TSG RAN1#122, Bengaluru, India, August 25-29, 2025

R1- 2507057, “Overview of 6GR air interface”, Huawei, HiSilicon, 3GPP TSG RAN1#122b, Prague, Czech Republic, October 13th – 17th, 2025