3GPP TSG RAN WG1 #123		                        R1-2508835

Dallas, USA, Nov 17th-21st, 2025

Source: 	Transsion Holdings

Title:	Discussion on D2R transmission

Agenda item:	10.3.2.2

Document for:	Discussion & Decision

Introduction

In RAN1#122bis meeting, the following agreements[1] on D2R transmission were achieved:

Agreement Study necessity and feasibility of D2R CDM(A) for Device 2b and for Device C with sequence-based Msg1 transmission and potential impact to random access procedures. - FFS: other cases, except for data transmission Agreement The study assumes single-carrier waveform for D2R. - FFS whether to use 2SB or 1SB for D2R Agreement Regarding D2R preamble for PDRCH, study the following aspects of the D2R preamble. - Preamble length (e.g., longer than Rel-19 D2R preamble) - Sequence type - Number of sequences using the same length (e.g., for reader/device differentiation) - Considering other factors affecting preamble design (e.g., D2R modulation and detection schemes) Agreement Regarding D2R midamble for PDRCH, study the following aspects of the D2R midamble. - Midamble pattern(s) including the length(s) and interval(s) within (and possibly at the end of) a PDRCH - Sequence type, and whether the same or different sequence type is used for D2R preamble and D2R midamble - Considering other factors affecting midamble design (e.g., D2R modulation and detection schemes) Agreement Study interleaving for PDRCH with FEC, including the necessity and feasibility of the following: - Alt 1: reuse both LTE sub-block interleaver and LTE bit collection scheme - Alt 2: reuse LTE bit collection scheme without LTE sub-block interleaver - Alt 3: based on LTE bit collection scheme with some update (e.g., for memory reduction) without LTE sub-block interleaver - Combination of alternatives can be studied Agreement Study interleaving for PRDCH with FEC, including the necessity and feasibility of the following: - Alt 1: reuse both LTE sub-block interleaver and LTE bit collection scheme - Alt 2: reuse LTE bit collection scheme without LTE sub-block interleaver - Alt 3: based on LTE bit collection scheme with some update (e.g., for memory reduction) without LTE sub-block interleaver - Combination of alternatives can be studied Agreement For PDRCH, further study the necessity of scrambling with following details: - Scrambling applied after block-level repetition (if applied) and channel coding (if applied, including interleaving if supported) - Scrambling sequence type and generation method Agreement Regarding D2R modulation for device 2b and device C, further study the following: - Option 1: BPSK - FFS the corresponding preamble and midamble design for coherent detection at reader side - Option 2: OOK - FFS the corresponding preamble and midamble design for coherent or non-coherent detection at reader side - Option 3: DBPSK - FFS the corresponding preamble and midamble design for non-coherent detection at reader side - Option 4: MSK - FFS the corresponding preamble and midamble design for coherent or non-coherent detection at reader side Agreement For R2D and D2R, study the order of channel coding (including interleaver if supported) and block-level repetition (if supported) for complexity reduction, considering necessity and feasibility.

In this contribution, we provide our views on D2R transmission for Device 2b/C.

Discussion

Modulation

In RAN1#122bis meeting, the following agreement was reached.

Agreement Regarding D2R modulation for device 2b and device C, further study the following: - Option 1: BPSK - FFS the corresponding preamble and midamble design for coherent detection at reader side - Option 2: OOK - FFS the corresponding preamble and midamble design for coherent or non-coherent detection at reader side - Option 3: DBPSK - FFS the corresponding preamble and midamble design for non-coherent detection at reader side - Option 4: MSK - FFS the corresponding preamble and midamble design for coherent or non-coherent detection at reader side

- Firstly, for BPSK, its strong resistance to noise and multipath fading makes it well-suited to meeting the long-distance coverage requirements of Rel-20 outdoor scenarios. Furthermore, it has higher spectral efficiency compared to OOK. However, BPSK performance depends on accurate CFO estimation and compensation; therefore, insufficient CFO estimation will significantly degrade BPSK performance.

- Secondly, for OOK, since it supports envelope detection, it’s insensitive to the impact of CFO. This means OOK is more robust than BPSK under large CFO. In addition, OOK has a simple implementation architecture and low power consumption, making it suitable for low power devices. However, square wave OOK introduces high odd harmonic interference, requiring the addition of guard bands or the introduction of filters to suppress adjacent-channel interference, which significantly reduces resource utilization.

- Thirdly, for DBPSK, since its detection process relies on comparing the phase differences between adjacent bits, which inherently provides robustness against CFO. This means it maintains stable performance even under large CFO conditions. However, since the differential demodulation process amplifies noise power, DBPSK is quite sensitive to noise under low signal-to-noise ratio conditions.

- Finally, for MSK, it achieves theoretical bit error rate (BER) performance comparable to that of BPSK, while occupying a narrower bandwidth and offering higher spectral efficiency. However, similar to BPSK, MSK remains highly sensitive to CFO. Furthermore, the hardware implementation complexity of MSK is significantly higher than that of BPSK, which also diminishes its advantages to some extent..

- In summary, considering the complexity of device’s RF architecture together with sensitivity to CFO and noise, BPSK remains the preferred choice for Rel-20 outdoor coverage scenarios.

- Proposal 1  It is recommended that BPSK can be considered as the D2R modulation scheme.

FEC

In RAN1#122 meeting, the following agreement was achieved.

Agreement Study necessity and feasibility of enhancements on existing Rel-19 D2R FEC for Device 2b and for Device C including the following aspects: - additional FEC code rate(s) (e.g., 1/2, 1/4) - interleaver, or LTE bit collection scheme for TBCC - Note: no change to Rel-19 TBCC polynomial.

In Rel-20, the primary deployment scenarios are outdoor environments with longer transmission distances, therefore, the requirements for link coverage and communication reliability are significantly higher. To address these requirements, it is necessary to lower the current tail-biting convolutional coding (TBCC) rate of 1/3 supported in Rel-19 D2R, for example, by adopting a 1/4 coding rate. Although a lower coding rate decreases effective data throughput, the additional redundancy can significantly enhance link robustness. In particular, under long-distance transmission or low signal-to-noise ratio (SNR) conditions, this robustness improvement can effectively boost decoding performance. In conclusion, it is recommended that Rel-20 D2R support lower FEC coding rates, such as 1/4.

- Proposal 2  It is recommended that Rel-20 D2R support lower FEC coding rates, such as 1/4.

Repetitions

In RAN1#122 meeting, the following agreement was achieved.

Agreement For PDRCH, study necessity and feasibility of larger (>2) number of repetitions for block-level repetitions for Device 2b and for Device C.

Firstly, in Rel-19, D2R has adopted block-level repetition as the repetition scheme. Therefore, Rel-20 doesn't need to study another new repetition scheme, and can directly reuse the same scheme as Rel-19, thereby reducing the workload involved in specification discussions.

Secondly, as the primary deployment scenario for Rel-20 shifts from indoor to outdoor, and the target coverage distance increases significantly to 500 meters, the system must maintain stable and reliable communication over long distances and under low signal-to-noise ratio conditions. To achieve this goal, the most direct and effective approach is to increase the repetition factor in D2R, for example, by setting the block-level repetition factor to greater than 2.

In conclusion, to meet Rel-20’s objective for outdoor long distance coverage, it is recommended to adopt a larger block-level repetition factor in D2R.

- Proposal 3  It is recommended that Rel-20 D2R could adopt a larger block-level repetition factor .

Scrambling

In RAN1#122bis meeting, the following agreement was achieved.

Agreement For PDRCH, further study the necessity of scrambling with following details: - Scrambling applied after block-level repetition (if applied) and channel coding (if applied, including interleaving if supported) - Scrambling sequence type and generation method

To enhance interference randomization and reduce the collision probability between transmissions from different devices or between different transmission blocks of the same device, it is recommended to support scrambling in PDRCH.

- Proposal 4  It is recommended that scrambling in PDRCH can be considered .

SFS

In the frequency domain resource allocation for D2R transmission, Device 2b/C can internally generate RF signals directly, without relying on a carrier wave or additional frequency shift operations. Therefore, the small frequency shift factor in the specification is no longer necessary for Device 2b/C.

Proposal 5  It is recommended that Device 2b/C is not required to support the small frequency shift factor.

Conclusions

In this contribution,some views on D2R transmission for Device 2b/C are discussed, and the following proposals are made.

- Proposal 1  It is recommended that BPSK can be considered as the D2R modulation scheme.

- Proposal 2  It is recommended that Rel-20 D2R support lower FEC coding rates, such as 1/4.

- Proposal 3  It is recommended that Rel-20 D2R could adopt a larger block-level repetition factor .

Proposal 4  It is recommended that scrambling in PDRCH can be considered .

Proposal 5  It is recommended that Device 2b/C is not required to support the small frequency shift factor.

References

- “Final Report of 3GPP TSG RAN WG1 #122-bis v1.0.4”, RAN1#122bis