3GPP TSG RAN WG1 #123	R1-2508924

Dallas, USA, Nov 17th – 21st, 2025

Agenda item:	9.2

Source: 	Fujitsu

Title:	UE features for Msg.3 repetition in SBFD operation

Document for:		Discussion and decision

Introduction

This contribution will discuss UE features for Msg.3 repetition in SBFD operation.

Discussion

In RAN1#122bis meeting, it was agreed to down-select one of the 3 alternatives as below.

Agreement: For Msg.3 repetition in SBFD symbols, RAN1 to down-select one of the following alternatives below in RAN1#123: - Alt-1: A new FG for Msg.3 repetition in SBFD symbols is introduced as follows: 60-6; Msg.3 repetition in SBFD symbols; Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI in SBFD symbols with separate power control parameter; 60-3 or 60-4, 30-6; YES; n/a; Msg.3 repetition in SBFD symbols is not supported; Per Band; TDD only; n/a; n/a; Optional with capability signalling. - Alt-2: Adopt the following updates in FG 60-3 and 60-4 - Add a new component: “X. Support of Msg.3 repetition in SBFD symbols” - Add in Note column: “Component X applies only when the UE reports FG 30-6 in the same band” - Alt-3: Neither new component nor FG is added for support of Msg.3 repetition in SBFD symbols

For a cell configured with second ROs (additional ROs), RO type switching in a random-access procedure may be enabled. When RO type switching is enabled, the UE would switch from first ROs (legacy ROs)/second ROs to second ROs/first ROs after a certain number of RACH attempts.

In RAN2#131 meeting, the following agreement was made for the case of RO type switching. In RO type switching, the UE selects the set of resources associated with the same feature or feature combination.

In RO type switching, for the other RO type, UE can select the set of Random Access resources associated with the same feature or feature combination, and with higher Msg1 repetition number, if the set with the same Msg1 repetition number is not available.

According to the agreement, for Msg.3 repetition, if the UE initially selects a set of resources associated with Msg.3 repetition in the first ROs, after RO type switching to the second ROs, the UE will select a set of resources associated with Msg.3 repetition in the second ROs, as illustrated in Figure 1, and vice versa.

Figure 1. example of selection of set of RA resources in case of RO type switching

Therefore, a SBFD-aware UE supporting FG 60-3 or 60-4 should support both Msg.3 repetition with Configuration 1 and the legacy Msg.3 repetition or not support either. Otherwise, assuming that the UE only supports Msg.3 repetition with Configuration 1 or the legacy Msg.3 repetition, if the UE initially selects a set of resources associated with Msg.3 repetition for a RO type, after RO type switching, the UE would be not able to select the set of resources associated with Msg.3 repetition for the other RO type.

Proposal 1: A SBFD-aware UE supporting FG 60-3 or 60-4 shall either support both Msg.3 repetition with Configuration 1 and the legacy Msg.3 repetition, or support neither of them.

With the above, some restrictions should be specified for UE features for Msg.3 repetition in SBFD operation. Besides, we think ‘Msg.3 repetition with Configuration 1’ is more appropriate than ‘Msg.3 repetition in SBFD symbols’, because the legacy Msg.3 repetition may also be scheduled in SBFD symbols which are semi-statically configured as flexible.

To sum up, we have the following proposal.

Proposal 2: Select one of the updated alternatives below. Alt-2 is slightly preferable for simplicity and signaling overhead saving.

- Alt-1: A new FG for Msg.3 repetition in SBFD symbols with Configuration 1 is introduced as follows:

| 60-6 | Msg.3 repetition in SBFD symbols with Configuration 1 | Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI in SBFD symbols with Configuration 1 with separate power control parameter | 60-3 or 60-4, 30-6 | YES | n/a | Msg.3 repetition is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - Add in Note collum: “If a UE supports FG 60-3 or 60-4, the UE does not support FG 30-6 without support of Msg.3 repetition with Configuration 1.”

- Alt-2: Adopt the following updates in FG 60-3 and 60-4

  - Add a new component: “X. Support of Msg.3 repetition in SBFD symbols with Configuration 1”

  - Add in Note column: “Component X applies only when the UE reports FG 30-6 in the same band”

- Alt-3: Neither new component nor FG is added for support of Msg.3 repetition in SBFD symbols with Configuration 1

  - Add in Note column of any related FG: “If a UE supports FG 60-3 or 60-4, the UE also supports Msg.3 repetition with Configuration 1 when the UE supports FG 30-6 in the same band.”

Conclusion

Based on the discussion above, we have the following proposals.

Proposal 1: A SBFD-aware UE supporting FG 60-3 or 60-4 shall either support both Msg.3 repetition with Configuration 1 and the legacy Msg.3 repetition, or support neither of them.

Proposal 2: Select one of the updated alternatives below. Alt-2 is slightly preferable for simplicity and signaling overhead saving.

- Alt-1: A new FG for Msg.3 repetition in SBFD symbols with Configuration 1 is introduced as follows:

| 60-6 | Msg.3 repetition in SBFD symbols with Configuration 1 | Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI in SBFD symbols with Configuration 1 with separate power control parameter | 60-3 or 60-4, 30-6 | YES | n/a | Msg.3 repetition is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - Add in Note collum: “If a UE supports FG 60-3 or 60-4, the UE does not support FG 30-6 without support of Msg.3 repetition with Configuration 1.”

- Alt-2: Adopt the following updates in FG 60-3 and 60-4

  - Add a new component: “X. Support of Msg.3 repetition in SBFD symbols with Configuration 1”

  - Add in Note column: “Component X applies only when the UE reports FG 30-6 in the same band”

- Alt-3: Neither new component nor FG is added for support of Msg.3 repetition in SBFD symbols with Configuration 1

  - Add in Note column of any related FG: “If a UE supports FG 60-3 or 60-4, the UE also supports Msg.3 repetition with Configuration 1 when the UE supports FG 30-6 in the same band.”