3GPP TSG RAN WG1 #123                                                               R1-2509183

Dallas, USA, Nov 17th – 21st, 2025

Source:	Sharp

Title:	Discussions on other aspects for NR air interface enhancements

Agenda Item:	10.1.1.2

Document for:	Discussion and Decision

Introduction

The Rel-20 work item on AI/ML for NR air interface enhancements [1] has the following objectives to provide specification support for CSI feedback enhancement .

CSI feedback enhancement, encompassing (two-sided model) [RAN1, RAN2]: - CSI spatial/frequency compression without temporal aspects (“Case 0”), - Specify necessary signalling/mechanism(s) as per the identified potential specification impacts in FS_NR_AIML_Air , including: - Model pairing procedure including ID and applicability reporting - Inference aspects including target CSI type, measurement and report configuration, CQI RI determination, payload determination, quantization configuration codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports - NW and UE side data collection for training, - Target CSI format - Note The framework defined in BM and CSI prediction use cases could be reused - Specify performance monitoring

In this contribution, we discuss specification support for aspects on NW and UE data collection for training, performance monitoring.

Discussions

Data collection for training

NW side data collection

In previous RAN1 meeting, agreements regarding the NW side data collection were reached as below.

Agreement (RAN1#122) For NW side data collection of CSI compression, discuss the content and format of the collected data by assuming higher layer based signalling. - Details on signaling is up to RAN2. Agreement(RAN1#122) For NW side data collection, study the solution for quantizing Target CSI, from the aspects of overhead, quantization loss/performance, and complexity. E.g.,: - Option 0: Reusing legacy e-Type II codebook - Option 1: Scalar quantization to the Target CSI. FFS number of bits for real/imaginary, whether coefficients for quantization are obtained from transformation of Target CSI. - Option 2: Enhanced methods to reduce the SVD/EVD decomposition complexity, reduce frequency/spatial/beam domain basis searching, with potentially relaxed overhead, e.g., Type II like codebook with scalar quantized W2 similar to Option 1 scalar quantization. - Option 3: Enhanced method for improved overhead saving, e.g., reporting the dominant eigenvalues/eigenvectors of per layer precoding matrix. - Option 4: eType II like codebook with new parameter values (e.g. larger L, pv, beta, amplitude, phase) - Note: the quantizaton format should consider the maximum payload size per reporting (e.g., 9KBytes) of the current higher layer signaling. - Note: At least take precoding matrix as Target CSI type. FFS channel matrix. - FFS: EVM, metric, benchmark (e.g., FP32, eT2 with PC6/8), by reusing R18 study EVM as baseline. Agreement (RAN1#122bis) For NW side data collection with higher layer reporting, for further study of Option 1 (scalar quantization), consider the following definition for evaluation: - Content: for a number of S=1 (as baseline) Target CSI samples each with the dimensions of 2N1N2 ports, N3 subbands, v layers for precoding matrix, and real+imaginary (or amplitude+phase) parts, which has overall 2N1N2*N3*v complex elements, and any complex element has a real (or amplitude) part of x1 and an imaginary (or phase) part of x2. - Note: Companies to report whether real value and imaginary value are quantized, or amplitude value and phase value are quantized. - Format: x1 is quantized to k1 bits within range of [T1min, T1max], and x2 is quantized to k2 bits within range of [T2min, T2max]. E.g., for Scalar8, k1=8 bits for quantizing the real part, and k2=8 bits for quantizing the imaginary part. Uniform quantization is assumed. - Note: Companies to report whether/how the normalization is performed, and whether it is normalized within or across 2N1N2 ports/ N3 subbands/ v layers/ real and imaginary (or amplitude and phase) parts. - Fixed parameter values of k1/k2/T1min/T1max/T2min/T2max across complex elements, as a baseline for evaluations. Companies to report the values. - Note: It is up to company to use number of samples S>1 in their evaluations. In this case, companies need to provide information how S samples are quantized in their evaluation. Agreement (RAN1#122bis) For NW side data collection with higher layer reporting, regarding Option 4 (eType II like codebook with new parameter), further study the following candidate enhanced parameter values with potential down selection. - L= 4/6/12/16/(N1*N2), pv=0.6/0.75/1, Beta = 0.5/0.8/1. Reference amplitude = 4bits/6bits, differential amplitude = 3bits/4bits, and phase = 4bits/6bits - Company to report the quantization step for reference amplitude for 6bits. - No restriction to the total number of non-zero coefficients for a specific rank. - Other combinations can be considered and reported by companies. Agreement (RAN1#122bis) For the study of NW side data collection with higher layer reporting, following evaluation assumptions are considered: - Benchmark#1: FP32 (k1=k2=32 bits) - Benchmark#2: legacy eType II with PC6 (for layer 1/2/3/4), PC8(for layer 1/2) - CSI payload size: at least consider large CSI payload Z ≥ 230 bits per layer, optionally consider low/medium CSI payload size X/Y per layer. - Tx port: 32, 64, or 128 up to companies. - Subband number: 13, or 19 up to companies. - Performance metric: - Metric#1: SGCS for specific layer(s) between Target CSI of FP32 and recovery CSI, for a model trained with quantized Target CSI with a specific quantization option. - Metric#2: SGCS for specific layer(s) between the Target CSI of FP32 with its quantized version subject to a specific quantization solution - Overhead per sample and per layer SGCS results are reported per layer - Note: Complexity should be analysed. - For Option 1, consider k1+k2=4, 6 ,7, 8, 16, 32 - Other assumptions follow R18 study EVM

For NW side data collection for training, reusing legacy e-Type II codebook (option 0) is feasible via legacy CSI report. In addition to the option 0, options 1 and 4 have received majority support compared to options 2 and 3. Furthermore, evaluation assumptions for options 1 and 4 were discussed in the last meeting to assess them. From our perspective, to reduce UE implementation complexity for data collection, it is not necessary to support both options. Supporting a single option should be sufficient for NW side data collection with higher layer reporting.

Proposal 1: For CSI compression, for NW side data collection with higher layer reporting, down-selection to one of option 1 and option 4.

In legacy CSI reporting, UE autonomously selects an optimal rank by itself for CSI reporting based on the propagation channel and SINR level on the UE. For NW side data collection purpose, NW may wish to obtain target CSI at an intended rank number for the purpose of ground-truth/target CSI. Allowing the network to configure a rank information for data collection improves dataset collection for desired model training. Therefore, it is beneficial that NW can indicate rank information to a UE for data collection for training.

Proposal 2: For CSI compression, for NW side data collection for training, support that NW can indicate rank information for target CSI reporting.

UE side data collection

Regarding UE side data collection, it has been agreed to reuse the CSI-ReportConfig to configure resources in last RAN1 meeting.

Agreement (RAN1#122): To enable UE side data collection, CSI-ReportConfig can be used for configuring the resources for data collection purpose without CSI report - P-CSI-RS and /SP CSI-RS are supported

Additionally, it was agreed that 1 CPU would be used for UE side data collection, while the duration of CPU occupancy remains to be FFS.

Agreement(RAN1#122bis) For CPU counting of UE side data collection, - OCPU=1 - FFS: The CPU occupancy duration

Since the UE does not report CSI for UE side data collection, the mechanism for CPU occupancy duration can reuse the legacy CPU occupancy duration for CSI reports configured with reportQuantity set to 'none', 'none-bm-r19', or 'none-csi-r19'. Specifically, for SP CSI report on PUSCH, the CPU occupancy starts from the first symbol of earliest P/SP-CSI-RS occasion until Z3' symbols after the last symbol of the latest P/SP-CSI-RS occasion. While for AP CSI report, the CPU occupancy starts from the first symbol after the PDCCH triggering the AP CSI report until the last symbol between Z3 symbols after the first symbol after the PDCCH and Z3' symbols after the last symbol of the latest P/SP-CSI-RS occasion.

Proposal 3: For UE side data collection, CPU occupancy duration can reuse the legacy CPU occupancy rule for CSI reports  configured to 'none', 'none-bm-r19', or 'none-csi-r19'.

For UE side data collection, the reportQuantity parameter in the CSI-ReportConfig shall be set to a dedicated value to indicate that the purpose of CSI reporting is for CSI compression data collection. This dedicated value explicitly notifies the UE of the intent behind the data collection, aligning with the approach used in Rel-19 AI/ML-based Beam Management (BM) and CSI prediction, where reportQuantity is also assigned dedicated values.

Proposal 4: For data collection for UE side model, reportQuantity is set to a dedicate value to notify UE of the data collection for CSI compression.

Performance monitoring

During the study phase in Rel-19, the following agreements and conclusions related to performance monitoring of CSI compression were reached.

Agreement (RAN1#117) Further study following monitoring options in Rel-19, including the necessity and feasibility, - NW-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1) - SRS-based monitoring - UE-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the output of the CSI reconstruction model at the UE (Case 2-1) - Note: CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side. - Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI (Case 2-2) - Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI - Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model - Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook Regarding monitoring metrics: - Monitoring accuracy also includes generalization considerations, if applicable. - Complexity also includes LCM complexity, if applicable. - Monitoring overhead, latency, complexity, and accuracy analysis may have to consider using at N>1 CSI feedback occasions. - Testability of UE reported metrics Discussion may include the following aspects: - Consideration of Options 1-5 and their sub-options for alleviating / resolving the issues related to inter-vendor training collaboration - Temporal domain aspects of CSI compression - How the above monitoring approaches or combination of them may help identifying the cause (e.g., NW side, UE side, data drift) of the performance degradation Note: for UE-side monitoring, the final reported monitoring output, if specified, may be different, e.g., be further derived based on the output of the above approaches. Note: implementation-based monitoring solutions can be considered in assessing the necessity of the above monitoring approaches.

Conclusion (RAN1#120bis) For NW-side monitoring with target CSI reporting - Target CSI reporting via legacy CSI codebooks can be used for NW-side monitoring - Target CSI reporting with CSI codebook enhancement via higher-resolution parameter combination may be beneficial for improving NW-side monitoring with additional cost of complexity and overhead at UE side. Conclusion (RAN1#120bis) UE-side monitoring is feasible. Observation (RAN1#120bis) Some companies think that, at least in some UE-side monitoring options, NW-side monitoring with target CSI reporting is needed to check the reliability of UE-side monitoring reports

NW side monitoring

It was concluded in RAN1#120bis that target CSI reporting via legacy eTypeII CSI codebooks can be used for NW-side monitoring. Additional, target CSI with high resolution (option 1) or legacy eTypeII codebook with new parameters (option 4) may offer improved NW-side performance monitoring, with increased complexity and overhead on the UE side. Therefore, at least target CSI reporting via legacy CSI codebook could be adopted for NW side monitoring as baseline. Further consideration can be given to the target CSI with high resolution or the legacy CSI codebook with high resolution parameter to enhance monitoring accuracy, depending on which option is supported for NW side data collection.

Proposal 5: For NW side monitoring, target CSI reporting via eTypeII codebook can be supported as baseline.

- Support target CSI reporting with configured rank by network

For container of the target CSI reporting for NW side monitoring, L1 signalling by reusing CSI framework can be supported at least for target CSI via eType II codebook. In existing CSI report framework, periodic CSI reporting on PUCCH supports only Type I CSI and semi-persistent report on PUCCH supports only part 1 of Type II CSI feedback. Therefore, to report the target CSI with eTypeII codebook, the monitoring report should use the semi-persistent CSI report on PUSCH and aperiodic CSI report as the monitoring report type. To keep L1 overhead efficient, we should review whether certain CSI fields like CQI in existing quantities are necessary for NW side monitoring and can be omitted. And whether a new report quantity for target CSI report for the NW side monitoring is needed.

Proposal 6: For NW side monitoring, support to reuse CSI framework for monitoring report in L1 signalling to carry target CSI via eTypeII codebook.

- For monitoring report type, semi-persistent on PUSCH and aperiodic CSI report are supported.

- FFS: whether some contents in existing quantity can be omitted and whether a new report quantity is configured.

In addition, the option 1 and option 4 for NW side data collection is under evaluation for down-selection based on the simulation results and analysis. If RAN1 agrees to consider one of options 1 and 4 for NW side data collection, it may also be possible to consider the same format of NW side data collection for target CSI reporting for monitoring. In this case, higher layer signalling could be used to report the target CSI of high resolution. This approach can avoid the need of using L1 signalling, which would otherwise heavily impact RAN1 spec by requiring the definition of new report quantity, CPU resource determination, CSI omission handling, and other factors. Therefore, higher layer signalling is a proper alternative for reporting target CSI of high resolution for monitoring

Proposal 7: For NW side monitoring, consider using higher layer signalling to report target CSI of high resolution in the same format as NW side data collection, provided that one of options 1 and 4 was agreed for NW side data collection.

UE side monitoring

For UE-side monitoring, it was concluded in RAN1#120bis that UE side monitoring is feasible, leaving potential solutions on how to implement the UE-side monitoring as below which were made in RAN1#117.

Agreement (RAN1#117) Further study following monitoring options in Rel-19, including the necessity and feasibility, - NW-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1) - SRS-based monitoring - UE-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the output of the CSI reconstruction model at the UE (Case 2-1) - Note: CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side. - Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI (Case 2-2) - Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI - Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model - Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook

The first option (i.e., Case 2-1) is that the UE runs an actual/reference/proxy CSI reconstruction model to derive the monitoring output. The Case 2-1 may achieve high monitoring accuracy especially when the actual CSI reconstruction model used at the NW-side can be deployed at UE side. It also keeps low UL overhead since the performance metric (e.g., SGCS) can be reported. The Case 2-1 may best match NW behaviour if the reconstruction model is ensured. While the trade-off is increased UE complexity to run the decoder in terms of additional computation and memory.

The second and third options are that the UE computes an intermediate KPI (SGCS or others) via estimation without reconstructing a target CSI. These options achieve low UL overhead for performance metric reporting. However, the determination of the intermediate KPI is up to UE implementation, and the monitoring accuracy would be concerned given the accuracy depends on the UE side estimator, especially for the lower-tier devices.

The fourth option is that the NW transmits the precoded RS to reflect the reconstructed CSI. Then the UE derives the monitoring results from measurement on the precoded RS. This option requires the precoded CSI-RS transmission should be dedicated to a specific UE. When many UEs support the CSI compression functionality in a cell, it leads to spectral inefficiency and scheduling complexity. In addition, the correlation between the monitored target and the precoded RS can be degraded by channel aging.

The last option is that the NW will explicitly transmit the reconstructed CSI. This option will require less processing complexity at the UE while require additional DL transmission. The storage capability is required that UE needs to buffer the measured CSI for subsequent comparison with reconstructed CSI.

Based on above analysis, each UE side monitoring option introduce trade-offs between accuracy, overhead, increasing burden on the UE side or the NW side. It is essential to assess whether the UE side monitoring is necessary, in addition to the NW side monitoring. If the UE side monitoring is supported, we need to further justify these UE side monitoring options including detailed design for specification support.

Proposal 8: For UE side monitoring, RAN1 should further conclude the necessity of supporting the UE side monitoring for CSI compression.

- If UE side monitoring is supported, study details for each option among UE-side monitoring options agreed in RAN1#117.

Conclusion

In this contribution, we have discussed our views on specification support for aspects of CSI compression on NW and UE data collection for training, performance monitoring.

Proposal 1: For CSI compression, for NW side data collection with higher layer reporting, down-selection to one of option 1 and option 4.

Proposal 2: For CSI compression, for NW side data collection for training, support that NW can indicate rank information for target CSI reporting.

Proposal 3: For UE side data collection, CPU occupancy duration can reuse the legacy CPU occupancy rule for CSI reports  configured to 'none', 'none-bm-r19', or 'none-csi-r19'.

Proposal 4: For data collection for UE side model, reportQuantity is set to a dedicate value to notify UE of the data collection for CSI compression.

Proposal 5: For NW side monitoring, target CSI reporting via eTypeII codebook can be supported as baseline.

- Support target CSI reporting with configured rank by network

Proposal 6: For NW side monitoring, support to reuse CSI framework for monitoring report in L1 signalling to carry target CSI via eTypeII codebook.

- For monitoring report type, semi-persistent on PUSCH and aperiodic CSI report are supported.

- FFS: whether some contents in existing quantity can be omitted and whether a new report quantity is configured.

Proposal 7: For NW side monitoring, consider using higher layer signalling to report target CSI of high resolution in the same format as NW side data collection, provided that one of options 1 and 4 was agreed for NW side data collection.

Proposal 8: For UE side monitoring, RAN1 should further conclude the necessity of supporting the UE side monitoring for CSI compression.

- If UE side monitoring is supported, study details for each option among UE-side monitoring options agreed in RAN1#117.

References

RP-251870 “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements”, RAN#108, Qualcomm.