3GPP TSG RAN WG1 Meeting #123		R1- 2509380

Dallas, USA, 17th – 21st November 2025

Agenda item:	10.3.2.1

Source: 	Sequans

Title: 	On AIoT air interface R2D design for Rel.20 Device 2b/C

Document for:	Discussion and Decision

- Introduction

The Rel.20 study item for Ambient IoT was approved in RAN#108 and SID was revised in RAN#109 [1]. The general scope is focused on Device 2b and Device C for outdoor. This contribution provides further discussion and suggestions on air interface enhancements for Rel.20 Ambient IoT, focusing on R2D design.

# R2D design

## Waveform and Modulation

In RAN1#122, the following was agreed:

Agreement

For R2D, study necessity of addition/removal of M values from Rel-19 M value set for Device 2b and for Device C for at least the following M values:

- Addition of M=1

- Addition of M=32

- Removal of M=24

Outdoor channels (e.g., TDL-C, 300 ns) introduce inter-chip/ISI that can raise BLER floors above 1% and coverage degradation as M grows. Larger chip duration mitigates multipath delay-spread, provides better time diversity, and improves energy collection per bit. Hence, considering M=1 is technically justified for outdoor PRDCH with weak-signal conditions. Conversely, very large  M (24, 32) make correlation/detection more fragile for low-power receivers and may not meet 50-500 m coverage target without heavy repetition or FEC. The Rel-19 minimum-BW mapping (i.e., M=2→1 RB, M=6→1 RB, M=12→2 RB, M=24→3 RB) also indicates that pushing M upward increases occupied BW without proportional robustness gains in outdoor. Since robust coverage is more crucial than data rate enhancement in AIoT, RAN1 may prioritize study for M=1 first (with Manchester and FEC/repetition options), and treat M=24 removal and M=32 addition study as lower-priority.

Proposal 1: RAN1 prioritizes the R2D M-value study on addition of M=1 for outdoor Devices 2b/C.

Rel.19 R2D uses DFT-s-OFDM waveform with OOK-4 modulation, Manchester line coding and no channel coding, enabling basic connectivity for indoor use cases (inventory, control commands) under controlled conditions. Rel.20 shifts focus to outdoor AIoT scenarios and more capable Device Types 2b and C will require enhanced waveform/modulation to cope with certain limitations of the Rel.19 R2D design. As Rel.19 R2D waveform lacks any FEC mechanism or physical-layer repetition, in noisy outdoor channels with greater path loss, the uncoded OOK can suffer high error rates (even a single bit error causes a CRC failure with no chance of recovery). Moreover, a deep fade or strong interferer, which may be common in outdoor deployments, can erase an OOK “ON” pulse entirely. Furthermore, the basic Rel.19 OOK modulation with Manchester coding limits data rate which may be more important for active AIoT Device 2b/C. But more importantly, Rel.19 design forces any AIoT device to listen and decode the address mask in payload which is inefficient in dense deployments. These limitations combined will make the support of outdoor use cases practically unfeasible. Since Devices 2b/C should be able to handle modest increases in complexity from pure envelope detection, waveform and modulation enhancements to address aforementioned limitations should be studied.

Proposal 2: For Rel.20 R2D, study necessary enhancements to waveform and modulation to support Devices 2b/C for outdoor.

## SIP

In RAN1#122bis, the following was agreed:

Agreement

For the functionality of indicating the start of the R2D transmission that contains PRDCH, study at least the following options if the functionality relies on SIP:

- Option 1: binary sequence-based SIP (e.g M-sequence or Golay sequence).

- Option 2: Reuse the Rel-19 SIP.

A sequence-based SIP would increase device-side complexity and power consumption. On the other hand, Rel-19 SIP pattern has already demonstrated adequate detection reliability within its intended operating conditions. Considering that Rel-20 Device 2b/C types will feature improved oscillator stability and processing capability, it is reasonable to expect equal or better detection performance by re-using the existing SIP design. It is still to be seen if there is indeed a necessity for introducing a sequence-based SIP for outdoor operation which introduces higher interference levels and extended coverage requirements.

Proposal 3: For the functionality of indicating the start of the R2D transmission that contains PRDCH, support Option 2: Reuse the Rel-19 SIP.

## Chip duration determination and CAP

In RAN1#122bis, the following was agreed:

Agreement

For the functionality of chip duration determination of the data payload in the PRDCH, study the following options:

- Option 1: same as the chip duration of the corresponding L1 R2D control information

- Option 2: indicated by the corresponding L1 R2D control information

Agreement

For the functionality of chip duration determination of the L1 R2D control information, study at least the following options:

- Option 1: Fixed chip duration. No use of CAP for chip duration determination.

- Option 2: Blind detection from a predefined set of the chip durations of L1 R2D control information. No use of CAP for chip duration determination.

- Option 3: Use CAP

  - Alt.3-1 Rel-19 CAP pattern

  - Alt.3-2 Enhanced CAP

    - Alt.3-2a: Rel-19 CAP pattern with repetition

    - Alt.3-2b: CAP pattern and the association between chip durations for CAP and chip durations of L1 R2D control information

- Option 4: Use a set of different binary sequences with fixed or variable length

  - Alt.4-1: using binary sequences for SIP. No use of CAP for chip duration determination.

  - Alt.4-2: using binary sequences for CAP

- Option 5: Use broadcast information (if any) to indicate the chip duration

For the chip duration configuration of the PRDCH data payload, it is most practical to align it with that of the associated L1 R2D control information, if there is no clear benefit or need for separate configuration (as also seen in Rel.19 study). Maintaining a common M value across both segments offers multiple advantages: it simplifies receiver design, guarantees consistent link performance between control and data, and eliminates the need for additional signalling to convey a second M value.

Proposal 4: For the functionality of chip duration determination of the data payload in the PRDCH, support Option 1: same as the chip duration of the corresponding L1 R2D control information

Regarding the selection of chip duration for the L1 R2D control information, Option 1 suggests a fixed chip duration which can limit transmission adaptability across different scenarios (choosing fixed smaller M would reduce the overall system throughput, whereas fixed larger M can degrade R2D reception for cell-edge devices). Option 2 relies on blind detection which increases the detection complexity at device. Option 5 limits flexibility as it only allows indication through broadcast information. Options 3 and Alt.4-2 of using CAP are preferred as they offer a balance between detection complexity and performance. If there is no clear benefit or need for new CAP patterns, Alt.3-1 of Rel-19 CAP pattern should be the most suitable approach for determining chip duration for L1 control information (and PRDCH data payload, if same configuration is agreed).

Proposal 5: For the functionality of chip duration determination of the L1 R2D control information, support Alt.3-1: Rel-19 CAP pattern.

## CFO calibration signal

In RAN1#122, the following was agreed:

Agreement

For R2D signal(s), study at least the following functionalities (if needed), and study whether to reuse/enhance existing R2D signal(s) or to introduce new R2D signal(s) for the following functionalities (if needed):

- R2D chip duration determination

- indication of the start of R2D

- indication of the end of PRDCH

- frequency synchronization (including frequency acquisition)

- CFO estimation / LO calibration

- SFO calibration

- timing synchronization/tracking

- measurement (e.g., signal strength)

- reader identification/differentiation

In RAN1#122bis, the following was agreed:

Agreement

For the SFO and CFO in LLS assumption of Device C, update the previous RAN1 agreement (in RAN1#122) as follows,

Agreement

For Rel-20 study, SFO and CFO in LLS assumption for evaluation purpose is from the followings:

- Device C is smaller than Device 2b

|  | Device 2b | Device C |
| --- | --- | --- |
| Initial SFO | Randomly select a value from the range of 10^3~10^4 ppm | [50] ppm |
| SFO after clock sync/calibration | 10^2 (O), 10^3 (M) ppm | [10] ppm |
| Initial CFO | Randomly select a value from the range of 10^3~10^4 ppm | [50] ppm |
| CFO after clock sync/calibration | 100 (M), 200 (O), 50 (O) ppm | [10] ppm |

Agreement

For the functionality of CFO estimation/LO calibration at the device side, study CFO calibration signal with the following aspects:

- Whether the signal is

  - Option a: unmodulated sinusoid single tone

  - Option b: unmodulated multiple sinusoid single tones

  - Option c: modulated multiple tones

    - Whether the CFO calibration signal is the same or different from the synchronization signal (if supported)

- Whether the CFO calibration signal is transmitted periodically and/or as needed (aperiodically)

- The time location(s) of CFO calibration signal.

  - Option 1: in relation to the L1 R2D control information

  - Option 2: in relation to the data payload of the PRDCH

  - Option 3: in relation to the PDRCH

  - Option 4: in relation to the synchronization signal (if supported)

  - Option 5: in relation to broadcast information (if supported)

  - Combination of Options is not precluded

Other details (e.g., time duration, frequency location(s) of the CFO calibration signal)

Single unmodulated sinusoidal tone provides the most practical and efficient approach for CFO calibration in R2D. A simple pure continuous wave segment can be considered to calibrate the device’s carrier frequency. The reader can transmit for a brief interval an unmodulated single-tone signal on the assigned PRB(s). An active device 2b/C can use its narrowband filter to extract this signal and mix it down with its LO to measure the frequency error. Then, LO can be effectively adapted before its own transmission and ideally also before reception. This enables highly reliable frequency offset estimation even under large oscillator mismatches (hundreds to thousands of ppm), while maintaining minimal receiver complexity and energy consumption. The constant-amplitude single-tone allows simple phase-based CFO estimation without correlation or symbol detection.

In contrast, multiple unmodulated tones increase PAPR and require FFT-based tone separation, raising both transmitter and receiver complexity. Modulated multiple tones further complicate synchronization and demands symbol-level demodulation. such a composite, chopped by ON/OFF gating, multi-tone chunk faces multi-tone beating, edge transients at every ON/OFF (phase jitter), and unknown per-ON phase contributions from mapping/windowing and the channel. Knowing the envelope transitions will not give the per-subcarrier phase reference needed.

Proposal 6: For CFO calibration signal, support Option a: unmodulated sinusoid single tone.

Another important question is whether this signal should be periodic, on-demand, or part of every R2D transmission. Including it as part of every R2D transmission would be inefficient. Regarding the on-demand case, with a CFO drift of 0.1-1 ppm/sec (as considered in TR 38.769), the accumulated CFO over time will be a small fraction of even the calibrated 100/10ppm CFO. A long R2D transmission, e.g. for commonly considered 96 bits PRDCH and M=2, will be performed in a bit less than 10ms. After calibration within a 10ms R2D transmission, the CFO drift will represent max 0.01% of the 100ppm for Device 2b, and max 0.1% of 10ppm for Device C. 1 second after the calibration, the CFO drift will represent max 1% of the 100ppm for Device 2b, and max 10% of 10ppm for Device C. Therefore, a relatively sparse and periodic transmission of the CFO calibration signal should make more sense, for devices to not monitor for trigger signaling as well as for reduced signalling overhead. Periodicity could be considered in the order of 100s of ms, to support also calibration for DO-A traffic case (i.e., device initiates D2R transmission after fast fix from such periodic R2D) without introducing large latency penalty.

Proposal 7: For CFO calibration signal, support periodic transmission.

An additional important factor is the location of the signal. CFO calibration signal is mainly important for supporting D2R transmission. Active Devices 2b/C will generate internally the D2R transmission, not relying on backscattering, and may implement architectures with independent transmit/receive oscillators. In that case, the initial CFO to be used for subsequent D2R can be off by hundreds of kHz or even MHz-level without high-precision crystals which are prohibitive for those devices. Furthermore, CFO calibration signal can be useful for R2D reception. Devices 2b/C can use the calibrated LO and their narrow filters to improve the reliability of PRDCH reception. According to the architecture definitions of Device 2b and C in TR 38.769, those devices will have the capability to track the R2D carrier frequency via their IF/ZIF narrowband receivers. Especially if FD multiplexing is eventually supported for R2D (as done in Rel.19 D2R), calibrated LOs of Devices 2b/C will mean reduced guard-bands among multiplexed R2D transmissions and increased R2D coverage and capacity.

Observation 1: For Rel.20 R2D, a CFO calibration signal is useful to support D2R and R2D transmissions for Devices 2b/C.

Having a separate CFO calibration signal transmission will be inefficient for the ultra-low-power AIoT devices (extra monitoring, wake/sleep process). The way forward should be to support transmission of the signal together with conventional R2D messages.

Observation 2: It is preferable for Rel.20 R2D design to include CFO calibration signal together with PRDCH transmission.

Possible clearly distinguished options for CFO calibration signal location within R2D transmission include: (a) before R-TAS; (b) before PRDCH or L1 R2D control information; (c) after PRDCH or postamble. Case (a) has the strong advantage that Device 1 reception is not affected as it will not consider any samples before SIP detection. The disadvantage however is that Device 2b/C needs to be monitoring before R-TAS and consume power (e.g., to at least buffer samples). Case (b) advantage is that CFO calibration signal can still be used for reception of PRDCH. Case (c) main disadvantage is that CFO calibration signal cannot then be used for the preceding PRDCH reception.

One other important parameter affecting efficiency and accuracy of the CFO calibration process at LO, is the received power of the single-tone signal. It should be studied further what calibration levels can be feasible in considered outdoor scenarios, whether signal fading causes issues at cell edge, and whether longer signal durations can amplify the signal strength adequately.

Regardless of the received power conditions, the signal’s exact duration must be specified so as its time edges are clearly known to the device for robust CFO calibration. The total signal duration shall be considered such as it also satisfies the targeted calibration accuracies (e.g., 100/10 ppm for Device 2b/C) in all reasonable outdoor scenarios for Rel.20 AIoT.

The design of CFO calibration signal should be studied further to identify an optimal design considering all above aspects.

Proposal 8: Further study the design of a single-tone CFO calibration signal, considering aspects of:

- received power at cell edge

- duration

- periodicity

- location within R2D transmission

- R2D backwards compatibility for Device 1 reception

## L1 Control

In RAN1#122bis, the following was agreed:

Agreement

Study L1 R2D control information with separate CRC for Device 2b and for Device C considering the following aspects

- Necessary information to be included in the L1 R2D control

- Details on how the L1 R2D control information is transmitted

Agreement

For the L1 R2D control information preceding and scheduling the data payload of PRDCH, study the following aspects:

- Time location of L1 R2D control information:

  - Option 1: L1 R2D control information and data payload of PRDCH are contiguous in time.

  - Option 2: L1 R2D control information and data payload of PRDCH are not contiguous in time.

- Study frequency resource of L1 R2D control information

- Whether L1 R2D control information is included in the PRDCH that carries that data payload, or in another PRDCH, or in another channel

Introduction of L1 control in Rel.20 AIoT shall enable the reader to convey control information related to scheduling, including TB size, resource allocation as well as FEC code rate and repetition indication (if introduced), etc. On how the L1 R2D control information is transmitted, it should be considered whether L1 control information should be sent through a PRDCH-like payload, or conveyed in an implicit manner (e.g., overlaid signal), or both. For outdoor dense deployments, it will be beneficial for example if R2D control information such as ID is detectable without decoding the entire PRDCH. This will allow devices to conserve energy by only processing self-intended transmissions.​ This is especially important considering that Devices 2b/C may require larger data rates and payload transmissions. Supporting implicit and variable-sized R2D control information ​will also ensure robustness and backward/forward compatibility. In case both options are deemed necessary, auxiliary information carried in implicit information could be used to reduce search space of PRDCH-like control and unnecessary “up” time, reducing significantly device consumption.

Proposal 9: Further study design options for conveying L1 control information with R2D transmission, including implicit and PRDCH-like control.

## Repetition

In RAN1#122, the following was agreed:

Agreement

Study necessity and feasibility of R2D repetitions for R2D for Device 2b and for Device C considering the following repetition types:

- Block-level repetitions

- Bit-level Type-2 repetitions

- Chip-level repetitions (in case of M=1 with FEC if applied)

Agreement

- For R2D and D2R, study the order of channel coding (including interleaver if supported) and block-level repetition (if supported) for complexity reduction, considering necessity and feasibility.

Repetition offers gains via non-coherent combining at the reader and provides time/frequency diversity (e.g., same-RB time repetition or light frequency-hopped repetition across adjacent RBs) while keeping device complexity and occupancy low. Block-level repetition is more straightforward to specify and clarify the gains (as in Rel.19 D2R) since it was well studied and specified for Rel.19 D2R design. Further study of bit-level and chip-level repletion could reveal exact conditions of benefits for R2D. To improve outdoor robustness without overdoing it in terms of latency and device energy consumption, limited repetition should be studied on PRDCH, e.g., N_rep in {1, 2, 4}. Higher repetition factors have diminishing returns, increase channel occupancy and reader load, and can mask design issues better handled M-value or FEC selection. Therefore N_rep > 4 should be out of scope, unless coverage targets cannot be met otherwise.

Proposal 10: Study repetitions for Rel.20 R2D, with priority on block-level and low (e.g., <=4) number of repetitions.

Regarding the order between coding and repetition, from AIoT device decoding point of view it is less complex (processing and memory) to receive repeated versions of coded blocks.

Proposal 11: For R2D transmission procedure, support FEC before repetition.

## FEC

In RAN1#122bis, the following was agreed:

Agreement

For the study of necessity and feasibility of FEC for R2D for Device 2b and for Device C, further study the following aspects:

- Coding schemes

  - LTE TBCC

  - Tailed convolutional code with the same polynomial as LTE TBCC

  - RM code

- FEC code rates: 1/2, 1/3, 1/4

The interaction between FEC and repetition for R2D for Devices 2b/C is important, as combining modest FEC coding with limited repetition may provide the best coverage-complexity trade-off. TBCC reuse minimizes standardization effort and allows smooth evolution from Rel.19 AIoT (and LTE IoT), while RM codes could offer potential gains in block-error performance for very short block sizes but at the cost of implementation complexity. Therefore, LTE TBCC code should be considered as the practical baseline for Rel.20 R2D transmission. Additional study could assess whether RM codes or modified convolutional encoders can deliver measurable coverage improvements within acceptable complexity limits.

Proposal 12: Support LTE TBCC as baseline coding scheme for Rel.20 R2D study

- Conclusions

In this contribution, we discuss necessary R2D air interface enhancements for Device 2b/C for outdoor.

Proposal 1: RAN1 prioritizes the R2D M-value study on addition of M=1 for outdoor Devices 2b/C.

Proposal 2: For Rel.20 R2D, study necessary enhancements to waveform and modulation to support Devices 2b/C for outdoor.

Proposal 3: For the functionality of indicating the start of the R2D transmission that contains PRDCH, support Option 2: Reuse the Rel-19 SIP.

Proposal 4: For the functionality of chip duration determination of the data payload in the PRDCH, support Option 1: same as the chip duration of the corresponding L1 R2D control information

Proposal 5: For the functionality of chip duration determination of the L1 R2D control information, support Alt.3-1: Rel-19 CAP pattern.

Proposal 6: For CFO calibration signal, support Option a: unmodulated sinusoid single tone.

Proposal 7: For CFO calibration signal, support periodic transmission.

Observation 1: For Rel.20 R2D, a CFO calibration signal is needed to support D2R and R2D transmissions for Devices 2b/C.

Observation 2: It is preferable for Rel.20 R2D design to include CFO calibration signal together with PRDCH transmission.

Proposal 8: Further study the design of a single-tone CFO calibration signal, considering aspects of:

- received power at cell edge

- duration

- periodicity

- location within R2D transmission

- R2D backwards compatibility for Device 1 reception

Proposal 9: Further study design options for conveying L1 control information with R2D transmission, including implicit and PRDCH-like control.

Proposal 10: Study repetitions for Rel.20 R2D, with priority on block-level and low (e.g., <=4) number of repetitions.

Proposal 11: For R2D transmission procedure, support FEC before repetition.

Proposal 12: Support LTE TBCC as baseline coding scheme for Rel.20 R2D study

# References

- RP-252964, Revised SID: Study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices, 3GPP TSG RAN Meeting #109, Beijing, China, September 15-18, 2025.

- TS 38.291, Ambient IoT Physical Layer, V19.1.0.

- TR 38.769, Study on A-IoT (Internet of Things) in RAN, V19.0.0.