3GPP TSG RAN WG1 Meeting #73	R1-132417

Fukuoka, Japan, 20th May – 24th May, 2013

Agenda Item:	6.2.5.1.2

Source:	CATR

Title:	Considerations and simulation results for downlink 256QAM

Document for:	Discussion

# Introduction

In RAN#58, “New Study Item Proposal for Small Cell Enhancements for E-UTRA and E-UTRAN – Physical-layer Aspects” was approved. One object of this SI is “Introduction of a higher order modulation scheme (e.g. 256 QAM) for the downlink [1]”. In RAN1 #72 and #72bis, simulation assumptions were approved [2]. In this contribution, we will provide our views and simulation results for 256QAM.

# Discussion

The deployment of small cell is usually for indoor and hotspot. It implies that the distance between small cell and UE is short and the possibility of high SINR is high, especially for sparse small cell deployment scenarios. 256QAM is suitable for high SINR region.

In [2], 3 typical scenarios are provided for simulation where both sparse and dense are included. For actually small cell deployment, the percentage of dense and sparse scenario is not easy to estimate. The density of small cells and users receiving SINR depends on many factors. Firstly, the available operation carrier number will affect the density of small cell using the same carrier. Small cells could choose different operation carrier to avoid intra-frequency interference. If over 100MHz bandwidth is used, like 3.5GHz, the density of small cell using the same carrier will decrease. Secondly, dynamic cell on/off will have effect on UE receiving SINR. To avoid interference and save energy, dynamic cell on/off should be applied for small cell deployment. If this technique is used, the density of small cells operating in the same carrier might be further reduced and users SINR could be improved. Thirdly, interference cancellation can improve users SINR. For small cell deployment, if many cells are located in a small area, it is easier to find out several strong interferences and make interference cancellation. Besides, the variation of channels in indoor and hotspot scenarios is slow. It is also an advantage for interference cancellation. Therefore, we propose

Proposal 1: some potential factors/techniques that have effect on SINR distribution should be considered for 256QAM evaluation.

EVM and UE receiver impairments play key role for 256QAM implement. According to the LS from RAN4[3], improved EVM values compared to existing requirement of 8% can be achieved for low power BS. It is implies that a lower EVM value for 256QAM simulation could be used. However, the effect of Rx impairment is still FFS for RAN4. We might add a maximum SINR constraint to simulate all the factors together.

# Simulation results

The design of 256QAM is Gray mapping. Simulation assumptions are listed in table 1 which is given in [2] as scenario #3(sparse). We set the EVM constraint as 4% and 6 coding rate is used.

Table 1 system level simulation assumptions on 256QAM

| Parameters | Assumption |
| --- | --- |
| Carrier frequency | 3.5 GHz |
| System bandwidth | 10 MHz |
| Distance-dependent path loss | ITU InH [referring to Table B.1.2.1-1 in TR36.814] 3D distance between an eNB and a UE is applied |
| Penetration | 0 |
| Shadowing | ITU InH [referring to Table A.2.1.1.5-1 in TR36.814] |
| Antenna Height: | 6m |
| Antenna gain + connector loss | 5dBi |
| Fast fading channel between eNB and UE | ITU InH |
| Number of UEs | 10 |
| UE dropping | Randomly and uniformly distributed over area per floor |
| Minimum distance (2D distance) | Small cell-UE: 3m |
| Traffic model | FTP model 3 |
| CRS interference | Without CRS interference |
| EVM constraint | 4% |
| 256QAM coding rate | 0.7667,0.8,0.8333,0.8667,0.9,0.93 |
| Transmission scheme | SU-MIMO with rank adaptation |
| CRS configuration | Antenna ports 0,1 |

Figure 1 shows the geometry of scenario 3(sparse) without EVM constraint. We can see, over 45% users’ geometry are over 20dB, which means the potential area for 256QAM might be wide for indoor scenario.

Figure 1 Geometry of scenario 3(sparse) without EVM constraint

Figure 2 gives the relative gain of 256QAM for cell average throughput and 5% cell edge user throughput. The baseline is system simulation results without 256QAM. The EVM constraint is 4%. From Figure 2, 256QAM will bring about 24% cell average throughput gain comparing with no 256QAM. The benefit of introducing 256QAM is quite small for cell edge users. It brings less than 1% throughput gain for 5% cell edge user.

Figure 2 Relative gain of 256QAM in scenario 3(sparse)

Obeservation 1: 256QAM will bring about 24% cell average throughput gain and less than 1% cell edge user throughput gain with 4% EVM constraint in scenario 3(sparse).

# Conclusion

In this contribution, we show the performance of 256QAM in scenario 3(sprase) and get the following proposal and observation:

Proposal 1: some potential factors/techniques that have effect on SINR distribution should be considered for 256QAM evaluation.

Obeservation 1: 256QAM will bring about 24% cell average throughput gain and less than 1% cell edge user throughput gain with 4% EVM constraint.

Reference

- [1] RP-122032, “New Study Item Proposal for Small Cell Enhancements for E-UTRA and E-UTRAN – Physical-layer Aspects”, 3GPP RAN#58, Barcelona, Spain, Dec. 2012.

[2] R1-130856, “Evaluation assumptions for small cell enhancements-physical layer”, Huawei, HiSilicon

[3] R1-131712(R4-132019), “ LS on 256 QAM Support”, Alcatel Lucent