3GPP TSG-RAN WG1 #75 	R1-135313

November 11 – 15, 2013

San Francisco, United States

Agenda item:	6.2.7.2

Source: 	Qualcomm Incorporated

Title: 	Discussion on RSRP modeling

Document for:	Discussion/Decision

# Introduction

In order to compare the evaluation results, it is important to use the same RSRP calculation method.  In the email discussion following RAN1#74, five RSRP models were proposed:

- Alt-1: Based on LOS direction only

- Alt-2: Based on mean angles

- Alt-3: Based on angle of all clusters

- Alt-4: Based on angle of all rays of all clusters (R1-133967)

- Alt-5: Based on channel realizations H

In RAN1#74bis, these alternatives were discussed and the following was agreed [1].

- For RSRP calculations needed for UE attachment (including coupling loss calculations), all rays of all clusters shall be used for a given link between a UE and a transmission point

Clearly, both Alt-4 and Alt-5 fits the agreement.  However, the equation in Alt-4 cannot be applied with cross-polarized antenna.  In this contribution, we discuss the RSRP modeling with cross-polarized antennas.

# RSRP modeling with cross-polarized antennas

According to the agreed working assumption [2], the ITU channel coefficient generation procedure [3] shall be reused.  The impulse response of the fast fading channel from one eNB antenna to one UE antenna is given by

|  | hτ;t=n=1NPnMm=1MFϑφnm,ϑnmFφφnm,ϑnmTejΦnmϑθκejΦnmϑϕκejΦnmφθejΦnmφϕFθϕnm,θnmFϕϕnm,θnmejνnmtδ(τ-τn), | (1) |
| --- | --- | --- |

where Pn is the power of the nth cluster; τn is the excess delay of the nth cluster; ϕnm, θnm, φnm and ϑnm denote the AoD, EoD, AoA, and EoA of the mth ray in the nth cluster, respectively;  Φnmϑθ, Φnmϑϕ, Φnmφθ, Φnmφϕ are the initial phases, νnm denotes the Doppler shift contributed by the mth ray in the nth cluster;  κ is the inverse of the XPD;  Fθ and Fϕ are the eNB antenna field pattern, Fϑ(⋅,⋅) and Fφ(⋅,⋅) are the UE antenna field pattern.  Note that the antenna field pattern is the effective antenna pattern of the CRS port which is determined by the element pattern and the CRS virtualization weight.

Given a subcarrier at frequency f, the frequency channel response is

|  | Hf;t=0∞hτ;te-j2πfτdτ=n=1Nm=1MPnMαnmejνnm-2πfτn | (2) |
| --- | --- | --- |

where

|  | αnm=αnmϑθejΦnmϑθ+καnmϑϕejΦnmϑϕ+αnmφθejΦnmφθ+αnmφϕejΦnmφϕ, αnmϑθ=Fϑφnm,ϑnmFθϕnm,θnm, αnmϑϕ=Fϑφnm,ϑnmFϕϕnm,θnm, αnmφθ=Fφφnm,ϑnmFθϕnm,θnm, αnmφϕ=Fφφnm,ϑnmFϕϕnm,θnm. | (3) |
| --- | --- | --- |

The averaged channel gain over the time range [t0,t0+T] is

|  | Gf;T=1Tt0t0+THf;tHf;t*dt =1Tn=1Nn'=1Ne-j2πfτn-τn'm=1Mm'=1MPnPn'Mαnmαn'm'*t0t0+Tejνnm-νn'm'tdt. | (4) |
| --- | --- | --- |

The second equation holds under the assumption that νnm≠νn'm' for n≠n' or m≠m'.  When T goes to infinity, we have

|  | G=limT→∞G(f;T)=n=1Nm=1MPnMαnm2 | (5) |
| --- | --- | --- |

Hence, the long-term RSRP can be modeled as

|  | RSRP=P⋅n=1Nm=1MPnMFϑφnm,ϑnmFφφnm,ϑnmTejΦnmϑθκejΦnmϑϕκejΦnmφθejΦnmφϕFθϕnm,θnmFϕϕnm,θnm2, | (6) |
| --- | --- | --- |

where P is the product of the eNB transmit power, the path loss, and the shadow fading.  Note that in practical systems, the RSRP obtained by average the CRS power over multiple CRS REs and over multiple subframes.  The average over frequency may shorten the time needed to reach convergence.

When co-polarized antenna is employed, we have

Fθϕnm,θnm=GeNBϕnm,θnm,
Fϑφnm,ϑnm=GUEφnm,ϑnm,
Fϕθnm,ϕnm=Fφφnm,ϑnm=0,

where GeNB is the CRS port gain pattern and GUE is the gain pattern of the UE antenna.  Consequently, the RSRP model (6) may be rewritten as

|  | RSRP=P⋅n=1Nm=1MPnM⋅GeNBϕnm,θnm⋅GUEφnm,ϑnm | (7) |
| --- | --- | --- |

Note that this model is identical to Alt-4.

Proposal: Adopt the RSRP model in (6) for calibration and evaluation.

# Conclusion

In summary, we propose the following in this contribution regarding the RSRP calculation with cross-polarized antennas.

Proposal: Adopt the RSRP model in (6) for calibration and evaluation.

# References

- Draft report of 3GPP TSG RAN WG1 #74bis v0.2.0.

- R1-132748, WF on introduction of elevation domain parameters in 3D channel modeling.

- 3GPP TR 36.814, “Further advancements for E-UTRA physical layer aspects.”