3GPP TSG RAN WG1 Meeting #75	R1-135829

San Francisco, America, November 11-15, 2013

Agenda Item:	6.2.6.3

Source:	CATR

Title:	Simulation results on radio-interface based synchronization

Document for:	Discussion/Decision

# Introduction

In RAN1 #74bis meeting, evaluation assumptions for radio-interface based synchronization are agreed in [1]. In this contribution, we provide simulation results for the radio-interface based synchronization.

# Discussion

We provide both link level and system level simulation results. Evaluation assumptions are listed in appendix, which follow the proposals in [1].

## Link level simulation results

Figure 1 gives the simulation results with ideal frequency provider. With ideal frequency provider, for each hop, the performance of CRS based radio-interface synchronization degraded with SINR decrease. An obviously gap happens between -10dB and -12dB.

Figure 1 Time estimation error with ideal frequency provider

Figure 2 provide the simulation results of time estimation without ideal frequency provider. The frequency estimation results are also given in figure 3. We can see, with frequency estimation error, the accuracy of time frequency decreases. Purely considering the time estimation accuracy, our observation is that

Observation 1: Link level simulations show that, for each hop, the received SINR needs to be at least -8 dB to achieve certain synchronization accuracy.

Figure 2 Time estimation results without ideal frequency provider

Figure 3 Frequency estimation results

## System level simulation results

Simulation results of system level are given in Figure 4. Both 1 and 2 floors per building in scenario 2b with different thresholds are provided. The reference timing is from the overlaid macro cell and only one small cell within each cluster is selected to acquire this reference timing. This small cell is chosen with the maximum received power of macro cells. Then other small cells will select the cell with the smallest stratum level as its source cell to perform synchronization if the SINR between the source and slave cells is higher than some certain threshold, e.g. -8dB to -20 dB. In table 1, the distribution of different synchronization stratum levels with different SINR threshold are listed. With different SINR threshold, we also observe some small cells can’t acquire synchronization. The percentage of small cells which can’t achieve synchronization from their source cells is given in the last column of table 1.

From Figure 4 and table 1, we can see there is a balance between SINR threshold and distribution. If we choose a higher SINR threshold to improve synchronization accuracy, it will lead to hop number and percentage of unsynchronized small cells increasing. Considering the simulation results of both link level and system level, we notice that it is important to satisfy the link level requirements for RIBS first. According to the simulation results in link level simulation, -8dB threshold is proposed to meet the time estimation accuracy.

Observation 2: System level simulation results show that with -8dB threshold, for 2 floor -8dB threshold case, 19.51% small cells will lose synchronization and 9.59% small cells require 5~ 6 hops to achieve synchronization.

Figure 4 SINR distribution for network listening for small cells in scenaro 2b (dense)

Table 1 Ratio of small cells with different stratum levels under different thresholds (%)

| Stratum level Threshold(dB) | 1 | 2 | 3 | 4 | 5 | 6 | unsynchronized |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1floor-8dB | 25 | 35.48 | 21.48 | 6.62 |  |  | 11.42 |
| 1floor-10dB | 25 | 43.16 | 21.29 | 3.54 |  |  | 7.01 |
| 1floor-12dB | 25 | 48.86 | 19.44 | 1.64 |  |  | 5.06 |
| 1floor-14dB | 25 | 50.88 | 18.26 | 1.24 |  |  | 4.62 |
| 1floor-20dB | 25 | 57.56 | 16.01 | 0.82 |  |  | 0.61 |
| 2floor-8dB | 12.5 | 23.43 | 22.13 | 12.84 | 6.11 | 3.48 | 19.51 |
| 2floor-10dB | 12.5 | 29.53 | 32.68 | 13.4 | 4.22 | 1.25 | 6.42 |
| 2floor-12dB | 12.5 | 32.79 | 38.72 | 10.39 | 2.43 | 0.49 | 2.68 |
| 2floor-14dB | 12.5 | 34.05 | 41.35 | 9.45 | 2.09 | 0.17 | 0.39 |
| 2floor-20dB | 12.5 | 36.08 | 42.77 | 8.23 | 0.42 | 0 | 0 |

# Conclusion

In this contribution, we provide the simulation results for RIBS. Both link level and system level simulation results are given. The following two observations are

Observation 1: Link level simulations show that, for each hop, the received SINR needs to be at least -8 dB to achieve certain synchronization accuracy.

Observation 2: System level simulation results show that with -8dB threshold, for 2 floor -8dB threshold case, 19.51% small cells will lose synchronization and 9.59% small cells require 5~ 6 hops to achieve synchronization.

# Reference

R1-134966 Evaluation assumptions for RIBS.  Huawei, HiSilicon, CATR, CATT, CMCC

# Appendix

Table 2 link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Channel bandwidth | Baseline: 10 MHz |
| Carrier frequency | Macro cell as source cell: 2 GHz Small cell as source cell: 3.5GHz |
| Channel profile | EPA with low mobility, e.g. 0.01km/h |
| Total number of subframes measured | One subframe within one measurement interval The baseline for measurement interval is 10s. |
| Time drift | Frequency synchronization error multiplied by the measurement interval. Baseline: The frequency synchronization error is assumed to be +0.1ppm. Optional: The frequency synchronization error is the estimated error if frequency synchronization is obtained via air interface. |
| Network listening RS design | Baseline: CRS for synchronization maintenance. Other types of reference signal are not precluded. Number of antenna ports for CRS: 2 |
| Performance metrics | Time synchronization error for single hop under different SINR; Frequency synchronization error for single hop under different SINR for the case that frequency synchronization is obtained via air interface. |

Table 3 system level simulation assumptions

| Parameter | Value |
| --- | --- |
| Scenario | Scenario #2b (dense) with 1 or 2 floors |
| Channel model between Macro cell and small cell | Refer to channel model between Macro cell and indoor UEs; Some modification is needed on the receiver side, i.e, ‘antenna height’: replace 1.5m with 6m; ‘antenna gain + connector loss’: replace 0dBi with 5dBi ‘Noise Figure’: 9dB |
| Channel model between small cell and small cell | Refer to channel model between small cell and indoor UEs; Some modification is needed on the receiver side, i.e, ‘antenna height’: replace 1.5m with 6m; ‘antenna gain + connector loss’: replace 0dBi with 5dBi ‘Noise Figure’: replace 9dB with 7dB |
| Source cell selection | Each small cell selects a cell with acceptable received signal quality and small synchronization stratum as its source cell; Only one small cell within each cluster directly acquires the reference timing from Macro layer or GPS. This small cell can be the one which has the maximum received power of macro cells, or can be randomly selected. |
| Performance metrics | SINR distribution; Synchronization stratum distribution and Maximum hop number; The probability of the small cell cluster using more than one Macro cell to be the source cells when Case 2 of source cell selection is used. |