3GPP TSG RAN WG1 Meeting #76bis  	                R1-141265

Shenzhen, China, 31st March – 4th April 2014

Source: 	ETRI

Title: 	Remaining issues for TDD-FDD CA

Agenda Item:	7.2.3 LTE TDD-FDD joint operation including carrier aggregation

Document for:	Discussion/Decision

- Introduction

In RAN #63 plenary meeting, RAN1 agreements for the WI “LTE TDD-FDD joint operation including carrier aggregation (CA)” were approved and there remains one open issue for further discussion as below with minimal time unit allocation at this RAN1 meeting [1]:

- TDD PCell DL HARQ timing for TDD-FDD carrier aggregation with self-carrier scheduling on FDD SCell

In this paper, we further discuss candidate options for the downlink HARQ timing of the FDD SCell when the PCell is the TDD Cell.

- Discussion

In RAN1 #76, two alternative schemes (three if including the LG’s compromised solution in R1-140958) were discussed for the FDD SCell HARQ timing [2]. One is Option 1-1 which tries to fully achieve the benefit of the CA, i.e., high peak data rate (Table 1), and the other is Option 2c-B2 whose basic concept is not to fully optimize and reuse the existing timings for minimum specification impact (Table 2).

Table 1. Option 1-1

| UL/DL Configuration | Subframe n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | - | - | 6, [5] | [5], [4] | 4 | - | - | 6, [5] | [5], [4] | 4 |
| 1 |  |  | 7, 6 | [6], [5], 4 |  |  |  | 7, 6 | [6], [5], 4 |  |
| 2 | - | - | 8, 7, 6, [5], 4 | - | - | - | - | 8, 7, 6, [5], 4 | - | - |
| 3 | - | - | 11, [10], [9], [8], 7, 6 | 6, 5 | 5, 4 | - | - | - | - | - |
| 4 | - | - | 12, 11, [10], [9], 8, 7 | 7, 6, 5, 4 |  |  |  |  |  |  |
| 5 | - | - | 13, 12, 11, [10], 9, 8, 7, 6, 5, 4 | - | - | - | - | - | - | - |
| 6 | - | - | [8], 7 | 7, [6] | [6], 5 | - | - | 7 | 7, [6], [5] | - |

Table 2. Option 2c-B2

| TDD PCell UL/DL configuration | Allowed DL reference TDD UL/DL configuration for FDD SCell (2 aggregated cells) | Allowed DL reference TDD UL/DL configuration for FDD SCell (more than 2 aggregated cells) |
| --- | --- | --- |
| 0 | 5 | 2 |
| 1 | 5 | 2 |
| 2 | 5 | 2 |
| 3 | 5 | 4 |
| 4 | 5 | 4 |
| 5 | 5 | N/A |
| 6 | 5 | 2 |

Option 2c-B2 adopts UL/DL configuration 5 (DL:UL=9:1) as the reference configuration for the case of two aggregated cells. Different from other previously discussed options, i.e., Option 2c-B and 2c-B1, Option 2c-B2 uses a fixed reference timing for each TDD PCell UL/DL configuration. Thus no higher layer signaling is needed and the timings are clear with TDD eIMTA-enabled cells. However, as a trade-off for utilizing the FDD SCell DL subframes as many as possible, Option 2c-B2 suffers from ACK/NACK concentration to UL subframe #2 of the TDD PCell regardless of TDD PCell’s UL/DL configuration.

Figures 1 and 2 describe Option 1-1 and Option 2c-B2 when the TDD PCell’s UL/DL configuration is 1 as an example. In this case, while Option 1-1 guarantees balanced allocation of PDSCH ACK/NACK to all four UL subframes of the TDD PCell, with Option 2c-B2, the maximum 11 ACK/NACK information can be concentrated in one UL subframe (Table 3). This may degrade the PUCCH performance. Moreover, Option 1-1 provides an additional 6.7% downlink resource usage and also gives shorter retransmission delay.

Figure 1. Option 1-1 with TDD UL/DL configuration 1

Figure 2. Option 2c-B2 with TDD UL/DL configuration 1

Table 3. The number of associated DL subframes of FDD SCell

| UL subframe # | #2 | #3 | #7 | #8 |
| --- | --- | --- | --- | --- |
| Option 1-1 | 4 | 4 | 4 | 4 |
| Option 2c-B2 | 11 | 1 | 2 | 1 |

Similar advantages of Option 1-1 are found for other TDD PCell’s UL/DL configurations except UL/DL configuration 5 (UL/DL configuration 5 may not be a frequently used configuration). Considering that nevertheless the specification effort to support Option 1-1 is minimal, we suggest to adopt Option 1-1 for the TDD PCell case.

Proposal 1: For the case of PUCCH transmission on PCell only, when PCell is TDD carrier and SCell is FDD carrier, for PDSCH transmitted on SCell with self-scheduling, HARQ timing should follow Option 1-1.

- Conclusions

In this contribution, we have discussed our views on the open issue for TDD-FDD CA and proposed the following.

Proposal 1: For the case of PUCCH transmission on PCell only, when PCell is TDD carrier and SCell is FDD carrier, for PDSCH transmitted on SCell with self-scheduling, HARQ timing should follow Option 1-1.

- References

[1] RP-140517, “Status report LTE TDD-FDD joint operation including carrier aggregation,” 3GPP RAN #63, Fukuoka, Japan, March 3th-6th, 2014.

[2] Chairman’s notes RAN1 #76, Prague, Czech Republic, February 10th-14th, 2014.