3GPP TSG RAN WG1 Meeting #78,	R1-143322

Dresden, Germany, 18th – 22rd August 2014

Source:	Ericsson

Title:	Correction to DL HARQ feedback table for TDD

Agenda Item:	7.1

Document for:	Discussion and Decision

# Discussion

In section 10.1.3.1 and 10.1.3.1A of [1], the downlink association set  is defined as a set of M elements  to indicate the subframe offsets  with respect to a subframe n, where . The set is defined in Tables 10.1.3.1-1, 10.1.3.1-1A and 10.1.3A-1.

There seem to be a typo in the table headers where the word “index” is present, but K is not an index, it is a set of indices. This should be corrected. Since it is such a small edit, it can be handled by the editor, if agreed.

Proposal: The TS 36.213 editor should remove the word “index” from the headers of Table 10.1.3.1-1 10.1.3.1-1A and 10.1.3A-1 at the next spec update.

Table 10.1.3.1-1: Downlink association set index:  for TDD

| UL/DL Configuration | Subframe n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | - | - | 6 | - | 4 | - | - | 6 | - | 4 |
| 1 | - | - | 7, 6 | 4 | - | - | - | 7, 6 | 4 | - |
| 2 | - | - | 8, 7, 4, 6 | - | - | - | - | 8, 7, 4, 6 | - | - |
| 3 | - | - | 7, 6, 11 | 6, 5 | 5, 4 | - | - | - | - | - |
| 4 | - | - | 12, 8, 7, 11 | 6, 5, 4, 7 | - | - | - | - | - | - |
| 5 | - | - | 13, 12, 9, 8, 7, 5, 4, 11, 6 | - | - | - | - | - | - | - |
| 6 | - | - | 7 | 7 | 5 | - | - | 7 | 7 | - |

Table 10.1.3.1-1A: ModeA downlink association set index : for TDD

| Higher layer parameter ‘ModeA-dlReferenceConfig-r12’ | Higher layer parameter ‘subframeAssignment’ | Subframe n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 2 | 0 | - | - | 7,8,4 | - | - | - | - | 7,8,4 | - | - |
|  | 1 | - | - | 8,4 | - | - | - | - | 8,4 | - | - |
|  | 6 | - | - | 6,8,4 | - | - | - | - | 8,6,4 | - | - |
| 4 | 0 | - | - | 12,7,11,8 | 7,4,5,6 | - | - | - | - | - | - |
|  | 1 | - | - | 12,8,11 | 7,5,6 | - | - | - | - | - | - |
|  | 3 | - | - | 12,8 | 4,7 | - | - | - | - | - | - |
|  | 6 | - | - | 12,11,8 | 4,5,6 | - | - | - | - | - | - |
| 5 | 0 | - | - | 12,7,11,13,8,4,9,5 | - | - | - | - | - | - | - |
|  | 1 | - | - | 13,12,8,11,4,9,5 | - | - | - | - | - | - | - |
|  | 2 | - | - | 13,12,9,11,5 | - | - | - | - | - | - | - |
|  | 3 | - | - | 13,12,5,4,8,9 | - | - | - | - | - | - | - |
|  | 4 | - | - | 13,5,4,6,9 | - | - | - | - | - | - | - |
|  | 6 | - | - | 13,12,11,6,8,4,9,5 | - | - | - | - | - | - | - |

Table 10.1.3A-1: Downlink association set index:  for FDD-TDD and serving cell frame structure type 1

| DL-reference UL/DL Configuration | Subframe n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | - | - | 6, 5 | 5, 4 | 4 | - | - | 6, 5 | 5, 4 | 4 |
| 1 | - | - | 7, 6 | 6, 5, 4 | - | - | - | 7, 6 | 6, 5, 4 | - |
| 2 | - | - | 8, 7, 6, 5, 4 | - | - | - | - | 8, 7, 6, 5, 4 | - | - |
| 3 | - | - | 11, 10, 9, 8, 7, 6 | 6, 5 | 5, 4 | - | - | - | - | - |
| 4 | - | - | 12, 11, 10, 9, 8, 7 | 7, 6, 5, 4 | - | - | - | - | - | - |
| 5 | - | - | 13, 12, 11, 10, 9, 8, 7, 6, 5, 4 | - | - | - | - | - | - | - |
| 6 | - | - | 8, 7 | 7, 6 | 6, 5 | - | - | 7 | 7, 6, 5 | - |

# References

3GPP TS 36.213