3GPP TSG RAN WG1 Meeting #80		R1-150649

Athens, Greece, 9th – 13th February 2015

Source:	Ericsson

Title:	Proposed Clarification on Out of Coverage Sync Resource Selection

Agenda Item:	7.1

Document for:	Discussion and Decision

# Discussion

During RAN1 #78bis the following was agreed:

Agreement:

- Out-of-coverage UEs do not transmit D2DSS on more than 1 D2DSS resource

  - 2 D2DSS resources are used for out-of-coverage

    - FFS whether the locations are preconfigured, signalled or fixed in the spec w.r.t. DFN#0

During RAN1#79 the criteria for selecting either of the 2 resources in different coverage scenarios are provided, including the following agreement:

Agreement:

- If a UE is transmitting D2DSS/PD2DSCH

  - If the UE does not select any D2DSS/PD2DSCH as its transmission timing reference and it is not camping/connected to an eNB

    - The PD2DSCH contents are determined by the pre-configuration, except for

      - In-coverage indicator: 0

      - DFN: using preconfigured value of syncOffsetIndicator, with the rest of the DFN being up to UE implementation for the first transmission

    - D2DSS sequence is arbitrarily selected from D2DSSue_oon, and can only be reselected if there is a change of transmission timing reference

However for the case where a UE is isolated and OoC it is ambiguous which D2DSS resource should be used. We further notice that it is reasonable to leave the choice of the OoC D2DSS resource up to UE implementation or based on a random selection, but it is preferable to limit the reselection frequency in order to facilitate convergence for the synchronization procedure. We suggest to complete existing agreements as follows, and to liaise with RAN2 once agreement is reached.

Proposals:

- If a UE is transmitting D2DSS/PD2DSCH

  - If the UE does not select any D2DSS/PD2DSCH as its transmission timing reference and it is not camping/connected to an eNB

    - One of the two out-of-coverage synchronization resources is arbitrarily selected, and can only be reselected if there is a change of transmission timing reference;

- Send an LS to RAN2 informing them of the above agreement.