3GPP TSG-RAN WG1 Meeting #80bis	R1-152087

Belgrade, Serbia, 20th – 24th April 2015

Agenda Item:	6.3.2

Source: 	Ericsson

Title:  	Text proposal on system simulation assumptions and performance metrics for Network Assisted Interference Cancellation and Suppression for UMTS

Document for:	Discussion

# Introduction

This contribution provides the text proposal for the system simulation assumptions and performance metrics for the study on network assisted interference cancellation and suppression [1], reported in [2].

# Text Proposal

[------------------------------------------------------ TEXT START ---------------------------------------------------------]

## A.1	Link simulation assumptions

## A.2 System simulation assumptions

Both hetnet and homonet types of network are evaluated using the two agreed receivers, type 3i and NAICS pre-decoding. Optionally NAICS post-decoding can be evaluated.  The traffic models shall account for both full buffer and bursty traffic.

## A.2.1 Simulation parameters

Parameters for hetnet and homonet scenarios are given in table 1a and 1b.  Details on hetnet parameters can be found in TR25.800 [3].

Table １a: System simulation parameters

| Parameters | Value |  |
| --- | --- | --- |
| Cell plan | 57 cell hexagonal (19 NodeB, 3 sectors per Node B with wrap-around) (21 cell hexagonal optional) |  |
| Carrier Frequency | 2000 MHz |  |
| Carrier bandwidth | 5MHz |  |
| Channel Model | PA3, VA3, (VA120 optional) |  |
| Thermal noise density | -174dBm/Hz |  |
| Number of HARQ processes | 8 |  |
| Number of LPNs/LPN deployment | 1, 2, 4; 8 (optional); 16 (optional) Minimum distance between LPN and macro cell: 75m Minimum distance between LPNs: 40m |  |
| Dropping criteria for LPNs | - LPNs are randomly and uniformly distributed within a macro cell. - (Optional) LPNs are deployed according to the received CPICH RSCP of the macro cell: CPICH RSCP = TxPow_CPICH + AntGain - PL – PenLoss TxPow_CPICH is the CPICH tx power of macro cell (33dBm) AntGain is the antenna gain PL is large scale fading calculated according to path loss model PenLoss is the penetration loss The deployment of LPNs will be labelled as centre, near, middle, far, edge, from the macro cell depending on the CPICH RSCP value, P(dBm). P=-46dBm, centre (the min distance between UE and macro cell, and UE is in main beam of antenna); P=-66dBm, near (1/3 of distance centre-edge of the macro cell) P=-74dBm, middle (1/2) P=-80dBm, far (2/3) P=-88dBm, edge |  |
| Inter-site distance | 500 m 1000 m (optional) |  |
| Number of UEs | - For full buffer (DL) - 16, optional 32 for the case of 16 LPNs - For full buffer (UL) - 8 - For bursty traffic model variable up to system stability level The minimum distance between UE and macro cell is 35m The minimum distance between UE and LPN is 10m |  |
| Dropping criteria for UEs | - Random: UE randomly and uniformly distributed within a macro cell - Hotspot: Randomly and uniformly dropping with Photspot of the total users within a radius, r, of LPN base station, and randomly and uniformly dropping of the remaining users in the entire macro geographical area of the given macro cell (including LPN area). Type 1: Photspot = ½ Type 2: Photspot = ¾ (optional) The radius r of the LPN is equal to 20m, 35m, and 60m when the LPN power is 24dBm, 30dBm, and 37dBm, respectively. |  |
| RoT | 6dB (both NodeB and LPN NodeB) |  |
| Path Loss | Macro Node: L=128.1 + 37.6log10(R), R in kilometres LPN: L=140.7 + 36.7log10(R), R in kilometres |  |
| Log Normal Fading (outdoor) | Standard Deviation: 8dB (macro cell); 10 dB (LPN) Inter-Node B Correlation: 0.5 Intra-Node B Correlation :1.0 Correlation Distance: 50m |  |

Table １b: System simulation parameters, cont’d.

| Penetration loss | 20dB |
| --- | --- |
| Maximum UE EIRP | 24dBm |
| Maximum Tx Power of NodeB | Macro Node: 43dBm LPN: 37 dBm, 30 dBm, 24 dBm |
| Max BS Antenna Gain | Macro cell: 14dBi LP cell: 5 dBi |
| Max UE Antenna Gain | 0dBi |
| NodeB Noise Figure | Macro Node: 5 dB LPN: 5 dB; 11 dB (optional) |
| UE Noise Figure | 9 dB |
| Thermal noise density | -174dBm/Hz (reception bandwidth 3.84MHz) |
| HS-DSCH | Up to 15 SF 16 codes per carrier for HS-PDSCH Total available power for HS-PDSCH is 80% (SIMO) / 75% (MIMO) of Node B Tx power, with HS-SCCH transmit power being driven by 1% HS-SCCH BLER. HS-PDSCH HARQ: Both chase combining and IR based can be used. Maximum of 4 transmissions with 10% target BLER after the first transmission. Retransmissions are of highest priority. HS-DPCCH decoding is assumed ideal. UL HARQ operating point: 1% residual BLER after 4th transmission |
| Number of HARQ processes | 6 |
| HS-SCCH code number | 4 |
| Total overhead power | 20% (SIMO) 25% (MIMO, optional) |
| UE Receiver | Type 3i (interference suppression receiver) Pre-decoding IC (post-decoding IC , optional) |
| Soft Handover | Consideration Scenarios with and without SHO |
| Soft Handover Parameters | SHO available - R1a (reporting range constant) = 4.5dB - R1b (reporting range constant) = 4.5dB Consideration of scenarios without SHO |
| CIO | 0dB 3 dB |
| Max active set size | 3 |
| Power control/harq operation | UL: Target 10% IBLER after the first transmission DL: Based on CQI. No IBLER control |
| Network Configuration | SIMO MIMO (optional) |

Parameters for downlink and uplink bursty traffic model are given in Table 2 and Table 3, respectively.

Table ２: Downlink bursty traffic model

| Component | Distribution | Parameters | PDF |
| --- | --- | --- | --- |
| File size (S) | Truncated Lognormal | Mean = 0.25 Mbytes Std. Dev. = 0.0902 Mbytes Maximum = 1.25 Mbytes |  |
| Inter-burst time | Exponential | Mean = 5 sec |  |

Table ３: Uplink bursty traffic model

| Component | Distribution | Parameters | PDF |
| --- | --- | --- | --- |
| File size (S) | Truncated Lognormal | Mean = 0.0625 Mbytes Std. Dev. = 0.0225 Mbytes Maximum = 0.3125 Mbytes |  |
| Inter-burst time | Exponential | Mean = 5 sec |  |

## A.2.2 System performance evaluation metrics

### A.2.2.1 Full buffer traffic

For full buffer traffic, the following performance measures are used for evaluation:

- System  throughput

- UE throughput: average, 90%,  50%, and 5%

- Percentage of UEs served by LPNs

- Average and CDF of RoT for UL

### A.2.2.2 Bursty traffic

The burst patterns in a burst traffic model are characterized by the following measurements:

- The burst rate, defined as the ratio between the data burst size in bits and the total time the burst spent in the system.

- The total time the burst spent in the system is the time difference measured between the instant the data burst arrives at the Node B and the instant when the transfer of the burst over the air interface is completed.. The total time the burst spent in the system is equal to the sum of the transmission time over the air and the queuing delay.

For bursty traffic, the performance metrics described in subclause 2.3.1 may be re-used.

[------------------------------------------------------TEXT END-----------------------------------------------------]

# Conclusion

Upon reviewing the content of the Text Proposal, it is proposed to:

Proposal: Agree on the text proposal and capture the content presented in this document in the Technical Report on Network-Assisted Interference Cancellation and Suppression for UMTS [2].

# References

- RP-142250, " New Study Item proposal: Study on Network-Assisted Interference Cancellation and Suppression for UMTS ", Huawei, HiSilicon.

- 3GPP TR 25.766 v0.1.0, Technical Report on Network-Assisted Interference Cancellation and Suppression for UMTS.