3GPP TSG RAN WG1 Meeting #81		R1-153251

Fukuoka, Japan, 25th – 29th May 2015

Agenda item:	6.2.5.1

Source:	InterDigital

Title:	Performance results of Cat-2 baseline

Document for:	Discussion

# 1	Introduction

In RAN1 #79, it is agreed that for a given antenna array configuration, an enhancement proposal should be evaluated against a baseline case, where the enhancement case requires specification change and the baseline case may be considered as a standard transparent scheme which has no specification impact based on Rel-12 [1]. In this contribution, we provide assumptions for the baseline and its performance results which provide the best performance achievable using Rel-12 specifications.

- For a given antenna array configuration, an enhancement proposal that requires specifications change should at least be provided with the following:

  - A baseline case

    - A baseline is considered to have no specification impact to Rel-12 and providing the best performance achievable using Rel-12 specifications

  - An enhancement case

    - An enhancement is considered to have specification impact to Rel-12

  - The enhancement case should at least be evaluated against the  baseline case(s)

  - Antenna array configuration is given by the parameters {M,N,P,Q}

  - Baseline and Enhancement cases assume the same values for M, N, P, Q

    - 1D TXRU virtualization: The total number of associated TXRUs:  Q= MTXRU * N * P according to TXRU model-1 (as defined in RAN1#78bis)

    - 2D TXRU virtualization: The total number of TXRUs Q should be described by the proponent

In this contribution, we evaluate the performance of the cat-3 baseline with 64 TXRUs in 3D-UMa and 3D-UMi.

# 2	Cat-2 baseline assumptions

In [2], it is shown that the adaptive downtilt scheme which optimize downtilt angle in each cell provides reasonable amount of performance gains by exploiting elevation beamforming gain compared to the fixed downtilt case. The elevation beamforming gain could be further optimized by adapting the beams in a UE-specific manner with the assumption that eNB has a full capability to change the elevation beam per UE at the same time (i.e. the number of TXRU is the same as the number of antenna elements). In this case, a UE may report a preferred elevation beam (such as the one giving the highest received RSRP or wideband SINR) by using multiple CSI processes, thus up to 3 elevation beams may be used in a standard transparent manner.

The table 1 shows the assumption used for beamformed CSI-RS with 64 TXRUs and three CSI processes as a Cat-2 baseline scheme.

Table 1. Baseline assumptions

| Parameters | Values for reference | Values for baseline |
| --- | --- | --- |
| Scenarios | (3D-UMa, ISD 500m, 2GHz), (3D-UMi, ISD 200m, 2GHz) | (3D-UMa, ISD500m, 2GHz), (3D-UMi, ISD 200m, 2GHz) |
| Scheme | Fixed downtilt | Beamformed CSI-RS |
| Antenna array configuration (M,N,P,Q) | (8,4,2,8) | (8,4,2,64) |
| TXRU virtualization weights | 1 vertical TXRU per polarization, DFT weighted, all TXRU using the same fixed downtilt full connection based. | One to one mapping from AE to TXRU, total 64 TXRUs |
| CSI-RS to TXRU virtualization | one TXRU mapped to one CRI-RS port , total 8 CSI-RS ports, one vertical eight horizontal | Each CSI-RS process has 8 horizontal CSI-RS ports. One vertical port per polarization, DFT weighted through all TXRU in one column. Each CSI-RS process is beamformed with a different angle to support UE specific Elevation BF. |
| CSI-RS port indexing | As in Rel.10, port 15-18 mapped to one polarization (+45 deg) and port 19-22 mapped to the other polarization (-45 deg) | As in Rel.10, port 15-18 mapped to one polarization (+45 deg) and port 19-22 mapped to the other polarization (-45 deg) 1) port 15-22 for downtilt angle 1 in CSI process-1. 2) port 15-22 for downtilt angle 2 in CSI process-2. 3) port 15-22 for downtilt angle 3 in CSI process-3. |
| Cell association weights | CRS port 0 mapped same as CSI-RS port 15, i.e., [1 0 0 0 0 0 0 0] | CRS port 0 mapped same as CSI-RS port 15, i.e., [1 0 0 0 0 0 0 0] |
| Cell-association method | RSRP on CRS port 0 fixed at 100 deg downtilt | RSRP on CRS port 0 fixed at 100 deg downtilt |
| DFT weighted downtilt (set) | [99, 102, 105] for 3D-UMa and 3D-UMi | Three elevation angles (i.e. three elevation beams), each applied to a CSI process [99, 102, 105] for 3D-UMa and 3D-UMi |
| Beamformed CSI-RS selection | A single CSI-RS configuration with a fixed elevation beam | A single CSI-RS configuration is selected among three CSI-RS configurations based on RSRP. The CSI feedback is only for one CSI-RS configuration selected. |

# 3	Evaluation Results

The Cat-2 baseline performance is evaluated in this section based on the assumptions listed in the table 1. The beamformed CSI-RS performance is compared with a fixed downtilt (i.e. cell-specific elevation beamforming, phase-1 baseline) in non-full buffer cases with different traffic loads. Further details of the simulation assumptions are listed in Table A, which is based on the agreement in [3]-[4].

Table 2 shows the performance of the beamformed CSI-RS and the phase-1 baseline in 3D-UMa channel.  As seen in the table, the cat-2 baseline achieves 31%, 22%, and 20%  mean throughput gain respectively for high load, medium load, and low load cases, as compared with that of the phase-1 baseline (i.e. best fixed downtilt case).

Table 2: Non-full buffer results (Mbps) – 3D-UMa, 500m, 2 GHz

| Traffic type | High load |  |  |  | Medium load |  |  |  | Low load |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Offered Load (Mbits/cell) and λ | 14 ( λ = 3.5) |  |  |  | 10 (λ = 2.5) |  |  |  | 4 (λ = 1.0) |  |  |  |
|  | RU | Mean | 50% | 5% | RU | Mean | 50% | 5% | RU | Mean | 50% | 5% |
| Beam-formed CSI-RS with beam angles = [99 102 105] | 75.2% | 12.46 | 9.45 | 2.39 | 44.4% | 19.42 | 17.55 | 4.93 | 15.8% | 29.75 | 26.31 | 9.24 |
| Fixed beam, 99 deg | 89.6% | 7.45 | 4.68 | 0.35 | 73.7% | 10.56 | 7.93 | 0.69 | 23.2% | 22.64 | 21.33 | 3.78 |
| Fixed beam, 102 deg | 82.5% | 9.03 | 6.04 | 0.75 | 61.8% | 14.52 | 11.75 | 3.19 | 21.3% | 24.78 | 23.57 | 4.74 |
| Fixed beam, 105 deg | 81.1% | 9.53 | 6.59 | 1.06 | 54% | 15.92 | 13.09 | 4.15 | 22.1% | 24.83 | 22.71 | 4.96 |

Table 3 shows the performance of the beamformed CSI-RS and the phase-1 baseline in 3D-UMi channel.  As seen in the table, the cat-2 baseline achieves 22%, 22%, and 2%  mean throughput gain respectively for high load, medium load, and low load cases, as compared with that of phase-1 baseline from the downtilt angle set used in the UE specific downtilt scheme. Also, note that the relative gain gets reduced in lower traffic cases due to the fact that the system performance for all the schemes is restricted by the traffic status and available UEs with packets.

Table 3: Non-full buffer results (Mbps) – 3D-UMi , 200m, 2 GHz

| Traffic type | High load |  |  |  | Medium load |  |  |  | Low load |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Offered Load (Mbits/cell) and λ | 14 ( λ = 3.5) |  |  |  | 10 (λ = 2.5) |  |  |  | 4 (λ = 1.0) |  |  |  |
|  | RU | Mean | 50% | 5% | RU | Mean | 50% | 5% | RU | Mean | 50% | 5% |
| Beam-formed CSI-RS with beam angles = [99 102 105] | 73.5% | 13.23 | 9.59 | 1.71 | 50% | 20.37 | 16.89 | 3.31 | 15.9% | 35.43 | 35.30 | 5.88 |
| Fixed beam, 99 deg | 83% | 10.22 | 7.30 | 0.78 | 60% | 15.73 | 12.49 | 1.55 | 17% | 32.31 | 30.48 | 4.97 |
| Fixed beam, 102 deg | 86% | 9.62 | 6.71 | 0.77 | 59% | 16.49 | 12.50 | 2.04 | 18.5% | 31.22 | 30.31 | 4.56 |
| Fixed beam, 105 deg | 80% | 10.88 | 7.43 | 0.87 | 56.3% | 16.65 | 12.61 | 2.46 | 13.5% | 34.89 | 33.60 | 9.73 |

# 4	Summary

In this contribution, we provided performance results of the cat-2 baseline with 64 TXRUs.

# References

- RAN1 #79 Chairman’s note

- R1-144210, “Initial Evaluation Results for 2D antenna array”, InterDigital

- R1-144444, “WF on simulation assumptions”, Nokia Networks, Nokia Corporation, CMCC, Samsung

- R1-144437, “WF on Antenna modeling”, Samsung, Nokia Networks, Nokia Corporation, CMCC, Huawei, HiSilicon, Interdigital, LGE, Motorola Mobility, Qualcomm, CHTTL

# Annex

The 2D Antenna array used in the simulation is configured as below.

Table A. System Level Simulation Assumptions

| Parameter | Assumption |
| --- | --- |
| Network layout | 7-site 21-cell wraparound |
| Channel model | 3D Urban Macro (3D-UMa) 3D Urban Micro (3D-UMi) |
| eNB antenna configuration | (M,N,P,Q)=(8,4,2,8) for fixed downtilt, (M,N,P,Q)=(8,4,2,64) for UE specific elevation BF. 0.5 λ spacing in H, 0.8 λ spacing in V, cross-polarization (X-pol) [4] |
| UE antenna configuration | nR=2, cross-polarization, 0o/90o |
| UE attachment | RSRP on CRS port 0 [4] |
| Electrical Downtilt | beam angles for UE specific elevation BF [99 102 105 ] for 3D-UMa and 3D-UMi; One single elevation angle is chosen as θtilt from above set for fixed downtilt case. |
| UE-specific elevation beam selection | based on CSI feedback from multiple CSI process |
| Number of UEs per cell | 10 |
| UE distribution | uniformly dropped according to [3] |
| Traffic model | non-full buffer FTP model 1, packet size 0.5M bytes |
| Scheduler | proportional fair (PF) |
| Transmission scheme | SU/MU-MIMO dynamic switching with SU-MIMO feedback, Non-transparent MU-MIMO |
| Codebook | Rel.10 8Tx codebook |
| Link adaptation | AMC with OLLA, 10% BLER target |
| Receiver | MMSE-IRC, ideal channel estimation, ideal interference modelling |
| Feedback | PUSCH 3-1, CQI and PMI reporting triggered every 5ms |
| Receiver | feedback delay is 5 ms |
| Maximum number of HARQ retransmission | 4 |