# 3GPP TSG RAN WG1 Meeting #82		 R1-154579

# Beijing, China, 24th – 28th August 2015	(Revision of R1-152992)

# Title: 	CSI Measurement and Reporting for LAA

# Source: 	Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

# Agenda Item:	7.2.4.4

# Document for: 	Discussion/Decision

# Introduction

In RAN#68, the study item on licensed-assisted access (LAA) using LTE on unlicensed spectrum [1] was closed, with the technical report approved in [2]. In addition, a work item on LAA [3] was approved to specify DL-only LAA while taking into account some of the UL design. The specification work should follow the recommendations and agreements in the study item phase, and further narrow down the detailed design.

For LAA, discontinuous transmission needs to be supported due to coexistence with other networks and the listen-before-talk requirements in some regions/countries. It is important to enable an LAA carrier to support fast on/off on the subframe or multi-subframe level to improve the efficiency. One of the important issues to address for the support of fast on/off is how to support CSI measurement and reporting.

For CSI measurement, it was agreed in RAN1 LAA ad hoc meeting that

Agreement:

- Interference measurement for CSI is not allowed outside of the serving cell transmission periods

It was further agreed in RAN1#80bis that

Agreements:

Below is a list of design options for CSI measurements, CSI reporting, and transmission schemes for LAA, for the case that LAA supports the transmission of CRS and/or CSI-RS and/or CSI-IM.

- For the purpose of CSI measurement, if LAA supports CSI measurements on CSI-RS and CSI-IM (or only CSI-RS for TM9)

  - NZP CSI-RS transmission may be subject to LBT

  - The presence of NZP CSI-RS and CSI-IM in a subframe is indicated implicitly or explicitly

    - Note: indication may or may not involve signaling

  - Potential subframes for NZP CSI-RS and CSI-IM:

    - Opt 1-A: The potential subframes may be occurring periodically from UE perspective

    - Opt 1-B: The potential subframes may be occurring aperiodically from UE perspective

    - Opt 1-C: a combination of opt 1-A and opt 1-B

    - Note: same or different options for NZP CSI-RS and CSI-IM is not precluded

- For the purpose of CSI measurement, if LAA supports CSI measurements on CRS

  - CRS transmission may be subject to LBT

  - The presence of CRS in a subframe is indicated implicitly or explicitly

    - Note: indication may or may not involve signaling

- The following CSI reporting options have been identified

  - Opt 2-A: Support for both aperiodic and periodic CSI reporting for an LAA Scell

  - Opt 2-B: Support only for aperiodic CSI reporting for an LAA SCell

- For aperiodic CSI reporting:

  - Opt 3-A: Aperiodic CSI can be reported on PUSCH transmitted on a carrier in licensed spectrum or on an LAA SCell

  - Opt 3-B: Aperiodic CSI can only be reported on PUSCH on a carrier in licensed spectrum

- The following options for supported transmission schemes have been identified

  - Opt 4-A: DMRS-based demodulation and CRS-based demodulation for PDSCH

  - Opt 4-B: Only DMRS-based demodulation for PDSCH

  - Further down selection of the supported transmission schemes is possible for the options above

On aperiodic CSI reporting, it was agreed in RAN1#81 to adopt Opt 3-A, that is:

Agreements:

- Aperiodic CSI reporting can be carried on PUSCH transmitted on a carrier in licensed spectrum or on an LAA SCell

  - If the PUSCH containing aperiodic CSI is scheduled on LAA SCell and cannot be transmitted in the scheduled subframe on the scheduled carrier, the aperiodic CSI based on the same scheduling grant is not transmitted in another subframe or another carrier

  - Otherwise, it is recommended to follow the existing mechanism in CA for aperiodic CSI reporting (if no issue will be identified), including potential additional Rel-13 CA enhancement(s) if no issue is identified for using Rel-13 CA enhancement(s)

In this contribution we discuss the different options to support CSI measurement and reporting and share our views.

# CSI Measurement

In LTE, the CSI measurement has been performed based on CRS, or CSI-RS, or CSI-RS and CSI-IM, depending on the transmission mode. For TM1 to TM8, the channel measurement is based on CRS, and the interference measurement is up to UE implementation (e.g. based on CRS). For TM9, the channel measurement is based on CSI-RS, and the interference measurement is up to UE implementation (e.g. based on CRS). For TM10, the channel measurement is based on CSI-RS, and the interference measurement is based on CSI-IM.

It was agreed that the interference measurement for CSI is not allowed outside of the serving cell transmission periods. This means that there should be a way for the UE to know whether a subframe belongs to a DL transmission burst or not. This can be done either implicitly or explicitly.

If the interference measurement is based on CSI-IM, a straightforward way is to pre-configure a set of periodic subframes as the potential subframes containing CSI-IM, and the UE assumes CSI-IM is present as long as it is part of a DL transmission burst. A similar way can be used for NZP CSI-RS. If the interference measurement is not based on CSI-IM, the UE can measure interference potentially in every subframe within a DL transmission burst. In this case, the UE only needs to know which subframes belong to a DL transmission burst.

In addition, there could be explicit indication to inform the UE the presence of CSI-RS and CSI-IM in a subframe, e.g. whenever aperiodic CSI report is triggered. The additional benefit of this explicit signalling depends on the periodicity of the configured periodic subframes. If the periodicity is sufficiently small, there may not be much motivation to have the explicit signalling.

Proposal 1: If LAA supports CSI measurements based on CSI-RS and CSI-IM (or only CSI-RS for TM9), the configuration of a set of periodic subframes as the potential subframes for NZP CSI-RS and CSI-IM should be supported. FFS explicit indication of aperiodic subframes for NZP CSI-RS and CSI-IM.

Proposal 2: A mechanism should be provided to allow a UE (with or without being scheduled on the DL) to know whether a subframe belongs to a DL transmission burst.

The interference on an LAA carrier is typically considered to have more variation compared to that on a licensed carrier due to the potential interference from a large number of nodes and bursty nature of traffic on each node on the unlicensed carrier. Because of this larger variation, it becomes more difficult to track instantaneous interference and try to adapt to the instantaneous condition. In legacy LTE system, there is a minimum of 4 ms delay between the CSI measurement and report. Even if this delay is reduced, e.g. to 2 ms, it is still quite questionable how strong the correlation is between the interference of the two subframes that are at least 3 ms apart (2ms reporting delay plus 1ms decoding/scheduling delay) given the bursty traffic.

- If the interference comes from the nodes that are out of the sensing range (including the hidden nodes), the transmission bursts from these nodes are completely independent from the burst in the serving cell. Considering two subframes that are at least 3 ms apart, there can be different nodes starting or finishing their transmission bursts between these two subframes, so the correlation of interference for these two subframes can be very weak.

- If the interference comes from a colliding transmission from a node that is within the sensing range, that could be stronger correlation between the subframes within the same burst. Two colliding transmissions start roughly at the same time, so if two bursts have the same length, they will collide for the entire transmission. This means high correlation for the interference in all the subframes, because the interference from the colliding node is typically higher than the interference from the nodes out of the sensing range and dominates the interference.

If there is a good contention window adaptation algorithm in place using LBT Cat 4, it should control the collision probability fairly well. With a low collision probability, the strong interference correlation case only occurs once a while. For majority cases, there is no strong correlation. Even though strong correlation cannot be assumed, it is still reasonable to assume a similar statistical property for the interference in the subframes which are not too far apart in time.

In addition, the accuracy of the instantaneous interference measurement may not be very good, especially if it is measured based on the few REs of a CSI-IM resource.

The analysis above suggests that it is still more reasonable to rely on the average interference measurement, as in legacy LTE systems. The interference measured in the current transmission burst can be potentially averaged with the interference measured in the previous transmission bursts. Of course different weighting factors can be applied depending on how outdated the previous interference measurements were done.

Proposal 3: The interference measurement for CSI should be averaged and not instantaneous.

Due to the discontinuous transmission on LAA, the opportunities for the UE to make CSI measurement can no longer be guaranteed when LBT is required. Aperiodic RS has been discussed as a possible enhancement to increase the measurement opportunites.

Aperiodic RS could be embedded in the data transmission, and it can be indicated to the UE whenever it is transmitted, as discussed earlier.

In addition, aperiodic RS could be transmitted by itself when the channel is free. This makes sense if the data buffer is empty, because otherwise it is more efficient to embed aperiodic RS in the data transmission. However, when the data buffer is empty, it becomes debatable whether it is still very meaningful to have the UE measure the CSI, because the aperiodic RS transmission is additional overhead. The consequence of not providing any intermediate opportunity (i.e. transmitting aperiodic RS) during the long idle time between two transmission bursts is that the CSI report can be very outdated. So the scheduling decision cannot be made effectively in the first few subframes in the transmission burst, before the eNB receives new CSI report from the UE. This tradeoff between the overhead and performance gain needs to be evaluated further. An example of how to do aperiodic RS transmission was provided in [4].

Observation 1: The benefit of aperiodic RS only transmission should be carefully evaluated against the overhead.

# CSI Reporting

Periodic and aperiodic CSI reporting are supported in legacy LTE systems. Usually it is considered natural to continue to support both, but the question has been raised on whether it is still useful to support periodic CSI reporting.

With aperiodic CSI (A-CSI) reporting only, the eNB needs to trigger CSI report every time, and this is expected to occur fairly frequently. For each A-CSI report, it needs a UL grant for the triggering and PUSCH resources to transmit A-CSI. When the trigger needs to be done frequently, this is significant overhead involved.

Periodic CSI (P-CSI) does not require any triggering, so it does not need any grant. It is carried periodically on PUCCH using preconfigured resource instead of PUSCH for A-CSI, regardless of whether there is any data for the UE or not. The major concern on P-CSI is whether periodic CSI is useful considering that the LAA carrier is not always on, so the P-CSI report can be wasted during the carrier off duration, especially when the off duration is long. This may be true in some cases. But in many other cases, P-CSI can still be useful. For example, in a typical environment, when multiple nodes with similar LBT mechanism contend for a channel, all the nodes get more or less equal access to the channel, so they roughly take turn to transmit. This means that a node will get the chance to transmit once a while (e.g. once every tens of ms). With these transmissions, the UE has the opportunities to measure the CSI. Via P-CSI feedback, the eNB always have a reasonably accurate CSI report. This saves the overhead to trigger the report as for A-CSI. So for these cases, it is still beneficial to support periodic CSI report.

In addition, LAA design targets towards low-mobility UEs, so the channel varies slowly. Given that we believe the interference measurement should be averaged (as explained above), the periodicity for CSI feedback does not need to be very small. For UEs with reasonable amount of data to be delivered (which is also the main design target for LAA), the overhead from periodic CSI reporting would be worthwhile.

When both periodic and aperiodic CSI reporting are supported, it can be left to the eNB implementation to decide which reporting mode to use. Different mode, or a combination of both, could be used in different scenarios.

Proposal 4: Both periodic and aperiodic CSI reporting should be supported (Option 2-A).

# Conclusion

In this contribution, we have discussed CSI measurement in LAA and have the following proposals and observations:

Proposal 1: If LAA supports CSI measurements based on CSI-RS and CSI-IM (or only CSI-RS for TM9), the configuration of a set of periodic subframes as the potential subframes for NZP CSI-RS and CSI-IM should be supported. FFS explicit indication of aperiodic subframes for NZP CSI-RS and CSI-IM.

Proposal 2: A mechanism should be provided to allow a UE (with or without being scheduled on the DL) to know whether a subframe belongs to a DL transmission burst.

Proposal 3: The interference measurement for CSI should be averaged and not instantaneous.

Proposal 4: Both periodic and aperiodic CSI reporting should be supported (Option 2-A).

Observation 1: The benefit of aperiodic RS only transmission should be carefully evaluated against the overhead.

# References

- RP-141664, “Study on Licensed-Assisted Access using LTE,” Ericsson, Qualcomm, Huawei, Alcatel-Lucent, RAN#65, September 2014.

- 3GPP TR 36.889 v13.0.0, “Study on Licensed-Assisted Access to Unlicensed Spectrum,” June 2015.

- RP-151045, “New Work Item on Licensed-Assisted Access to Unlicensed Spectrum,” Ericsson, Huawei, Qualcomm, Alcatel-Lucent, RAN#68, June 2015.

- R1-151481, “On the CSI Measurement and Report for LAA,” Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, RAN1#80bis, April 2015.