3GPP TSG RAN WG1 Meeting #82bis	                    R1-155335

Malmö, Sweden, 5th - 9th October 2015

Source:                 Panasonic

Title: 	Evaluation on frequency domain synchronization based on PBCH/PSS/SSS in eMTC

Agenda Item:		7.2.1.6

Document for:	Discussion and Decision

Introduction

Synchronization performance is critical for receiver and such topic was discussed in RAN1#82 meeting. Based on agreements PBCH repetitions could be considered for improving frequency tracking loop operation. Details are as follows.

Agreement:

- In subframes with PBCH repetition:

  - CSI-RS punctures PBCH repetition REs

  - Working assumption: consider mapping PBCH repetition symbols for improving frequency tracking loop operation

    - FFS detailed mapping (within and across subframes)

  - FFS: signal if the eNB is using PBCH repetition, e.g. for R13 or later regular UEs to rate match around

This document shows our evaluation results on frequency domain synchronization based on PBCH/PSS/SSS.

Discussion

Evaluation methods on frequency synchronization

PBCH based FO estimation [1]: correlation between repeated OFDM symbols then average in both time and frequency domains, shown in Fig.1. The obtained one subframe result is averaged 10 times or 100 times.

Figure 1 PBCH based FO estimation [1]

PSS/SSS based FO estimation: correlation between SSS and PSS over the frequency domain in one subframe. The obtained one subframe result is averaged 10 times or 100 times.

The other simulation assumptions are shown in Appendix.

We didn't evaluate PSS collision issue among different PCID. We didn't evaluate the frequency estimation among different PBCH subframes. These may make the performance more difference.

Simulation results

Our simulation results on SNR -15 dB condition are shown in Fig.2. We have following observations,

      - PBCH 10 times averaging shows similar performance with PSS/SSS 100 times averaging. PBCH 10 times averaging requires totally 50ms assuming two subframes PBCH transmission in a radio frame and PSS/SSS 100 times averaging requires 500ms assuming 5ms internal as well. PBCH based solution would reduce UE power consumption significantly. The reason of the performance improvement based on PBCH repetitions would be more REs for averaging and finer accuracy.

      - ±0.1 PPM at 2 GHz corresponds to ±200 Hz frequency offset. When frequency synchronization is within certain range like ±200 Hz, multiple subframe based frequency offset estimation based CRS or PBCH is possible. Using such technique, we expect ±0.1 PPM at 2 GHz would be feasible even -15dB condition.

Figure 2 FO estimation

Conclusion

In this contribution, we discussed evaluation results on frequency domain synchronization based on PBCH/PSS/SSS. Based on the observations, we propose

Proposal 1: Confirm the working assumption to use PBCH as frequency tracking.

- Reference

[1] R1-153851, “PBCH repetition for MTC”, Qualcomm

- Appendix

| Parameters | Value |
| --- | --- |
| Number of antennas | 2×1 with low correlation |
| Transmission mode | 1 |
| Frame Structure | Type 1: FDD |
| Channel model | EPA (Doppler frequency, fD=1Hz) |
| Frame format | Central 6RB |
| Carrier frequency | 2GHz |
| System bandwidth | 20MHz |
| Modulation order | QPSK for PBCH |
| Timing error | Ideal |
| Transmit power control error | Ideal |
| Frequency offset | 0 Hz |
| Estimation over number of subframe | Within one subframe: 1 per frame; 10 subframes, 100 subframes |