3GPP TSG RAN WG1 Meeting #82bis	R1-155563

Malmö, Sweden, 5th – 9th October 2015

Source:	Sharp

Title:	Frequency allocation for M-PDCCH

Agenda Item:	7.2.1.2

Document for:	Discussion and Decision

Introduction

In RAN1 #80bis, it was agreed Rel-11 EPDCCH is a starting point for design of M-PDCCH for MTC UEs in normal coverage and coverage enhancement [1], and it was agreed that both CSS and USS are supported for M-PDCCH [2].

Agreements: - Confirm the working assumption to use M-PDCCH scheduled PDSCH carrying paging record(s) - One or more narrowbands can be configured by eNB for paging - A UE monitors a paging occasion in one of the configured narrowbands in the subframe based on at least UE ID at least for paging record(s) - On indicators for system information update, ETWS, and CMAS: - Option A: Transmit in M-PDCCH - Option B: Transmit in PDSCH - FFS if the paging occasion for SI update, ETWS, and/or CMAS is a cell common or not - FFS additional information (e.g. CFI, TDD configuration) in the associated DCI or paging message - The M-PDCCH search space for paging includes M-PDCCH candidate(s) with the highest CE level configured in the cell Note: the ‘CE level’ can be replaced by a proper terminology later if needed

Agreement: - For coverage enhancement, an M-PDCCH candidate is composed by consecutive valid subframes - For an M-PDCCH UE-specific search space for a UE at least in normal coverage/ small coverage enhancement - M-PDCCH candidates with different L (aggregation level) is supported - FFS: other coverage enhancement case(s)

In this contribution, we share our considerations on M-PDCCH frequency allocation for Rel-13 MTC UEs.

M-PDCCH frequency allocation

Rel-13 LC MTC UEs will support narrow bandwidth operations of 6 PRBs in both RF and baseband. Therefore, instead of receiving wideband PDCCH, such UEs can only receive M-PDCCH in one group of 6 PRB-pairs (i.e., narrow-band). Rel-13 LC MTC UEs should know in which narrow-band within the system bandwidth and in which PRB-pairs within the group of 6 PRB-pairs to receive M-PDCCH.

In RAN1#82, it was agreed to confirm the working assumption to use M-PDCCH scheduled PDSCH carrying paging record(s) and as working assumption to support only option 1 (M-PDCCH-scheduled PDSCH carrying the message) for RAR in a narrowband [2]. This means an M-PDCCH common search space will be defined for Rel-13 MTC UEs, which can be also used for the initialization of UE-specific M-PDCCH set.

In RAN1#80, the agreement was made that multiple repetition levels in time domain are supported for M-PDCCH [3]. In order to provide higher frequency utilization efficiency, this agreement should be applicable to the M-PDCCH common search space. The narrow-band and MPDCCH-PRB-sets corresponding to an M-PDCCH common search space can be configured independently for each repetition level by system information (e.g. SIB2).

Proposal 1:

- The narrow-band and MPDCCH-PRB-sets of M-PDCCH common search space are configured independently for each repetition level by system information (e.g. SIB2).

In Rel-11, UE-specific higher layer signalling can configure a UE with one or two EPDCCH-PRB-sets for EPDCCH monitoring. The PRB-pairs corresponding to an EPDCCH-PRB-set are indicated by higher layers as well. Since multiple repetition levels in time domain are supported for M-PDCCH, in order to provide flexibility for frequency utilization, the PRB-pairs corresponding to an EPDCCH-PRB-set should be configured independently for each repetition level, and only one narrow-band is configured for a UE by dedicated RRC signalling.

Proposal 2:

- The narrow-band for M-PDCCH UE-specific search space is per UE configured by dedicated RRC signalling.

Proposal 3:

- The MPDCCH-PRB sets per repetition level can be either predefined or configured by dedicated RRC signalling for M-PDCCH UE-specific search space.

In Rel-11, both localized and distributed transmissions are supported for EPDCCH. Localised transmission can achieve frequency selection gain based on reliable CSI feedback. Distributed transmission can achieve frequency diversity gain when reliable CSI feedback is not available. However, Rel-13 MTC UEs only receive M-PDCCH within a 6PRB narrow-band, the frequency diversity gain may not provide any meaningful gain.

Proposal 4:

- Only localized transmission is used for M-PDCCH.

Conclusion

In this contribution, we have the following proposals:

Proposal 1:

- The narrow-band and MPDCCH-PRB-sets of M-PDCCH common search space are configured independently for each repetition level by system information (e.g. SIB2).

Proposal 2:

- The narrow-band for M-PDCCH UE-specific search space is per UE configured by dedicated RRC signalling.

Proposal 3:

- The MPDCCH-PRB sets per repetition level can be either predefined or configured by dedicated RRC signalling for M-PDCCH UE-specific search space.

Proposal 4:

- Only localized transmission is used for M-PDCCH.

References

3GPP RAN1 #80bis chairman’s notes, April 2015.

3GPP RAN1 #82 chairman’s notes, August 2015

3GPP RAN1 #80 chairman’s notes, February 2015