3GPP TSG RAN WG1 Meeting #82bis	R1-155564

Malmö, Sweden, 5th – 9th October, 2015

Source:	Sharp

Title:	Aperiodic CSI reporting for CA beyond 5 carriers

Agenda Item:	7.2.2.1.3

Document for:	Discussion and Decision

Introduction

In the RAN1#82 meeting, the following working assumption was made for aperiodic CSI transmission for Rel. 13 CA [1].

Working assumption: - To increase the number of aperiodic CSI sets for flexible aperiodic CSI triggering in Rel-13 CA, - Size of aperiodic CSI request field is increased to 3 bits. - RRC-configured aperiodic CSI sets are commonly configured for all UL cells/subframes as in Rel-12 CA.

In this contribution, we share our views on aperiodic CSIs reporting.

Discussions

Aperiodic CSI request field on Rel. 13 CA

The motivation to enhance the aperiodic CSI reporting in Rel. 13 CA is to improve the flexibility of aperiodic CSI triggering considering the number of DL carriers more than 5 CCs as shown on the agreement of increasing the number of aperiodic CSI sets [1]. Moreover, it is needed that the design of CSI feedback mechanism supports CSIs of 32 DL CCs on one UL carrier. Therefore, to achieve enough flexibility in aspects of CC-domain and time-domain, the working assumption in last meeting should be confirmed.

Proposal 1:

- Confirm the following working assumption in RAN1#82

  - To increase the number of aperiodic CSI sets for flexible aperiodic CSI triggering in Rel-13 CA,

    - Size of aperiodic CSI request field is increased to 3 bits.

      - RRC-configured aperiodic CSI sets are commonly configured for all UL cells/subframes as in Rel-12 CA.

Wideband-only aperiodic CSI reporting mode

The main benefit to introduce wideband-only aperiodic CSI reporting mode is UL overhead saving while specification impact is marginal [2], [3]. Generally, when more than 5 CCs are configured for a UE, the frequency scheduling within a CC may not be required. For example, we can consider LAA is one typical use case. Moreover, as mentioned in [4], DCI and PDSCH CRC overhead would be high when the transmitted data is allocated over too many CCs with small number of PRBs per CC by subband scheduling.

On the other hand, as the opposite perspective, current periodic CSI can configure shorten reporting period if needed and it may be instead of the aperiodic CSI enhancement. However, shorter periodic CSI period may consume more UL resources resulting in increasing UL overhead.

Comparing the specification impact of the reporting mode enhancement and increase of UL resource consumption by shorten reporting period, it seems the wideband-only aperiodic CSI reporting has enough advantage to introduce from UL overhead saving perspective.

Proposal 2:

- Wideband-only aperiodic CSI reporting should be introduced.

Conclusion

In this contribution, we propose:

Proposal 1:

- Confirm the following working assumption in RAN1#82

  - To increase the number of aperiodic CSI sets for flexible aperiodic CSI triggering in Rel-13 CA,

    - Size of aperiodic CSI request field is increased to 3 bits.

      - RRC-configured aperiodic CSI sets are commonly configured for all UL cells/subframes as in Rel-12 CA.

Proposal 2:

- Wideband-only aperiodic CSI reporting should be introduced.

References

- “Chairman’s notes RAN1 #82,” August 2015.

- R1-154967, Panasonic, Ericsson, Nokia Networks, “Introduction of Wideband-Only Aperiodic CSI Reporting Mode”, RAN1#82, August, 2015.

- R1-154422, Ericsson, “CSI feedback enhancements for up to 32 CCs”, RAN1#82, August, 2015.

- R1-153973, Panasonic, “CSI Reporting Granularity for eCA”, RAN1#82, August, 2015.