3GPP RAN WG1 Meeting #82bis 	R1-156338

Malmo, Sweden, October 5 – 9, 2015

Title:	Draft LS reply on PHY aspects for SC-PTM transmission

Response to:	R2-153965

Release:	Rel-13

Work Item:	LTE_SC_PTM-Core

Source:	Huawei,[RAN1]

To:	RAN2

Cc:

Contact Person:

Name:	Yongxia Lyu

E-mail Address:	lvyongxia@huawei.com

Attachments:

1. Overall Description:

RAN1 thanks RAN2 for their LS R2-153965 on SC-PTM transmission, and provides answers to the questions listed in the LS as follows.

Question 1)	Discuss and decide what PDCCH search space is used to transmit the DCI for SC-PTM scheduling, and to make the necessary changes in RAN1 specifications after the decision is made;

Answer:

RAN1 agreed that the existing CSS is used to transmit the DCI for SC-PTM transmission and therefore does not intend to develop an additional search space for SC-PTM.

In addition, it is agreed that only legacy DCI format 1A /1C in the existing CSS are supported to schedule the SC-PTM transmission, and therefore large delay CDD transmission in TM3 is not supported for SC-PTM transmission.

Question 2)	RAN2 would like to ask RAN1 whether a UE can support the reception of multiple services (i.e. multiple transport blocks scheduled via different Group-RNTIs) with different Transmission Modes (i.e. TM 1, 2 or 3) in one subframe

Answer: The concept of transmission mode does not apply to the SC-PTM transmission since only DCI format 1A/1C is used for SC-PTM transmission. Therefore, it is not necessary for the eNB to configure the transmission mode for the SC-PTM transmission. For DL SC-PTM transmissions, RAN1 concluded that the single-antenna port transmission scheme will be used if the number of PBCH antenna ports is one, (see subclause 7.1.1 in 36.213), otherwise transmit diversity (see subclause 7.1.2 in 36.213) will be used. This is the same behaviour as for SIB, random access response and paging messages.

2. Actions:

To RAN WG2: RAN1 respectfully asks RAN2 to take the above information into consideration.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #83	16 - 20 Nov 2015	Anaheim, US

TSG RAN WG1 Meeting #84	15 - 19 Feb 2016   	Malta, MT