3GPP RAN WG1 Meeting #82bis 	R1- 156355

Malmo, Sweden, October 5 – 9, 2015

Title:	Draft LS on eMTC UE transmission aspect

Response to:	R4-152495

Release:	Rel-13

Work Item:	LTE_MTCe2_L1-Core

Source:	Samsung, Panasonic [RAN1]

To:	RAN4

Cc:

Contact Person:

Name:	Donghan Kim

E-mail Address:	donghan7.kim at samsung.com

Name:	Hidetoshi Suzuki

E-mail Address:	suzuki.hidetoshi at jp.panasonic.com

Attachments:	none

1. Overall Description:

RAN1 discussed UE transmission power and agreed the following.

Agreement:

- For a UE transmitting PUSCH or PUCCH with repetitions, the PUSCH or PUCCH transmission power remains constant over X > 1 subframes.

  - For small repetitions, the UE transmission power is determined at the first subframe among the X subframes.

  - For other repetition cases,

    - For PUSCH, the power is always Pcmax,c(i)

      - Ask RAN4 whether or not Pcmax,c(i) can be a constant over X subframes. If not, a constant power based on Pcmax,c(i) is requested from RAN4 – LS to be prepared by Donghan (Samsung)

    - For PUCCH, FFS the power level

  - FSS: the value(s) of X.

Regarding the agreement above, RAN1 would like to ask RAN4 to provide feedback whether it is feasible to keep Pcmax,c(i) constant over X subframes for a UE transmitting PUSCH with repetitions, and to provide a constant power level if it is not feasible to have constant Pcmax,c(i) over X subframe.

Regarding the phase continuity, RAN1 would like to ask RAN4 to provide feedback about the questions below.

Question 1:

Regarding the phase continuity, RAN1 would like to ask RAN4 to provide feedback on whether the phase continuity in the transmitted signal can be maintained if the UE suspends transmission in one SC-FDMA symbol (due to cell-specific SRS) and resumes transmission after the SC-FDMA symbol.

Question 2:

RAN1 would like to ask RAN4 to provide feedback on the impact on the phase continuity in the transmitted signal if the UE suspends transmission in more than one SC-FDMA symbol (e.g., one or more subframes) and then resumes transmission.

2. Actions:

To RAN WG4:

RAN1 would like to ask RAN4 to provide feedback whether it is feasible to keep Pcmax,c(i) constant over X subframes for a UE transmitting PUSCH with repetitions, and to provide guidance for a constant power based on Pcmax,c(i) if it is not feasible to have constant Pcmax,c(i) over X subframe.

Also, RAN1 would like to ask RAN4 to provide feedback about the questions regarding the phase continuity

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #83	16 - 20 Nov 2015	Anaheim, US

TSG RAN WG1 Meeting #84	15 - 19 Feb 2016   	Malta, MT