3GPP TSG-RAN WG1 #83 	R1-157058

15th – 22th November 2015

Anaheim, USA

Agenda item:	6.2.4.3.2

Source: 	Qualcomm Incorporated

Title: 	Remaining details for CSI reporting class A

Document for:	Discussion/Decision

# Introduction

In RAN1#81, the study of performance and feasibility of elevation beamforming and full-dimensional (FD) MIMO [1] was completed.  It has been concluded that [2]

- Rel.13 class A codebook configured with 5 RRC parameters:

  - N1, N2 = {1,2,3,4,8} where the valid candidates are (N1, N2) = (8,1), (2,2), (2,3), (3,2), (2,4), (4,2)

  - O1, O2 = {2,4,8}

    - For each (N1, N2), configurability of (O1, O2)  is restricted to two possible fixed pairs

  - Config = {1, 2, 3, 4}

- Given the set of values of N1,N2,O1,O2:

  - W1 matrices with (L'1,L'2) = (4,2), (2,4) are constructed for N1>=N2 and N1<N2, respectively

  - W1=X1m1⊗X2m200X1m1⊗X2m2    where mi is the index for Xi

- An associated codebook table is defined in terms of i'2, i11 and i12

- Given the value of Config, a subset of codewords from the codebook table is selected as an active subset of values of i'2, associated with one of the following 4 configurations:

  - Config=1: (L1,L2) = (1,1) for rank 1-2

  - Config=2: (L1,L2) = (2,2) for rank 1-2 [square]

  - Config=3: (L1,L2) = (2,2) for rank 1-2 [non-adjacent 2D beams/checkerboard]

  - Config=4: (L1,L2) = (4,1), (1,4) for N1>=N2 and N1<N2 respectively for rank 1-2

In this contribution, we discuss necessary enhancements for periodic CSI reporting on PUCCH to enable the use of the agreed codebook for class A.

# Periodic CSI reporting on PUCCH

The feedback overhead increases as the number of CSI-RS ports. We consider the worst case where 16 CSI-RS ports are configured.  In Table 1, we listed the feedback overhead of each component PMI for a (2, 4, 2) antenna port configuration with the proposed rank 1-2 codebook according to [2].

Table 1: Feedback overhead for the (N1, N2) = (2, 4) antenna port configuration

| Config | (O1, O2) | i11 | i12 | i2 |
| --- | --- | --- | --- | --- |
| 1 | (8, 4) | 0 – 7 (3-bit) | 0 – 7 (3-bit) | 0 – 15 (4-bit) |
|  | (8, 8) | 0 – 7 (3-bit) | 0 – 15 (4-bit) | 0 – 15 (4-bit) |
| 2, 3 and 4 | (8, 4) | 0 – 15 (4-bit) | 0 – 15 (4-bit) | 0 – 15 (4-bit) |
|  | (8, 8) | 0 – 15 (4-bit) | 0 – 31 (5-bit) | 0 – 15 (4-bit) |

For rank-1 and rank-2 transmission, both first PMIs require up to 9 bits depending on RRC configuration.  In current Rel-10 8-TX codebook, the first PMI requires only 4-bit.  So, existing CSI reporting types in PUCCH mode 1-1 submodes 1 and 2 and PUCCH mode 2-1 cannot be directly reused with the new codebook.  In following sections, we discuss how to enhance existing PUCCH modes/submodes to support the new codebook.

## Mode 1-1

To support wideband PMI/CQI reporting via PUCCH mode 1-1, an extension of Sub-mode 1 can be considered by dividing the CSI reporting into 3 reports.

- Option-1a: the CSI reporting consists of 3 reports

  - A first report for RI and i11 feedback

  - A second report for i12 feedback

  - A third report for wideband CQI and i2 feedback

- Option-1b: the CSI reporting consists of 3 reports

  - A first report for RI and i12 feedback

  - A second report for i11 feedback

  - A third report for wideband CQI and i2 feedback

The difference between Option-1a and Option-1b is in how to transmit the first PMIs.  In Option-1a, i11 is multiplexed with RI in the 1st report which is reported in a longer period; i12 is reported in the 2nd report.  In Option-1b, i12 is multiplexed with RI in the 1st report which is reported in a longer period; i11 is reported in the 2nd report.  A UE can be configured with either of Option-1a or Option-1b to report one of the first PMIs in a more frequent basis than the other.  This can be useful when the angular spread in one dimension is much larger than that in the other.  The dimension with smaller angular spread can be reported less frequently, since the PMI tends to vary slowly than the other.

If it needs to adapt the channel variation in both vertical and horizontal, UE can be configured with following submode.

- Option-2: the CSI reporting consists of 3 reports

  - A first reports for RI and PTI feedback

  - A second report, depending on the value of PTI, for either i11 or i12 feedback

  - A third report, depending on the value of PTI, for either

    - i12, wideband CQI and i2 feedback, or

    - i11, wideband CQI and i2 feedback.

The feedback duty cycle of i11 and i12 can be controlled by UE via reporting a proper PTI.  Note that codebook subsampling may be needed for the third reporting type in Option-2 in order to keep the payload size within 11 bits.

Proposal 1: Consider Option-1a, 1b and 2 as three submodes of PUCCH mode 1-1 for the new codebook.

Proposal 2: Further study how to subsample the codebook for the third report in Option-2.

## Mode 2-1

To support subband CQI reporting via PUCCH mode 2-1, we still need to break the CSI reporting into 3 reporting types.  As shown in Figure 1, the 3 reports are

- A first report for RI and PTI feedback

- A second report, depending on the value of PTI, for either

  - the joint encoding of i11 or i12, or

  - wideband CQI and i2 feedback.

- A third report, depending on the value of PTI, for either

  - wideband CQI and i2 feedback, or

  - subband CQI and i2 feedback.

Note that for the subband CQI and i2 feedback, codebook subsampling may be needed to fulfill the 11-bit payload size constraints.  All other reporting types fulfill the payload size requirements.

Proposal 3: Consider the proposed feedback mode for subband CQI reporting.

Proposal 4: Further study if the codebook subsampling is needed or not.

## Collision handling

In Table 2, we summarized all CSI reporting types for the proposed PUCCH modes 1-1 and 2-1.

Table 2: Definition of CSI reporting types for the proposed PUCCH modes 1-1 and 2-1.

| ID | Contents of CSI | Existing Reporting Type |
| --- | --- | --- |
| 1a | RI and wideband 1st PMI i11 feedback | New |
| 1b | RI and wideband 1st PMI i12 feedback | New |
| 2a | Wideband 1st PMI i11 feedback | New |
| 2b | Wideband 1st PMI i12 feedback | New |
| 3 | Wideband CQI and 2nd PMI feedback | Type 2c |
| 4 | RI and PTI feedback | Type 6 |
| 5a | Wideband CQI, 1st PMI i11, and 2nd PMI feedback | New |
| 5b | Wideband CQI, 1st PMI i12, and 2nd PMI feedback | New |
| 6 | 1st PMIs i11 and i12 feedback | New |
| 7 | Subband CQI and 2nd PMI feedback | Type 1a |

All these reporting types can be divided into several priority groups.  For example

- Group 1 (with highest priority) includes Types 1a, 1b and 4 which contain the RI

- Group 2 include Types 2a, 2b and 6 which contain the first PMI(s)

- Group 3 includes Types 3, 5a, 5b and 7 which contain wideband CQI

- Group 4 includes only Type 8 where the subband CQI is reproted

If collision happens between different priority groups, the reporting type with lower priority can be dropped.  If the collision happens within the same priority group, the dropping criteria can be based on the serving cell index, CSI process index and/or subframe set index, etc.

Proposal 5: Consider the proposed collision handling mechanism.

# Conclusions

In summary, we proposed several feedback modes extension to support periodic CSI feedback on PUCCH for the codebook proposed in [2].  Our proposals are as follows.

Proposal 1: Consider Option-1a, 1b, and 2 as three submodes of PUCCH mode 1-1 for the new codebook.

- Option-1a: the CSI reporting consists of 3 reports

  - A first report for RI and i11 feedback

  - A second report for i12 feedback

  - A third report for wideband CQI and i2 feedback

- Option-1b: the CSI reporting consists of 3 reports

  - A first report for RI and i12 feedback

  - A second report for i11 feedback

  - A third report for wideband CQI and i2 feedback

- Option-2: the CSI reporting consists of 3 reports

  - A first reports for RI and PTI feedback

  - A second report, depending on the value of PTI, for either i11 or i11 feedback

  - A third report, depending on the value of PTI, for either

    - i12, wideband CQI and i2 feedback, or

    - i11, wideband CQI and i2 feedback.

Proposal 2: Further study how to subsample the codebook for the third report in Option-2.

Proposal 3: Consider following feedback mode for subband CQI reporting.

- A first report for RI and PTI feedback

- A second report, depending on the value of PTI, for either

  - the joint encoding of i1V or i1H, or

  - wideband CQI and i2 feedback.

- A third report, depending on the value of PTI, for either

  - wideband CQI and i2 feedback, or

  - subband CQI and i2 feedback.

Proposal 4: Further study if the codebook subsampling is needed or not for the third report in subband CQI reporting.

Proposal 5: Consider the proposed collision handling mechanism.

# References

- Draft Report of 3GPP TSG RAN WG1 #82bis.

- R1-156217, WF on class A and class B CSI reporting for Rel.13 EB/FD-MIMO, AT&T, et al.