3GPP TSG RAN WG1 Meeting #83	R1-157756

# Anaheim, CA, USA, 16th - 20th November 2015

# Agenda item:	6.2.4.3.1

Source: 	Samsung, Ericsson, NTT DOCOMO, CATT

Title:	Joint proposal on rank 3-8codebook

Document for:	Discussion and Decision

# Background

This contribution provides the following additional details and revisions as compared to the joint proposal in R1-156390 submitted to the e-mail reflector:

- Proposal for 8-Tx rank 5-8 codebook;

- (s1,s2) values for rank 5-8 codebook; and

- Revisions of (s1,s2) and (p1,p2) parameters for Config 2, 3, 4 for rank 3-4 codebook.

# Rank 3-4 Class A Codebooks

For rank 3-4, the proposed codebook for a given rank value is characterized by four parameters: i11,i12,k,i2. Different values of the parameter k are used to construct different orthogonal beam groups for rank 3-4 codebooks. Figure 1 illustrates three example orthogonal-beam groups, indexed by k = 0,1,2.

Figure 1: Rank 3-4 orthogonal beam pair construction with k = 0, 1, and 2

Table 2 and Table 3 show the rank 3-4 codebook tables that can be used for any of Q = 8, 12, and 16 antenna port configurations, where are selected from Table 1 depending on the k value, the corresponding rank 3 precoder is either , or , and the corresponding rank 4 precoder is.

UE feeds back k in PMI as part of the W1 indication.  In particular, k is jointly encoded with i1 indication(s).

There are two alternatives for the number of values of k:

- If N1>1 and N2>1: k = 0,1 in Table 1

- If N2=1: k = 0,1,2 in Table 1

Table 1: Orthogonal beam type to ( ) mapping

| k |  | 0 | 1 | 2 |
| --- | --- | --- | --- | --- |
| If N1>1 and N2>1 |  | O1 | 0 |  |
|  |  | 0 | O2 |  |
| If N2=1 |  | O1 | 2O1 | 3O1 |
|  |  | 0 | 0 | 0 |

i1,1=0,1,…,O1N1/s1-1

i1,2=0,1,…,O2N2/s2-1

Table 2: Codebook for 3 layer CSI reporting

|  | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| i1,1, i1,2, k |  |  |  |  |
|  | 4 | 5 | 6 | 7 |
| i1,1, i1,2, k |  |  |  |  |
|  | 8 | 9 | 10 | 11 |
| i1,1, i1,2, k |  |  |  |  |
|  | 12 | 13 | 14 | 15 |
| i1,1, i1,2, k |  |  |  |  |
|  | 16 – 31 |  |  |  |
| i1,1, i1,2, k | Entries 16-31 constructed with replacing in third and fourth subscripts with in entries 0 – 15. |  |  |  |

Table 3: Codebook for 4 layer CSI reporting

|  | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| i1,1, i1,2, k |  |  |  |  |
|  | 4 | 5 | 6 | 7 |
| i1,1, i1,2, k |  |  |  |  |
|  | 8 – 15 |  |  |  |
| i1,1, i1,2, k | Entries 8-15 constructed with replacing in third and fourth subscripts with in entries 0 – 7. |  |  |  |

Codebook Subset Selection

Figure 2: An illustration of orthogonal beams for rank 3-4 when k = 0

Table 4: Selected   indices for rank-3 CSI reporting (in Table 2)

| Config | Selected indices | (s1,s2) | (p1,p2) |
| --- | --- | --- | --- |
| 1 | 0,2 | (1,1) | (-,-) |
| 2 | 0 - 7, 16 –23 | (O12,O22) | (O14,O24) |
| 3 | 8-23 | (O1,O22) | (O14,O24) |
| 4 | 0 – 15 | (O1,O24) | (O14,-) |

Table 5: Selected   indices for rank-4 CSI reporting (in Table 3)

| Config | Selected indices | (s1,s2) | (p1,p2) |
| --- | --- | --- | --- |
| 1 | 0, 1 | (1,1) | (-,-) |
| 2 | 0 - 3, 8 – 11 | (O12,O22) | (O14,O24) |
| 3 | 4-11 | (O1,O22) | (O14,O24) |
| 4 | 0 – 7 | (O1,O24) | (O14,-) |

With the (s1,s2) and (p1,p2) parameters proposed in Table 5:

- When O1=8 the effective oversampling factor is the same as legacy (i.e., 4), and;

- When O1=4 the effective oversampling factor is same as the configured one (i.e., 4).

# Rank 5-8 Class A Codebooks

For ranks 5-8, the proposed codebooks are characterized by two parameters: i11,i12, and these are used to form i1 indication(s), rather than i11,i12,k that is used for ranks 3-4.

For rank 5, 6, 7, 8, the precoding matrices are as in the following, where ,,,,, are determined by the RRC ‘Config’ parameter, and

- (s1,s2) = (1,1) for Config 1; and

- (s1,s2) = (O14,O24) for Config 2,3,4

.

Figure 3 Illustration of orthogonal beam grouping for rank 5-8: 16 ports

For 16 ports, ,,,,, are defined as:

Table 6: Delta values for 16-port rank 5-8 codebooks

|  | Antenna configuration |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| config = 4 | N1≥N2 | O1 | 0 | 2O1 | 0 | 3O1 | 0 |
|  | N1<N2 | 0 | O2 | 0 | 2O2 | 0 | 3O2 |
| config =3 | N1≥N2 | O1 | 0 | 2O1 | O2 | 3O1 | O2 |
|  | N1<N2 | 0 | O2 | O1 | 2O2 | O1 | 3O2 |
| Config =1, 2 | Both | O1 | 0 | O1 | O2 | 0 | O2 |

Figure 4 Illustration of orthogonal beam grouping for rank 5-8: 12 ports

For 12 ports, ,,,,, are defined as:

Table 7: Delta values for 12-port rank 5-8 codebooks

| Type | Configuration |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Config= 4 | N1≥N2 | O1 | 0 | 2O1 | 0 | 0 | O2 |
|  | N1<N2 | 0 | O2 | 0 | 2O2 | O1 | 0 |
| Config= 3 | N1≥N2 | O1 | 0 | 2O1 | O2 | O1 | O2 |
|  | N1<N2 | 0 | O2 | O1 | 2O2 | O1 | O2 |
| Config= 1, 2 | Both | O1 | 0 | O1 | O2 | 0 | O2 |

Figure 5 Illustration of orthogonal beam grouping for rank 5-8: 8 ports

For 8 ports, ,,,,, are defined as:

Table 8: Delta values for 8-port rank 5-8 codebooks

| Type | Configuration |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Config= 1, 2, 3, 4 | N1=N2 | O1 | 0 | O1 | O2 | 0 | O2 |

3   Rank 3-8 Class B K=1 Codebooks

Rank 3-4 class B K=1 codebook

.

(Class B codebook)

- Based on class A codebook tables: Table 1 to Table 5

- Configuration = 4, δ1=1, p1=1, δ2=0

- Assume i11=i12=0

- Assume vm1=Q2emod(m1,Np/2)Np/2, um2=1 where eiN is a length-N selection vector

Table 9: Rank 3 Codebook

| NP | Single PMI i2 (Rel.13) | Precoder |  |
| --- | --- | --- | --- |
|  |  | Precoder description based on class A CB (reuse of W2) | Resulting precoder |
| 4 | 0, 1, 2, 3 | i11=i12=0, i2'=0, 1, 2, 3 | e0e0e1e0-e0-e1, e1e0e1e1-e0-e1, e0e1e1e0e1-e1, e1e0e0e1e0-e0 |
| 8 | 0, 1, …, 15 | i11=i12=0, i2'=0, 1, …, 15 | eieiei+1ei-ei-ei+1, ei+1eiei+1ei+1-ei-ei+1, eiei+1ei+1eiei+1-ei+1, ei+1eieiei+1ei-ei, i=0,1,2 and e3e3e0e3-e3-e0, e0e3e0e0-e3-e0, e3e0e0e3e0-e0, e0e3e3e0e3-e3 |

Table 10: Rank 4 Codebook

| NP | Single PMI i2 (Rel.13) | Precoder |  |
| --- | --- | --- | --- |
|  |  | Precoder description based on class A CB (reuse of W2) | Resulting precoder |
| 4 | 0, 1 | i11=i12=0, i2'=0, 1 | e0e1e0e1jke0jke1-jke0-jke1, k=0,1 |
| 8 | 0, 1, …, 7 | i11=i12=0, i2'=0, 1, …, 7 | eiei+1eiei+1jkeijkei+1-jkei-jkei+1, k=0,1; i=0,1,2 and e3e0e3e0jke3jke0-jke0-jke0, k=0,1 |

Rank 5-8 class B K=1 codebook

(Class B codebook)

- Based on class A codebook tables: Section 3

- δ1,1=1, δ1,2=2, δ1,3=3

- Assume i11=i12=0

- Assume vm1=Q2em1Np/2, um2=1 where eiN is a length-N selection vector

Table 11: Rank 5-8 Codebook

| NP | Rank ν | Single PMI i2 (Rel.13) | Precoder |  |
| --- | --- | --- | --- | --- |
|  |  |  | Precoder description based on class A CB (reuse of W2) | Resulting precoder |
| 8 | 5 | 0 | i11=i12=0 | e0e0 e0-e0 e1e1 e1-e1 e2e2 |
|  | 6 | 0 | i11=i12=0 | e0e0 e0-e0 e1e1 e1-e1 e2e2 e2-e2 |
|  | 7 | 0 | i11=i12=0 | e0e0 e0-e0 e1e1 e1-e1 e2e2 e2-e2 e3e3 |
|  | 8 | 0 | i11=i12=0 | e0e0 e0-e0 e1e1 e1-e1 e2e2 e2-e2 e3e3 e3-e3 |