3GPP TSG RAN WG1 #85	R1-164453

May 23rd – 27th, 2016

Nanjing, China

Agenda item:	6.2.8

Source: 	Qualcomm Incorporated

Title: 	MCS table design considerations for UL 256QAM

Document for:	Discussion and Decision

# 1	Introduction

In RAN#71, the study item “New Work Item on Uplink Capacity Enhancements for LTE” as approved [1]. One objective of the WID is to add 256QAM support for LTE UL, with emphasis on small cell operation, for both licensed band and unlicensed band operation. In RAN1 #84b, multiple agreements have been reached, but many details are still FFS, including the detailed design of MCS/TBS table, UE category details, and MCS table configuration details.

In this paper, we provide some simulations results under realistic EVM as inputs to MCS table design.

# 2	Discussion

The current UL MCS table is defined as in 	Table 1.

Table 1. Current LTE UL MCS Table

| MCS Index | Modulation Order | TBS Index | Redundancy Version rvidx |
| --- | --- | --- | --- |
| 0 | 2 | 0 | 0 |
| 1 | 2 | 1 | 0 |
| 2 | 2 | 2 | 0 |
| 3 | 2 | 3 | 0 |
| 4 | 2 | 4 | 0 |
| 5 | 2 | 5 | 0 |
| 6 | 2 | 6 | 0 |
| 7 | 2 | 7 | 0 |
| 8 | 2 | 8 | 0 |
| 9 | 2 | 9 | 0 |
| 10 | 2 | 10 | 0 |
| 11 | 4 | 10 | 0 |
| 12 | 4 | 11 | 0 |
| 13 | 4 | 12 | 0 |
| 14 | 4 | 13 | 0 |
| 15 | 4 | 14 | 0 |
| 16 | 4 | 15 | 0 |
| 17 | 4 | 16 | 0 |
| 18 | 4 | 17 | 0 |
| 19 | 4 | 18 | 0 |
| 20 | 4 | 19 | 0 |
| 21 | 6 | 19 | 0 |
| 22 | 6 | 20 | 0 |
| 23 | 6 | 21 | 0 |
| 24 | 6 | 22 | 0 |
| 25 | 6 | 23 | 0 |
| 26 | 6 | 24 | 0 |
| 27 | 6 | 25 | 0 |
| 28 | 6 | 26 | 0 |
| 29 | reserved |  | 1 |
| 30 |  |  | 2 |
| 31 |  |  | 3 |

In [2], a modified UL MCS table with new 256QAM entries introduced and existing entries modified. The proposed MCS table is repeated in Table 2 below.

Table 2: Proposed MCS table for PUSCH (green MCS entries for 64QAM, blue MCS entries for 256QAM)

| MCS Index | Modulation Order | TBS Index | Spectral Efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 0 | 0.19 |
| 1 | 2 | 2 | 0.31 |
| 2 | 2 | 4 | 0.49 |
| 3 | 2 | 6 | 0.72 |
| 4 | 2 | 8 | 0.97 |
| 5 | 2 | 10 | 1.22 |
| 6 | 4 | 11 | 1.40 |
| 7 | 4 | 13 | 1.78 |
| 8 | 4 | 14 | 1.99 |
| 9 | 4 | 15 | 2.13 |
| 10 | 4 | 16 | 2.26 |
| 11 | 4 | 17 | 2.51 |
| 12 | 4 | 18 | 2.75 |
| 13 | 4 | 19 | 2.99 |
| 14 | 6 | 20 | 3.23 |
| 15 | 6 | 21 | 3.49 |
| 16 | 6 | 22 | 3.72 |
| 17 | 6 | 23 | 3.98 |
| 18 | 6 | 24 | 4.24 |
| 19 | 6 | 25 | 4.41 |
| 20 | 6 | 27 | 4.62 |
| 21 | 6 | 28 | 4.94 |
| 22 | 6 | 29 | 5.12 |
| 23 | 8 | 30 | 5.47 |
| 24 | 8 | 31 | 5.64 |
| 25 | 8 | 32 | 5.89 |
| 26 | 8 | 33 | 6.80 |
| 27 | 8 | 34 | 7.08 |
| 28 | 8 | 35 | 7.37 |
| 29 | reserved |  |  |
| 30 |  |  |  |
| 31 |  |  |  |

In this section, we provide some link level simulation results for the proposed MCS entries under realistic RX and TX side EVM conditions.

The simulation setup is summarized in Table 3. We studied the new 64QAM and 256QAM MCS under different TX and RX EVM values. Note that in DL 256QAM, the EVM is specified to be 3.5%. In this paper, we try a few tentative EVM values for the UL around 3.5%. We also show 6% and 0% EVM values for reference.

Table 3. Simulation setup

| Parameter | Value |
| --- | --- |
| System bandwidth | 10MHz |
| CP type | Normal |
| Duplex mode | FDD |
| Antenna configuration | 1x2 |
| PUSCH transmission mode | Single layer transmission with 1TX |
| RB allocation | Full 50 PRB allocation |
| PUSCH scheduling | All SF |
| MCS simulated | MCS 20~28 in Table 3, with an MCS23* with same spectrum efficiency as MCS23 but using 64QAM |
| Propagation channel model | AWGN, EVA5 |
| TX EVM | 6%, 4%, 3.5%, 3%, 0% |
| RX EVM | 3.5%, 3%, 0% |

The detailed simulation results are included in Appendix A. Here we only provide a summary of the SNR point to reach 70% of the max throughput for each MCS.

Table 4. SNR to reach 70% maximum throughput under AWGN channel

| MCS | Mod Order | Spectrum Efficiency | AWGN |  | EVA5 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0% RX EVM 0% TX EVM | 3.5% RX EVM 3.5% TX EVM | 0% RX EVM 0% TX EVM | 3.5% RX EVM 3.5% TX EVM |
| 20 | 64QAM | 4.62 | 13.8 dB | 13.8 dB | 16.4 dB | 17 dB |
| 21 | 64QAM | 4.94 | 14.8 dB | 14.8 dB | 17.6 dB | 18.5 dB |
| 22 | 64QAM | 5.12 | 15.8 dB | 15.8 dB | 18.4 dB | 19.4 dB |
| 23* | 64QAM | 5.47 | 17.8 dB | 17.8 dB | 21.2 dB | 22.9 dB |
| 23 | 256QAM | 5.47 | 19.1 dB | 21.2 dB | 24.3 dB | 30 dB |
| 24 | 256QAM | 5.64 | 20 dB | 21.5 dB | 25.4 dB | 32 dB |
| 25 | 256QAM | 5.89 | 20.4 dB | 22.5 dB | 26 dB | 37 dB |
| 26 | 256QAM | 6.80 | 21.2 dB | 24.8 dB | 28.2 dB | - |
| 27 | 256QAM | 7.08 | 22.3 dB | 27.4 dB | 30 dB | - |
| 28 | 256QAM | 7.37 | 23.5 dB | - | 33.5 dB | - |

Observation 1: For 64QAM, the EVM of 3.5% at both RX and TX does not affect performance, but for 256QAM, the same EVM will lead to more than 2 dB loss.

Observation 2: Across MCS 24, 25 and 26, under 3.5% TX/RX EVM, the SNR threshold of MCS 25 is very close to that of MCS 24 while there is a 1 bit/s/Hz spectrum efficiency distance from MCS 26.

For MCS 23, though in [2], the SNR decoding thresholds are the same when using 64QAM and 256QAM, under 3.5% EVM for both RX and TX, the SNR decoding threshold is lower when 64QAM is used. It might be more robust to use 64QAM for that MCS.

Proposal 1: For MCS 23 using TBS index 30, uses 64QAM instead of 256QAM.

Proposal 2: Increase the spectrum efficiency of MCS 25 to around 6.2 to make it more uniformly spaced.

# 3	Conclusions

Based on the discussion presented in the paper, we have the following observations and proposals:

Observation 1: For 64QAM, the EVM of 3.5% at both RX and TX does not affect performance, but for 256QAM, the same EVM will lead to more than 2 dB loss.

Observation 2: Across MCS 24, 25 and 26, under 3.5% TX/RX EVM, the SNR threshold of MCS 25 is very close to that of MCS 24 while there is a 1 bit/s/Hz spectrum efficiency distance from MCS 26.

Proposal 1: For MCS 23 using TBS index 30, uses 64QAM instead of 256QAM.

Proposal 2: Increase the spectrum efficiency of MCS 25 to around 6.2 to make it more uniformly spaced.

# 3	Appendix A: Detailed Simulation Results

## 3.1 AWGN Channel Results

Figure 1. MCS20, 64QAM, AWGN

Figure 2. MCS21, 64QAM, AWGN

Figure 3. MCS22, 64QAM, AWGN

Figure 4. MCS23*, 64QAM, AWGN

Figure 5. MCS23, 256QAM, AWGN

Figure 6. MCS24, 256QAM, AWGN

Figure 7. MCS25, 256QAM, AWGN

Figure 8. MCS26, 256QAM, AWGN

Figure 9. MCS27, 256QAM, AWGN

Figure 10. MCS28, 256QAM, AWGN

## 3.2 EVA5 Channel Results

Figure 11. MCS20, 64QAM, EVA5

Figure 12. MCS21, 64QAM, EVA5

Figure 13. MCS22, 64QAM, EVA5

Figure 14. MCS23*, 64QAM, EVA5

Figure 15. MCS23, 256QAM, EVA5

Figure 16. MCS24, 256QAM, EVA5

Figure 17. MCS25, 256QAM, EVA5

Figure 18. MCS26, 256QAM, EVA5

Figure 19. MCS27, 256QAM, EVA5

Figure 20. MCS28, 256QAM, EVA5

# 4	References

[1] RP-160664 “New Work Item on Uplink Capacity Enhancements for LTE”, RAN #71, Feb 2016

- [2] R1-162140, “MCS table for UL 256QAM”, Huawei, HiSilicon, RAN#84bis, April. 11-15, 2016.