3GPP TSG RAN WG1 #85		R1-164802

Nanjing, China 23rd - 27th May 2016

Agenda item:	7.2.5.1

Source:	Samsung, Nokia

Title:	E-mail discussion summary of the large scale calibration

Document for:	Discussion & Decision

# Introduction

From the e-mail discussion of [84b-16] for large scale channel modeling calibration, the following conclusion has been agreed.

Conclusion:

- Large-scale calibration is completed, and the calibration results in the latest version of the excel sheet in the ftp folder are captured in the TR.

  - In the TR, average cdf plots will be provided for the calibration parameters with citing rapporteur’s calibration summary tdoc similarly as done for TR36.873.

  - In the rapporteur’s calibration summary tdoc, the below table on the O2I parameters is included.

- For the full calibration, companies continue discuss how to align the three O2I parameters in [84b-17].

This contribution is the rapporteur’s calibration summary tdoc to be included as a reference in the TR38.900 according to the agreed conclusion. The calibration results can be found in the enclosed excel sheet.

During the e-mail discussion, it has been found that the calibration assumption did not clarify some parameters related to O-I modeling, and the calibration results from different companies are derived with different assumptions. Table 1, summarizes individual companies’ assumptions for the O-I parameters.

Table 1: Individual companies’ assumptions for the O-I parameters

|  | Samsung | Ericsson | CATT | ZTE | Huawei | Nokia | InterDigital | CHTTL | LGE | CATR | ITRI | Intel | Sharp | Qualcomm | Xinwei | NTT DOCOMO | CMCC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| High or low loss | UE-specific | UE-specific | UE-specific | UE-specific | UE-specific | UE-specific | UE-specific | UE-specific | UE-specific | UE-specific | UE-specific | UE-specific | UE-specific | UE- specific | UE-specific | UE-specific | UE-specific |
| d_in | UE-specific | UE-specific | UE-specific | UE-specific | UE-specific | Link Speicfic | UE-specific | Link specific | Link specific | UE-specific | UE-specific | UE-specific | UE-specific | UE-specific | UE-specific | Link-specific | Link-specific |
| O-I penetration loss deviation | UE-specific | Link specific* | UE-specific | Link specific* | Link specific* | Link Specific | UE-specific | Link specific | Link specific* | Link specific* | Link specific | Link specific* | Link specific | Link-specific | Link specific* | Link-specific* | Link-specific* |

* Note: O-I penetration loss deviation has replaced the shadow fading term