3GPP TSG RAN WG1 Meeting #85                                                                   R1-155199

Nanjing, China 23rd - 27th May 2016

Source:	NTT DOCOMO

Title:	Discussion on CSI-RS Port Extension

Agenda Item:	6.2.3.1.1

Document for: 	Discussion

# Introduction

At the RAN1 #84bis meeting, there were intensive discussions on CSI reporting for hybrid CSI-RS especially on potential technologies [1]. Agreements and observations are captured as follows.

Agreements:

- For {20, 24, 28, 32} ports, a CSI-RS resource for class A CSI reporting is composed as an aggregation of K CSI-RS configurations [i.e. RE patterns].

  - The number of REs in the kth configuration Nk ∈ {4, 8}

    - The same Nk = N can be used for all k

      - FFS whether the same Nk = N for all k is the only permitted configuration

      - FFS whether the set of values of Nk might be further restricted for some numbers of CSI-RS ports

    - FFS whether a different set of Nk might apply in case of CDM4

    - FFS on including Nk=2.

  - Aim to enable the support of CSI-RS port sharing with Rel-13 and Rel-12 UEs

  - The per-port CSI-RS density is FFS based on one or more of the following alternatives:

    - FDM

    - TDM

    - Partial port

    - Partial overlapping, e.g. for 32 ports, ports 15-38 in PRB#1, ports 23-46 in PRB#2

    - Aperiodic CSI-RS with partial bandwidth

    - Measurement restriction in frequency domain

    - CDM, e.g. 2 x Nk ports transmitted in a single Nk resource

    - Other schemes

    - Note that the following are not precluded:

      - per-port CSI-RS density per PRB = 1

      - different per-port CSI-RS densities for different CSI-RS ports is not precluded

In this contribution, we further discuss CSI-RS enhancement to achieve {20, 24, 28, 32} CSI-RS ports with mechanism to reduce CSI-RS overhead.

# Discussion

Fig. 1 shows RE mapping of CSI-RS for legacy LTE releases. The RE mapping were determined to avoid collisions to other physical channels and signals such as PDCCH, CRS and DM-RS. Totally, 40 REs are reserved per PRB pair for all of the different AP configurations, i.e., {1, 2, 4, 8, 12, 16}-Tx CSI-RS. According to the increase in the number of CSI-RS APs, overhead becomes non-negligible. For example, if we assume similar CSI-RS design, overhead of 32-Tx CSI-RS will be 3.8 (11.4) % for the reuse factor of 1 (3) with the transmission periodicity of 5 ms. In addition, if we keep current RE pool and density of CSI-RS, reuse factor of CSI-RS per subframe reduces to 1 for {24, 28, 32}-Tx CSI-RS. Considering the discussion above, it is important to consider reducing CSI-RS overhead and/or increasing CSI-RS capacity per subframe.

Figure 1: RE mapping of conventional CSI-RS design

Followings show some candidate schemes for Rel. 14 CSI-RS design, which can be jointly applied.

- TDM and FDM with multiple PRB pairs

Figs. 2 show examples of TDM and FDM based CSI-RS multiplexing schemes. In Rel. 13 LTE, CSI-RS resource is configured by aggregating multiple legacy CSI-RS resources. There is a restriction that legacy resources are multiplexed in a single subframe. Considering the number of supported APs in Rel. 14, it can be beneficial to consider TDM and FDM based multiplexing scheme using multiple PRB pairs. Figs. 2a and 2b show two different implementations of enhanced CSI-RS transmission with TDM. There are two different CSI-RS sets in the figures that can be different in terms of APs, e.g., APs 15-30 and 31-46, polarization, e.g., polarization with +45 and -45 degs, etc. In order to avoid the influence of frequency offset, the interval between set 1 and 2 should be small such as in Fig. 2(b). The effect of frequency offset should be carefully studied for these TDM based schemes. Fig. 2(c) and 2(d) show similar multiplexing scheme with FDM and TDM+FDM, respectively. These schemes don’t suffered from frequency offset as in TDM based schemes, but should be carefully considered in terms of backward compatibility, since both of CSI-RS sets don’t follow legacy CSI-RS resource mapping.

(a) TDM (example 1)                                                                     (b) TDM (example 2)

(c) FDM                                                                                 (d) TDM+FDM

Figures 2: Multiplexing of CSI-RS with TDM and FDM

- Reduction of CSI-RS RE density per RB and AP

In the legacy LTE releases, CSI-RS density per RB is 1RE/RB/port. It may be beneficial that the density is reduced in Rel. 14. One option is to reduce CSI-RS density for all APs, e.g., 0.5RE/RB/port for APs 15-46. Another option can be reducing CSI-RS density for a part of APs. For example, CSI-RS with APs 15-30 (31-46) can be multiplexed with density of 1 (0.5) RE/RB/port. The first option may have an impact on backward compatibility, since CSI-RS design for legacy antenna ports are modified.

- Partial transmission of CSI-RS

In order to obtain CSI, which is just enough for precoding, CSI-RS doesn’t need to be transmitted from all the APs. For example, AoD and EoD for different polarizations may exhibit similar characteristics. In this case, some antenna ports for one of the polarizations can be omitted. As another example, legacy precoding technology is designed to utilize channel information in horizontal and vertical domains together with co-phasing. It can be also considered that CSI-RS is transmitted in order to obtain horizontal and vertical channel information and co-phasing.

- Increase of the number of CSI-RS RE

As described above, Rel. 13 LTE utilizes 40 REs for CSI-RS transmission. If we don’t follow the restriction of existing RE pool, residual REs can be used for CSI-RS REs. For instance, 49 REs are newly reserved, if PDSCH region is replaced to CSI-RS. Furthermore, 27 REs are additionally reserved, if DM-RS regions are reused. These subframes with a number of CSI-RS REs may be no longer useful for PDSCH transmission, since the number of residual REs is quite limited. Hence, they can be used as subframes dedicated for CSI-RS transmission and other controlling objectives. If this scheme is introduced, performance is degraded for legacy UEs. More specifically, Rel. 8 doesn’t have method to know that PDSCH is punctured by CSI-RS. Similarly, Rel. 13 UE doesn’t recognize RE locations of additional CSI-RS resources. These UEs are suffered by interference caused by unknown CSI-RS.

New CSI-RS design should be carefully determined considering multiple factors such as performance benefit, CSI-RS overhead, backward compatibility and flexibility for further enhancement, reuse factor per subframe, etc. In addition, it should be further discussed whether CSI-RS without overhead reduction scheme, i.e., CSI-RS transmission with 1RE/RB/port for all APs, is also supported. According the discussion above, we can make the following proposals.

Proposal 1: Rel. 14 CSI-RS design should take into account following factors.

- Performance benefit

- CSI-RS overhead

- Backward compatibility and flexibility for further enhancement

- Reuse factor per subframe

Proposal 2: Following schemes are considered as potential technologies for non-precoded CSI-RS enhancement to enable {20, 24, 28, 32}-Tx CSI-RS.

- TDM and FDM using multiple PRB pairs

- Reduction of RE density per RB and AP

- Partial transmission of CSI-RS

- Increase of the number of CSI-RS RE

Proposal 3: Necessity of CSI-RS transmission without overhead reduction technology is further studied.

# Summary

In this contribution, we discussed CSI-RS enhancement to achieve {20, 24, 28, 32} CSI-RS ports with mechanism to reduce CSI-RS overhead. Based on the discussion, we made the following proposals.

Proposal 1: Rel. 14 CSI-RS design should take into account following factors.

- Performance benefit

- CSI-RS overhead

- Backward compatibility and flexibility for further enhancement

- Reuse factor per subframe

Proposal 2: Following schemes are considered as potential technologies for non-precoded CSI-RS enhancement to enable {20, 24, 28, 32}-Tx CSI-RS.

- TDM and FDM using multiple PRB pairs

- Reduction of RE density per RB and AP

- Partial transmission of CSI-RS

- Increase of the number of CSI-RS RE

Proposal 3: Necessity of CSI-RS transmission without overhead reduction technology is further studied.

# References

- 3GPP, “Draft Report of 3GPP TSG RAN WG1 #84b v0.1.0.”