3GPP TSG RAN WG1 Meeting #85 	R1-165206

Nanjing, China 23rd - 27th May 2016

Source:	NTT DOCOMO

Title:	Evaluation for CRS-based MUST with 4 Tx antennas

Agenda Item:	6.2.4.3

Document for: 	Discussion

Introduction

At the RAN#71 meeting, a new work item regarding Downlink Multiuser Superposition Transmission (MUST) for LTE was agreed [1]. In the WI, up to 4 Tx CRS-based or 8 Tx DMRS-based transmission schemes can be used for MUST.

- For all three Cases using up to 4 Tx CRS-based or up to 8 Tx DMRS-based transmission schemes, evaluate the system-level performance based on the evaluation methodology and assumptions in TR36.859.

In this contribution, the system-level performance of DL MUST with 4 Tx CRS-based transmission scheme are evaluated and discussed based on the evaluation methodology and assumptions in [2].

Simulation assumptions

The system-level simulations are conducted with MUST scenario 1 and evaluation assumptions in [2]. The detailed simulation parameters are shown in Table A-I in the Annex A. We assume 2x2 and 4x2 antenna configurations with SU-MIMO of TM4. Note that the size of codebook for precoding is 4 for 2x2 antenna configurations, while that for 4x2 antenna configuration is 16.

MUST with CRS-based transmission scheme is considered. When MUST is applied, up to two superposed data layers from two co-scheduled UEs per spatial layer are supported and the maximum number of spatial layers in a cell is two. For the superposed UEs per spatial layer, the same precoder is applided. We assume 4 power sets for multiple transmission power ratios, where power ratios which are most likely to be chosen are selected, i.e., (α1, α2), where α1=0.14, 0.17, 0.23, 0.36. Transmission power alignment for different subbands is assumed to reduce signaling overhead and receiver complexity. The scheduling SINRs of superposed users are approximated from LTE OFDMA implicit feedback and OLLA is further applied to compensate the CQI imperfections in the evaluations. The R-ML receiver is adopted for MUST-Near UE to deal with the inter-user interference.

In the evaluations, FTP traffic model 1 with resource utilization (RU) ranging from 70% to 90% are assumed. File dropping is modeled according to [3]. For performance metrics of FTP traffic, 5/50/95%, mean user perceived throughput (UPT) and mean UPT below 5% CDF are evaluated. In addition, ratio of served cell throughput over offered cell throughput is also provided.

Simulation results

Table I: Performance of MUST with 2x2 and 4x2 SU-MIMO for subband scheduling for FTP traffic (~70%, 80% RU)

| Throughput (Mbps) | 2x2 antennas |  |  | 4x2 antennas |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | MUST | Gain | Baseline | MUST | Gain |
| Mean UPT | 11.8688 | 12.2950 | 3.59% | 13.7610 | 13.9363 | 1.27% |
| 95%ile UPT | 38.0952 | 38.0953 | 0.00% | 42.1053 | 42.1053 | 0.00% |
| 50%ile UPT | 7.6924 | 8.0808 | 5.05% | 9.3026 | 9.5239 | 2.38% |
| 5%ile UPT | 1.4626 | 1.5595 | 6.63% | 1.8736 | 1.9231 | 2.64% |
| Mean UPT below 5% | 1.0437 | 1.1238 | 7.67% | 1.3810 | 1.4207 | 2.87% |
| RU (%) | 71.75% | 70.85% | ---- | 70.03% | 69.72% | ---- |
| Served/Offered | 99.57% | 99.63% | ---- | 99.77% | 99.78% | ---- |
| λ / packet size | 12.0 / 100 Kbytes |  |  | 14.5 / 100 Kbytes |  |  |
| Throughput (Mbps) | 2x2 antennas |  |  | 4x2 antennas |  |  |
|  | Baseline | MUST | Gain | Baseline | MUST | Gain |
| Mean UPT | 8.7860 | 9.3872 | 6.84% | 9.1146 | 9.4538 | 3.72% |
| 95%ile UPT | 29.6297 | 30.7693 | 3.85% | 30.7692 | 30.7697 | 0.00% |
| 50%ile UPT | 5.2981 | 5.7556 | 8.64% | 5.4422 | 5.7555 | 5.76% |
| 5%ile UPT | 1.0001 | 1.1060 | 10.59% | 1.0382 | 1.1128 | 7.19% |
| Mean UPT below 5% | 0.6646 | 0.7592 | 14.23% | 0.7129 | 0.7805 | 9.48% |
| RU (%) | 82.60% | 81.51% | ---- | 84.87% | 84.24% | ---- |
| Served/Offered | 98.95% | 99.21% | ---- | 99.16% | 99.33% | ---- |
| λ / packet size | 13.5 / 100 Kbytes |  |  | 17.0 / 100 Kbytes |  |  |

Table I presents the system performance of MUST with 2x2 and 4x2 SU-MIMO for subband scheduling for FTP traffic, with RU around 70% and 80%. As shown in Table I, it can be observed that: MUST can achieve performance gains with FTP RU~70% and 80% traffic load for 2 Tx and 4 Tx antennas. For FTP traffic with RU~70%, MUST with 4 Tx antennas can achieve about 1.27% gains for mean UPT and 2.64% gain for 5%ile UPT. When high traffic load is assumed, i.e. RU~80%, performance gains of MUST with 4 Tx antennas are about 3.72% for mean UPT and 7.19% for 5%ile UPT. The performance gains of MUST with 4 Tx reduce compared to those with 2 Tx antennas.

Observation 1: Performance gains of MUST for 4 Tx are lower than those for 2 Tx. MUST with 4 Tx antennas can achieve 3.72% for mean UPT 7.19% for 5%ile UPT with RU around 80%.

Figure 1 shows the statistics of UE pairing for MUST with 2 Tx and 4 Tx antennas for FTP traffic (RU~80%). From the results, we can observe that the UE pairing probability for MUST decreases as number of antennas increases. For 4 Tx, the percentage of MUST UE pairing is about 7%, while 12% of UE pairing for 2 Tx. Since the codebook size in 4 Tx is larger than that in 2 Tx, the UE pairing probability for MUST with same precoder restriction becomes smaller with 4 Tx antennas. Therefore, the performance gains of MUST with 4 Tx reduce in respect to those with 2 Tx.

Fig. 1. UE pairing probability for MUST with 2x2 and 4x2 antennas configurations (FTP traffic, RU~80%)

Observation 2: UE pairing probability for MUST in 4 Tx with same precoder restriction is smaller due to larger size of codebook in 4 Tx compared to 2 Tx.

Observation 3: Compared to 2CRS APs, additional specification impact by supporting 4CRS APs may not be expected.

Conclusion

In this contribution, we discussed the evaluation of MUST with 4 Tx antennas CRS-based transmission scheme. The system-performance of MUST with 4x2 SU-MIMO assuming non-full buffer traffic and R-ML receiver is provided and discussed. According to the discussion, we have the following observations.

Observation 1: Performance gains of MUST for 4 Tx are lower than those for 2 Tx. MUST with 4 Tx antennas can achieve 3.72% for mean UPT 7.19% for 5%ile UPT with RU around 80%.

Observation 2: UE pairing probability for MUST in 4 Tx with same precoder restriction is smaller due to larger size of codebook in 4 Tx compared to 2 Tx.

Observation 3: Compared to 2CRS APs, additional specification impact by supporting 4CRS APs may not be expected.

- References

- 3GPP RP-160680, Mediatek, “New work item proposal: Downlink Multiuser Superposition Transmission for LTE”, Mar. 2016.

- 3GPP TR 36.859 V13.0.0, “Study on Downlink Multiuser Superposition Transmission (MUST) for LTE,” Dec. 2015.

- 3GPP TR 36.814 V9.0.0, “Further advancements for E-URRA physical layer aspects”, Mar. 2010.

- Annex A: Simulation assumption

Table A-I: Simulation parameters

| Layout | Hexagonal grid, 3 sectors per site, 19 macro sites (ISD = 500 m) |
| --- | --- |
| System bandwidth per carrier | 10 MHz |
| Carrier frequency | 2.0 GHz |
| Total BS TX power (Ptotal per carrier) | 46 dBm |
| Distance-dependent path loss | ITU UMa |
| Penetration loss | For outdoor UEs:0dB For indoor UEs: 20dB+0.5din (din : independent uniform random value between [ 0, min(25,d) ] for each link) |
| Shadowing | ITU Uma |
| Antenna pattern | 3D (referring to TR36.819) |
| Antenna Height: | 25 m |
| UE antenna Height | 1.5 m |
| Antenna gain + connector loss | 17 dBi |
| Antenna gain of UE | 0 dBi |
| Fast fading channel between eNB and UE | ITU UMa |
| Antenna configuration | BS: 2/4 Tx (0.5 lambda), cross-polarized UE: 2Rx (0.5 lambda), cross-polarized |
| UE dropping | 20% UEs are outdoor and 80% UEs are indoor. |
| minimum distance from macro-cell to UEs | 35 m |
| Traffic model | FTP: 0.1 MByte, RU=60%~90% |
| UE receiver | - MMSE-IRC is assumed for inter-cell interference suppression - R-ML for inter-spatial-layer - R-ML/ideal SIC for inter-user interference |
| Transmission mode | TM4 |
| UE noise figure | 9 dB |
| UE speed | 3 km/h |
| Cell selection criteria | RSRP |
| Handover margin | 3 dB |
| Scheduling algorithm | Proportional fairness maximization |
| Control delay (scheduling, AMC) | 5 msec |
| HARQ | Chase combining |
| Round trip delay (HARQ) | 8 msec |
| Granularity of CSI feedback | 5 msec |
| Granularity of rank adaptation | 100 msec |
| CQI quantization | Yes |
| Codebook | LTE Rel. 8 |
| Power ratio sets | Adaptive power allocation: 4 power sets, i.e. (α1, α2), where α1=0.14, 0.17, 0.23, 0.36; |
| OLLA | Yes |
| Number of superposed signals in superposition transmission | 2 |
| Receiver impairment modeling for demodulation | Non-ideal CRS-based channel estimation |
| EVM | Tx EVM: 8%, UE Rx EVM: 4% |
| Duration of simulation | 50000 msec (5000 pre-run + 45000 simulation) |
| Maximum transfer time (T_drop) | 1.6 sec |