3GPP TSG RAN WG1 Meeting #85			R1-165412

Nanjing, China, 23th - 27th May 2016

Agenda Item:	6.2.2.1.1

Source: 	CATR

Title: 	Link level performance of DM RS for V2V

Document for:	Discussion and Decision

Introduction

At the RAN1 #84 meeting, the following agreements for DMRS enhancements for V2V were reached [1]:

- Confirm the working assumption:

  - 15 kHz subcarrier spacing with 1 msec TTI length

- Working assumption is alt 5:

  - “4V structure” for PSSCH/PSCCH is kept (which is already an agreement in RAN1)

  - In order to support 500 km/h relative speed case,  lowering the coding rate can be used

    - FFS how to adapt MCS, RB size, and/or number of transmission subframes depending on the situation

      - This may or may not have any specification impact

  - adapt MCS, the number of RBs, and number of transmission subframes depending on the UE absolute speed and UE synchronization source (e.g, GNSS or eNB)

  - FFS: One or more PSCCH format(s) need to be supported

- All proponents are requested to analyze/evaluate Alt. 5 until the next meeting, and RAN1 will decide the final decision in RAN1 #85 meeting

In this contribution, we compare the performance of the “4V structure” DM RS with timing error for PSSCH. According to the revised WID [2], up to 500km/h relative speed needs to be supported. The performance of the “4V structure” DM RS for PSSCH is evaluated for the 500km/h scenario. The evaluation results show that the the “4V structure” DM RS is robust to timing errors, and it can also work in case of the extreme high speed scenario with adaptation of MCS/RB size.

Evaluation performance of DM RS for V2V

According to the revised WID [2]: The outcome of this work item should be able to support a relative speed of up to 500 km/h with enhancements/changes (e.g., adaptation of code-rate, the DMRS mapping/ structure) (if necessary) to the physical layer structure designed for the relative speed up to 280 km/h.

In this section, we evaluated the performance of the “4V structure” DM RS (DM RS symbols are located in symbol 2, 5, 8 and 11) with timing error for PSSCH for extremely high speed scenarios. The scenarios of 500km/h relative speed (250km/h per vehicle) are simulated.Linear interpolation is used for channel estimation. The evaluation parameters are given in Appendix.

Figure 1 performance of PSSCH with relative speed of 500km/h(190 bytes)

Figure 2 performance of PSSCH with relative speed of 500km/h(300 bytes)

Figure 1 is the performance of PSSCH with 0us or 2us timing error , when the relative speed is 500km/h there is an error floor for 190 bytes with MCS level corresponding to QPSK r=1/2. The error floor can be reduced using a lower coding rate of QPSK r=1/3.

Figure 2 is the performance of PSSCH with 0us or 2us timing error , when the relative speed is 500km/h there is an error floor for 300 bytes with MCS level corresponding to QPSK r=1/2. The error floor can be reduced using a lower coding rate of QPSK r=1/3.

Observation: For PSSCH, to support extremly high relative speed of 500km/h, the lower MCS level could get better performance with the “4V structure” DM RS.

Conclusion

Performance of “4V structure” D< RS for PSSCH with timing error is evaluated in scenarios of extremly relative high speed to500km/h, based on the results and analysis, the following observation is made:

Observation: For PSSCH, to support extremly high relative speed of 500km/h, the lower MCS level could get better performance with the “4V structure” DM RS.

- References

3GPP RAN1 #84bis, Chairman Notes

RP-160649, “Revised WID: Support for V2V services based on LTE sidelink”, LGE, Huawei, HiSilicon, CATT, CATR,  Mar. 7-10, 2016

R1-162118	Further evaluations of DMRS for PSSCH/PSCCH	Huawei, HiSilicon

- Appendix A: Simulation setting

Table 1. Parameter setting of link level simulation

| Parameters | Assumptions |
| --- | --- |
| General parameters: |  |
| System bandwidth | 10 MHz |
| Carrier frequency | 6 GHz |
| Channel model | ITU-R UMi NLOS |
| Antenna configuration | 1 antenna @ transmitter 2 antennas @ receiver |
| UE relative speed | 500 km/h |
| Number of transmissions | 1 |
| CFO | B1 (case B + case 1); A2 (case A + case 2); |
| Frequency offset estimation | time domain |
| Channel estimation | Linear interpolation |
| PSSCH parameters: |  |
| TBS | {190, 300} bytes |
| Modulation | QPSK |
| Coding rate | 1/2,1/3 |