3GPP TSG RAN1 #85		R1-165773

# Nanjng, China, May 23 – 27, 2016

Source:	Qualcomm, Intel, Samsung, Ericsson

Title:	TP for blockage modeling

Agenda Item:	7.2.3.1

Document for:	Discussion and Decision

# Text Proposal to Section 7.6.4

======================== START OF TEXT PROPOSAL ===========================

Blockage modeling is an add-on feature to the channel model. The method described in the following applies only when this feature is turned on. In addition, the temporal variability of the blockage modelling parameters is on an on-demand basis. It is also noted that the modeling of the blockage does not change LOS/NLOS state of each link.

When blockage model is applied, the channel generation in Section 7.5 should have several additional steps between Steps 9 and 10, as illustrated in Figure 7.6.4-1.

Figure.7.6.4-1 Channel generation procedure with blockage model

Two options (Option A and Option B) are provided for the specific methodology of blockage modeling.

Option A:

Option A proposes a stochastic approach based on a geometric model for capturing human and vehicular blocking.

Step a: Determine the number of blockers

A number K of 2-dimensional (2D) angular blocking regions, in terms of azimuth and elevation angles, , are generated around the UT:

.

Optionally, one of the K angular blocking regions represents self-blocking (i.e. human/hand holding the UT); this blocking region is not spatially consistent and generated specific to each UT.

The total number of non self-blocking angular regions is 4.

Step b: Generate the size and location of each blocker

The k-th blocking region is made of degrees in azimuth anddegrees in elevation. The center angle pair in azimuth and elevation of the k-th blocking region is (φk,θk). These parameters are given as follows:

| Blocker index | φk | xk | θk | yk |
| --- | --- | --- | --- | --- |
| k = 1 (Portrait mode) | 260o | 120o | 100o | 80o |
| k = 1 (Landscape mode) | 40o | 160o | 110o | 75o |
| k = 2, …, 5 (InH scenario) | Uniform in [0o, 360o] | Uniform in [15o, 45o] | 90o | Uniform in [5o, 15o] |
| k = 2, …, 5 (UMi, UMa, RMa scenarios) | Uniform in [0o, 360o] | Uniform in [5o, 10o] | 90o | 5o |

Step c: Determine the attenuation of each cluster due to blockers

The attenuation of each cluster, which is characterized by the center angle pair (φAoA,θZoA), due to self-blocking is 30 dB provided that φAoA- φ1<x12 and θZoA- θ1<y12.

The attenuation of each cluster due to the other blockers is given as

LdB=-20log10 1-FA1+FA2FZ1+FZ2

provided that φAoA- φk<xk and θZoA- θk<yk. The terms in the above equation are given as

F A1A2Z1|Z2=tan-1±π2πλ r1cosA1|A2|Z1|Z2- 1π

where

A1=xk2-φAoA- φk, A2=xk2+φAoA-φk,Z1= yk2-θZoA- θk, Z2= yk2+θZoA-θk.

The appropriate signs within the tan-1(.) term are according to the following table:

|  | -yk<θAoA- θk≤-yk2 | -yk2<θAoA- θk≤yk2 | yk2<θAoA- θk≤yk |
| --- | --- | --- | --- |
| xk2<φAoA- φk≤xk | (-, -) for A1,A2 (-, -) for Z1,Z2 | (-, +) for A1,A2 (-, -) for Z1,Z2 | (-, -) for A1,A2 (-, -) for Z1,Z2 |
| -xk2<φAoA- φk≤xk2 | (-, -) for A1,A2 (+, -) for Z1,Z2 | (+, +) for A1,A2 (+, +) for Z1,Z2 | (-, -) for A1,A2 (-, +) for Z1,Z2 |
| -xk<φAoA- φk≤-xk2 | (-, -) for A1,A2 (-, -) for Z1,Z2 | (+, -) for A1,A2 (-, -) for Z1,Z2 | (-, -) for A1,A2 (-, -) for Z1,Z2 |

In the above formula for F(.), r is the distance between the UT and the blocker which is fixed at r=2 meters for the InH scenario and r=10 meters for the UMi/UMa/RMa scenarios and λ is the wavelength.

Step d: Spatial and temporal consistency of each blocker

The blocking parameters (center of blocker and attenuation loss) are generated in a spatially consistent manner using the method described in Section 7.6.3 with the following correlation distance table.

|  | UMi |  |  | UMa |  |  | RMa |  |  | InH |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LOS | NLOS | O-I | LOS | NLOS | O-I | LOS | NLOS | O-I |  |
| Correlation distance (in m) for cluster specific random variables | 10 | 10 | 5 | 10 | 10 | 5 | 10 | 10 | 5 | 5 |

To model the mobility of the blockage, a linear interpolation approach interpolating two attenuation values (obtained at two temporal points corresponding to the scenario simulation in time) is used.

Option B:

Option B proposes a hybrid approach based on a geometric model for capturing human and vehicular blocking.

======================== END OF TEXT PROPOSAL ===========================