3GPP TSG-RAN WG1 #86                                                          	            R1-166259

22nd – 26th August, 2016

Gothenburg, Sweden

Agenda item:	7.2.2.2.3

Source: 	Qualcomm Incorporated

Title: 	Measurement metric for congestion control

Document for:     Discussion/Decision

# 1	Introduction

During RAN1#85 the following agreements were achieved on measurements of congestion level for LTE V2V:

Agreements:

- A measurement metric is specified to reflect the congestion level of a PC5 carrier, e.g., similarly to the channel busy ratio defined in ETSI.

  - This measurement is available to higher layers in the UE.

    - FFS whether the measurement can be reported to eNB.

  - FFS the details of the measurement and the UE behavior, for example:

    - The principle to drop PC5 transmissions as a function of this measurement and priority is supported.

    - The radio adjusts radio parameters (max tx power, nr of retx restriction, MCS range restriction, nr of PRBs restriction) as a function of priority and this measurement.

    - Packets with different priorities are transmitted on the same resource pool

The motivation of defining such a measurement in 3GPP arises from the fact that the ETSI harmonized standard EN 302.571 for the unlicensed ITS spectrum in Europe mandates the use of decentralized congestion control (DCC). This is to make sure that radio channel is not congested with too many transmissions in a certain geographical area [Section 4.2.10, ETSI EN 302.571 v2.0.0]. However, we note that the current requirements were specified with 802.11p as the technology, and are currently being debated for technology neutrality [3].

Given that the target of defining this measurement for radio congestion for LTE V2V is to meet the requirements of the harmonized standard in EU, and that the current requirements are themselves being debated and revised for technology neutrality, we attempt to provide a more holistic discussion for DCC in this paper.

The paper is organized as follows:

- Section 2 provides a background on the existing requirements for DCC in ETSI EN 302.571,

- Section 3 provides a generalized formulation for DCC and discusses how it maps to 802.11p and LTE-V2V,

- Section 4 provides some simulation results with DCC,

- Section 5 concludes the contribution.

# 2	Background: DCC requirements in EN 302.571

In this section, we briefly summarize the existing requirements for decentralized congestion control as specified in the existing harmonized specification ETSI EN 302.571 v2.0.0:

Channel busy time / channel busy ratio (CBR) is used to determine the transmission behaviour. channelBusyTime=(1 ∀ probes with channel busy)/Np Where Np is the total number of probes, and channel is declared as busy if the received signal strength exceeds a threshold Sth. The parameters used to determine the channelBusyTime are specified as follows: Parameters; Value; Description. Tp; 8 us; Probing time. Tm; 100 ms; Measuring interval. Sth; ≤ -85dBm; Signal strength threshold for determining when the channel is busy or not. Np; 12500; Number of probes during the measurement interval Tm. The requirements for DCC for CBR < 62% are specified as: Parameters; Value; Description. Ton; 4 ms; A Ton of 4ms corresponds to transmitting a packet of length 1500bytes at a transfer rate of 3Mbit/s. Toff; 25 ms; A Toff of 25ms corresponds to 40Hz. Duty cycle; 3%; The duty cycle is defined as the ratio, expressed as a percentage of the maximum transmitter “on” time Ton on one carrier frequency, relative to a 1 sec period. Frame body; 1500 byte; The Frame Body field in IEEE 802.11-2012 [12], Figure 8-1, shall not exceed 1500 bytes. The requirements for DCC for CBR ≥ 62% are specified as: ToffLImit=1CW×Ton×4000×CBR-CTHCBR-1 Where Parameters; Value; Description. CTH; 0.6; The congestion threshold. CW; 1; The weight factor.

From the requirements above, it can be readily seen that the requirements are derived assuming 802.11p as the technology. Some of these are

- Channel busy time measurement is defined assuming TDMA access (no FDM)

- Radio parameters are 802.11p specific

  - Probe time corresponds to the OFDM symbol time of 802.11p

  - Signal strength threshold Sth corresponds to the reference sensitivity for 802.11p

  - Ton is calculated using the minimum data rate for 802.11p

- CBR limits of 62% were derived using system simulation with 802.11p channel access

- For CBR > 62%, the DCC limits the channel utilization at a UE by limiting the Toff / Ton time. This relates to TDMA access. Further, the formulation is also 802.11p specific as derived in the next section.

Observation 1: The existing DCC requirements in ETSI harmonized specification EN 302.571 v2.0.0 are derived assuming 802.11p based channel access.

Motivated by the observation above, we take a step back to look at DCC from a technology-neutral viewpoint and derive the 802.11p requirements as a subset of a more technology neutral formulation.

# 3	Formulation for DCC

## 3.1	Generalized formulation

In the generalized formulation, the channel busy ratio can be defined as the ratio of radio resources (i.e. time-frequency instead of only time dimension) that were deemed busy. A resource is deemed busy if the received signal strength on the time-frequency resource exceeds a given threshold (Sth).

channelBusyRatio(CBR)=(1 ∀ probes with radio resource busy)/Np,

where Np are the total number of radio resources probed for CBR measurement within a measurement window. Further, a resource is defined as the minimum time-frequency unit of allocation possible for the technology (e.g. 8us x 8.125MHz for 802.11p, 1 PRB pair for LTE-V2V). The threshold Sth will be technology specific.

The idea of the CBR measurement is to obtain an indirect measure of the total number of transmissions in the UEs proximity [4]. Thus the CBR measurement relates to the number of ITS stations (Nsta) in the proximity. This relation is expressed as

CBR=fNsta

The function f() on mapping of Nsta to CBR depends on the channel access method, e.g. asynchronous vs synchronous channel access and on channel access parameters as well.

The idea of DCC is then to limit the channel utilization by each UE to a maximum allowed limit (CBRlimit) of the ratio of total radio resources that can be used in a given geographical area. One way to do it is to split the maximum allowed fraction of busy radio resources among the ITS stations in the given geographical area. The limit of the channel resource utilization (CRlimit) can then be expressed as:

CRlimit=CBRlimitNsta=CBRlimitf-1CBRmeasured

The DCC loop in the UE then ensures that the channel resource utilization (CR) remains below CRlimit, i.e. CR≤ CRlimit.

In a slight alteration to the above formulation (to resemble the harmonized specification), the DCC loop can be triggered only when the CBRmeasured exceeds the CBRlimit. With that, the CRlimit is expressed as a function of CBRmeasured alone as:

If CBRmeasured> CBRlimit, then

CRlimit=CBRmeasuredf-1CBRmeasured

The above formulation thus takes the current channel utilization (if exceeding the CBRlimit) and splits the utilized resources among the ITS stations in a given geographical area.

Observation 2: DCC can be described in a generalized form that applies to all technologies.

## 3.2	Specialization of the generalized formulation for 802.11p

In this subsection, we derive the existing requirements in the harmonized specification EN 302.571 using certain assumptions specific to 802.11p channel access.

The subsection thus serves two purpose:

- Validating the generalized formulation and the existing requirements as a subset of the generalized formulation, and

- Emphasizes certain assumptions that are specific to 802.11p (that may not be evident otherwise) in the existing requirements in EN 302.571. Furthermore, it allows us to question such assumptions from a technology-neutrality viewpoint (or at least for LTE V2V).

Assumption 1: CR usage is based on TDMA access, i.e.

CR=TonTon+Toff

Assumption 2: CBR is assumed to be a linear function of Nsta, i.e.,

CBRmeasured=a×Nsta+b

Substituting these two assumptions in the generalized formulation, it leads to

1CRlimit=1+ToffLimitTon=f-1CBRmeasuredCBRmeasured
=1aCBRmeasured-bCBRmeasured 
The ToffLimit is thus specified as:

ToffLimit=Ton×1aCBRmeasured-bCBRmeasured-1

This is the same as the existing requirements in the harmonized specification, with the choice of the parameters a = 1/4000 and b = 0.6. The parameters (a,b) were derived based on system level simulations to measure the sensitivity of CBR as a function of the number of stations, and thereby are related to 802.11p channel access parameters.

Observation 3: The generalized formulation can be easily specialized to obtain the DCC formulation for 802.11p as described in EN 302.571.

## 3.3	Discussion on CBR and DCC for LTE-V2V

With the generalized formulation in mind, this subsection provide a further discussion on certain assumptions and their applicability for LTE-V2V.

Issue 1: On the functional form (linear vs. non-linear) for CBR = f(Nsta)

One the key components of DCC is the functional mapping of CBR to the number of ITS stations in a given geographical area. This is of key importance since the idea of DCC is to distribute the total number of resources allowed to be used (say 60%) equally among all the UEs. The number of UEs is of course not measurable by itself, and is only obtained by inverting the function with the measured CBR as observed by the UE.

If the assumed function is incorrect, then the UE will either underestimate or overestimate the number of stations in the given geographical area. The congestion of such a system will thus not converge at the design operating point, but rather over or under that operating point depending if the number of stations are underestimate or overestimated, respectively.

To investigate the CBR vs Nsta relationship for 802.11p and LTE-V2V, we plot the measured CBR as a function of the number of stations in within a radius equal to the link budget for that system.

Figure 1: CBR vs number of ITS stations within link budget for LTE-V2V and 802.11p; urban scenario; 15km/hr

From Figure 1 it can be seen that the functional form for CBR vs. Nsta is quite different for LTE-V2V and 802.11p. For 802.11p, the linear assumption is validated using this simulation and is expected due to CSMA. To elaborate, for low density of UEs, it is expected that CBR will grow linearly with Nsta. This behaviour is maintained for 802.11p due to CSMA wherein the density of actual UEs that get to transmit is still maintained low. The non-linear behaviour for 802.11p still arises due to hidden nodes etc., but is not expected to dominate the shape of the function f(). For LTE-V2V, however, since there is no CSMA, as the density grows more rapidly with Nsta and starts to plateau as the resource reuse increases with the geographical area (with radius of link budget).

Depending on the desired system operating point with respect to the CBRlimit, the linear assumption for f() as made currently in the harmonized specification may not be appropriate for LTE-V2V. Even with a linear function assumption, the parameters (slope, intercept) of the function may be different for LTE-V2V compared to 802.11p.

We hence propose that the function f() can be configured in the UE (as its technology specific). This also allows for future flexibility with other technologies (NR) that may have different channel access methods leading to a different relationship between CBR and Nsta.

Observation 4: Linear mapping from Nsta to CBR as used in EN 302.571 is not sufficient for LTE-V2V. For LTE-V2V the mapping between CBR and Nsta should be (pre-) configurable.

Issue 2: On DCC on control vs. data resource

In the current version of harmonized standard, the CBR measurement is specified for the entire channel together. This works well for 802.11p where the radio resources are not split between control and data separately. For LTE-V2V, however, we have split of radio resources between control and data.

If a single CBR measurement and DCC loop is used for LTE-V2V using the set of all radio resources, then it may not work well in scenarios where (for example) only the control resources are congested, while the overall congestion of the channel (control + data) is still under the desired limit. In such an example, the system will still fail as control becomes the bottleneck.

We hence propose that the CBR measurement and DCC can be done independently for control and data resource pools, with separate CBR limits specified for control and data.

Observation 5: For LTE-V2V it is better to define separate CBR for PSCCH and PSSCH resource pools.

Issue 3: Requirement on limiting the channel resource utilization (CR) (Tofflimit vs. CRlimit)

From the discussion above, we noted that a generalized formulation of DCC is to limit the channel resource utilization (CR) at the UE as:

If CBRmeasured> CBRlimit, then

CRlimit=CBRmeasuredf-1CBRmeasuredor CBRlimitf-1CBRmeasured

For 802.11p, this translated to a limit on Toff period due to TDMA access, since for 802.11p

CR=TonTon+Toff for 802.11p

For LTE-V2V, however, a better DCC implementation is possible. For example, assume that the CRlimit for the system is 0.004, which is 20 RBs every 100ms. When the system is not congested, the UE can still transmit over 30RBs every 100ms (i.e. CR = 0.008). However, when the system is congested, i.e. CBRmeasured> CBRlimit, the UE has both time and frequency dimensions to limit the CR below CRlimit. For example, it can either increase MCS to occupy 20RBs every 100ms, or increasing Toff as in the current specification.

We hence propose that the generalized formulation of CRlimit should be applicable for LTE-V2V (and all technologies in general). For 802.11p, this is automatically calculated as the Tofflimit as currently specified.

Observation 6: For LTE-V2V a generalized formulation of CRlimit should be adopted to account the FDM nature of LTE-V2V and can be considered as limit on the fraction of time-frequency resources occupied.

Issue 4: CBR threshold of 0.62%

The existing limit of 62% to trigger DCC is based on system simulations using 802.11p channel access method. Clearly, it cannot be reasoned that the same limit will apply to LTE-V2V and other technologies in general. We hence propose the CBRlimit (currently set as 62% for 802.11p) should be a configurable parameter for the technology.

Issue 5: Interaction between PPPP and DCC

One specific aspect for LTE-V2V is the presence of PPPP. The question then is how congestion control should work when the system has packets of different priorities. A natural requirement is that low priority packets should not congest the system too much so that the higher-priority packets suffer (by either longer backoff or even performance impacts).

We hence propose that we have separate CBR limits per packet priority. Taking an example of two priority packets in the systems, the UE is then configured with CBRlimit_p0 and CBRlimit_p1, with p0 having higher priority than p1. Then the CBR limits can be set such that CBRlimit_p1 < CBRlimit_p0, so that low priority packets can congest the system only to a certain limit and still leave room for higher priority packets to enter the system. With increasing congestion, the low priority packets will then backoff first and reduce the effect on high priority packets.

Observation 7: For LTE-V2V the CBRlimit should be configurable and should be on a PPPP basis.

Issue 6: Measurement of Received Signal Strength

We now discuss the measurements that need to be defined in 3GPP RAN1 to meet the requirements on CBR measurements and DCC requirements in the harmonized ETSI ITS standard. We propose that for PSCCH received signal strength is defined as RSSI over 2 PRB pairs. For PSSCH it is the RSSI over N PRB pairs. Here N is the length of ‘channel’ as discussed in our companion contribution [5].

Proposal 1: For LTE-V2V define received signal strength as:

- RSSI over 2 PRB pairs for PSCCH

- RSSI over N PRB pairs for PSSCH. Here N is the length of channel configured for data resource pool.

## 3.4	Technology neutral proposal CBR measurement and DCC

Based on the discussions in the previous subsection, we hence formulate the following proposal for technology neutral CBR measurement and DCC.

We recognize that defining these are not necessarily within 3GPP V2V/V2X WI scope, but rather within the scope of EU harmonized standard / ETSI. Nonetheless, it is important from 3GPP view point to formulate the CBR measurement and DCC requirements as would be seen appropriate from LTE-V2V perspective. Furthermore, these formulations are indeed required to be technology neutral, and hence we have formulated them as such.

Importantly, we also note that the existing requirements are merely a subset of this tech-neutral proposal and remain unchanged for 802.11p. So it is merely expanding of the existing requirements in the harmonized specification to incorporate LTE-V2V and possibly other technologies, without affecting requirements for 802.11p.

Table 1: Technology neutral formulations for CBR measurements and DCC. Note existing requirements are merely a subset and hence remain unchanged for 802.11p.

| CBR measurement | channelBusyRatio(CBR)=(1 ∀ probes with radio resource busy)/Np - Np is the total number of radio resources probed with a measurement window - Resource is defined as the minimum time-frequency unit of allocation for the technology (e.g., 8us x channel BW for 802.11p, N PRB pairs for LTE-V2V) - Resource is deemed busy if received signal power on that resource is above a threshold Sth. - The threshold Sth is configured by the technology; if not, the default value is as currently specified in ETSI EN 302.571 (≤-85dBm) |
| --- | --- |
| Control + Data resource | If the set of radio resources are divided into subsets used for control and data transmissions, respectively, then CBR measurements and DCC can be performed separately on control and data set of resources. - Enabled by configuring different CBR limits |
| CBR limit | Configured in the UE for the technology. - Separate limits for control and data resource sets can be configured, if applicable. - Separate limits for different packet priorities can be configured, if applicable. If not configured, the default value is as currently specified in ETSI EN 302.571 (62%) |
| DCC | The channel radio resource utilization is limited as follows: If CBRmeasured> CBRlimit, then CRlimit=CBRmeasuredf-1CBRmeasured - f() is either configured in the UE; if not configured the default function is as currently specified in ETSI EN 302.571: 1/a * (CBRmeasured – b), with (a,b) = (1/4000, 0.6). - Note for 802.11p, CR = Ton / (Ton+Toff) |

Observation 8: Table 1 provides a generalized technology neutral framework for DCC and CBR that can be adopted by the harmonized standard. Existing requirements are obtained as a subset and remain unchanged for 802.11p.

# 4	Simulation Results

In this section, we provide some simulation results with and without DCC for LTE-V2V. We use the generalized formulation as developed in the previous section. A naïve form of DCC is implemented wherein the channel utilization (CR) is kept below CRlimit by simply dropping packets when required – i.e., periodicity of the packets is still maintained at 100ms, however, a packet is dropped if its transmission would cause CR to increase over CRlimit.

We simulated Urban 15 km/hr with a total of 2360 UEs are present, by introduced to the system over time. Every 1 sec 236 UEs are added to the system. Simulations are performed with and without DCC enabled. When DCC is enabled, CBRlimit is set at 0.6.

The result on the CBR vs time is shown in Figure 2. The average (over all ITS stations) CBR measured at the UE is plotted with time with and without DCC enabled. Without DCC, it can be seen that the CBR keeps increasing. With DCC, it can be seen that the CBR starts to floor a little above 0.6. This result captures the effect of DCC in limiting the system congestion to a CBR limit.

Figure 2: CBR vs Time for LTE-V2V system with and without DCC

Observation 9: Generalized formulation for DCC can control the CBR for LTE-V2V.

# 5	Conclusion

In this contribution we made the following proposal and observations.

Observation 1: The existing DCC requirements in ETSI harmonized specification EN 302.571 v2.0.0 are derived assuming 802.11p based channel access.

Observation 2: DCC can be described in a generalized form that applies to all technologies.

Observation 3: The generalized formulation can be easily specialized to obtain the DCC formulation for 802.11p as described in EN 302.571.

Observation 4: Linear mapping from Nsta to CBR as used in EN 302.571 is not sufficient for LTE-V2V. For LTE-V2V the mapping between CBR and Nsta should be (pre-) configurable.

Observation 5: For LTE-V2V it is better to define separate CBR for PSCCH and PSSCH resource pools.

Observation 6: For LTE-V2V a generalized formulation of CRlimit should be adopted to account the FDM nature of LTE-V2V and can be considered as limit on the fraction of time-frequency resources occupied.

Observation 7: For LTE-V2V the CBRlimit should be configurable and should be on a PPPP basis.

Proposal 1: For LTE-V2V define received signal strength as:

- RSSI over 2 PRB pairs for PSCCH

- RSSI over N PRB pairs for PSSCH. Here N is the length of channel configured for data resource pool.

Observation 8: Table 1 provides a generalized technology neutral framework for DCC and CBR that can be adopted by the harmonized standard. Existing requirements are obtained as a subset and remain unchanged for 802.11p.

Observation 9: Generalized formulation for DCC can control the CBR for LTE-V2V.

# References

- Chairman’s Notes, RAN1#85

- ETSI EN 302.571 v2.0.0

- ERMTG37(16)000037r2, “AP 20160623: Comments on Draft ETSI EN 302 571 V2.0.0 (2016-03)”.

- ETSI TR 101.612 v1.1.1, “ITS: Cross Layer DCC Management Entity for operation in the ITS G5A and ITS G5B medium; Report on Cross layer DCC algorithms and performance evaluation”, Sep 2014.

- R1-166260, “Details of resource pool design”, Qualcomm Incorporated