3GPP TSG RAN WG1 Meeting #86	R1-166608

Gothenburg, Sweden, 22nd - 26th August 2016

Source:	OPPO

Title:	Discussion on forward compatibility of subframe structure for NR

Agenda Item:	8.1.1

Document for:	Discussion and Decision

Introduction

According to the discussion in RAN1#85 on forward compatibility and numerology for NR, we have the following agreements:

Agreements:

- Forward compatibility of NR shall ensure smooth introduction of future services and features with no impact on the access of earlier services and UEs

  - Multiplexing different numerologies within a same NR carrier bandwidth (from the network perspective) is supported

    - FDM and/or TDM multiplexing can be considered

- For the numerology with 15 kHz and larger subcarrier spacing ,1 msec alignment is supported

Also, we have the following working assumption for NR numerology:

Working assumption:

- RAN1 concludes on alternative 1 (15 kHz) as the baseline design assumption for the NR numerology

- RAN1 concludes on scale factors N =2n for subcarrier spacing as the baseline design assumption for the NR numerology

- In the case of subcarrier spacing 15 kHz and 14 symbols per 1ms, the following applies:

  - Baseline: Symbol boundary is aligned with LTE of normal CP

From the conclusion, NR would support multiple numerologies and the numerologies can be multiplexed in the same carrier e.g. via FDM/TDM. If more numerologies are introduced in the future for new services and features, e.g. numerology with subcarrier spacing smaller than 15 kHz, the forward compatibility should also be ensured. In this contribution, we discuss the forward compatibility of subframe structure for NR to support multiplexing of different numerologies.

Discussion

In the past meeting, the frame structure was discussed and some high level conclusions were made. It is clear that different numerologies may lead to different durations of TTI or OFDM symbol, different number of TTI or symbols within a time interval, and also different CP lengths. To simplify the standardization effort and keep the forward compatibility, the same subframe structure for different numerologies is preferred. The design aspects of the subframe structure include:

- The length of frame.

- The time domain structure of TTI (e.g. time interval X) including the number of slots/OFDM symbols within a TTI.

- The indication/determination mechanism of timing relationship for both TDD and FDD, including scheduling timing, HARQ timing, retransmission timing etc.

- The HARQ mechanism including timing mechanism and resource allocation.

Other designs corresponding to signal transmission, e.g. RS position, CP length, can be respectively considered for different numerologies.

Proposal 1: The subframe structure should be designed independently from the numerologies.

Since active UEs may have different traffic types, data using multiple numerologies could be TDMed or FDMed in the same carrier. It would be very flexible if different numerologies could be multiplexed without any restriction as shown in Fig.1. Nevertheless, there may be also some issues:

- It would be very hard for receiver to determine the size of detection window if the transmission boundary does not align in different bands as shown in Fig.1.

- The timing would be mixed if different numerologies are arbitrarily TDMed. Since the same number of “time interval” does not mean the same time length, it would be very difficult for eNB to indicate the timing for scheduling/HARQ.

- There would be strong scheduling restriction and additional scheduling complexity to ensure 1 ms alignment.

- Inter-cell interference coordination would be difficult if there is no time alignment among cells.

Fig.1: The example of multiplexing of multiple numerologies

To solve these issues, a unified transmission duration can be introduced for different numerologies. That is, each data transmission using the same numerology should last the time of the transmission duration. The transmission duration can be determined according to all the supported numerologies. For example, if only 15 kHz and larger subcarrier spacing is supported, the transmission duration can be set to 1ms or several ms. With the same transmission duration, data using different numerologies may last one or multiple time intervals as shown in Fig.2. The transmission duration can be used as detection window in receiver, or used as scheduling window at eNB. For example, single grant can be used to schedule data within a window of transmission duration at some cases.

Fig.2: A unified transmission duration for different numerologies

Proposal 2: A unified transmission duration is defined for different numerologies.

Conclusions

In this contribution, we discuss the design of subframe structure to ensure forward compatibility. Since multiple numerologies are supported for NR and more numerologies may be introduced in future, it is important to design a subframe structure compatible to different numerologies. From the discussion, we propose that:

Proposal 1: The subframe structure should be designed independently from the numerologies.

Proposal 2: A unified transmission duration is defined for different numerologies.

References

- RP-160671, New SID Proposal: Study on New Radio Access Technology