3GPP TSG RAN WG1 Meeting #86	R1-167164

Gothenburg, Sweden, August 22-26, 2016

Agenda Item:	7.2.11.1.4

Source:	Huawei, HiSilicon

Title:	Capacity limit evaluation

Document for:	Discussion and decision

# Introduction

In last RAN#72 plenary meeting, WID for enhancement of NB-IoT was approved and the following was agreed on introducing positioning functionality [1]:

Support of UTDOA or OTDOA:

- Study accuracy, UE complexity, UE power consumption for both UTDOA and OTDOA using NB-IoT and provide recommendation to RAN#73 on which one solution to adopt [RAN1]

  - 3GPP network operators are invited to provide inputs to RAN1#86 on their positioning requirements. Companies are encouraged to include both methods in their evaluations.

- Based on the study make a choice (either uplink positioning or OTDOA) during RAN#73

We study on the above three aspects and show the evaluation in other contributions [2][3][4]. In this paper, we intend to provide additional analysis from comparison perspective on the capacity of positioning UE number over the NB-IoT air interface.

# Discussion

As discussed in [2], there are big differences in procedure flows between the downlink positioning and the uplink positioning. For OTDOA, UE needs to support LPP protocol to communicate with E-SMLC and performs physical layer RSTD measurement while UE is transparent for UTDOA where no additional protocol is added for the UE and it only needs to receive the configuration from eNB then transmits the configured reference signal.

To simplify the capacity study and do the comparisons for a recommendation to RAN#73, we only focus on the different key steps of OTDOA and UTDOA and omit the common parts. Particularlyy for OTDOA we assume UE is static and has already got the assistance data from E-SMLC thus other LPP messages are not required in every positioning. These steps are summarized in the Table 1.

Table 1 Summary of key procedure flows for positioning

| Positioning method | Procedure flows | UE status |
| --- | --- | --- |
| OTDOA | - UE is asked for positioning via LPP message OTDOA-RequestLocationInformation | Receive |
|  | - UE performs RSTD measurement for N neighbor cells | Receive |
|  | - UE report measurement results for N neighbor cells via LPP message OTDOA-ProvideLocationInformation | Transmit |
| UTDOA | - UE get reference signal configuration from eNB | Receive |
|  | - UE transmit reference signal | Transmit |

Both the first flow of the OTDOA and of UTDOA refer to UE receiving control messages from the network in above table thus we here omit the detail difference of the message size and assume it is not necessary to compare this equivalent common part from the comparison perspective.

Then taking the second flow of OTDOA, it means UE performs measurement based on downlink broadcast reference signals transmitted by base station [5]. The overhead is constant and will not be changed by increasing the number of UEs conducting positioning, which means the supporting number of measurement UE is not limited by this.

Thus for OTDOA, the key limit of capacity over the air interface is the third flow of reporting measurement results via NPUSCH, while for UTDOA transmitting the reference signal is the key limit.

For this comparison, we assume a UTDOA positioning reference signal is transmitted at the maximum repetition number (128) configurable for NPRACH [4], regardless of the actual MCL for any uplink positioning UE. This maximizes the coverage of UTDOA. We can regard this as a minimal capacity baseline where total 48 users within 180 kHz and 819.2 ms frequency-time resource for UTDOA shown in Table 2.

Table 2 Summary of equivalent capacity over the air interface for UTDOA

| Positioning method | Frequency bandwidth | Duration (repetition=128) | User number (single-tone 3.75kHz) |
| --- | --- | --- | --- |
| UTDOA | 180kHz | 819.2ms | 48 |

For OTDOA, the measurement results report can be regarded as per different level of MCL. We summarize the equivalent capacity within same 180 kHz by 819.2 ms frequency-time resource for different cases in Table 4 where the figures are deduced by NPUSCH demodulation performance as shown in Table 3.

Table 3 NPUSCH demodulation performance for single-tone 3.75 kHz

| MCL | TBS | MCS | RU | Repetition |
| --- | --- | --- | --- | --- |
| 144dB | 328 | 7 | 3 | 1 |
| 154dB | 328 | 3 | 6 | 1 |
| 164dB | 328 | 3 | 6 | 8 |

Table 4 Summary of equivalent capacity for OTDOA

| Positioning method | LPP message [6] payload size of reporting N neighbor cell measurement results (including header and octet padding) | Number of UEs (MCL=144dB) | Number of UEs (MCL=154dB) | Number of UEs (MCL=164dB) |
| --- | --- | --- | --- | --- |
| OTDOA | N=3, ~80 Bytes | 210 | 105 | 13.1 |
|  | N=6, ~120 Bytes | 140 | 70 | 8.7 |
|  | N=16, ~250 Bytes | 67.2 | 33.6 | 4.2 |
|  | N=24, ~350 Bytes | 48 | 24 | 3 |

It can be seen from the Table 4 and Table 2, OTDOA can have a higher UE capacity for normal coverage level (MCL=144dB) while UTDOA has higher capacity outperforms for extreme coverage level (MCL=164dB),.For extended coverage level (MCL=154 dB), OTDOA has a higher UE capacity when the number of reported measured cells is small while UTDOA has a higher UE capacity when the number of reported measured cells is large. The capacity of OTDOA normalized by UTDOA is summarized in Table 5. Note that these results are assuming the minimal capacity baseline for UTDOA.

# Conclusion

In this contribution, we provide additional analysis from comparison perspective on positioning capacity over the NB-IoT air interface. The observations can be summarized in the following table as follow.

Table 5 Summary of OTDOA vs. UTDOA capacity (normalized by UTDOA capacity)

| LPP message payload size of reporting N neighbor cell measurement results for OTDOA (including header and octet padding) | Number of UEs (MCL=144dB) | Number of UEs (MCL=154dB) | Number of UEs (MCL=164dB) |
| --- | --- | --- | --- |
| N=3, ~80 Bytes | 4.4 | 2.2 | 0.27 |
| N=6, ~120 Bytes | 2.9 | 1.5 | 0.18 |
| N=16, ~250 Bytes | 1.4 | 0.7 | 0.09 |
| N=24, ~350 Bytes | 1 | 0.5 | 0.06 |

Observation 1: For extreme coverage (MCL=164dB), UTDOA has a much higher user capacity.

Observation 2: For extended coverage (MCL=154 dB), OTDOA has a higher user capacity when the number of reported measured cells is smaller, and UTDOA has a higher user capacity when the number of reported measured cells is larger.

Observation 3: For normal coverage (MCL=144dB), OTDOA has a higher user capacity.

# References

RP-161324, “New work item proposal: Enhancements of NB-IoT”, RAN#72, Vodafone, Huawei, HiSilicon, Ericsson, Qualcomm, Busan, Korea, June 2016.

R1-166176, “UE complexity and power consumption analysis for NB-IoT positioning”. Huawei, HiSilicon, RAN1#86, Gothenburg, Sweden, August 2016.

R1-166172, “Evaluation of NB-IoT downlink positioning”. Huawei, HiSilicon, RAN1#86, Gothenburg, Sweden, August 2016.

R1-166174, “Evaluation of NB-IoT uplink positioning”. Huawei, HiSilicon, RAN1#86, Gothenburg, Sweden, August 2016.

R1-167163, “Design of NB-IoT downlink positioning”. Huawei, HiSilicon, RAN1#86, Gothenburg, Sweden, August 2016.

3GPP TS36.355, “LTE Positioning Protocol (LPP)”